3GPP TSG-RAN5 Meeting #91-e	R5-213152r2

# Electronic Meeting, 17th May 2021 – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1919 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to UE Policy Delivery messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The default messages for UE Policy Delivery have been defined as V2X specific messages. As they can also be used for Policy Delivery of URSP and ANDSF policies, they should be moved to a separate section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The default messages for UE Policy Delivery have been moved to a new clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE Policy Delivery messages can only be used for V2X policy delivery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.4, 4.7.5 and 4.7.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Incorporated updates from CRs in TDocs R5-212484, R5-212485 and R5-212486. R2: Added some missing fields. |  |  |  |  |  |  |  |  |

### 4.7.4	Contents of V2X messages

#### –	DIRECT LINK ESTABLISHMENT REQUEST

Table 4.7.4-7: DIRECT LINK ESTABLISHMENT REQUEST

| Derivation Path: TS 24.587 Table 7.3.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK ESTABLISHMENT REQUEST message identity |  | FFS |  |  |
| Sequence number |  | FFS |  |  |
| V2X service identifiers |  | FFS |  |  |
| Source user info |  | FFS |  |  |
| UE security capabilities |  | FFS |  |  |
| UE PC5 unicast signalling security policy |  | FFS |  |  |
| Key establishment information container |  | FFS |  |  |
| Nonce_1 |  | FFS |  |  |
| MSBs of KNRP-sess ID |  | FFS |  |  |
| Target user info |  | FFS |  |  |
| KNRP ID |  | FFS |  |  |

#### –	DIRECT LINK ESTABLISHMENT ACCEPT

Table 4.7.4-8: DIRECT LINK ESTABLISHMENT ACCEPT

| Derivation Path: TS 24.587 Table 7.3.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK ESTABLISHMENT ACCEPT message identity |  | FFS |  |  |
| Sequence number |  | FFS |  |  |
| Source user info |  | FFS |  |  |
| QoS flow descriptions |  | FFS |  |  |
| Configuration of UE PC5 unicast user plane security protection |  | FFS |  |  |
| IP address configuration |  | FFS |  |  |
| Link local IPv6 address |  | FFS |  |  |

#### –	DIRECT LINK MODIFICATION REQUEST

Table 4.7.4-9: DIRECT LINK MODIFICATION REQUEST

| Derivation Path: TS 24.587 Table 7.3.4.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK MODIFICATION ACCEPT

Table 4.7.4-10: DIRECT LINK MODIFICATION ACCEPT

| Derivation Path: TS 24.587 Table 7.3.5.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK RELEASE REQUEST

Table 4.7.4-11: DIRECT LINK RELEASE REQUEST

| Derivation Path: TS 24.587 Table 7.3.6.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK RELEASE ACCEPT

Table 4.7.4-12: DIRECT LINK RELEASE ACCEPT

| Derivation Path: TS 24.587 Table 7.3.7.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK KEEPALIVE REQUEST

Table 4.7.4-13: DIRECT LINK KEEPALIVE REQUEST

| Derivation Path: TS 24.587 Table 7.3.8.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK KEEPALIVE RESPONSE

Table 4.7.4-14: DIRECT LINK KEEPALIVE RESPONSE

| Derivation Path: TS 24.587 Table 7.3.9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK AUTHENTICATION REQUEST

Table 4.7.4-15: DIRECT LINK AUTHENTICATION REQUEST

| Derivation Path: TS 24.587 Table 7.3.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK AUTHENTICATION RESPONSE

Table 4.7.4-16: DIRECT LINK AUTHENTICATION RESPONSE

| Derivation Path: TS 24.587 Table 7.3.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK AUTHENTICATION REJECT

Table 4.7.4-17: DIRECT LINK AUTHENTICATION REJECT

| Derivation Path: TS 24.587 Table 7.3.12.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK SECURITY MODE COMMAND

Table 4.7.4-18: DIRECT LINK SECURITY MODE COMMAND

| Derivation Path: TS 24.587 Table 7.3.13.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK SECURITY MODE COMPLETE

Table 4.7.4-19: DIRECT LINK SECURITY MODE COMPLETE

| Derivation Path: TS 24.587 Table 7.3.14.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK SECURITY MODE REJECT

Table 4.7.4-20: DIRECT LINK SECURITY MODE REJECT

| Derivation Path: TS 24.587 Table 7.3.15.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK REKEYING REQUEST

Table 4.7.4-21: DIRECT LINK REKEYING REQUEST

| Derivation Path: TS 24.587 Table 7.3.16.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK REKEYING RESPONSE

Table 4.7.4-22: DIRECT LINK REKEYING RESPONSE

| Derivation Path: TS 24.587 Table 7.3.17.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK IDENTIFIER UPDATE REQUEST

Table 4.7.4-23: DIRECT LINK IDENTIFIER UPDATE REQUEST

| Derivation Path: TS 24.587 Table 7.3.18.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK IDENTIFIER UPDATE ACCEPT

Table 4.7.4-24: DIRECT LINK IDENTIFIER UPDATE ACCEPT

| Derivation Path: TS 24.587 Table 7.3.19.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK IDENTIFIER UPDATE ACK

Table 4.7.4-25: DIRECT LINK IDENTIFIER UPDATE ACK

| Derivation Path: TS 24.587 Table 7.3.20.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK IDENTIFIER UPDATE REJECT

Table 4.7.4-26: DIRECT LINK IDENTIFIER UPDATE REJECT

| Derivation Path: TS 24.587 Table 7.3.21.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK MODIFICATION REJECT

Table 4.7.4-27: DIRECT LINK MODIFICATION REJECT

| Derivation Path: TS 24.587 Table 7.3.22.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

#### –	DIRECT LINK ESTABLISHMENT REJECT

Table 4.7.4-28: DIRECT LINK ESTABLISHMENT REJECT

| Derivation Path: TS 24.587 Table 7.3.23.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FFS |  |  |  |  |

### 4.7.5	V2X information elements

#### 4.7.5.1	Void

#### 4.7.5.2	Void

#### 4.7.5.3	Void

#### 4.7.5.4	V2X information elements for UE policy part

##### –	UE policy part when UE policy part type = {V2XP}

Table 4.7.5.4-1: UE policy part when UE policy part type = {V2XP}

| Derivation Path: TS 24.588 Figure 5.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| Spare |  | ‘0000’B |  |  |
| UE policy part type |  | ‘0001’B | UE policy part type={ V2XP } |  |
| UE policy part contents={V2XP contents} |  | See Table 4.7.5.4-2 |  |  |

##### –	V2XP contents

Table 4.7.5.4-2: V2XP contents

| Derivation Path: TS 24.588 Figure 5.2.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| V2XP info #1 |  | See Table 4.7.5.4-3 |  |  |
| V2XP info #2 |  | FFS |  |  |

##### –	V2XP info

Table 4.7.5.4-3: V2XP info

| Derivation Path: TS 24.588 Figure 5.3.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Spare |  | ‘0000’B |  |  |
| V2XP info type |  | ‘0001’B | UE policies for V2X communication over PC5 |  |
| Length of V2XP info contents |  | Set to the actual length of 'V2XP info contents' in bytes |  |  |
| V2XP info contents |  | See Table 4.7.5.5-1 |  |  |

#### 4.7.5.5	V2X information elements of UE policies for V2X communication over PC5

##### –	V2XP info = {UE policies for V2X communication over PC5}

Table 4.7.5.5-1: V2XP info = {UE policies for V2X communication over PC5}

| Derivation Path: TS 24.588 Figure 5.3.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Spare |  | ‘0000’B |  |  |
| V2XP info type |  | ‘0001’B | UE policies for V2X communication over PC5 |  |
| Length of V2XP info contents |  | Set to the actual length of 'V2XP info contents' in bytes |  |  |
| Validity timer |  | FFS |  |  |
| VSITPMRI |  | ‘1’B | ‘V2X service identifier to PC5 RAT and Tx profiles mapping rules’ is present |  |
| Spare |  | ‘000 0000’B |  |  |
| Served by E-UTRA or served by NR |  | See Table 4.7.5.5-2 |  |  |
| Not served by E-UTRA and not served by NR |  | See Table 4.7.5.5-6 |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rules |  | See Table 4.7.5.5-12 |  |  |
| Privacy config |  | See Table 4.7.5.5-15 |  |  |
| V2X communication over PC5 in E-UTRA-PC5 |  | See Table 4.7.5.5-19 |  |  |
| V2X communication over PC5 in NR-PC5 |  | See Table 4.7.5.5-31 |  |  |

##### –	Served by E-UTRA or served by NR

Table 4.7.5.5-2: Served by E-UTRA or served by NR

| Derivation Path: TS 24.588 Figure 5.3.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of served by E-UTRA or served by NR contents |  | Set to the actual length of 'served by E-UTRA or served by NR contents' in bytes |  |  |
| Authorized PLMN and RATs combinations |  | See Table 4.7.5.5-3 |  |  |

##### –	Authorized PLMN and RATs combinations

Table 4.7.5.5-3: Authorized PLMN and RATs combinations

| Derivation Path: TS 24.588 Figure 5.3.1.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of authorized PLMN and RATs combinations contents |  | Set to the actual length of 'authorized PLMN and RATs combinations contents' in bytes |  |  |
| Authorized PLMN and RATs combination 1 |  | See Table 4.7.5.5-4 |  |  |
| Authorized PLMN and RATs combination 2 |  | FFS |  |  |

##### –	Authorized PLMN and RATs combination

Table 4.7.5.5-4: Authorized PLMN and RATs combination

| Derivation Path: TS 24.588 Figure 5.3.1.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PLMN ID |  | Set according to parameter given in test case |  |  |
| EPIEN |  | FFS |  |  |
| NPIEN |  | FFS |  |  |
| Spare |  | ’00 0000’B |  |  |

##### –	PLMN ID

Table 4.7.5.5-5: PLMN ID

| Derivation Path: TS 24.588 Figure 5.3.1.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MCC digit 2 |  | Set according to parameter given in test case |  |  |
| MCC digit 1 |  | Set according to parameter given in test case |  |  |
| MNC digit 3 |  | Set according to parameter given in test case |  |  |
| MCC digit 3 |  | Set according to parameter given in test case |  |  |
| MNC digit 2 |  | Set according to parameter given in test case |  |  |
| MNC digit 1 |  | Set according to parameter given in test case |  |  |

##### –	Not served by E-UTRA and not served by NR

Table 4.7.5.5-6: Not served by E-UTRA and not served by NR

| Derivation Path: TS 24.588 Figure 5.3.1.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of not served by E-UTRA and not served by NR contents |  | Set to the actual length of 'not served by E-UTRA and not served by NR contents' in bytes |  |  |
| EPIEN |  | FFS |  |  |
| NPIEN |  | FFS |  |  |
| Spare |  | ’00 000’B |  |  |
| VPNENNI |  | FFS |  |  |
| E-UTRA radio parameters per geographical area list |  | See Table 4.7.5.5-7 |  |  |
| NR radio parameters per geographical area list |  | See Table 4.7.5.5-7 |  |  |

##### –	Radio parameters per geographical area list

Table 4.7.5.5-7: Radio parameters per geographical area list

| Derivation Path: TS 24.588 Figure 5.3.1.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area list contents |  | Set to the actual length of 'radio parameters per geographical area list contents' in bytes |  |  |
| Radio parameters per geographical area info 1 |  | See Table 4.7.5.5-8 |  |  |
| Radio parameters per geographical area info 2 |  | FFS |  |  |

##### –	Radio parameters per geographical area info

Table 4.7.5.5-8: Radio parameters per geographical area info

| Derivation TS 24.588 Figure 5.3.1.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area contents |  | Set to the actual length of 'radio parameters per geographical area contents' in bytes |  |  |
| Geographical area |  | See Table 4.7.5.5-9 |  |  |
| Radio parameters |  | See Table 4.7.5.5-11 |  |  |
| MI |  | FFS |  |  |
| Spare |  | ‘000 0000’B |  |  |

##### –	Geographical area

Table 4.7.5.5-9: Geographical area

| Derivation Path: TS 24.588 Figure 5.3.1.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of geographical area contents |  | Set to the actual length of 'geographical area contents' in bytes |  |  |
| Coordinate 1 |  | See Table 4.7.5.5-10 |  |  |
| Coordinate 2 |  | FFS |  |  |

##### –	Coordinate area

Table 4.7.5.5-10: Coordinate area

| Derivation Path: TS 24.588 Figure 5.3.1.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Latitude |  | FFS |  |  |
| Longitude |  | FFS |  |  |

##### –	Radio parameters

Table 4.7.5.5-11: Radio parameters

| Derivation Path: TS 24.588 Figure 5.3.1.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters contents |  | Set to the actual length of 'radio parameters contents' in bytes |  |  |
| Radio parameters contents |  | See Table 4.10.1.1-1: SL-V2X-Preconfiguration in TS 36.508 [2] |  | E-UTRAN V2X |
|  |  | See Table 4.10.1-1: SL-PreconfigurationNR |  | NR V2X |

##### –	V2X service identifier to PC5 RAT and Tx profiles mapping rules

Table 4.7.5.5-12: V2X service identifier to PC5 RAT and Tx profiles mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 RAT and Tx profiles mapping rules contents |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rule 1 |  | See Table 4.7.5.5-13 |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rule 2 |  | FFS |  |  |

##### –	V2X service identifier to PC5 RAT and Tx profiles mapping rule

Table 4.7.5.5-13: V2X service identifier to PC5 RAT and Tx profiles mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 RAT and Tx profiles mapping rule contents |  | Set to the actual length of 'V2X service identifier to PC5 RAT and Tx profiles mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| Spare |  | ‘0000 00’B |  |  |
| PC5 RAT |  | ‘00’B | E-UTRA-PC5 |  |
|  |  | ‘01’B | NR-PC5 |  |
| Length of E-UTRA-PC5 Tx profiles |  | Set to the actual length of 'E-UTRA-PC5 Tx profiles' in bytes |  |  |
| E-UTRA-PC5 Tx profiles |  | FFS | Refer to IE v2x-TxProfileList in TS 36.331 |  |
| Length of NR-PC5 Tx profiles |  | Set to the actual length of 'NR-PC5 Tx profiles' in bytes |  |  |
| NR-PC5 Tx profiles |  | FFS |  |  |

##### –	V2X service identifiers

Table 4.7.5.5-14: V2X service identifiers

| Derivation Path: TS 24.588 Figure 5.3.1.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifiers contents |  | Set to the actual length of 'V2X service identifiers contents' in bytes |  |  |
| V2X service identifier 1 |  | FFS |  |  |
| V2X service identifier 2 |  | FFS |  |  |

##### –	Privacy config

Table 4.7.5.5-15: Privacy config

| Derivation Path: TS 24.588 Figure 5.3.1.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of privacy config contents |  | Set to the actual length of 'privacy config contents' in bytes |  |  |
| V2X services requiring privacy |  | See Table 4.7.5.5-16 |  |  |
| Privacy timer |  | FFS |  |  |

##### –	V2X services requiring privacy

Table 4.7.5.5-16: V2X services requiring privacy

| Derivation Path: TS 24.588 Figure 5.3.1.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X services requiring privacy contents |  | Set to the actual length of 'V2X services requiring privacy contents' in bytes |  |  |
| V2X service requiring privacy 1 |  | See Table 4.7.5.5-17 |  |  |
| V2X service requiring privacy 2 |  | FFS |  |  |

##### –	V2X service requiring privacy

Table 4.7.5.5-17: V2X service requiring privacy

| Derivation Path: TS 24.588 Figure 5.3.1.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service requiring privacy contents |  | Set to the actual length of 'V2X service requiring privacy contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Geographical areas |  | See Table 4.7.5.5-18 |  |  |

##### –	Geographical areas

Table 4.7.5.5-18: Geographical areas

| Derivation Path: TS 24.588 Figure 5.3.1.18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of geographical areas contents |  | Set to the actual length of 'geographical areas contents' in bytes |  |  |
| Geographical area 1 |  | FFS |  |  |
| Geographical area 2 |  | FFS |  |  |

##### –	V2X communication over PC5 in E-UTRA-PC5

Table 4.7.5.5-19: V2X communication over PC5 in E-UTRA-PC5

| Derivation Path: TS 24.588 Figure 5.3.1.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X communication over PC5 in E-UTRA-PC5 contents |  | Set to the actual length of 'V2X communication over PC5 in E-UTRA-PC5 contents' in bytes |  |  |
| DDL2II |  | FFS |  |  |
| VSIEFMRI |  | FFS |  |  |
| VSAPI |  | FFS |  |  |
| PPMR |  | FFS |  |  |
| Spare |  | ‘0000’B |  |  |
| V2X service identifier to destination layer-2 ID mapping rules |  | See Table 4.7.5.5-20 |  |  |
| PPPP to PDB mapping rules |  | See Table 4.7.5.5-22 |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rules |  | See Table 4.7.5.5-24 |  |  |
| V2X services authorized for PPPR |  | See Table 4.7.5.5-29 |  |  |
| Default destination layer-2 ID |  | FFS |  |  |

##### –	V2X service identifier to destination layer-2 ID mapping rules

Table 4.7.5.5-20: V2X service identifier to destination layer-2 ID mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID mapping rule 1 |  | See Table 4.7.5.5-21 |  |  |
| V2X service identifier to destination layer-2 ID mapping rule 2 |  | FFS |  |  |

##### –	V2X service identifier to destination layer-2 ID mapping rule

Table 4.7.5.5-21: V2X service identifier to destination layer-2 ID mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Destination layer-2 ID |  | FFS |  |  |

##### –	PPPP to PDB mapping rules

Table 4.7.5.5-22: PPPP to PDB mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of PPPP to PDB mapping rules contents |  | Set to the actual length of 'PPPP to PDB mapping rules contents' in bytes |  |  |
| PPPP to PDB mapping rule 1 |  | See Table 4.7.5.5-23 |  |  |
| PPPP to PDB mapping rule 2 |  | FFS |  |  |

##### –	PPPP to PDB mapping rule

Table 4.7.5.5-23: PPPP to PDB mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Spare |  | ‘0000 0’B |  |  |
| PPPP |  | FFS |  |  |
| PDB |  | FFS |  |  |

##### –	V2X service identifier to V2X E-UTRA frequency mapping rules

Table 4.7.5.5-24: V2X service identifier to V2X E-UTRA frequency mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.24 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rules contents |  | Set to the actual length of 'V2X service identifier to V2X E-UTRA frequency mapping rules contents' in bytes |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 1 |  | See Table 4.7.5.5-25 |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 2 |  | FFS |  |  |

##### –	V2X service identifier to V2X E-UTRA frequency mapping rule

Table 4.7.5.5-25: V2X service identifier to V2X E-UTRA frequency mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rule contents |  | Set to the actual length of 'V2X service identifier to V2X E-UTRA frequency mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| V2X E-UTRA frequencies with geographical areas list |  | See Table 4.7.5.5-26 |  |  |

##### –	V2X E-UTRA frequencies with geographical areas list

Table 4.7.5.5-26: V2X E-UTRA frequencies with geographical areas list

| Derivation Path: TS 24.588 Figure 5.3.1.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies with geographical areas list contents |  | Set to the actual length of 'V2X E-UTRA frequencies with geographical areas list contents' in bytes |  |  |
| V2X E-UTRA frequencies with geographical areas info 1 |  | See Table 4.7.5.5-27 |  |  |
| V2X E-UTRA frequencies with geographical areas info 2 |  | FFS |  |  |

##### –	V2X E-UTRA frequencies with geographical areas info

Table 4.7.5.5-27: V2X E-UTRA frequencies with geographical areas info

| Derivation Path: TS 24.588 Figure 5.3.1.27 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies with geographical areas info contents |  | Set to the actual length of 'V2X E-UTRA frequencies with geographical areas info contents' in bytes |  |  |
| V2X E-UTRA frequencies |  | See Table 4.7.5.5-28 |  |  |
| Geographical areas |  | FFS |  |  |

##### –	V2X E-UTRA frequencies

Table 4.7.5.5-28: V2X E-UTRA frequencies

| Derivation Path: TS 24.588 Figure 5.3.1.28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies contents |  | Set to the actual length of 'V2X E-UTRA frequencies contents' in bytes |  |  |
| V2X E-UTRA frequency 1 |  | FFS |  |  |
| V2X E-UTRA frequency 2 |  | FFS |  |  |

##### –	V2X services authorized for PPPR

Table 4.7.5.5-29: V2X services authorized for PPPR

| Derivation Path: TS 24.588 Figure 5.3.1.29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X services authorized for PPPR contents |  | Set to the actual length of 'V2X services authorized for PPPR contents' in bytes |  |  |
| V2X service authorized for PPPR 1 |  | See Table 4.7.5.5-30 |  |  |
| V2X service authorized for PPPR 2 |  | FFS |  |  |

##### –	V2X service authorized for PPPR

Table 4.7.5.5-30: V2X service authorized for PPPR

| Derivation Path: TS 24.588 Figure 5.3.1.30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service authorized for PPPR contents |  | Set to the actual length of 'V2X service authorized for PPPR contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Spare |  | ‘0000 0’B |  |  |
| PPPR |  | FFS |  |  |

##### –	V2X communication over PC5 in NR-PC5

Table 4.7.5.5-31: V2X communication over PC5 in NR-PC5

| Derivation Path: TS 24.588 Figure 5.3.1.31 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X communication over PC5 in NR-PC5 contents |  | Set to the actual length of 'V2X communication over PC5 in NR-PC5 contents' in bytes |  |  |
| DDL2IBI |  | FFS |  |  |
| VSINFMRI |  | FFS |  |  |
| Spare |  | ’00 0000’B |  |  |
| V2X service identifier to V2X NR frequency mapping rules |  | See Table 4.7.5.5-32 |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rules |  | See Table 4.7.5.5-37 |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rules |  | See Table 4.7.5.5-39 |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules |  | See Table 4.7.5.5-41 |  |  |
| V2X service identifier to PC5 QoS parameters mapping rules |  | See Table 4.7.5.5-43 |  |  |
| AS configuration |  | See Table 4.7.5.5-45 |  |  |
| Default destination layer-2 ID for broadcast |  | FFS |  |  |
| NR-PC5 unicast security policies |  | See Table 4.7.5.5-49 |  |  |
| V2X service identifier to default mode of communication mapping rules |  | See Table 4.7.5.5-52 |  |  |

##### –	V2X service identifier to V2X NR frequency mapping rules

Table 4.7.5.5-32: V2X service identifier to V2X NR frequency mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.32 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X NR frequency mapping rules contents |  | Set to the actual length of 'V2X service identifier to V2X NR frequency mapping rules contents' in bytes |  |  |
| V2X service identifier to V2X NR frequency mapping rule 1 |  | See Table 4.7.5.5-33 |  |  |
| V2X service identifier to V2X NR frequency mapping rule 2 |  | FFS |  |  |

##### –	V2X service identifier to V2X NR frequency mapping rule

Table 4.7.5.5-33: V2X service identifier to V2X NR frequency mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X NR frequency mapping rule contents |  | Set to the actual length of 'V2X service identifier to V2X NR frequency mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| V2X NR frequencies with geographical areas list |  | See Table 4.7.5.5-34 |  |  |

##### –	V2X NR frequencies with geographical areas list

Table 4.7.5.5-34: V2X NR frequencies with geographical areas list

| Derivation Path: TS 24.588 Figure 5.3.1.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies with geographical areas list contents |  | Set to the actual length of 'V2X NR frequencies with geographical areas list contents' in bytes |  |  |
| V2X NR frequencies with geographical areas info 1 |  | See Table 4.7.5.5-35 |  |  |
| V2X NR frequencies with geographical areas info 2 |  | FFS |  |  |

##### –	V2X NR frequencies with geographical areas info

Table 4.7.5.5-35: V2X NR frequencies with geographical areas info

| Derivation Path: TS 24.588 Figure 5.3.1.35 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies with geographical areas info contents |  | Set to the actual length of 'V2X NR frequencies with geographical areas info contents' in bytes |  |  |
| V2X NR frequencies |  | See Table 4.7.5.5-36 |  |  |
| Geographical areas |  | FFS |  |  |

##### –	V2X NR frequencies

Table 4.7.5.5-36: V2X NR frequencies

| Derivation Path: TS 24.588 Figure 5.3.1.36 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies contents |  | Set to the actual length of 'V2X NR frequencies contents' in bytes |  |  |
| V2X NR frequency 1 |  | FFS |  |  |
| V2X NR frequency 2 |  | FFS |  |  |

##### –	V2X service identifier to destination layer-2 ID for broadcast mapping rules

Table 4.7.5.5-37: V2X service identifier to destination layer-2 ID for broadcast mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.37 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for broadcast mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 1 |  | See Table 4.7.5.5-38 |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 2 |  | FFS |  |  |

##### –	V2X service identifier to destination layer-2 ID for broadcast mapping rule

Table 4.7.5.5-38: V2X service identifier to destination layer-2 ID for broadcast mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.38 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for broadcast mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Destination layer-2 ID for broadcast |  | FFS |  |  |

##### –	V2X service identifier to destination layer-2 ID for groupcast mapping rules

Table 4.7.5.5-39: V2X service identifier to destination layer-2 ID for groupcast mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.39 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for groupcast mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 1 |  | See Table 4.7.5.5-40 |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 2 |  | FFS |  |  |

##### –	V2X service identifier to destination layer-2 ID for groupcast mapping rule

Table 4.7.5.5-40: V2X service identifier to destination layer-2 ID for groupcast mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.40 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for groupcast mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Destination layer-2 ID for groupcast |  | FFS |  |  |

##### –	V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

Table 4.7.5.5-41: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.41 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 1 |  | See Table 4.7.5.5-42 |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 2 |  | FFS |  |  |

##### –	V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

Table 4.7.5.5-42: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.42 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Destination layer-2 ID for unicast initial signalling |  | FFS |  |  |

##### –	V2X service identifier to PC5 QoS parameters mapping rules

Table 4.7.5.5-43: V2X service identifier to PC5 QoS parameters mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.43 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 QoS parameters mapping rules contents |  | Set to the actual length of 'V2X service identifier to PC5 QoS parameters mapping rules contents' in bytes |  |  |
| V2X service identifier to PC5 QoS parameters mapping rule 1 |  | See Table 4.7.5.5-44 |  |  |
| V2X service identifier to PC5 QoS parameters mapping rule 2 |  | FFS |  |  |

##### –	V2X service identifier to PC5 QoS parameters mapping rule

Table 4.7.5.5-44: V2X service identifier to PC5 QoS parameters mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.46 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 QoS parameters mapping rule contents |  | Set to the actual length of 'V2X service identifier to PC5 QoS parameters mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| GFBRI |  | FFS |  |  |
| MFBRI |  | FFS |  |  |
| PLAMBRI |  | FFS |  |  |
| RI |  | FFS |  |  |
| Spare |  | ‘0000’B |  |  |
| PQI |  | FFS |  |  |
| Guaranteed flow bit rate |  | FFS |  |  |
| Maximum flow bit rate |  | FFS |  |  |
| Per-link aggregate maximum bit rate |  | FFS |  |  |
| Range |  | FFS |  |  |

##### –	AS configuration

Table 4.7.5.5-45: AS configuration

| Derivation Path: TS 24.588 Figure 5.3.1.46a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of AS configuration contents |  | Set to the actual length of 'AS configuration contents' in bytes |  |  |
| SLRB mapping rules |  | See Table 4.7.5.5-46 |  |  |

##### –	SLRB mapping rules

Table 4.7.5.5-46: SLRB mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.47 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of SLRB mapping rules contents |  | Set to the actual length of 'SLRB mapping rules contents' in bytes |  |  |
| SLRB mapping rule 1 |  | See Table 4.7.5.5-47 |  |  |
| SLRB mapping rule 2 |  | FFS |  |  |

##### –	SLRB mapping rule

Table 4.7.5.5-47: SLRB mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.48 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of SLRB mapping rule contents |  | Set to the actual length of 'SLRB mapping rule contents' in bytes |  |  |
| PC5 QoS profile |  | See Table 4.7.5.5-48 |  |  |
| Length of SLRB |  | Set to the actual length of 'SLRB' in bytes |  |  |
| SLRB |  | FFS |  |  |

##### –	PC5 QoS profile

Table 4.7.5.5-48: PC5 QoS profile

| Derivation Path: TS 24.588 Figure 5.3.1.49 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of PC5 QoS profile contents |  | Set to the actual length of 'PC5 QoS profile contents' in bytes |  |  |
| GFBRI |  | FFS |  |  |
| MFBRI |  | FFS |  |  |
| PLAMBRI |  | FFS |  |  |
| RI |  | FFS |  |  |
| PLOI |  | FFS |  |  |
| AWI |  | FFS |  |  |
| MDBVI |  | FFS |  |  |
| Spare |  | ‘0’B |  |  |
| PQI |  | FFS |  |  |
| Guaranteed flow bit rate |  | FFS |  |  |
| Maximum flow bit rate |  | FFS |  |  |
| Per-link aggregate maximum bit rate |  | FFS |  |  |
| Range |  | FFS |  |  |
| Spare |  | ‘0000 0’B |  |  |
| Priority level |  | FFS |  |  |
| Averaging window |  | FFS |  |  |
| Maximum data burst volume |  | FFS |  |  |

##### –	NR-PC5 unicast security policies

Table 4.7.5.5-49: NR-PC5 unicast security policies

| Derivation Path: TS 24.588 Figure 5.3.1.50 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policies contents |  | Set to the actual length of 'NR-PC5 unicast security policies contents' in bytes |  |  |
| NR-PC5 unicast security policy 1 |  | See Table 4.7.5.5-50 |  |  |
| NR-PC5 unicast security policy 2 |  | FFS |  |  |

##### –	NR-PC5 unicast security policy

Table 4.7.5.5-50: NR-PC5 unicast security policy

| Derivation Path: TS 24.588 Figure 5.3.1.51 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policy contents |  | Set to the actual length of 'NR-PC5 unicast security policy contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Security policy |  | See Table 4.7.5.5-51 |  |  |
| Geographical areas |  | FFS |  |  |

##### –	Security policy

Table 4.7.5.5-51: Security policy

| Derivation Path: TS 24.588 Figure 5.3.1.52 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Spare |  | ‘0’B |  |  |
| Signalling ciphering policy |  | FFS |  |  |
| Spare |  | ‘0’B |  |  |
| Signalling integrity protection policy |  | FFS |  |  |
| Spare |  | ‘0’B |  |  |
| User plane ciphering policy |  | FFS |  |  |
| Spare |  | ‘0’B |  |  |
| User plane integrity protection policy |  | FFS |  |  |

##### –	V2X service identifier to default mode of communication mapping rules

Table 4.7.5.5-52: V2X service identifier to default mode of communication mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.53 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to default mode of communication mapping rules contents |  | ‘Set to the actual length of 'V2X service identifier to default mode of communication mapping rules contents' in bytes |  |  |
| V2X service identifier to default mode of communication mapping rule 1 |  | See Table 4.7.5.5-53 |  |  |
| V2X service identifier to default mode of communication mapping rule 2 |  | FFS |  |  |

##### –	V2X service identifier to default mode of communication mapping rule

Table 4.7.5.5-53: V2X service identifier to default mode of communication mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.54 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to default mode of communication mapping rule contents |  | ‘Set to the actual length of 'V2X service identifier to default mode of communication mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Spare |  | ‘0000 00’B |  |  |
| DMC |  | FFS |  |  |

### 4.7.6	Contents of UE Policy Delivery messages

#### –	MANAGE UE POLICY COMMAND

Table 4.7.6-1: MANAGE UE POLICY COMMAND

| Derivation Path: TS 24.501 Table D.5.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| MANAGE UE POLICY COMMAND message identity |  | ’0000 0001’B | MANAGE UE POLICY COMMAND message |  |
| UE policy section management list |  |  |  |  |  |  |  |  |  |  |  |
| Length of UE policy section management list contents |  | Set to the actual length of 'UE policy section management list contents' in bytes |  |  |
| UE policy section management list contents |  | 1 entry |  |  |
| UE policy section management sublist (PLMN-1) |  |  |  |  |  |  |  |  |  |  |  |
| Length of UE policy section management sublist |  | Set to the actual length of 'UE policy section management sublist' in bytes |  |  |
| PLMN ID |  |  | Set to the PLMN value used in the test case |  |  |  |  |  |  |  |  |
| UE policy section management sublist contents |  |  |  |  |
| Instruction 1 |  |  |  |  |  |  |  |  |  |  |  |
| Instruction contents length |  | Set to the actual length of 'Instruction contents' in bytes |  |  |
| UPSC |  | FFS |  |  |
| UE policy section contents |  |  |  |  |
| UE policy part 1 |  |  |  |  |  |  |  |  |  |  |  |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| Spare |  |  | ‘0000’B |  |  |  |  |  |  |  |  |
| UE policy part type |  |  | ‘0001’B |  |  |  |  |  | URSP |  |  |
|  |  |  | ‘0010’B |  |  |  |  |  | ANDSP |  |  |
|  |  |  | ‘0011’B |  |  |  |  |  | V2XP |  |  |
| UE policy part contents |  |  | FFS |  |  |  |  |  | URSP |  |  |
|  |  |  | FFS |  |  |  |  |  | ANDSP |  |  |
|  |  |  | See Table 4.7.5.4-1 |  |  |  |  |  | V2XP |  |  |

| Condition | Explanation |
| --- | --- |
| URSP | UE Route Selection Policy (URSP) |
| ANDSP | UE Access Network Discovery and Selection Policy (ANDSP) |
| V2XP | UE policies for V2X (V2XP) |

#### –	MANAGE UE POLICY COMPLETE

Table 4.7.6-2: MANAGE UE POLICY COMPLETE

| Derivation Path: TS 24.501 Table D.5.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | The same value as the value set in MANAGE UE POLICY COMMAND message. |  |  |
| MANAGE UE POLICY COMPLETE message identity |  | ’0000 0010’B | MANAGE UE POLICY COMPLETE message |  |

#### –	MANAGE UE POLICY COMMAND REJECT

Table 4.7.6-3: MANAGE UE POLICY COMMAND REJECT

| Derivation Path: TS 24.501 Table D.5.3.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | The same value as the value set in MANAGE UE POLICY COMMAND message. |  |  |
| MANAGE UE POLICY COMMAND REJECT message identity |  | ’0000 0011’B | MANAGE UE POLICY COMMAND REJECT message |  |
| UE policy section management result |  |  |  |  |  |  |  |
| Length of UE policy section management result contents |  | Set to the actual length of 'UE policy section management result contents' in bytes |  |  |
| UE policy section management list contents |  | 1 entry |  |  |
| UE policy section management subresult (PLMN 1) |  |  |  |  |
| Number of results |  | 1 |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |  |  |  |
| UE policy section management subresult contents |  | 1 entry |  |  |
| Result 1 |  |  |  |  |  |  |  |
| UPSC |  | Set according to specific message content. |  |  |
| Failed instruction order |  | Set according to specific message content. |  |  |
| Cause |  | Set according to specific message content. |  |  |

#### –	UE STATE INDICATION

Table 4.7.6-4: UE STATE INDICATION

| Derivation Path: TS 24.501 Table D.5.4.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| UE STATE INDICATION message identity |  | ’0000 0100’B |  |  |
| UPSI list |  | 1 entry |  |  |
| Length of UPSI list contents |  | Set to the actual length of 'UPSI list contents' in bytes |  |  |
| UPSI sublist (PLMN 1) |  | 1 entry |  |  |
| Length of UPSI sublist |  | Set to the actual length of 'UPSI sublist' in bytes |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |
| UPSC 1 |  | Set according to specific message content. |  |  |
| UE policy classmark |  | Not checked |  |  |
| Length of Policy information contents |  | Set to the actual length of 'Policy information contents' in bytes |  |  |
| Spare |  | ‘0000 000’B |  |  |  |  |  |
| SupportANDSP |  | ‘1’B |  |  |  | ANDSP |  |
|  |  | Not checked |  |  |  |  |  |
| UE OS Id |  |  |  |  |
| Length of OS Id information contents |  | Set to the actual length of 'OS Id information contents' in bytes |  |  |
| OS Id_1 |  | Set according to parameter given in test case |  |  |

| Condition | Explanation |
| --- | --- |
| ANDSP | UE Access Network Discovery and Selection Policy (ANDSP) |

#### –	UE POLICY PROVISIONING REQUEST

Table 4.7.6-5: UE POLICY PROVISIONING REQUEST

| Derivation Path: TS 24.587 Table 7.2.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| UE POLICY PROVISIONING REQUEST message identity |  | ’0000 0101’B |  |  |
| Requested UE policies |  | FFS |  |  |

#### –	UE POLICY PROVISIONING REJECT

Table 4.7.6-6: UE POLICY PROVISIONING REJECT

| Derivation Path: TS 24.587 Table 7.2.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | The same value as the value set in UE POLICY PROVISIONING REQUEST message. |  |  |
| UE POLICY PROVISIONING REJECT message identity |  | ’0000 0110’B |  |  |
| UPDS cause |  | FFS |  |  |