3GPP TSG-RAN5 Meeting #91-e	R5-212440

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5018 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE TC 6.1.1.6a-User PLMN selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, Cell 4 in TC 6.1.1.6a are replaced with Cell 11 (Corresponding CR R5-211342). However, "Cell 4" in test procedure step 26-30 is still left. Furthermore, “PLMN3” in Table 6.1.1.6a.3.1-2 shall be replaced by "PLMN4". |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. "Cell 4" in test procedure step 26-30 is replaced with "Cell 11". 2. “PLMN3” in Table 6.1.1.6a.3.1-2 is replaced by "PLMN4". 3. Editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.1.6a	PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Single Frequency operation

6.1.1.6a.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.1.6.1

6.1.1.6a.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.6.2

6.1.1.6a.3	Test description

6.1.1.6a.3.1	Pre-test conditions

System Simulator:

-	Four intra-frequency multi-PLMN cells as specified in TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2.

System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.6a.3.1-1.

Table 6.1.1.6a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 2 | PLMN2 |
| 4 | PLMN3 |
| 11 | PLMN4 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.6a.3.1-2.

Table 6.1.1.6a.3.1-2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 | PLMN1 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 2 | PLMN2 PLMN3 Remaining defined entries use default values | E-UTRAN E-UTRAN |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFUST |  | Service 71 is not supported |  |

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.1.6a.3.2	Test procedure sequence

Table 6.1.1.6a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -85 | "Off" | (NOTE 1) |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | "Off" | "Off" | Only Cell 2 is available. (NOTE 1) |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -79 | -85 | -85 | "Off" | (NOTE 1). |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | -85 | -79 | (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |  |

Table 6.1.1.6a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level setting according to the row “T1” in table 6.1.1.6a.3.2-1 | - | - | - | - |
| 2 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 4-8 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 9 | Check: Is PLMN 2 indicated by the UE? | - | - | 1 | P |
| 10 | The SS changes the power level setting according to the row “T2” in table 6.1.1.6a.3.2-1. | - | - | - | - |
| 11 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 12 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within 90 s? | --> | RRCConnectionRequest | 2 | F |
| 13 | Check: Is PLMN 2 indicated by the UE? | - | - | 2 | P |
| 14 | The SS changes the power level setting according to the row “T3” in table 6.1.1.6a.3.2-1. | - | - | - | - |
| 15 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 16 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 3 | P |
| 17-21 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 22 | Check: Is PLMN 1 indicated by the UE? | - | - | 3 | P |
| 23 | The SS changes the power level setting according to the row “T4” in table 6.1.1.6a.3.2-1. | - | - | - | - |
| 24 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 25 | Check: Does the UE send an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 4 | P |
| 26-30 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 31 | Void | - | - | - | - |
| - | EXCEPTION: Step 32a1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 32a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 33 | Check: Is PLMN 4 indicated by the UE? | - | - | 4 | P |
| Note: MMI command “TRIGGER_USER_RESELECTION” to be used. AT command AT+COPS is not suitable to achieve the test purpose.. |  |  |  |  |  |

6.1.1.6a.3.3	Specific message contents

Table 6.1.1.6a.3.3-1: SystemInformationBlockType3 for cells 1 and 2 (preamble and all steps, table 6.1.1.6a.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst |  | dB24 |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>