3GPP TSG-RAN5 Meeting #91-e	R5-212602

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5298 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of band 71 in cat M2 MPR test case, 6.2.3EC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2021-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Band 71 deosn’t have the same output power tolerances as band 28 and should therefore not be grouped together. As per Table 6.2.2E1 in 36.101: Band 28 power class 3, 5 tolerance: +2/-2.5 Band 71 power class 3, 5 tolerance: ±2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed band 71 from table 6.2.3EC.5-1, table 6.2.3EC.5-1a, table 6.2.3EC.5-3, table 6.2.3EC.5-3a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 71 will not be tested correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3EC.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

#### 6.2.3EC.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3EC.5-1, 6.2.3EC.5-2 and6.2.3EC.5-3.

Table 6.2.3EC.5-1: UE Power Class 3 and 5 test requirements for bands without TC,c (Low/Mid/High range) and bands with TC,c (Mid range) except band 28

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 0 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 3 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 4 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 5 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 6 | 0 | 23 | 2 | 25.7 | 20.3 | 0 | 20 | 2 | 22.7 | 17.3 |
| 7 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 8 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 9 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 10 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 11 | 0 | 23 | 2 | 25.7 | 20.3 | N/A |  |  |  |  |
| 12 | N/A |  |  |  |  | 0 | 20 | 2 | 22,7 | 17,3 |
| 13 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 14 | 0 | 23 | 2 | 25.7 | 20.3 | N/A |  |  |  |  |
| 15 | 1 | 22 | 2 | 25.7 | 19.3 | 0 | 20 | 2 | 22.7 | 17.3 |
| 16 | NA |  |  |  |  | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 17 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20,7 | 13,3 |
| 18 | 0 | 23 | 2 | 25,7 | 20,3 | N/A |  |  |  |  |
| 19 | N/A |  |  |  |  | 0 | 20 | 2 | 22,7 | 17,3 |
| 20 | 1 | 22 | 2 | 25,7 | 19,3 | 1 | 19 | 3,5 | 21,7 | 14,8 |
| 21 | 0 | 23 | 2 | 25,7 | 20,3 | N/A |  |  |  |  |
| 22 | 1 | 22 | 2 | 25,7 | 19,3 | 0 | 20 | 2 | 22,7 | 17,3 |
| 23 | 2 | 21 | 2 | 25,7 | 18,3 | 2 | 18 | 4 | 20,7 | 13,3 |
| 24 | 0 | 23 | 2 | 25,7 | 20,3 | 0 | 20 | 2 | 22,7 | 17,3 |
| 25 | 1 | 22 | 2 | 25,7 | 19,3 | 1 | 19 | 3,5 | 21,7 | 14.8 |
| 26 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 27 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 28 | 0 | 23 | 2 | 25.7 | 20.3 | 0 | 20 | 2 | 22.7 | 17.3 |
| 29 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 30 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 31 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4.0 | 20.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3EC.5-1a: UE Power Class 3 test requirements for bands without TC,c (Low/Mid/High range) and bands with TC,c (Mid range) except band 28, subPRB allocation

| Configuration ID | Power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 2 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 3 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 4 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 5 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 6 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 7 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 8 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 9 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 10 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 11 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 12 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 13 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 14 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 15 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 16 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 17 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 18 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 19 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 20 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 21 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 22 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 23 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 24 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 25 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 26 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 27 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 28 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 29 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 30 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |

Table 6.2.3EC.5-2: UE Power Class 3 and 5 test requirements for bands with TC,c (Low/High range)

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 2 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 3 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 4 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 5 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 6 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 7 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 8 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 9 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 10 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 11 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 12 | N/A |  |  |  |  | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 13 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 14 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 15 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 16 | NA |  |  |  |  | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 17 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 18 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 19 | N/A |  |  |  |  |  |  |  |  | 0 |
| 20 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 21 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 22 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 5 | 24.2 | 12.8 |
| 23 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 24 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 25 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 26 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 27 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 28 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 29 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 30 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 31 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3EC.5-2a: UE Power Class 3 test requirements for bands with TC,c (Low/High range), subPRB allocation

| Configuration ID | Power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 2 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 3 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 4 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 5 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 6 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 7 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 8 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 9 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 10 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 11 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 12 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 13 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 14 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 15 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 16 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 17 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 18 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 19 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 20 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 21 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 22 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 23 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 24 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 25 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 26 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 27 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 28 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 29 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 30 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |

Table 6.2.3EC.5-3: UE Power Class 3 and 5 test requirements for band 28

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2.5 | 26.2 | 19.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 2 | 1 | 22 | 2.5 | 26.2 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 3 | 2 | 21 | 2.5 | 26.2 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 4 | 1 | 22 | 2.5 | 26.2 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 5 | 2 | 21 | 2.5 | 26.2 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 6 | 0 | 23 | 2.5 | 26.2 | 19.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 7 | 1 | 22 | 2.5 | 26.2 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 8 | 2 | 21 | 2.5 | 26.2 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 9 | 1 | 22 | 2.5 | 26.2 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 10 | 2 | 21 | 2.5 | 26.2 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 11 | 0 | 23 | 2.5 | 26.2 | 19.8 | N/A |  |  |  |  |
| 12 | N/A |  |  |  |  | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 13 | 1 | 1 | 19 | 3.5 | 23.2 | 1 | 19 | 3.5 | 23.2 | 14,8 |
| 14 | 0 | 23 | 2.5 | 26.2 | 19.8 | N/A |  |  |  |  |
| 15 | 1 | 22 | 2.5 | 26.2 | 18.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 16 | N/A |  |  |  |  | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 17 | 2 | 21 | 2.5 | 25.7 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 18 | 0 | 23 | 2.5 | 25.7 | 19.8 | N/A |  |  |  |  |
| 19 | N/A |  |  |  |  | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 20 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 21 | 0 | 23 | 2.5 | 25.7 | 19.8 | N/A |  |  |  |  |
| 22 | 1 | 22 | 2.5 | 25.7 | 18.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 23 | 2 | 21 | 2.5 | 25.7 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 24 | 0 | 23 | 2.5 | 25.7 | 19.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 25 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 26 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 27 | 2 | 21 | 2.5 | 25.7 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 28 | 0 | 23 | 2.5 | 25.7 | 19.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 29 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 30 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 31 | 2 | 21 | 2.5 | 25.7 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3EC.5-3a: UE Power Class 3 test requirements for band 28, subPRB allocation

| Configuration ID | Power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 2 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 3 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 4 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 5 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 6 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 7 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 8 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 9 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 10 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 11 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 12 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 13 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 14 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 15 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 16 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 17 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 18 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 19 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 20 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 21 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 22 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 23 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 24 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 25 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 26 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 27 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 28 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 29 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 30 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |

Table 6.2.3EC.5-4: UE Power Class 2 test requirements for bands without TC,c (Low/Mid/High range) and bands with TC,c (Mid range)

| Configuration ID | Power class 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 2 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 3 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 4 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 5 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 6 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 7 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 8 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 9 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 10 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 11 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 12 | N/A |  |  |  |  |
| 13 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 14 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 15 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 16 | N/A |  |  |  |  |
| 17 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 18 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 19 | N/A |  |  |  |  |
| 20 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5. Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |

Table 6.2.3EC.5-5: UE Power 2 test requirements for bands with TC,c (Low/High range)

| Configuration ID | Power class 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| 2 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 3 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 4 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 5 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 6 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| 7 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 8 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 9 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 10 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 11 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 12 | N/A |  |  |  |  |
| 13 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 14 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 15 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| 16 | N/A |  |  |  |  |
| 17 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| 18 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 19 | N/A |  |  |  |  |
| 20 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5. Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |

Table 6.2.3EC.5-6: UE Power Class 6 test requirements for bands without TC,c (Low/Mid/High range) and bands with TC,c (Mid range)

| Configuration ID | Power class 6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 2 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 3 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 4 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 5 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 6 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 7 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 8 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 9 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 10 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 11 | N/A |  |  |  |  |
| 12 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 13 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 14 | N/A |  |  |  |  |
| 15 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 16 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 17 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 18 | N/A |  |  |  |  |
| 19 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 20 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 21 | N/A |  |  |  |  |
| 22 | 0 | 14 | 2.5 | 22.7 | 17.3 |
| 23 | 2 | 13 | 2.5 | 20.7 | 13.3 |
| 24 | 0 | 14 | 2.5 | 22.7 | 17.3 |
| 25 | 1 | 13 | 2.5 | 21.7 | 14.8 |
| 26 | 1 | 13 | 2.5 | 21.7 | 14.8 |
| 27 | 2 | 12 | 3.5 | 20.7 | 13.3 |
| 28 | 0 | 14 | 2.5 | 22.7 | 17.3 |
| 29 | 1 | 13 | 2.5 | 21.7 | 14.8 |
| 30 | 1 | 13 | 2.5 | 21.7 | 14.8 |
| 31 | 2 | 12 | 3.5 | 20.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5. Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1. |  |  |  |  |  |

Table 6.2.3EC.5-7: UE Power 6 test requirements for bands with TC,c (Low/High range)

| Configuration ID | Power class 6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 13 | 2.5 | 15.7 | 9.3 |
| 2 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 3 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 4 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 5 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 6 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 7 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 8 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 9 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 10 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 11 | N/A |  |  |  |  |
| 12 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 13 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 14 | N/A |  |  |  |  |
| 15 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 16 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 17 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 18 | N/A |  |  |  |  |
| 19 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 20 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 21 | N/A |  |  |  |  |
| 22 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 23 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 24 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 25 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 26 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 27 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 28 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 29 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 30 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 31 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5. Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1. |  |  |  |  |  |

<< End of changes>>