3GPP TSG-RAN5 Meeting #91-e	R5-212891

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1899 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default RRM test frequencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According 38.508-1 cl. 7.2.3.1, test frequencies defined in 38.508-1 cl.4.3.1.1 will be directly used in FR1 RRM testing. To be more specific, test frequencies corresponding to Mid range shall be used according to 38.533 annex E.1.2. On the other hand, the CORESET #0 reference configurations used in RRM test is defined by RAN4 in 38.133 cl.A.3.1.3 and further referred by RAN5 in 38.533 A.1.2, and CORESET #0 ref. config is incompatiable with 38.508-1 cl.4.3.1.1 on several bands. Take band N79 as an example. 38.508-1 Table 4.3.1.1.1.79-2 indicates that CORESET offset index 1 shall be used(offset = 4RB) according to 38.213 table 13-6 while CORESET #0 reference configuration indicates UE to use index 0 (offset = 0RB). So we purpose to clarify parameters related to CORESET #0 shall be configured according to reference configuration rather than cl.4.3.1.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify that parameters related to CORESET #0 shall be configured according to reference configuration in RRM testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.2.3	Default test frequencies

7.2.3.1	Default test frequencies FR1 NR operating bands

For FR1 NR operating bands the test frequencies for RRM testing are specified in clause 4.3.1.1 except that:

- Offset Carrier CORESET#0, CORESET#0 Index and offsetToPointA(SIB1) are determined according to the CORESET #0 reference configuration used in RRM testing. CORESET #0 reference configuration are given in TS 38.533 [18] Annex A.1.2.

<End of modified section 1>