3GPP TSG-RAN5 Meeting #91-e	R5- 213391

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1367 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Test frequencies for NB-IoT FDD MFBI for band 4, 10 and 66 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, LTE_bands_R15_M1_NB1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NB-IoT Test frequencies for overlapping band 4, 10 and 66 is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for NB-IoT FDD MFBI for band 4, 10 and 66 have been added to Table 8.3.2.3.1-1a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a revision of R5-212677 that was referring to the wrong specification and using “doc” format instead of “docx”-format. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## [Start of changes]

##### 8.3.2.3.1	NB-IoT Test frequencies for signalling test

Test frequencies for signalling test are specified in table 8.3.2.3.1-1 in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.

Table 8.3.2.3.1-1: Test frequencies for NB-IoT FDD (200 kHz)

| NB-IoT Operating Band |  | Bandwidth [MHz] |  | f1, f5 |  |  |  | f2 |  |  |  | f3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NUL |  | NDL |  | NUL |  | NDL |  | NUL |  | NDL |  |
| 1 |  | 60 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 2 |  | 60 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 3 |  | 75 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 4 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 5 |  | 25 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  | 35 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  | 20 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 12 |  | 17 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 13 |  | 10 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 14 |  | 10 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  | 12 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 18 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 19 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 20 |  | 30 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 21 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  | 65 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 26 |  | 35 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 66 |  | 70+901 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 70 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 71 |  | 35 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 73 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 85 |  | 18 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 87 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 88 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| NOTE 1: Asymmetric operating band (UL + DL). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Test frequencies for signalling test of MFBI are specified in Table 8.3.2.3.1-1a for FDD in terms of Low, Mid and High which are referred to as Low Range, Mid Range and High range in clause 8.1.3.1.

Table 8.3.2.3.1-1a: Test frequencies for NB-IoT FDD MFBI (200 kHz)

| NB-IoT Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL |
| 2 | 25 | 60 | Mid | Mid | High | High | Low | Low |
| 4 | 10 | 45 | Mid | Mid | High | High | Low | Low |
| 4 | 66 | 45 | Mid | Mid | High | High | Low | Low |
| 5 | 18 | 15 | Mid | Mid | 23999 | 5999 | Low | Low |
| 5 | 19 | 15 | Mid | Mid | 20609 | 2609 | Low | Low |
| 5 | 26 | 25 | Mid | Mid | High | High | Low | Low |
| 12 | 17 | 12 | Mid | Mid | High | High | Low | Low |
| 17 | 12 | 12 | Mid | Mid | High | High | Low | Low |
| 19 | 5 | 15 | Mid | Mid | High | High | Low | Low |
| 19 | 26 | 15 | Mid | Mid | High | High | Low | Low |
| 25 | 2 | 60 | Mid | Mid | 26639 | 8639 | Low | Low |
| 26 | 5 | 25 | Mid | Mid | High | High | Low | Low |
| 26 | 18 | 15 | Mid | Mid | High | High | Low | Low |
| 26 | 19 | 15 | Mid | Mid | High | High | Low | Low |
| 66 | 4 | 45 | Mid | Mid | 132421 | 66885 | Low | Low |
| 66 | 10 | 60 | Mid | Mid | 132571 | 67035 | Low | Low |

[End of changes]