3GPP TSG-RAN5 Meeting #91-e 	R5-213582

# Electronic Meeting, 17th May 2021 – 28th May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.509 | CR | 0039 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test function Set UL Message - NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of Set UL Message Function |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Set UE Capability Info description and message names "SET UE CAPABILITY INFO REQUEST" and SET UE CAPABILITY INFO RESPONSE" In Rel-16 RACS WI, the UE capability Information message can be segmented if the capability size > Max PDCP SDU size. Since there is no capability indicator from the UE on whether the capability size > Max PDCP SDU size, it can be challenging to find a definitive way to test this. On the same, UE capability message size depends on various parameters and operator requirements. To overcome this and to find a definitive way to test part of conformance requirement (TS38.331, cl 5.7.7.2 and 5.7.7.3), a test mode function which can indicate to the UE to use a preconstructed message which is > Max PDCP SDU size. This will also indicate to conformance tests on the number of segments that will be received from the UE. A test command can set the UE's UEcapabilityInformation message, to be included in the rrc-MessageSegmentContainer IE of the UlDedicatedMessageSegment message(s). Based the on this test message, UE will then trigger the UECapabilityInformation message based on the RRC-segAllowed-r16 IE of the UECapabilityEnquiry message, thus providing test coverage for this feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some aspect of the RACS WP wont be testable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10 (new), 6.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.509 CR 0209 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-212797->R5-212797r1: Editorial corrections are made R5-212797r1->R5-212797r2: 1. Clause 5.9 and 6.9 are updated to 5.10 and 6.10 to avoid merge issues with R5-212451. 2. Test function is now defined in TS 36.509 (R5-212796) and referred in this CR so relevant changes are made. 3. Changes from r1 are not present anymore due to point 2 above. R5-213582: This is the final version of R5-212797r2 |  |  |  |  |  |  |  |  |

<Start of Changes>

## 5.10	Set UL Message test function

### 5.10.1	General

Same as TS 36.509 [6], subclause 5.10.1.

### 5.10.2	Initiation

Same as TS 36.509 [6], subclause 5.10.2.

### 5.10.3	Reception of SET UL MESSAGE REQUEST message by UE

Same as TS 36.509 [6], subclause 5.10.3 with the following exception:

-	the predefined UE capability container is defined in [FFS].

<Unchanged Sections Omitted>

## 6.10	SET UL MESSAGE

### 6.10.1	SET UL MESSAGE REQUEST

Same as TS 36.509 [6], subclause 6.19.1

### 6.10.2	SET UL MESSAGE RESPONSE

Same as TS 36.509 [6], subclause 6.19.2

<End of Changes>