3GPP TSG-RAN5 Meeting #92-e	R5-214194

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1231 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion 7.7.1.2 including TT analysis results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The RRM FR2 test case for inter-frequency periodic measurements is not complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been updated with the following information: - Test requirements including TT analysis results - Test procedure - Message contents - Alignment to core requirements in TS 38.133 - Connection diagram |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM FR2 inter-frequency periodic measurements will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on R4-2112625 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### 7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

7.7.1.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP measurement accuracy is within the specified limits for all bands.

7.7.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.7.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.1.2.

7.7.1.2.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the inter-frequency NR FR2 neighbour cell.

7.7.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.1.2.4.1-1.

Table 7.7.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.1.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode |
| 7.7.1.2-2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.1.2.4.1-2.

Table 7.7.1.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.1.2.4.1-3.

2.	Message contents are defined in clause 7.7.1.2.4.3.

3.	There are two inter-frequency cells specified in the test, where Cell 1 is the serving cell on an NR FR2 carrier and Cell 2 is the neighbour cell on a different NR FR2 carrier and the target cell for the SS-RSRP measurements.

4.	The rx beam peak and directions in which the UE meets the EIS spherical coverage criteria have been found with one of the procedures from Annex I.

7.7.1.2.4.2	Test procedure

1.	Configure the positioning system for a valid test point as defined in A.9.4. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.1.2.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport for the following requirements:

- R1: The SS-RSRP value of Cell 1 reported by the UE is compared to the expected SS-RSRP for Cell 1. If the value is outside the limits in Table 7.7.1.2.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R1 is increased by one. Otherwise, the number of passed iterations for R1 is increased by one.

- R2: The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP for Cell 2. If the value is outside the limits in Table 7.7.1.2.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

- R3: The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 7.7.1.2.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G for each of the requirements is achieved. The evaluation of a specific requirement is concluded when the confidence level for that requirement is reached, even if more measurement reports are required for the remaining requirements.

8.	The SS shall select a new test point as defined in A.9.4 and rotate the positioning system for the selected test point.

9.	Set the parameters according to Test 2 in Table 7.7.2.1.5-2 as appropriate and repeat steps 5-7. In Step 6, the SS shall check the following requirements:

- R4: The SS-RSRP value of Cell 1 reported by the UE is compared to the expected SS-RSRP for Cell 1. If the value is outside the limits in Table 7.7.2.1.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

- R5: The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP for Cell 2. If the value is outside the limits in Table 7.7.2.1.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R5 is increased by one. Otherwise, the number of passed iterations for R5 is increased by one.

- R6: The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 7.7.2.1.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R6 is increased by one. Otherwise, the number of passed iterations for R6 is increased by one.

7.7.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-5 Table H.3.1-6 with condition Pattern #0 Table H.3.1-7 with condition INTER-FREQ Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.7.1.2.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.7.1.2.5	Test requirement

Table 7.7.2.1.5-1 defines the cell specific settings for all tests. Table 7.7.2.1.5-2 defines the OTA primary level settings including test tolerances for all tests.

The SS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clause 7.7.1.0.2.1 and relative accuracy requirements in clause 7.7.1.0.2.2. The following eight requirements are to be verified:

During T1:

R1: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in Table 7.7.2.1.5-3 for test configuration 1 and in Table 7.7.2.1.5-4 for test configuration 2.

R2: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in Table 7.7.2.1.5-3 for test configuration 1 and in Table 7.7.2.1.5-4 for test configuration 2.

R3: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 7.7.2.1.5-5.

During T2:

R4: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table 7.7.2.1.5-3 for test configuration 1 and in Table 7.7.2.1.5-4 for test configuration 2.

R5: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table 7.7.2.1.5-3 for test configuration 1 and in Table 7.7.2.1.5-4 for test configuration 2.

R6: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 7.7.2.1.5-5.

Table 7.7.2.1.5-1: SS-RSRP Inter frequency general test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN | 1~2 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1~2 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated | 1 |  | 24 |  | 24 |  |
|  | 2 |  | 48 |  | 48 |  |
| Gap pattern ID |  |  | 0 |  | 0 |  |
| Duplex mode | 1~2 |  | TDD |  | TDD |  |
| TDD configuration | 1~2 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.3. 2 TDD | - | SR.3. 2 TDD | - |
|  | 2 |  | SR.3.3 TDD |  | SR.3.3 TDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
|  | 2 |  | CR.3.2 TDD |  | CR.3.2 TDD |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
|  | 2 |  | CCR.3.7 TDD |  | CCR.3.7 TDD |  |
| SSB configuration | 1 |  | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  | 2 |  | SSB.4 FR2 |  | SSB.4 FR2 |  |
| PDSCH/PDCCH subcarrier spacing | 1~2 | kHz | 120 |  | 120 |  |
| OCNG Patterns | 1~2 |  | OP.3 |  | OP.3 |  |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |  | DLBWP.1.3 ULBWP.1.3 |  |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |  | TCI.State.2 |  |
| SMTC configuration | 1~2 |  | SMTC.1 |  | SMTC.1 |  |
| Time offset between Cell 2 and Cell 1 | 1~2 | s | 3 |  | 3 |  |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition | 1~2 | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration | 1~2 | - | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void. |  |  |  |  |  |  |

Table 7.7.2.1.5-2: SS-RSRP Inter frequency OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration | 1~2 |  | Setup 4b according to clause A.9.4 |  | Setup 4b according to clause A.9.4 |  |
|  |  |  | AoA1 Spherical coverage | AoA2 Rx Beam Peak | AoA1 Spherical coverage | AoA2 Rx Beam Peak |
| Assumption for UE beamsNote 7 | 1~2 |  | Rough |  | Rough |  |
| Note1 | 1, | dBm/15kHzNote4 | -96.3 | -96.3 | (Table B.2.3-2 Rx Beam PeakNote 8 -4.63dB) | (Table B.2.3-2 Rx Beam PeakNote 8 -3.03dB) |
|  | 2 |  | -99.3 | -99.3 |  |  |
| Note1 | 1 | dBm/SCSNote4 | -87.3 | -87.3 | (Table B.2.3-2 Rx Beam PeakNote 8 +4.4dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +6.0dB) |
|  | 2 |  | -87.3 | -87.3 | (Table B.2.3-2 Rx Beam PeakNote 8 +7.4dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +9.0dB) |
|  | 1~2 | dB | 6.0 | 6.0 | 17.0 | 1.0 |
| SSB_RPNote2 | 1 | dBm/SCS | -81.3 | -81.3 | (Table B.2.3-2 Rx Beam PeakNote 8 +21.4dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +7.0dB) |
|  | 2 |  | -81.3 | -81.3 | (Table B.2.3-2 Rx Beam PeakNote 8 +24.4dB) | (Table B.2. 3-2 Rx Beam PeakNote 8 +10.0dB) |
| (SSB_RPCell 1 – SSB_RPCell 2) | 1~2 | dB | 0 |  | 14.40 |  |
| BBNote6 | 1 | dB | 5.23 | 5.93 | 5.38 | -1.46 |
|  | 2 |  | 4. 58 | 5.87 |  |  |
| IoNote2 | 1 | dBm/95.04 MHz Note4 | -55.70 | -55.70 | (Table B.2.3-2 Rx Beam PeakNote 8 +46.08dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +35.13dB) |
|  | 2 |  | -55.7 | -55.7 | (Table B.2.3-2 Rx Beam PeakNote 8 +49.09dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +38.14dB) |
| (Iofreq 1 – Io freq 2) | 1~2 | dB | 0 |  | 11.95 |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot, Io, (SSB_RPCell 2 – SSB_RPCell 1) and (Iofreq 2 – Io freq 1) levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP or ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. Note 7: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation Note 8: The value in Table B.2.3-2 of TS 38.133 [6] is the Minimum SSB_RP for SCSSSB = 120 kHz, selected according to the operating band of Cell 2 and UE power class, without ∆MBP,n adjustment. |  |  |  |  |  |  |

Table 7.7.2.1.5-3: evaluation limits for the reported values for T1 and T2 absolute accuracy rules R1, R2, R4, R5 for test configuration 1

| UE power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Normal Conditions | Test 1 |  | Test 2 |  |
| Lowest reported value (Cell 1) | n257, n258, n261 | 41 | n257, n258, n261 | 33 |
|  | n260 | 39 | n260 | 34 |
|  | n259 | FFS | n259 | FFS |
| Highest reported value (Cell 1) | All bands: 109 |  | n257, n258, n261 | 101 |
|  |  |  | n260 | 104 |
|  |  |  | n259 | FFS |
| Lowest reported value (Cell 2) | All bands: 52 |  | n257, n258, n261 | 32 |
|  |  |  | n260 | 34 |
|  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | All bands: 109 |  | n257, n258, n261 | 87 |
|  |  |  | n260 | 90 |
|  |  |  | n259 | FFS |
| Extreme Conditions | Test 1 |  | Test 2 |  |
| Lowest reported value (Cell 1) | n257, n258, n261 | 41 + FFS | n257, n258, n261 | 33 + FFS |
|  | n260 | 39 + FFS | n260 | 34 + FFS |
|  | n259 | FFS | n259 | FFS |
| Highest reported value (Cell 1) | All bands: 109 + FFS |  | n257, n258, n261 | 101 + FFS |
|  |  |  | n260 | 104 + FFS |
|  |  |  | n259 | FFS |
| Lowest reported value (Cell 2) | All bands: 52 + FFS |  | n257, n258, n261 | 32+ FFS |
|  |  |  | n260 | 34+ FFS |
|  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | All bands: 109 + FFS |  | n257, n258, n261 | 87+ FFS |
|  |  |  | n260 | 90+ FFS |
|  |  |  | n259 | FFS |

Table 7.7.2.1.5-4: evaluation limits for the reported values for T1 and T2 absolute accuracy rules R1, R2, R4, R5 for test configuration 2

| UE power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Normal Conditions | Test 1 |  | Test 2 |  |
| Lowest reported value (Cell 1) | n257, n258, n261 | 41 | n257, n258, n261 | 36 |
|  | n260 | 39 | n260 | 37 |
|  | n259 | FFS | n259 | FFS |
| Highest reported value (Cell 1) | All bands: 109 |  | n257, n258, n261 | 104 |
|  |  |  | n260 | 107 |
|  |  |  | n259 | FFS |
| Lowest reported value (Cell 2) | All bands: 52 |  | n257, n258, n261 | 35 |
|  |  |  | n260 | 37 |
|  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | All bands: 109 |  | n257, n258, n261 | 90 |
|  |  |  | n260 | 93 |
|  |  |  | n259 | FFS |
| Extreme Conditions | Test 1 |  | Test 2 |  |
| Lowest reported value (Cell 1) | n257, n258, n261 | 41 + FFS | n257, n258, n261 | 36 + FFS |
|  | n260 | 39 + FFS | n260 | 37 + FFS |
|  | n259 | FFS | n259 | FFS |
| Highest reported value (Cell 1) | All bands: 109 + FFS |  | n257, n258, n261 | 104 + FFS |
|  |  |  | n260 | 107 + FFS |
|  |  |  | n259 | FFS |
| Lowest reported value (Cell 2) | All bands: 52 + FFS |  | n257, n258, n261 | 35+ FFS |
|  |  |  | n260 | 37+ FFS |
|  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | All bands: 109 + FFS |  | n257, n258, n261 | 90+ FFS |
|  |  |  | n260 | 93+ FFS |
|  |  |  | n259 | FFS |

Table 7.7.2.1.5-5: evaluation limits for the reported values for T1 and T2 relative accuracy rules R3, R6

| UE power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Normal Conditions | Test 1 |  | Test 2 All bands |  |
| Lowest value (RSRP report Cell 2 – RSRP report Cell 1) | All bands: -15 |  | -29 |  |
| Highest value (RSRP report Cell 2 – RSRP report Cell 1) | n257, n258, n261 | +25 | n257, n258, n261 | +11 |
|  | n260 | +27 | n260 | +13 |
|  | n259 | FFS | n259 | FFS |
| Extreme Conditions | Test 1 |  | Test 2 All bands |  |
| Lowest value (RSRP report Cell 2 – RSRP report Cell 1) | -15 + FFS |  | -29+ FFS |  |
| Highest value (RSRP report Cell 2 – RSRP report Cell 1) | n257, n258, n261 | +25 + FFS | n257, n258, n261 | +11 + FFS |
|  | n260 | +27+ FFS | n260 | +13 + FFS |
|  | n259 | FFS | n259 | FFS |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R6) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

< End of changes >