3GPP TSG RAN Meeting #92-e	 	R5-214202

Electronic Meeting, 16th – 27th August 2021

Source:		Rohde & Schwarz

Title:	On the achievable SNR for demod test cases

Agenda Item:		5.3.2.17

Document for:	Discussion and Endorsement

# Introduction

In the last RAN5 meetings the testability of high SNR demod test cases has been discussed [1 –  12] and several working assumptions have been agreed. However, the maximum achievable SNR is not yet finalized since details such as the backoff from the 1dB compression point of the PA have not been concluded. In the meantime, the available DL power at center of quiet zone at CW 1dB compression and a working assumption on the achievable SNR have been endorsed for frequencies n258/n257/n261 mid. In this contribution, we propose according values for the remaining bands n260 and n259.

# Discussion

At RAN5 #91-e the following proposals have been endorsed:

Proposal 3: For n258/n257/n261 mid, apply the values in Table 1 in the calculation of the achievable DL SNR and update the spreadsheet calculator accordingly.

Table 1: Proposed available DL power at QZ center

| Operating band and frequency range | Test frequency (GHz) | Available DL power at center of quiet zone at CW 1dB compression (dBm) |
| --- | --- | --- |
| n258 mid | 25.875 |  |
| n257 mid n261 mid | 28 27.925 | -32.8 |

Proposal 4: For n258/n257/n261 mid, use the values in Table 2 and Table 3 as interim working assumption of the maximum achievable SNR for 100MHz CHBW, PC3 and IFF with 30cm Quiet Zone. Enable testing of the test points with SNR less than or equal to these values for 100 MHz CHBW.

Table 2: Maximum achievable SNR for IFF with 30cm QZ, PC3 and fading

| Operating band and frequency range | Test frequency (GHz) | Maximum achievable SNR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n258 mid | 25.875 |  |  |  |
| n257 mid n261 mid | 28 27.925 | 24.0 | 20.8 | 17.8 |

Table 3: Maximum achievable SNR for IFF with 30cm QZ, PC3 without fading

| Operating band and frequency range | Test frequency (GHz) | Maximum achievable SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n258 mid | 25.875 |  |  |  |
| n257 mid n261 mid | 28 27.925 | 28.7 | 25.5 | 22.5 |

Based on the characteristics of our test system, we propose the values in Table 1 for the achievable DL power at 1dB compression for the remaining bands.

Table 1: Proposed available DL power at QZ center

| Operating band and frequency range | Test frequency (GHz) | Available DL power at center of quiet zone at CW 1dB compression (dBm) |
| --- | --- | --- |
| n260 mid | 38.5 | -36.4 |
| n259 mid | 41.5 | -39.4 |

Proposal 1: For n260 mid and n259 mid, apply the values in Table 1 in the calculation of the achievable DL SNR and update the spreadsheet calculator accordingly.

Based on the working assumption of a backoff of 17.71 dB of last meeting, this results in the maximum achievable SNR for IFF with 30cm QZ, PC3 and fading summarized in Table 2.

Table 2: Maximum achievable SNR for IFF with 30cm QZ, PC3 and fading

| Operating band and frequency range | Test frequency (GHz) | Maximum achievable SNR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n260 mid | 38.5 | 17.7 | 14.5 | 11.4 |
| n259 mid | 41.5 | 13.6 | 10.3 | 7.0 |

For the case without fading a backoff of 13 dB is assumed and the associated maximum achievable SNR for IFF with 30cm QZ, PC3 are summarized in Table 3.

Table 3: Maximum achievable SNR for IFF with 30cm QZ, PC3 without fading

| Operating band and frequency range | Test frequency (GHz) | Maximum achievable SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n260 mid | 38.5 | 22.5 | 19.3 | 16.3 |
| n259 mid | 41.5 | 18.4 | 15.2 | 12.1 |

The calculation is documented in the Excel sheet attached to the CR [13] associated with this paper.

Proposal 2: For n260 mid and n259 mid, use the values in Table 2 and Table 3 as interim working assumption of the maximum achievable SNR for PC3 and IFF with 30cm Quiet Zone. Enable testing of the test points with SNR less than or equal to these values.

Accompanying CRs [13] & [14] provide a documentation of the assumptions and limits in TR 38.903 [16] and an update in TS 38.521-4 [15], respectively.

# Conclusion

In summary, the maximum achievable SNR of demod test cases has been discussed.

Proposal 1: For n260 mid and n259 mid, apply the values in Table 1 in the calculation of the achievable DL SNR and update the spreadsheet calculator accordingly.

Proposal 2: For n260 mid and n259 mid, use the values in Table 2 and Table 3 as interim working assumption of the maximum achievable SNR for PC3 and IFF with 30cm Quiet Zone. Enable testing of the test points with SNR less than or equal to these values.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |

# References

- R5-205702, On Fading Impact on CREST and Demod Testability Analysis, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #89-e, November 2020

- R5-206614, On the achievable SNR for FR2 demod and perf test cases, Anritsu, 3GPP TSG RAN WG 5 Meeting #89-e, November 2020

- R5-211947, Input on fading crest factor margin for FR2 Demodulation test cases, 3GPP TSG RAN WG 5 Meeting #90-e, Qualcomm, February 2021

- R5-210775, On Testability Aspects for FR2 Demodulation Testing, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #90-e, February 2021

- R5-211950, On the achievable SNR for demod test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #90-e, February 2021

- R5-211936, Update of demod SNR testability, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #90-e, February 2021

- R5-211929, Update of FR2 demod test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #90-e, February 2021

- R5-213806, On the achievable SNR for demod test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-213851, Update of demod SNR testability, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-214058, Update of FR2 demod test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-212961, On the upper limit of testable SNR for FR2 demodulation tests, Anritsu, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-213819, On Demod achievable SNR and Tx EVM, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-214204, Update of demod SNR testability, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #92-e, August 2021

- R5-214203, Update of FR2 demod test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #9s-e, August 2021

- TS 38.521-4, V16.8.0, 3GPP TSG RAN WG5, June 2021

- TR 38.903, V16.8.0, 3GPP TSG RAN WG5, June 2021