3GPP TSG-RAN WG5 Meeting #92-e	R5-214334

# Electronic Meeting, 16th - 27th August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0220 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of ICS for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test cases for NR-U need ICS to be complete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added NR-U ICS that are required in test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-U test cases cannot be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.1, A.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# A.4	ICS proforma tables

## A.4.1	UE Implementation Types

Table A.4.1-1: UE Radio Technologies

| Item |  | UE Radio Technologies |  | Ref. |  | Release |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | NR FDD |  | 38.101-1 |  | Rel-15 |  | pc_nrFDD |  |  |  |
| 2 |  | NR TDD |  | 38.101-1, 38.101-2 |  | Rel-15 |  | pc_nrTDD |  |  |  |
| 3 |  | NR sidelink |  | 38.101-1 |  | Rel-16 |  | pc_nrSL |  |  |  |

Table A.4.1-2: UE general functionality

| Item | UE Functionality | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Multiple NR FDD bands | 38.101-1, 5.2 | Rel-15 | pc_nrFDD_MultiBand |  |
| 2 | Multiple NR TDD bands | 38.101-1, 5.2, 38.101-2, 5.2 | Rel-15 | pc_nrTDD_MultiBand |  |
| 3 | NR SUL | 38.101-1 | Rel-15 | pc_nrSUL |  |
| 4 | NR SDL | 38.101-1 | Rel-15 | pc_nrSDL |  |
| 5 | Multiple NR SUL bands | 38.101-1, 5.2 | Rel-15 | pc_nrSUL_MultiBand |  |
| 6 | Multiple NR SDL bands | 38.101-1, 5.2 | Rel-15 | pc_nrSDL_MultiBand |  |
| 7 | Frequency range FR1 | 38.101-1, 5.1 | Rel-15 | pc_nrFR1 |  |
| 8 | Frequency range FR2 | 38.101-2, 5.1 | Rel-15 | pc_nrFR2 |  |

Table A.4.1-3: RAN-CN Interface Options

| Item | UE support of RAN-CN Interface Options | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NG-RAN NR | 38.300 | Rel-15 | pc_NG_RAN_NR | Option 2 |
| 2 | EN-DC | 37.340 | Rel-15 | pc_EN_DC | Option 3 |
| 3 | NE-DC | 37.340 | Rel-15 | pc_NE_DC | Option 4 |
| 4 | NG-RAN E-UTRA | 38.300 | Rel-15 | pc_NG_RAN_EUTRA | Option 5 |
| 5 | NGEN-DC | 37.340 | Rel-15 | pc_NGEN_DC | Option 7 |

Table A.4.1-4: NSA DC UE Radio Technologies

| Item | NSA UE Radio Technologies | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Intra-Band Contiguous EN-DC | 38.101-3, 5.5B.2 | Rel-15 | pc_IntraBand_Contiguous_ENDC |  |
| 2 | Intra-Band Non-Contiguous EN-DC | 38.101-3, 5.5B.3 | Rel-15 | pc_IntraBand_Non_Contiguous_ENDC |  |
| 3 | Inter-Band EN-DC within FR1 | 38.101-3, 5.5B.4 | Rel-15 | pc_InterBand_ENDC_WithinFR1 |  |
| 4 | Inter-Band EN-DC including FR2 | 38.101-3, 5.5B.5 | Rel-15 | pc_InterBand_ENDC_IncludingFR2 |  |
| 5 | Inter-band EN-DC including FR1 and FR2 | 38.101-3, 5.5B.6 | Rel-15 | pc_InterBand_ENDC_IncludingFR1_FR2 |  |
| 6 | Inter-band NR-DC between FR1 and FR2 | 38.101-3, 5.5B.7 | Rel-15 | pc_InterBand_NRDC_BetweenFR1_FR2 |  |

Table A.4.1-4A: SA CA UE Radio Technologies

| Item | SA UE Radio Technologies | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Intra-Band Contiguous CA within FR1 | 38.101-1, 5.5A.1 | Rel-15 | pc_IntraBand_Contiguous_CA_WithinFR1 |  |
| 2 | Intra-Band Non-contiguous CA within FR1 | 38.101-1, 5.5A.2 | Rel-16 | pc_IntraBand_NonContiguous_CA_WithinFR1 |  |
| 3 | Intra-Band Contiguous CA within FR2 | 38.101-2, 5.5A.1 | Rel-15 | pc_IntraBand_Contiguous_CA_WithinFR2 |  |
| 4 | Intra-Band Non-contiguous CA within FR2 | 38.101-2, 5.5A.2 | Rel-15 | pc_IntraBand_NonContiguous_CA_WithinFR2 |  |
| 5 | Inter-Band CA within FR1 | 38.101-1, 5.5A.3 | Rel-15 | pc_InterBand_CA_WithinFR1 |  |
| 6 | Inter-Band CA within FR2 | 38.101-2, 5.5A.3 | Rel-16 | pc_InterBand_CA_WithinFR2 |  |
| 7 | Inter-band CA between FR1 and FR2 | 38.101-3, 5.5A.1 | Rel-15 | pc_InterBand_CA_BetweenFR1_FR2 |  |

Table A.4.1-5: 5GS UE Core Technologies

| Item | 5GS UE Core Technologies | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | UE Supports 5G Core Network | 24.501 | Rel-15 | pc_5GCN |  |
| 2 | UE Supports 5G Core Network over non-3GPP Access Network | 24.501, 4.7 | Rel-15 | pc_5GCN_N3AN |  |

## A.4.2	UE Service Capabilities

### A.4.2.1	3GPP Standardised UE Service Capabilities

#### A.4.2.1.1	Bearer Services

Table A.4.2.1.1-1: Definition of Bearer Services

| Item | Definition of Bearer Services | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | FFS |  |  |  |  |

## A.4.3	Baseline Implementation Capabilities

Table A.4.3-1: Supported protocols

| Item | Supported protocols | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | 5GS Mobility Management | 24.501 | Rel-15 |  |  |
| 2 | 5GS Session Management | 24.501 | Rel-15 |  |  |
| 3 | Radio Resource Control | 38.331 | Rel-15 |  |  |
| 4 | Service Data Adaptation Protocol | 37.324 | Rel-15 |  |  |
| 5 | Packet Data Convergence Protocol | 38.323 | Rel-15 |  |  |
| 6 | Radio Link Control | 38.322 | Rel-15 |  |  |
| 7 | Medium Access Control | 38.321 | Rel-15 |  |  |
| 8 | Physical Layer | 38.201 | Rel-15 |  |  |

Table A.4.3-2: Special Conformance Testing Functions

| Item | Special Conformance Testing Functions | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | UE test loop | 38.509 | Rel-15 |  |  |

### A.4.3.1	RF Baseline Implementation Capabilities

NOTE:	The values indicated in column "Release" in tables A.4.3.1-1 and A.4.3.1-2 below are to be understood as the specifications release version in which a band was introduced and not as a mandate that a UE conforming to particular release shall support a particular band. For further guidance to release independent bands see TS 38.307 [19].

Table A.4.3.1-1: NR FDD FR1 RF Baseline Implementation Capabilities

| Item | NR FDD FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1920-1980 MHz (UL), 2110-2170 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand1_Supp | NR FDD FR1 Band 1 |
| 2 | NR Frequency band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand2_Supp | NR FDD FR1 Band 2 |
| 3 | NR Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand3_Supp | NR FDD FR1 Band 3 |
| 4 | NR Frequency band: 824-849 MHz (UL), 869-894 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand5_Supp | NR FDD FR1 Band 5 |
| 5 | NR Frequency band: 2500-2570 MHz (UL), 2620-2690 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand7_Supp | NR FDD FR1 Band 7 |
| 6 | NR Frequency band: 880-915 MHz (UL), 925-960 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand8_Supp | NR FDD FR1 Band 8 |
| 6a to 6c | Reserved |  |  |  |  |
| 6d | NR Frequency band: 699-716 MHz (UL), 729-746 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand12_Supp | NR FDD FR1 Band 12 |
| 6e | Reserved |  |  |  |  |
| 6f | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand14_Supp | NR FDD FR1 Band 14 |
| 6g to 6i | Reserved |  |  |  |  |
| 6j | NR Frequency band: 815-830 MHz (UL), 860-875 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand18_Supp | NR FDD FR1 Band 18 |
| 6k | Reserved |  |  |  |  |
| 7 | NR Frequency band: 832-862 MHz (UL), 791-821 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand20_Supp | NR FDD FR1 Band 20 |
| 7a to 7d | Reserved |  |  |  |  |
| 7e | NR Frequency band: 1850-1915 MHz (UL), 1930- 1995 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand25_Supp | NR FDD FR1 Band 25 |
| 7f | NR Frequency band: 814-849 MHz (UL), 859-894 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand26_Supp | NR FDD FR1 Band 26 |
| 7g | Reserved |  |  |  |  |
| 8 | NR Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand28_Supp | NR FDD FR1 Band 28 |
| 8a | Reserved |  |  |  |  |
| 8b | NR Frequency band: 2305-2315 MHz (UL), 2350-2360 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand30_Supp | NR FDD FR1 Band 30 |
| 8c to 8d | Reserved |  |  |  |  |
| 8e | NR Frequency band: 1920-2010 MHz (UL),2110-2200 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand65_Supp | NR FDD FR1 Band 65 |
| 9 | NR Frequency band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand66_Supp | NR FDD FR1 Band 66 |
| 9a to 9c | Reserved |  |  |  |  |
| 10 | NR Frequency band: 1695-1710 MHz (UL), 1995-2020 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand70_Supp | NR FDD FR1 Band 70 |
| 11 | NR Frequency band: 663-698 MHz (UL), 617-652 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand71_Supp | NR FDD FR1 Band 71 |
| 12 to 13 | Reserved |  |  |  |  |
| 14 | NR Frequency band: 1427-1470 MHz (UL), 1475-1518 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand74_Supp | NR FDD FR1 Band 74 |
| 15 | NR Frequency band: 832-862 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand91_Supp | NR FDD FR1 Band 91 |
| 16 | NR Frequency band: 832-862 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand92_Supp | NR FDD FR1 Band 92 |
| 17 | NR Frequency band: 880-915 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand93_Supp | NR FDD FR1 Band 93 |
| 18 | NR Frequency band: 880-915 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand94_Supp | NR FDD FR1 Band 94 |

Table A.4.3.1-2: NR TDD FR1 RF Baseline Implementation Capabilities

| Item | NR TDD FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 0 | NR Frequency band: 2010-2025 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand34_Supp | NR TDD FR1 Band 34 |
| 0a to 0c | Reserved |  |  |  |  |
| 1 | NR Frequency band: 2570-2620 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand38_Supp | NR TDD FR1 Band 38 |
| 1a | NR Frequency band: 1880-1920 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand39_Supp | NR TDD FR1 Band 39 |
| 1b | NR Frequency band: 2300-2400 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand40_Supp | NR TDD FR1 Band 40 |
| 2 | NR Frequency band: 2496-2690 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand41_Supp | NR TDD FR1 Band 41 |
| 2a to 2d | Reserved |  |  |  |  |
| 2e | NR Frequency band: 5150-5925 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand46_Supp | NR TDD FR1 Band 46 |
| 2f | Reserved |  |  |  |  |
| 2g | NR Frequency band: 3550-3700 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand48_Supp | NR TDD FR1 Band 48 |
| 2h | Reserved |  |  |  |  |
| 2i | NR Frequency band: 1432-1517 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand50_Supp | NR TDD FR1 Band 50 |
| 2j | NR Frequency band: 1427-1432 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand51_Supp | NR TDD FR1 Band 51 |
| 2k | Reserved |  |  |  |  |
| 2l | NR Frequency band: 2483.5-2495 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand53_Supp | NR TDD FR1 Band 53 |
| 3 | NR Frequency band: 3300–4200 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand77_Supp | NR TDD FR1 Band 77 |
| 4 | NR Frequency band: 3300–3800 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand78_Supp | NR TDD FR1 Band 78 |
| 5 | NR Frequency band: 4400–5000 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand79_Supp | NR TDD FR1 Band 79 |
| 6 | NR Frequency band: 2496–2690 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand90_Supp | NR TDD FR1 Band 90 |
| 7-11 | Reserved |  |  |  |  |
| 12 | NR Frequency band: 5925–7125 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand96_Supp | NR TDD FR1 Band 96 |

Table A.4.3.1-3: NR TDD FR2 RF Baseline Implementation Capabilities

| Item | NR TDD FR2 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz | 38.101-2, 5.2 | Rel-15 | pc_nrBand257_Supp | NR TDD FR2 Band 257 |
| 2 | NR Frequency band: 24250-27500 MHz | 38.101-2, 5.2 | Rel-15 | pc_nrBand258_Supp | NR TDD FR2 Band 258 |
| 2a | NR Frequency band: 39500-43500 MHz | 38.101-2, 5.2 | Rel-16 | pc_nrBand259_Supp | NR TDD FR2 Band 259 |
| 3 | NR Frequency band: 37000–40000 MHz | 38.101-2, 5.2 | Rel-15 | pc_nrBand260_Supp | NR TDD FR2 Band 260 |
| 4 | NR Frequency band: 27500–28350 MHz | 38.101-2, 5.2 | Rel-15 | pc_nrBand261_Supp | NR TDD FR2 Band 261 |

Table A.4.3.1-4: NR FR1 PC2 RF Baseline Implementation Capabilities

| Item | NR FR1 PC2 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 0 | NR Frequency band: 2300-2400 MHz | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand40_PC2_Supp | NR FR1 PC2 Band 40 |
| 1 | NR Frequency band: 2496-2690 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand41_PC2_Supp | NR FR1 PC2 Band 41 |
| 2 | NR Frequency band: 3300-4200 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand77_PC2_Supp | NR FR1 PC2 Band 77 |
| 3 | NR Frequency band: 3300–3800 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand78_PC2_Supp | NR FR1 PC2 Band 78 |
| 4 | NR Frequency band: 4400–5000 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand79_PC2_Supp | NR FR1 PC2 Band 79 |

Table A.4.3.1-4a: NR FR2 PC2 RF Baseline Implementation Capabilities

| Item | NR FR2 PC2 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand257_PC2_Supp | NR FR2 PC2 Band 257 |
| 2 | NR Frequency band: 24250-27500 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand258_PC2_Supp | NR FR2 PC2 Band 258 |
| 3 | NR Frequency band: 27500–28350 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand261_PC2_Supp | NR FR2 PC2 Band 261 |

Table A.4.3.1-4b: NR FR1 PC1 RF Baseline Implementation Capabilities

| Item | NR FR1 PC1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 788-798 MHz, 758-768 MHz | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand14_PC1_Supp | NR FR1 PC1 Band 14 |

Table A.4.3.1-4c: NR FR2 PC1 RF Baseline Implementation Capabilities

| Item | NR FR2 PC1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand257_PC1_Supp | NR FR2 PC1 Band 257 |
| 2 | NR Frequency band: 24250-27500 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand258_PC1_Supp | NR FR2 PC1 Band 258 |
| 3 | NR Frequency band: 37000-40000 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand260_PC1_Supp | NR FR2 PC1 Band 260 |
| 4 | NR Frequency band: 27500–28350 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand261_PC1_Supp | NR FR2 PC1 Band 261 |

Table A.4.3.1-4d: NR FR2 PC4 RF Baseline Implementation Capabilities

| Item | NR FR2 PC4 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand257_PC4_Supp | NR FR2 PC4 Band 257 |
| 2 | NR Frequency band: 24250-27500 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand258_PC4_Supp | NR FR2 PC4 Band 258 |
| 3 | NR Frequency band: 37000-40000 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand260_PC4_Supp | NR FR2 PC4 Band 260 |
| 4 | NR Frequency band: 27500–28350 MHz | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand261_PC4_Supp | NR FR2 PC4 Band 261 |

Table A.4.3.1-5: NR SUL FR1 RF Baseline Implementation Capabilities

| Item | NR SUL FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1710-1785 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand80_Supp | NR SUL FR1 Band 80 |
| 2 | NR Frequency band: 880-915 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand81_Supp | NR SUL FR1 Band 81 |
| 3 | NR Frequency band: 832-862 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand82_Supp | NR SUL FR1 Band 82 |
| 4 | NR Frequency band: 703-748 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand83_Supp | NR SUL FR1 Band 83 |
| 5 | NR Frequency band: 1920-1980 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand84_Supp | NR SUL FR1 Band 84 |
| 6 | NR Frequency band: 1710-1780 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand86_Supp | NR SUL FR1 Band 86 |
| 6a to 6b | Reserved |  |  |  |  |
| 6c | NR Frequency band: 824-849 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand89_Supp | NR SUL FR1 Band 89 |
| 7 | NR Frequency band: 2010-2025 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand95_Supp | NR SUL FR1 Band 95 |

Table A.4.3.1-6: NR SDL FR1 RF Baseline Implementation Capabilities

| Item | NR SDL FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 0 | NR Frequency band: 717-728 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand29_Supp | NR SDL FR1 Band 29 |
| 1 | NR Frequency band: 1432-1517 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand75_Supp | NR SDL FR1 Band 75 |
| 2 | NR Frequency band: 1427-1432 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand76_Supp | NR SDL FR1 Band 76 |

Table A.4.3.1-7: UE Power Class implementation Capabilities (for one or more of the supported UE Power Class Implemented Capabilities in Table A.4.3.1-4, Table A.4.3.1-4a, Table A.4.3.1-4b, Table A.4.3.1-4c and Table A.4.3.1-4d)

| Item | UE Power Class implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | UE Power Class 1 in FR1 | 38.101-1, 6.2.1 | Rel-16 | pc_FR1_PC1 | Applicable to the bands in Table A.4.3.1-4b |
| 1a | UE Power Class 1 in FR2 | 38.101-2, 6.2.1 | Rel-15 | pc_FR2_PC1 | Applicable to the bands in Table A.4.3.1-4c |
| 2 | UE Power Class 2 in FR1 | 38.101-1, 6.2.1 | Rel-15 | pc_FR1_PC2 | Applicable to the bands in Table A.4.3.1-4 |
| 2a | UE Power Class 2 in FR2 | 38.101-2, 6.2.1 | Rel-15 | pc_FR2_PC2 | Applicable to the bands in Table A.4.3.1-4a |
| 3 | UE Power Class 3 in FR1 | 38.101-1, 6.2.1 | Rel-15 | pc_FR1_PC3 | All applicable FR1 NR bands |
| 3a | UE Power Class 3 in FR2 | 38.101-2, 6.2.1 | Rel-15 | pc_FR2_PC3 | All applicable FR2 NR bands |
| 4 | UE Power Class 4 in FR2 | 38.101-2, 6.2.1 | Rel-15 | pc_FR2_PC4 | Applicable to the bands in Table A.4.3.1-4d |

Table A.4.3.1-7a: NR FR1 2Rx/4Rx implementation Capabilities

| Item | UE 2Rx/4Rx implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | UE 2Rx in FR1 | 38.101-1, 7.3 | Rel-15 | pc_FR1_2Rx | If the capability is supported then the Band(s) for which it is supported shall be indicated in Table A.4.3.9-4c |
| 2 | UE FDD 4Rx in FR1 | 38.101-1, 7.3 | Rel-15 | pc_FR1_FDD_4Rx | If the capability is supported then the Band(s) for which it is supported shall be indicated in Table A.4.3.9-4a |
| 3 | UE TDD 4Rx in FR1 | 38.101-1, 7.3 | Rel-15 | pc_FR1_TDD_4Rx | If the capability is supported then the Band(s) for which it is supported shall be indicated in Table A.4.3.9-4b |

Table A.4.3.1-8: Void

Table A.4.3.1-9: NR Sidelink FR1 RF Baseline Implementation Capabilities

| Item | NR Sidelink FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 2570-2620 MHz (Transmission), 2570-2620 MHz (Reception) | 38.101-1, 5.2E | Rel-16 | pc_nrBand38_NRSL_Supp | NR Sidelink FR1 Band 38 |
| 2 | NR Frequency band: 5855-5925 MHz (Transmission), 5855-5925 MHz (Reception) | 38.101-1, 5.2E | Rel-16 | pc_nrBand47_NRSL_Supp | NR Sidelink FR1 Band 47 |

### A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item |  |  | UE Physical Layer Baseline Implementation Capabilities |  |  | Ref. |  |  | Release |  |  | Mnemonic |  |  | M |  |  | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Support PDSCH reception based on semi-persistent scheduling |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_downlinkSPS |  |  | No |  |  |  |  |  |  |  |  |
| 2 |  |  | Support 256QAM for PDSCH for FR1 |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_pdsch_256QAM_FR1 |  |  | Yes |  |  |  |  |  |  |  |  |
| 3 |  |  | Support 256QAM for PDSCH for at least one NR FR2 band |  |  | 38.306, 4.2.7.2 |  |  | Rel-15 |  |  | pc_pdsch_256QAM_FR2 |  |  | No |  |  |  |  |  |  |  |  |
| 4 |  |  | Support 256QAM for PUSCH for at least one NR FR1 band |  |  | 38.306, 4.2.7.2 |  |  | Rel-15 |  |  | pc_pusch_256QAM_FR1 |  |  | No |  |  |  |  |  |  |  |  |
| 4a |  |  | Support 256QAM for PUSCH for at least one NR FR2 band |  |  | 38.306, 4.2.7.2 |  |  | Rel-15 |  |  | pc_pusch_256QAM_FR2 |  |  | No |  |  |  |  |  |  |  |  |
| 5 |  |  | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_pdsch_MappingTypeA |  |  | Yes |  |  | Yes |  |  |  |  |  |
| 6 |  |  | Support receiving PDSCH using PDSCH mapping type B |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_pdsch_MappingTypeB |  |  | Yes |  |  |  |  |  |  |  |  |
| 7 |  |  | Support resource allocation Type 0 for PUSCH |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_ra_Type0_PUSCH |  |  | No |  |  |  |  |  |  |  |  |
| 8 |  |  | Support scaling factor 0.75 is applied to the band in the max data rate calculation |  |  | 38.306, 4.2.7 |  |  | Rel-15 |  |  | pc_scalingFactor0dot75 |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  | Support reconfiguration with sync using a contention free random access on PRACH resources that are associated with CSI-RS resources of the target cell |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_csi_RS_CFRA_ForHO |  |  | No |  |  |  |  |  |  |  |  |
| 10 |  |  | Support Type 1 PUSCH transmissions with configured grant |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_configuredUL_GrantType1 |  |  | No |  |  |  |  |  |  |  |  |
| 11 |  |  | Support Type 2 PUSCH transmissions with configured grant |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_configuredUL_GrantType2 |  |  | No |  |  |  |  |  |  |  |  |
| 12 |  |  | Support PDSCH Reception when configured with higher layer parameter aggregationFactorDL > 1 |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_pdsch_RepetitionMultiSlots |  |  | No |  |  |  |  |  |  |  |  |
| 13 |  |  | Supports supplemental uplink with dynamic switch (DCI based selection of PUSCH carrier) |  |  | 38.306, 4.2.7.7 |  |  | Rel-15 |  |  | pc_dynamicSwitchSUL |  |  | No |  |  |  |  |  |  |  |  |
| 14 |  |  | Supports MIMO layers at the UE for PUSCH transmission with codebook precoding. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset |  |  | 38.306, 4.2.7.8 |  |  | Rel-15 |  |  | pc_nrMIMO_CB_PUSCH |  |  | No |  |  |  |  |  | Set to true if maxNumberMIMO-LayersCB-PUSCH has value different from "oneLayer" |  |  |
| 15 |  |  | Supports MIMO layers at the UE for PUSCH transmission using non-codebook precoding |  |  | 38.306, 4.2.7.8 |  |  | Rel-15 |  |  | pc_nrMIMO_NonCB_PUSCH |  |  | No |  |  |  |  |  | Set to true if maxNumberMIMO-LayersNonCB-PUSCH has value different from "oneLayer" |  |  |
| 16 |  |  | Support receiving PDSCH with interleaved VRB-to-PRB mapping |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_interleavingVRB_ToPRB_PDSCH |  |  | Yes |  |  |  |  |  |  |  |  |
| 17 |  |  | Support dynamic EN-DC power sharing for at least one EN-DC band combination |  |  | 38.306, 4.2.7.9 |  |  | Rel-15 |  |  | pc_dynamicPowerSharing |  |  | Yes |  |  |  |  |  | If the UE supports this capability it will dynamically share the power between NR and LTE if P_LTE + P_NR > Pcmax. |  |  |
| 18 |  |  | Supports up to 10 search spaces in a SCell per BWP |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_maxNumberSearchSpaces |  |  | No |  |  |  |  |  |  |  |  |
| 19 |  |  | Supports spatial bundling of HARQ-ACK bits carried on PUCCH or PUSCH per PUCCH group. With spatial bundling, two HARQ-ACK bits for a DL MIMO data is bundled into a single bit by logical "AND" operation |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_spatialBundlingHARQ_ACK |  |  | Yes |  |  |  |  |  |  |  |  |
| 20 |  |  | Support alternative additional DMRS position for co-existence with LTE CRS |  |  | 38.306, 4.2.7.5 |  |  | Rel-15 |  |  | pc_additionalDMRS_DL_Alt |  |  | No |  |  |  |  |  |  |  |  |
| 21 |  |  | Supports transmitting PUSCH scheduled by DCI format 0_0 or 0_1 when configured with higher layer parameter aggregationFactorIUL > 1 |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_pusch_RepetitionMultiSlots |  |  | Yes |  |  |  |  |  |  |  |  |
| 22 |  |  | Support beam correspondence without UL beam sweeping |  |  | 38.306, 4.2.7.2 |  |  | Rel-15 |  |  | pc_beamCorrespondenceWithoutUL_BeamSweeping |  |  | Yes |  |  |  |  |  | A UE that can fulfil the requirements without UL beam sweeping then set the bit to 1. A UE that can fulfil the requirements with UL beam sweeping then set the bit to 0. |  |  |
| 23 |  |  | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |  |  | 38.306, 4.2.7.6 |  |  | Rel-15 |  |  | pc_maxNumberMIMO_LayersPDSCH |  |  | CY |  |  |  |  |  |  |  |  |
| 24 |  |  | Supports DCI and timer based active BWP switching delay type1 |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_bwp-SwitchingDelay_Type1 |  |  | No |  |  |  |  |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |  |  |
| 24A |  |  | Supports DCI and timer based active BWP switching delay type2 |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_bwp_SwitchingDelay_Type2 |  |  | No |  |  |  |  |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |  |  |
| 25 |  |  | Support modified MPR behaviour |  |  | 38.306 4.2.7.2 |  |  | Rel-15 |  |  | pc_modifiedMPR_behaviour |  |  | No |  |  |  |  |  |  |  |  |
| 26 |  |  | Support dynamic switching between resource allocation Types 0 and 1 for PDSCH |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_dynamicSwitchRA_Type0_1_PDSCH |  |  | No |  |  |  |  |  |  |  |  |
| 27 |  |  | Support dynamic switching between resource allocation Types 0 and 1 for PUSCH |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_dynamicSwitchRA_Type0_1_PUSCH |  |  | No |  |  |  |  |  |  |  |  |
| 28 |  |  | Support almost contiguous UL CP-OFDM transmissions in FR1 |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_almostContiguousCP_OFDM_UL_FR1 |  |  | No |  |  |  |  |  |  |  |  |
| 29 |  |  | Support almost contiguous UL CP-OFDM transmissions in FR2 |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_almostContiguousCP_OFDM_UL_FR2 |  |  | No |  |  |  |  |  |  |  |  |
| 30 |  |  | Support dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1, and the minimum scheduling offset for PDSCH and aperiodic CSI-RS triggering offset (K0), and PUSCH (K2), and the extended value range for aperiodic CSI-RS triggering offset |  |  | 38.306, 4.2.7.10 |  |  | Rel-16 |  |  | pc_crossSlotScheduling |  |  | No |  |  |  |  |  |  |  |  |
| 31 |  |  | Supports pi/2-BPSK modulation scheme for PUSCH in FR1 |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_pusch_halfpiBPSK_FR1 |  |  | No |  |  |  |  |  |  |  |  |
| 31a |  |  | Supports pi/2-BPSK modulation scheme for PUSCH in FR2 |  |  | 38.306, 4.2.7.10 |  |  | Rel-15 |  |  | pc_pusch_halfpiBPSK_FR2 |  |  | Yes |  |  |  |  |  |  |  |  |
| 32 |  |  | Support multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency |  |  | 38.306, 4.2.7.6 |  |  | Rel-16 |  |  | pc_multiDCI_MultiTRP_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 33 |  |  | Support receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS |  |  | 38.306, 4.2.7.2 |  |  | Rel-15 |  |  | pc_rateMatchingLTE_CRS |  |  | Yes |  |  |  |  |  |  |  |  |
| 34 |  |  | Support of BWP operation without bandwidth restriction |  |  | 38.306, 4.2.7.2 |  |  | Rel-15 |  |  | pc_bwp-WithoutRestriction |  |  | No |  |  |  |  |  |  |  |  |
| 35 |  |  | Support of receiving SCell dormancy indication on SPCell using DCI format 2_6 outside the active time |  |  | 38.306, 4.2.7.4 |  |  | Rel-16 |  |  | pc_scellDormancyOutsideActiveTime_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 36 |  |  | Supports pi/2-BPSK modulation scheme for power boosting in FR1 |  |  | 38.306, 4.2.7.2 |  |  | Rel-15 |  |  | pc_powerBoosting_pi2BPSK |  |  | No |  |  |  |  |  |  |  |  |
| 37 |  |  | Support of dynamic UL Tx switching |  |  | 38.306, 4.2.7.1 |  |  | Rel-16 |  |  | pc_ULTxSwitchingBandPair |  |  | No |  |  |  |  |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |  |  |
| 38 |  |  | Support uplink transmission power boost by suspension of in-band emission (IBE) in FR2 |  |  | 38.306 4.2.7.2 |  |  | Rel-16 |  |  | pc_mpr_PowerBoost_FR2 |  |  | No |  |  |  |  |  |  |  |  |
| 39 |  |  | Supports the alternative 64QAM MCS table for PDSCH |  |  | 38.306, 4.2.7.10 |  |  | Rel-16 |  |  | pc_dl_64qam_mcs_tableAlt |  |  | No |  |  |  |  |  |  |  |  |
| 40 |  |  | Supports the CQI table with target BLER of 10^-5 |  |  | 38.306, 4.2.7.10 |  |  | Rel-16 |  |  | pc_cqi_tableAlt |  |  | No |  |  |  |  |  |  |  |  |
| 41 |  |  | Supports of single DCI based spatial division multiplexing scheme |  |  | 38.306, 4.2.7.5 |  |  | Rel-16 |  |  | pc_singledci_sdm |  |  | No |  |  |  |  |  |  |  |  |
| 42 |  |  | Support of BWP adaptation (up to 2 BWPs) with the same numerology |  |  | 38.306, 4.2.7.2 |  |  | Rel-15 |  |  | pc_bwp_SameNumerology_upto2 |  |  | No |  |  |  |  |  |  |  |  |
| 43 |  |  | Support of BWP adaptation (up to 4 BWPs) with the same numerology |  |  | 38.306, 4.2.7.2 |  |  | Rel-15 |  |  | pc_bwp_SameNumerology_upto4 |  |  | No |  |  |  |  |  |  |  |  |
| 44 |  |  | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer |  |  | 38.306 4.2.7.2 |  |  | Rel-15 |  |  | pc_bwp_DiffNumerology |  |  | No |  |  |  |  |  |  |  |  |
| 45 |  |  | Support PUSCH repetition type B |  |  | 38.306, 4.2.7.7 |  |  | Rel-16 |  |  | pc_pusch_RepetitionTypeB_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 46 |  |  | Support of 2-Step RACH |  |  | 38.306, 4.2.7.10 |  |  | Rel-16 |  |  | pc_twoStepRACH_r16 |  |  | No |  |  |  |  |  |  |  |  |
| XX1 |  |  | Support of NR CA with NR shared spectrum channel access |  |  | 38.306, 4.2.7.2a |  |  | Rel-16 |  |  | pc_NRCASharedAccess |  |  | No |  |  |  |  |  | Deployment scenario A in Annex B.3 of TS 38.300 [21] |  |  |
| XX2 |  |  | Support of EN-DC with NR shared spectrum channel access |  |  | 38.306, 4.2.7.2a |  |  | Rel-16 |  |  | pc_ENDCNRSharedAccess |  |  | No |  |  |  |  |  | Deployment scenario B in Annex B.3 of TS 38.300 [21] |  |  |
| XX3 |  |  | Support of NR standalone shared spectrum channel access |  |  | 38.306, 4.2.7.2a |  |  | Rel-16 |  |  | pc_standaloneNRSharedAccess |  |  | No |  |  |  |  |  | Deployment scenario C in Annex B.3 of TS 38.300 [21] |  |  |
| XX4 |  |  | Support of NR shared spectrum channel access with UL in licensed band |  |  | 38.306, 4.2.7.2a |  |  | Rel-16 |  |  | pc_NRSharedAccessUlLic |  |  | No |  |  |  |  |  | Deployment scenario D in Annex B.3 of TS 38.300 [21] |  |  |
| XX5 |  |  | Support of NR-DC with NR shared spectrum channel access |  |  | 38.306, 4.2.7.2a |  |  | Rel-16 |  |  | pc_NRDCSharedAccess |  |  | No |  |  |  |  |  | Deployment scenario E in Annex B.3 of TS 38.300 [21] |  |  |

### A.4.3.2A	NR CA Physical Layer Baseline Implementation Capabilities