3GPP TSG-RAN5 Meeting #92-e	R5-214518

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5031 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NB-IoT test case 22.2.5 for TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The configuration of NB-IoT test case 22.2.5 is not applicable to TDD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the test case configruation for NB-IoT TDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 22.2.5 will not be applicable to NB-IoT TDD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

22.2.5	NB-IoT / Intra-frequency Cell reselection / Qhyst, Qoffset, Treselection and Cell-specific reselection parameters

22.2.5.1	Test Purpose (TP)

(1)

with { UE in  RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Qhyst is non-zero or its value changes in system information }

then { UE reselects the highest ranked cell taking the actual Qhyst value into account }

}

(2)

with { UE in  RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { cell reselection criteria are fulfilled during a time interval Treselection }

then { UE reselects the highest ranked cell }

}

(3)

with { UE in  RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Qoffset is non-zero or its value changes in system information }

then { UE reselects the highest ranked cell taking the actual Qoffset value into account }

}

(4)

with { the UE is in RRC_IDLE and SystemInformationBlockType4-NB contain a cell-specific Qoffset for a neighbour intra frequency cell }

ensure that {

when { the neighbour cell has lower power than the serving cell but it is higher ranked due to the cell-specific Qoffset }

then { the UE reselects the neighbour cell with cell-specific Qoffset }

}

(5)

with { the UE is in RRC_IDLE and SystemInformationBlockType4-NB contain a black listed cell }

ensure that {

when { a black listed cell becomes higher ranked than the serving cell }

then { the UE remains camped on the serving cell }

}

(6)

with { UE in  RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { SintrasearchP is non-zero in system information }

then { UE perform measurement and reselects the highest ranked cell upon Srxlev < SintrasearchP }

}

22.2.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.300, clause 10.1.1.2; TS 36.304, clauses 5.2.1, 5.2.4.1, 5.2.4.2a and 5.2.4.6; TS 36.331, clause 6.7.3.1; TS 36.133, clause 4.2.2.4. Unless otherwise stated these are Rel-13 requirements.

[TS 36.300, clause 10.1.1.2]

A UE in RRC_IDLE performs cell reselection. The principles of the procedure are the following:

-	The UE makes measurements of attributes of the serving and neighbour cells to enable the reselection process:

-	There is no need to indicate neighbouring cells in the serving cell system information to enable the UE to search and measure a cell i.e. E-UTRAN relies on the UE to detect the neighbouring cells;

-	For the search and measurement of inter-frequency neighbouring cells, only the carrier frequencies need to be indicated;

-	Measurements may be omitted if the serving cell attribute fulfils particular search or measurement criteria.

-	Cell reselection identifies the cell that the UE should camp on. It is based on cell reselection criteria which involves measurements of the serving and neighbour cells, except for NB-IoT:

…

For NB-IoT, cell reselection identifies the cell that the UE should camp on. It is based on cell reselection criteria which involve measurements of the serving and neighbour cells as follows:

-	Intra-frequency reselection is based on ranking of cells (potentially with cell specific offsets);

-	Inter-frequency reselection is based on ranking of frequencies (potentially with frequency specific offsets);

-	Blind redirection supported for load balancing.

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.4.1]

The UE shall not consider any black listed cells as candidate for cell reselection.

[TS 36.304, clause 5.2.4.2a]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NB-IoT inter-frequencies which are indicated in system information:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP, the UE may choose not to perform inter-frequency measurements.

-	Otherwise, the UE shall perform inter-frequency measurements.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise and for NB-IoT this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2 (5.2.3.2a for NB-IoT), but may exclude all CSG cells that are known by the UE not to be CSG member cells.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.7.3.1]

The IE SystemInformationBlockType3-NB contains cell re-selection information common for intra-frequency, and inter-frequency cell re-selection as well as intra-frequency cell re-selection information other than neighbouring cell related.

The IE SystemInformationBlockType4-NB contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters.

[TS 36.133, clause 4.2.2.4]

…

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in  TS 36.304 within Kcarrier * Tdetect,EUTRAN_Inter  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5dB for reselections based on ranking or 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

22.2.5.3	Test description

22.2.5.3.1	Pre-test conditions

System Simulator:

-	Ncells 1, 2, and 4 in different tracking areas according to TS 36.508 Table 8.1.4.2-6.

-	System information combination 2 as defined in TS 36.508 [18] clause 8.1.4.3.1 is used.

-	Tcell Offset for Ncell 2 and Ncell 4 is set to same value as Ncell 1.

UE:

None

Preamble:

-	UE is in state 3-NB Idle mode on Ncell 1 according to TS 36.508 [18].

22.2.5.3.2	Test procedure sequence

Table 22.2.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1", "T2", "T3", "T4", "T5", "T6", "T7", "T8", "T9", "T10" and "T11"are applied at the points indicated in the Main behaviour description in Table 22.2.5.3.2-2.

Table 22.2.5.3.2-1: Time instances of cell power level and parameter change

|  | Parameter | Unit | Ncell 1 | Ncell 2 | Ncell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific NRS EPRE (FDD) | dBm/15kHz | -91 | -85 | Off | Ncell 2 becomes stronger than Ncell 1 but Ncell 1 remains the highest ranked one due to QhystsCell1 |
|  | Cell-specific NRS EPRE (TDD) |  | -89 | -85 |  |  |
| T2 | Qhysts | dB | 0 | 0 | - | QhystsCell1 change causes Ncell 2 to become highest ranked cell |
| T3 | Cell-specific NRS EPRE (FDD) | dBm/15kHz | -85 | -91 | Off | Ncell 1 becomes the strongest and highest ranked one |
|  | Cell-specific NRS EPRE (TDD) |  | -85 | -89 |  |  |
|  | Qoffsets,n | dB | 24 | 0 | - | Qoffsets,nCell2 remains zero |
| T4 | Cell-specific NRS EPRE (FDD) | dBm/15kHz | -91 | -85 | Off | Ncell 1 becomes weaker but it remains the highest ranked one due to Qoffsets,nCell1 |
|  | Cell-specific NRS EPRE (TDD) |  | -89 | -85 |  |  |
| T5 | Qoffsets,n | dB | 0 | 0 | - | Ncell 2 becomes the highest ranked one due to Qoffsets,nCell1 change |
| T6 | Cell-specific NRS EPRE (FDD) | dBm/15kHz | -85 | -91 | Off | Ncell 1 becomes the highest ranked one |
|  | Cell-specific NRS EPRE (TDD) |  | -85 | -89 |  |  |
|  | Treselection | s | 21 | 0 | - |  |
| T7 | Cell-specific NRS EPRE (FDD) | dBm/15kHz | -91 | -85 | Off | Ncell 2 becomes the highest ranked cell |
|  | Cell-specific NRS EPRE (TDD) |  | -89 | -85 |  |  |
| T8 | Cell-specific NRS EPRE | dBm/15kHz | Off | -91 | -85 | Ncell 4 has higher power than Ncell 2 but is black listed |
| T9 | Cell-specific NRS EPRE (FDD) | dBm/15kHz | -91 | -85 | Off | Ncell 1 becomes the highest ranked one |
|  | Cell-specific NRS EPRE (TDD) |  | -89 | -85 |  |  |
|  | Treselection | s | 0 | 0 | - |  |
|  | Qoffsets,n | dB | 24 | -24 | - |  |
| T9A | Cell-specific NRS EPRE | dBm/15kHz | -91 | -94 | Off | Ncell 1 remains highest ranked cell |
| T10 | Cell-specific NRS EPRE | dBm/15kHz | -85 | Off | Off | Only Ncell 1 is on |
|  | Qoffsets,n | dB | 0 | - | - |  |
| T11 | Cell-specific NRS EPRE | dBm/15kHz | -91 | -79 | Off | Srxlev of Ncell 1 is less than SintrasearchP |
|  | SintrasearchP | dB | 22 | 22 | - |  |

Table 22.2.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 1 second to allow UE to enter RRC_IDLE state on Ncell 1. | - | - | - | - |
| 1 | SS re-adjusts the cell-specific reference signal levels according to row "T1" in table 22.2.5.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 2 within the next 10s? | --> | RRCConnectionRequest-NB | 1 | F |
| 3 | SS resets QhystsCell1 according to row "T2" in table 22.2.5.3.2-1. | - | - | - | - |
| 4 | The SS transmits a Paging-NB message on Ncell 1 to notify the UE of the system information change. The systemInfoValueTag-r13 / systemInfoValueTag-r15 in the MasterInformationBlock-NB / MasterInformationBlock-TDD-NB is increased on Ncell 1. | <-- | Paging-NB | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on Ncell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 6 | SS changes Qoffsets,nCell1 according to rows "T3" in table 22.2.5.3.2-1. | - | - | - | - |
| 7 | The SS transmits a Paging-NB message on Ncell 2 to notify the UE of the system information change. The systemInfoValueTag-r13 / systemInfoValueTag-r15 in the MasterInformationBlock-NB / MasterInformationBlock-TDD-NB is increased on Ncell 2. | <-- | Paging-NB | - | - |
| 8 | Wait for 2.1* modification period (Note 1) to allow the new system information to take effect. | - | - | - | - |
| 8A | SS re-adjusts cell-specific reference signal levels according to rows "T3" in table 22.2.5.3.2-1. | - | - | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on Ncell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 10 | Wait for 1 second to allow UE to enter RRC_IDLE state on Ncell 1. | - | - | - | - |
| 11 | SS re-adjusts cell-specific reference signal levels according to row "T4" in table 22.2.5.3.2-1. | - | - | - | - |
| 12 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 2 within the next 10s? | --> | RRCConnectionRequest-NB | 3 | F |
| 13 | SS resets Qoffsets,nCell1 according to row "T5" in table 22.2.5.3.2-1. | - | - | - | - |
| 14 | The SS transmits a Paging-NB message on Ncell 1 to notify the UE of the system information change. The systemInfoValueTag-r13 / systemInfoValueTag-r15 in the MasterInformationBlock-NB / MasterInformationBlock-TDD-NB is increased on Ncell 1. | <-- | Paging-NB | - | - |
| 15 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on Ncell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 3 | - |
| 16 | SS changes TreselectionCell1 according to rows "T6" in table 22.2.5.3.2-1. | - | - | - | - |
| 17 | The SS transmits a Paging-NB message on Ncell 2 to notify the UE of the system information change. The systemInfoValueTag-r13 / systemInfoValueTag-r15 in the MasterInformationBlock-NB / MasterInformationBlock-TDD-NB is increased on Ncell 2. | <-- | Paging-NB | - | - |
| 18 | Wait for 2.1* modification period (Note 1) to allow the new system information to take effect. | - | - | - | - |
| 18A | SS re-adjusts cell-specific reference signal levels according to rows "T6" in table 22.2.5.3.2-1. | - | - | - | - |
| 19 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on Ncell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 20 | Wait for 1 second to allow UE to enter RRC_IDLE state on Ncell 1. | - | - | - | - |
| 21 | SS re-adjusts cell-specific reference signal levels according to rows "T7" in table 22.2.5.3.2-1. | - | - | - | - |
| 22 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 2 within the next 20s? | --> | RRCConnectionRequest-NB | 2 | F |
| 23 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 2 within the next 26s? | --> | RRCConnectionRequest-NB | 2 | P |
| 24 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 8.1.5A.5 are performed on Ncell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 25 | The SS re-adjusts the cell-specific reference signal levels according to row "T8" in table 22.2.5.3.2-1. | - | - | - | - |
| 26 | Check: Does the UE initiate a random access procedure on Ncell 4 within the next 120s? | - | - | 5 | F |
| 27 | SS changes TreselectionCell1 according to rows "T9" in table 22.2.5.3.2-1. | - | - | - | - |
| 28 | The SS transmits a Paging-NB message on Ncell 2 to notify the UE of the system information change. The systemInfoValueTag-r13 / systemInfoValueTag-r15 in the MasterInformationBlock-NB / MasterInformationBlock-TDD-NB is increased on Ncell 2. | <-- | Paging-NB | - | - |
| 29 | Wait for 2.1* modification period (Note 1) to allow the new system information to take effect. | - | - | - | - |
| 29A | SS re-adjusts cell-specific reference signal levels according to rows "T9" in table 22.2.5.3.2-1. | - | - | - | - |
| 30 | Steps 1 to 5 of the generic test procedure in TS 36.508 subclause 8.1.5A.5 are performed on Ncell 1 | - | - | 4 | - |
| 30A | SS re-adjusts cell-specific reference signal levels according to rows "T9A" in table 22.2.5.3.2-1. | - | - | - | - |
| 30B | SS releases the RRC connection | <-- | RRCConnectionRelease-NB | - | - |
| 31 | SS resets QhystsCell1 according to row "T10" in table 22.2.5.3.2-1. | - | - | - | - |
| 32 | The SS transmits a Paging-NB message on Ncell 1 to notify the UE of the system information change. The systemInfoValueTag-r13 / systemInfoValueTag-r15 in the MasterInformationBlock-NB / MasterInformationBlock-TDD-NB is increased on Ncell 1. | <-- | Paging-NB | - | - |
| 33 | Wait for 2.1* modification period (Note 1) to allow the new system information to take effect. | - | - | - | - |
| 33A | The SS changes the SintrasearchP for Ncell 1 and N cell2 according to row “T11” in table 22.2.5.3.2-1 |  |  |  |  |
| 33B | The SS transmits a Paging-NB message on Ncell 1 to notify the UE of the system information change. The systemInfoValueTag-r13 / systemInfoValueTag-r15 in the MasterInformationBlock-NB / MasterInformationBlock-TDD-NB is increased on Ncell 1. | <-- | Paging-NB |  |  |
| 33C | Wait for 2.1* modification period (Note 1) to allow the new system information to take effect. | - | - | - | - |
| 34 | The SS re-adjusts the cell-specific reference signal levels according to row "T11" in table 22.2.5.3.2-1. | - | - | - | - |
| 35 | Check: Does the test result of generic procedure 8.1.5A.5 indicate that the UE is camped on Ncell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 6 | P |
| 36-40 | void | - | - | - | - |
| NOTE 1: The wait time of 2.1* modification period in step 8,step 18, step 29, step 33 and step 33C is to allow for the network to paging the system information change during the next modification period, and update the system information at the subsequent modification period. UE should acquire the updated system information within 90ms of the start of modification period. |  |  |  |  |  |

22.2.5.3.3	Specific message contents

Table 22.2.5.3.3-1: MasterInformationBlock-NB for Ncell 1 (preamble and all steps, Table 22.2.5.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.2-1 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |
| systemInfoValueTag-r13 | The value is increased by 1 in step 4, step 7, step 14, step 17, step 28 and step 32, step 33B |  |
| } |  |  |

Table 22.2.5.3.3-1A: MasterInformationBlock-TDD-NB for Ncell 1 (preamble and all steps, Table 22.2.5.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.2-1A |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| MasterInformationBlock-TDD-NB ::= SEQUENCE { |  |  |
| systemInfoValueTag-r15 | The value is increased by 1 in step 4, step 7, step 14, step 17, step 28 and step 32, step 33B |  |
| } |  |  |

Table 22.2.5.3.3-2: SystemInformationBlockType3-NB for Ncell 1 (preamble, table 22.2.5.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon-r13 SEQUENCE { |  |  |  |
| q-Hyst-r13 |  | dB24 | QhystsCell1 |
| } |  |  |  |
| } |  |  |  |

Table 22.2.5.3.3-3: SystemInformationBlockType4-NB for Ncell 2 (all steps, Table 22.2.5.3.2-2)

| Derivation path: 36.508 table 8.1.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element | Value/Remark | Comment |
| SystemInformationBlockType4-NB-r13 ::= SEQUENCE { |  |  |
| IntraFreqBlackCellList-r13 SEQUENCE { | 1 entry |  |
| start[1] | PhysicalCellID of Ncell 4 |  |
| range[1] | Not present |  |
| } |  |  |
| } |  |  |

Table 22.2.5.3.3-4: SystemInformationBlockType3-NB for Ncell 1 (step 4, table 22.2.5.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon-r13 SEQUENCE { |  |  |  |
| q-Hyst-r13 |  | dB0 | QhystsCell1 |
| } |  |  |  |
| } |  |  |  |

Table 22.2.5.3.3-5: SystemInformationBlockType4-NB for Ncell 1 (step 7, table 22.2.5.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType4-NB-r13 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList-r13 SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |
| physCellId [1] |  | Physical cell identity of Ncell 2 |  |
| q-OffsetCell [1] |  | dB24 | Qoffsets,nCell1 |
| } |  |  |  |
| } |  |  |  |

Table 22.2.5.3.3-6: SystemInformationBlockType4-NB for Ncell 1 (step 14, table 22.2.5.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType4-NB-r13 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |
| physCellId [1] |  | Physical cell identity of Ncell 2 |  |
| q-OffsetCell [1] |  | dB0 | Qoffsets,nCell1 |
| } |  |  |  |
| } |  |  |  |

Table 22.2.5.3.3-7: SystemInformationBlockType3-NB for Ncell 1 (step 17, table 22.2.5.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon-r13 SEQUENCE { |  |  |  |
| q-Hyst-r13 |  | dB0 | QhystsCell1 |
| } |  |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |  |
| t-Reselection-r13 |  | s21 | seconds |
| } |  |  |  |
| } |  |  |  |

Table 22.2.5.3.3-8: MasterInformationBlock-NB for Ncell 2 (step 28 and step 32, Table 22.2.5.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.2-1 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |
| systemInfoValueTag-r13 | The value is increased by 1 |  |
| } |  |  |

Table 22.2.5.3.3-8A: MasterInformationBlock-TDD-NB for Ncell 2 (step 28 and step 32, Table 22.2.5.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.2-1A |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| MasterInformationBlock-TDD-NB ::= SEQUENCE { |  |  |
| systemInfoValueTag-r15 | The value is increased by 1 |  |
| } |  |  |

Table 22.2.5.3.3-9: SystemInformationBlockType4-NB for Ncell 1 (step 28, Table 22.2.5.3.2-2)

| Derivation path: 36.508 table 8.1.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element | Value/Remark | Comment |
| SystemInformationBlockType4-NB-r13 ::= SEQUENCE { |  |  |
| IntraFreqNeighCellList-r13 { | 1 entry |  |
| physCellId[1] | PhysicalCellID of Cell 2 |  |
| q-OffsetCell[1] | dB24 |  |
| } |  |  |
| } |  |  |

Table 22.2.5.3.3-10: SystemInformationBlockType4-NB for Ncell 2 (step 28, Table 22.2.5.3.2-2)

| Derivation path: 36.508 table 8.1.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element | Value/Remark | Comment |
| SystemInformationBlockType4-NB-r13 ::= SEQUENCE { |  |  |
| intraFreqNeighCellList-r13 SEQUENCE { | 1 entry |  |
| physCellId[1] | PhysicalCellID of Ncell 1 |  |
| q-OffsetCell[1] | dB-24 |  |
| } |  |  |
| } |  |  |

Table 22.2.5.3.3-11: SystemInformationBlockType3-NB for Ncells 1 (step 28, table 22.2.5.3.2-2)

| Derivation path: 36.508 table 8.1.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |
| t-Reselection-r13 | s0 |  |
| } |  |  |
| } |  |  |

Table 22.2.5.3.3-12: SystemInformationBlockType4-NB for Ncell 1 (step 32, Table 22.2.5.3.2-2)

| Derivation path: 36.508 table 8.1.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element | Value/Remark | Comment |
| SystemInformationBlockType4-NB-r13 ::= SEQUENCE { |  |  |
| IntraFreqNeighCellList-r13 { | 1 entry |  |
| physCellId[1] | PhysicalCellID of Ncell 2 |  |
| q-OffsetCell[1] | dB0 |  |
| } |  |  |
| } |  |  |

Table 22.2.5.3.3-13: SystemInformationBlockType4-NB for Ncell 2 (step 32, Table 22.2.5.3.2-2)

| Derivation path: 36.508 table 8.1.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element | Value/Remark | Comment |
| SystemInformationBlockType4-NB-r13 ::= SEQUENCE { |  |  |
| intraFreqNeighCellList-r13 SEQUENCE { | 1 entry |  |
| physCellId[1] | PhysicalCellID of Ncell 1 |  |
| q-OffsetCell[1] | dB0 |  |
| } |  |  |
| } |  |  |

Table 22.2.5.3.3-14: SystemInformationBlockType3-NB for Ncells 1and 2 (step 33A, table 22.2.5.3.2-2)

| Derivation path: 36.508 table 8.1.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |
| s-IntraSearchP-r13 | 11 (22 dB) |  |
| } |  |  |
| } |  |  |

Table 22.2.5.3.3-14 to 17: Void