3GPP TSG-RAN5 Meeting #92-e	R5-214537

# Electronic Meeting, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5033 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on TC 8.2.4.30.1 intra-frequency DAPS handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the updated WP, TC 8.2.4.30.1 is limited to intra-frequency scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Title change in 8.2.4.30.1. Update TP in 8.2.4.30.1.1. Delete TS 36.300 requirements in 8.2.4.30.1.2. Add uplink data switching requirements reference in 8.2.4.30.1.2 for step 6. Update pre test condition in 8.2.4.30.1.3.1. Format update in 8.2.4.30.1.2 and 8.2.4.30.1.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 8.2.4.30.1 will be uncompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.30.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | . |  |  |  |  |  |  |  |  |

<Start of Change>

8.2.4.30	RRC connection reconfiguration / Handover / DAPS Handover

8.2.4.30.1	RRC connection reconfiguration / Handover / DAPS Handover / Success / Intra-Frequency

8.2.4.30.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover }

ensure that {

when { UE receives an RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

then { UE performs DAPS handover to the target cell. For the DRB configured daps-HO, the UE continues to receive downlink data from both source and target eNBs for the DL and the UE transmits UL data to the source eNB }

}

(2)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when {  UE has performed random access procedure to the target cell successfully }

then { UE transmits an RRCReconfigurationComplete message and  the UE switches its UL data transmission to the target eNB }

}

(3)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { UE receives an RRCConnectionReconfiguration message does not include the mobilityControlInfo and includes daps-SourceRelease }

then { UE stops to receive downlink data from source eNB and releases the radio link with source cell }

}

8.2.4.30.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.3.5.3 and 5.3.5.4, TS 36.323 clause 5.12. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if the received RRCConnectionReconfiguration includes the daps-SourceRelease:

2>	reset source MCG MAC and release the source MCG MAC configuration;

2>	for each DAPS bearer:

3>	re-establish the RLC entity or entities for the source PCell;

3>	release the RLC entity or entities and the associated DTCH logical channel for the source PCell;

3>	reconfigure the PDCP entity to release DAPS, as specified in TS 36.323 [8];

2>	for each SRB:

3>	release the PDCP entity for the source PCell;

3>	release the RLC entity and the associated DCCH logical channel for the source PCell;

2>	release the physical channel configuration for the source PCell;

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if daps-HO is configured for any DRB:

2>	establish a MAC entity for the target PCell, with the same configuration as the MAC entity for the source PCell;

2>	for each DRB configured with daps-HO:

3>	establish the RLC entity or entities and the associated DTCH logical channel for the target PCell, with the same configurations as for the source PCell;

3>	reconfigure the PDCP entity to configure DAPS as specified in TS36.323 [8].

2>	for each DRB not configured with daps-HO:

3>	re-establish PDCP;

3>	re-establish the RLC entity and associate it, and the associated DTCH logical channel, to the target PCell;

2>	for each SRB:

3>	establish a PDCP entity for the target PCell, with the same configuration as the PDCP entity for the source PCell;

3>	establish an RLC entity and an associated DCCH logical channel for the target PCell, with the same configuration as for the source PCell;

2>	suspend the SRBs for the source PCell;

NOTE 1c:	In order to understand if a daps-HO is configured, the UE needs to check the presence of the field daps-HO within the received RadioResourceConfigDedicated IE.

[TS 36.323, clause 5.12]

For DAPS bearers, when upper layers request uplink data switching, the transmitting PDCP entity shall:

-	for DRBs mapped on RLC AM, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by the RLC entity associated with the source cell, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to uplink data switching to the RLC entity associated with the target cell as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.5.4;

-	perform ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.6;

-	submit the resulting PDCP Data PDU to lower layer.

-	for DRBs mapped on RLC UM, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, perform transmission of PDCP SDU in ascending order of the COUNT values to the RLC entity associated with the target cell as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.5.4;

-	perform ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.6;

-	submit the resulting PDCP Data PDU to lower layer.

8.2.4.30.1.3	Test description

8.2.4.30.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the serving cell, Cell 2 is the intra-ferquency neighbour cell of Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

8.2.4.30.1.3.2	Test procedure sequence

Table 8.2.4.30.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant in Cell 1. | - | - | - | - |
| 2 | The SS sends one IP Packet to the UE on the DRB#1. | <-- |  | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo containing rach-ConfigDedicated to order UE to perform non-contention based DAPS handover to Cell 2. DRB #1 is configured as DAPS bearer. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: During the execution from Step4 to 6, parallel behaviour defined in Table 8.2.4.30.1.3.2-2 is executed repeatedly. |  |  |  |  |
| 4 | The SS allocates uplink grant in Cell 1, allowing the UE to return the IP packet received at step 2 and continues to respond to scheduling requests from UE. | <-- | (UL Grant) | - | - |
| 5 | Check: Does the UE loop back the IP packet received at step 2 in the Cell 1? | - | - | 1 | P |
| 6 | The SS sends one IP Packet to the UE via DRB#1 in the Cell 1 and stops sending RLC acknowledgements for the waiting time period (Note 1) | - | - | - | - |
| 7 | The SS transmit Random Access Response to respond to the latest preamble on Cell 2. | <- | Random Access Response |  |  |
| 8 | The SS configures UL grant for the UE in Cell 2 and continues to respond to scheduling requests from UE. |  | (UL Grant) | - | - |
| - | EXCEPTION: Steps 9 and 10 can occur in any order. | - | - | - | - |
| 9 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message in Cell 2? |  | RRCConnectionReconfigurationComplete | 1,2 | P |
| 10 | Check: Does the UE loop back the IP packet received at step 6 in the Cell 2? | - | - | 2 | P |
| 11 | The SS transmits an RRCConnectionReconfiguration message not including mobilityControlInfo and including daps-SourceRelease in Cell 2. |  | RRCConnectionReconfiguration | - | - |
| 12 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message in Cell 2? |  | RRCConnectionReconfigurationComplete | 3 | P |
| 13 | the SS transmits one IP Packet to verify data path on DRB#1 on Cell 1. | - | - | - | - |
| 14 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 1? | - | - | 3 | F |
| 15 | the SS transmits one IP Packet to verify data path on DRB#1 on Cell 2. | - | - | - | - |
| 16 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 2? | - | - | 1 | P |
| Note 1: The waiting timer starts after SS sending RRCConnectionReconfiguration message in step 3. The waiting time period should not exceed the value of T304. The default value of T304 is set to 1000ms currently as specified in table 4.6.5-1 in [18]. |  |  |  |  |  |

<End of Change>