3GPP TSG-RAN5 Meeting #92-e	R5-214597

# Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

Title: 	Discussion on the removal of IMSo5G test case 6.5 from 34.229-5

Source: 	Keysight Technologies UK

Agenda Item:	6.3.2.20

Document for:	Endorsement

- 1) Introduction

The test case 6.5 in TS 34.229-5 requires the UICC to send a REFRESH command to the UE indicating that contents of ISIM has been updated. However there does not seem to be any standard AT command to trigger this action. This is currently done via MMI command in TTCN.

The aim of the document is to discuss the possible options to perform this action and if there are no options available then to decide is this test case can be removed from 34.229-5.

- 2) Discussion

As stated in the introduction section, there does not seem to be any standard AT command available to perform the REFRESH command required in the test case. Based on the discussion we had with UE vendors, we haven’t come across any other option either e.g., via a UE application to perform this action.

Proposal 2.1: To take inputs from UE vendors for the options to perform REFRESH command.

One of the options available to trigger this action is via a SIM applet but then this would be something which is not expected to be done as part of protocol testing for 34.229-5 or 38.523-1 for that matter. And it would be quite an effort just for one test case to do a SIM applet development to perform this action. If that is the only option available, then this test case should be removed from TS 34.229-5.

Proposal 2.2: If using SIM applet is the only option available to perform REFRESH command then test case 6.5 should be removed from 34.229-5.

To test the test purposes of this test case, it could be considered to move this test to some other test specification if it is feasible, USAT conformance specification, TS 31.124, could be an option.

Proposal 2.3: Based on proposals 2.1 and 2.2, if it is agreed to remove 6.5 from TS 34.229-5 then send an LS to CT1 to consider testing the test purposes of 6.5 as part of TS 31.124, if it is feasible.

- 3) Proposals

As discussed in section 2, it is requested to consider the following proposals

Proposal 2.1: To take inputs from UE vendors for the options to perform REFRESH command.

Proposal 2.2: If using SIM applet is the only option available to perform REFRESH command then test case 6.5 should be removed from 34.229-5.

Proposal 2.3: Based on proposals 2.1 and 2.2, if it is agreed to remove 6.5 from TS 34.229-5 then send an LS to CT1 to consider testing the test purposes of 6.5 as part of TS 31.124, if it is feasible.

- 4) References

[1] 	TS 27.007		AT command set for User Equipment (UE)

[2]    TS 31.124		Mobile Equipment (ME) conformance test specification; Universal Subscriber Identity

Module Application Toolkit (USAT) conformance test specification