3GPP TSG-RAN5 Meeting #92-e	R5-214716

# Electronic Meeting, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5325 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Added subPRB allocation to cat M1 test cases, 6.2.4EA, 6.6.2.2EA and 6.6.3EA.3 for NS_07 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test points for subPRB allocation is missing in cat M1 test cases, 6.2.4EA, 6.6.2.2EA and 6.6.3EA.3 for NS_07 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test points for subPRB allocation in cat M1 test cases, 6.2.4EA, 6.6.2.2EA and 6.6.3EA.3 for NS_07 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | efeMTC WI cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4EA, 6.6.2.2EA, 6.6.3EA. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis in CR 0242 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

### 6.2.4EA	Additional Maximum Power Reduction (A-MPR) for UE category M1

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- NS_04 (B41), NS_12 (B26): Minimum requirements in 36.101 are incomplete. Test configuration table and test requirements are missing

#### 6.2.4EA.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2 EA -1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

#### 6.2.4EA.2	Test applicability

The requirements of this test apply in test case 6.6.2.2EA Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06,NS_07 and NS-35 to all types of E-UTRA UE release 13 and forward of UE category M1.

The requirements of this test apply in test case 6.6.3EA.3 Additional Spurious Emissions for network signalled values NS_05, NS_07, NS_08, NS_09, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_22, NS_23, NS_38 and NS_39 to all types of E-UTRA UE release 13 and forward of UE category M1.

NOTE:	As a result TC 6.2.4EA has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4EA.3	Minimum conformance requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2.2EA-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2.4EA-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4EA-1 and from 6.2.4EA-2 to 6.2.4EA-3 are in addition to the allowed MPR requirements specified in subclause 6.2.3EA.

Table 6.2.4EA-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value |  | Requirements (subclause) |  | E-UTRA Band |  | Resources Blocks (NRB) |  |  |  | A-MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NS_01 |  | 6.6.2.1.1 |  | Table 5.2-1 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_03 |  | 6.6.2.2.1 |  | 2, 4 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_04 |  | 6.6.2.2.2 |  | 41 |  | [TBD] |  |  |  |  | [TBD] |  |  |
| NS_05 |  | 6.6.3.3.3.2 |  | 1 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_06 |  | 6.6.2.2.3 |  | 12, 13, 14 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_07 |  | 6.6.2.2.3 6.6.3.3.3.3 |  | 13 |  | Table 6.2.4-2E |  |  |  |  |  |  |  |
| NS_08 |  | 6.6.3.3.3.4 |  | 19 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_09 |  | 6.6.3.3.3.5 |  | 21 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_10 |  |  |  | 20 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_12 |  | 6.6.3.3.3.7 |  | 26 |  | [TBD] |  |  |  |  |  |  |  |
| NS_13 |  | 6.6.3.3.3.8 |  | 26 |  | Table 5.4.2-1 |  |  |  |  |  | N/A |  |
| NS_14 |  | 6.6.3.3.3.9 |  | 26 |  | Table 5.4.2-1 |  |  |  |  |  | N/A |  |
| NS_15 |  | 6.6.3.3.3.10 |  | 26 |  | Table 6.2.4-9 |  |  |  |  |  |  |  |
| NS_16 |  | 6.6.3.3.3.11 |  | 27 |  | Table 5.4.2-1 |  |  |  |  | N/A |  |  |
| NS_17 |  | 6.6.3.3.3.12 |  | 28 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_18 |  | 6.6.3.3.3.13 |  | 28 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_22 |  | 6.6.3.3.3.17 |  | 42, 43 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_23 |  | 6.6.3.3.3.18 |  | 42, 43 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_32 |  | - |  | - |  | - |  |  |  | - |  |  |  |
| NS_35 |  | 6.6.2.2.3.5 |  | 71 |  | Table 5.4.2-1 |  |  |  |  | N/A |  |  |
| NS_38 |  | 6.6.3.3.3.27 |  | 74 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |
| NS_39 |  | 6.6.3.3.3.28 |  | 74 |  | Table 5.4.2-1 |  |  |  | N/A |  |  |  |

No other A-MPR requirement than those specified in tables 6.2.4EA-1 applies to category M1 UE.

…

Table 6.2.4EA-2: A-MPR for "NS_07“ for Cat-M1

| BW [MHz] | 5 |  |  |  | 10 |  |
| --- | --- | --- | --- | --- | --- | --- |
| (NBindex,RBstart) | (0,<6) | (0,<6) | (3,<6) | (3,<6) | (0,<6) | (7,<6) |
| LCRB | >4 and <7 | >1 and ≤4 | >4 and <7 | >1 and ≤4 | >2and <7 | >2and <7 |
| AMPR [dB] | 2 | 1 | 2 | 1 | 1 | 1 |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [8]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [9]. |  |  |  |  |  |  |

Table 6.2.4EA-3: A-MPR for "NS_15" for E-UTRA highest channel edge > 845 MHz and ≤ 849 MHz

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 1.4 | RBend [RB] |  |  | 4-5 |
|  | A-MPR [dB] |  |  | ≤3 |
| 3 | RBend [RB] | 0-1 | 8-12 | 13-14 |
|  | LCRB [RB] | ≤2 | ≥8 | >0 |
|  | A-MPR [dB] | ≤4 | ≤4 | ≤9 |
| 5 | RBend [RB] | 0-4 | 12-19 | 20-24 |
|  | LCRB [RB] | ≤2 | ≥8 | >0 |
|  | A-MPR [dB] | ≤4 | ≤5 | ≤9 |
| 10 | RBend [RB] | 0-12 | 23-36 | 37-49 |
|  | LCRB [RB] | ≤2 | ≥15 | >0 |
|  | A-MPR [dB] | ≤4 | ≤6 | ≤9 |
| 15 | RBend [RB] | 0-20 | 26-53 | 54-74 |
|  | LCRB [RB] | ≤2 | ≥20 | >0 |
|  | A-MPR [dB] | ≤4 | ≤5 | ≤9 |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2.4A-4 for category M1 UE in addition to the allowed MPR requirements specified in subclause 6.2.3EA.

Table 6.2.4EA-4: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4 | Table 6.2.4E-8 |
| NS_04 | 6.6.2.2.2 | 41 | Table 6.2.4E-9 |
| NS_05 | 6.6.3.3.1 | 1 | [N/A] |
| NS_06 | 6.6.2.2.3 | 12, 13, 14 | Table 6.2.4E-13 |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | Table 6.2.4E-23 |
| NS_08 | 6.6.3.3.3 | 19 | [N/A] |
| NS_09 | 6.6.3.3.4 | 21, 74 | [N/A] |
| NS_10 |  | 20 | [N/A] |
| NS_12 | 6.6.3.3.5 | 26 | Table 6.2.4E-14 |
| NS_13 | 6.6.3.3.6 | 26 | [N/A] |
| NS_14 | 6.6.3.3.7 | 26 | [N/A] |
| NS_15 | 6.6.3.3.8 | 26 | [N/A] |
| NS_16 | 6.6.3.3.9 | 27 | [N/A] |
| NS_17 | 6.6.3.3.10 | 28 | [N/A] |
| NS_18 | 6.6.3.3.11 | 28 | [N/A] |
| NS_22 | 6.6.3.3.16 | 42, 43 | [N/A] |
| NS_23 | 6.6.3.3.17 | 42, 43 | [N/A] |
| NS_32 | - | - | - |
| NS_35 | 6.6.2.2.7 | 71 | Table 6.2.4E-15 |
| NS_38 | 6.6.3.3.29 | 74 | Table 6.2.4E-16 |
| NS_39 | 6.6.3.3.30 | 74 | [N/A] |

Table 6.2.4EA-5: A-MPR for "NS_03" for Cat-M1 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |  |  |
| --- | --- | --- | --- | --- |
| (NBindex,SCstart) | (0, ≤9) | (0, ≥63) | (0, ≤9) | (0, ≥63) |
| Lcsc | 2 | 2 | 3,6 | 3,6 |
| AMPR [dB] | ≤[0.5] | ≤[0.5] | ≤[1] | ≤[1] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [8]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [9] NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |

Table 6.2.4EA-6: A-MPR for "NS_04" for Cat-M1 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |
| --- | --- | --- |
| (NBindex,SCstart) | (0, ≤9) | (0, ≤9) |
| Lcsc | 2 | 3,6 |
| AMPR [dB] | ≤[1] | ≤[1.5] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [8]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [9]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |

Table 6.2.4EA-7: A-MPR for "NS_06" for Cat-M1 with sub-PRB allocation

| BW [MHz] | 1.4 | 3 | 5 |
| --- | --- | --- | --- |
| (NBindex,SCstart) | (0, < 15) and (0, > 54) | ([0], < 15) | ([0], < 15) |
| Lcsc | [2] | [2] | [2] |
| AMPR [dB] | ≤[0.5] | ≤[0.5] | ≤[0.5] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [8]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [9]. NOTE2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |

Table 6.2.4EA-8: A-MPR for "NS_12" for Cat-M1 with sub-PRB allocation

| Channel bandwidth [MHz] | Parameters | Region |  |  |
| --- | --- | --- | --- | --- |
| 1.4 | (NBindex,SCstart ) | ([0], ≤ [3]) | ([0], ≤ [3]) | ([0], ≤ [3]) |
|  | Lcsc | [2] | [3] | [6] |
|  | A-MPR [dB] | ≤ [5] | ≤ [5] | ≤ [5] |
| 3 | (NBindex,SCstart) | ([0], ≤ [3]) | ([0], ≤ [3]) | ([0], ≤ [3]) |
|  | Lcsc | [2] | [3] | [6] |
|  | A-MPR [dB] | ≤ [5] | ≤ [5] | ≤ [5] |
| 5 | (NBindex,SCstart) | ([0], ≤ [3]) | ([0], ≤ [3]) | ([0], ≤ [3]) |
|  | Lcsc | [2] | [3] | [6] |
|  | A-MPR [dB] | ≤ [5] | ≤ [5] | ≤ [5] |

Table 6.2.4E-9: A-MPR for "NS_35" for Cat-M1 with sub-PRB allocation

| BW [MHz] | 5 |
| --- | --- |
| (NBindex,SCstart) | ([0], [0]) |
| Lcsc | [2] |
| AMPR [dB] | ≤ [0.5] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [8]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [9]. NOTE2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |

Table 6.2.4EA-10: A-MPR for "NS_38" for Cat-M1 with sub-PRB allocation for E-UTRA lowest channel edge > 1427 MHz and ≤ 1447 MHz

| Channel bandwidth [MHz] | Parameters | Region |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | (NBindex,SCstart) | ([0], < 27) | ([0], > 51) |  |  |  |  |
|  | Lcsc | 2,3,6 | [3] |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |  |
| 3 | (NBindex,SCstart) | ([0], < 21) | ([0], > 60) |  |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |  |
| 5 | (NBindex,SCstart) | ([0], < 21) | ([0], > 60) |  |  |  |  |
|  | Lcsc | [2] | [2] |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |  |
| 10 | (NBindex,SCstart) | (0, < 15) |  |  |  |  |  |
|  | Lcsc | 2,3,6 |  |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] |  |  |  |  |  |
| 15 | (NBindex,SCstart) | (0, < 12) |  |  |  |  |  |
|  | Lcsc | 2,3,6 |  |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] |  |  |  |  |  |

Table 6.2.4EA-11: A-MPR for “NS_07” for Cat-M1 with sub-PRB allocation

| BW [MHz] | 10 |
| --- | --- |
| (NBindex,SCstart) | (0, < 9) |
| Lcsc | 2 |
| AMPR [dB] | ≤ 0.5 |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [8]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [9]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |

The normative reference for this requirement is TS 36.101 [2] subclause 6.2.4EA and 6.2.4.

#### 6.2.4EA.4	Test description

##### 6.2.4EA.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.2.4EA.4.1-1 to 6.2.4EA.4.1-20. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2.4EA.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 0 |
| 8 | 3MHz |  | QPSK | 6 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 5 | 0 |
| 11 | 5MHz |  | QPSK | 6 | 0 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 13 | 5MHz |  | 16QAM | 3 | 0 |
| 14 | 5MHz |  | 16QAM | 5 | 0 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 16 | 10MHz |  | QPSK | 6 | 0 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 18 | 10MHz |  | 16QAM | 5 | 0 |
| 19 | 15MHz |  | QPSK | 6 | 0 |
| 20 | 15MHz |  | 16QAM | 5 | 0 |
| 21 | 20MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 1 |
| 8 | 3MHz |  | QPSK | 6 | 1 |
| 9 | 3MHz |  | 16QAM | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 5 | 1 |
| 11 | 5MHz |  | QPSK | 6 | 3 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 13 | 5MHz |  | 16QAM | 3 | 3 |
| 14 | 5MHz |  | 16QAM | 5 | 3 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 16 | 10MHz |  | QPSK | 6 | 7 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 18 | 10MHz |  | 16QAM | 5 | 7 |
| 19 | 15MHz |  | QPSK | 6 | 11 |
| 20 | 15MHz |  | 16QAM | 5 | 11 |
| 21 | 20MHz |  | 16QAM | 5 | 15 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. Note 4: Only for power class 3 UE. |  |  |  |  |  |

Table 6.2.4EA.4.1-2: Test Configuration Table (network signalled value "NS_04")

TBD

Table 6.2.4EA.4.1-3: Test Configuration Table (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EA.4.1-4: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 0 |
| 8 | 3MHz |  | QPSK | 6 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 5 | 0 |
| 11 | 5MHz |  | QPSK | 6 | 0 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 13 | 5MHz |  | 16QAM | 3 | 0 |
| 14 | 5MHz |  | 16QAM | 5 | 0 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 16 | 10MHz |  | QPSK | 6 | 0 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 18 | 10MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 1 |
| 8 | 3MHz |  | QPSK | 6 | 1 |
| 9 | 3MHz |  | 16QAM | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 5 | 1 |
| 11 | 5MHz |  | QPSK | 6 | 3 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 13 | 5MHz |  | 16QAM | 3 | 3 |
| 14 | 5MHz |  | 16QAM | 5 | 3 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 16 | 10MHz |  | QPSK | 6 | 7 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 18 | 10MHz |  | 16QAM | 5 | 7 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. Note 4: Only for power class 3 UE. |  |  |  |  |  |

Table 6.2.4EA.4.1-5: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 10MHz |  | QPSK | 1 | 0 |
| 2 | 10MHz |  | QPSK | 4 | 0 |
| 3 | 10MHz |  | QPSK | 6 | 0 |
| 4 | 10MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 10MHz |  | QPSK | 1 | 7 |
| 2 | 10MHz |  | QPSK | 4 | 7 |
| 3 | 10MHz |  | QPSK | 6 | 7 |
| 4 | 10MHz |  | 16QAM | 5 | 7 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EA.4.1-5a: Test Configuration Table, subPRB allocation (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |  |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) | SCstart |  |
| Low range, Mid range |  |  |  |  |  |  |  |
| 1 | 10MHz |  | π/2-BPSK | ¼ | 0 |  | 0 |
| High range |  |  |  |  |  |  |  |
| 1 | 10MHz |  | π/2-BPSK | ¼ | 0 |  | 8 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |  |  |

Table 6.2.4EA.4.1-6: Test Configuration Table (network signalled value "NS_08")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EA.4.1-7: Test Configuration Table (network signalled value "NS_09")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EA.4.1-8: Test Configuration Table (network signalled value "NS_10")

Void, not tested

Table 6.2.4EA.4.1-9: Test Configuration Table (network signalled value "NS_12")

TBD

Table 6.2.4EA.4.1-10: Test Configuration Table (network signalled value "NS_13")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | UL 819.7 MHz (NUL = 26747) |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 1.4MHz |  | QPSK | 1 | 0 |
| 2 | 1.4MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EA.4.1-11: Test Configuration Table (network signalled value "NS_14")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | UL 829 MHz (NUL = 26840) |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 10MHz |  | QPSK | 1 | 0 |
| 2 | 10MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EA.4.1-12: Test Configuration Table (network signalled value "NS_15")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Configuration ID 1-2: UL 844.3 MHz Configuration ID 3-8: High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 1.4MHz, 3MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 1.4MHz |  | QPSK | 1@0, 1@5 | 0 |
| 2 | 1.4MHz |  | QPSK | 6 | 0 |
| 3 | 1.4MHz |  | QPSK | 1@5 | 0 |
| 4 | 1.4MHz |  | QPSK | 6 | 0 |
| 5 | 3MHz |  | QPSK | 1@1 | 1 |
| 6 | 3MHz |  | QPSK | 1@5 | 1 |
| 7 | 3MHz |  | QPSK | 6 | 1 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 |  |  |  |  |  |

Table 6.2.4EA.4.1-13: Test Configuration Table (network signalled value "NS_16")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 1.4MHz |  | QPSK | 1 | 0 |
| 2 | 1.4MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EA.4.1-14: Test Configuration Table (network signalled value “NS_17”)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 720.5MHz |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EA.4.1-15: Test Configuration Table (network signalled value "NS_18")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EA.4.1-16: Test Configuration Table (network signalled value "NS_19")

FFS

Table 6.2.4EA.4.1-17: Test Configuration Table (network signalled value "NS_20")

FFS

Table 6.2.4EA.4.1-18: Test Configuration Table (network signalled value "NS_21")

FFS

Table 6.2.4EA.4.1-19: Test Configuration Table (network signalled value "NS_22")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5 MHz |  | QPSK | 1 | 0 |  |
| 2 | 5 MHz |  | QPSK | 6 | 0 |  |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS 36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |  |

Table 6.2.4EA.4.1-20: Test Configuration Table (network signalled value "NS_23")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5 MHz |  | QPSK | 1 | 0 |  |
| 2 | 5 MHz |  | QPSK | 6 | 0 |  |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS 36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.4EA.4.1-1 to Table 6.2.4EA.4.1-20.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.2.4EA.4.3.

##### 6.2.4EA.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via M-PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.4EA.4.1-1 to Table 6.2.4EA.4.1-20. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	For UE supporting subPRB allocation, repeat step 1-3 with UL RMC according to applicable Table from Table 6.2.4EA.4.1-1a to Table 6.2.4EA.4.1-20a.

##### 6.2.4EA.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and with the same exceptions as in clauses 6.2.4.4.3, for each network signalled value.

#### 6.2.4EA.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4EA.5-1 to Table 6.2.4EA.5-22. The allowed A-MPR values specified in Table 6.2.4EA-1 are in addition to the allowed MPR requirements specified in clause 6.2.3 EA. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5 EA.3-1 and 6.2.5EA.3-2 apply.

Table 6.2.4EA.5-1: UE Power Class test requirements (network signalled value "NS_03")
(for Band 2 Low/High range with TC,c=1.5 dB)

| Config ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 22.7 | 13.8 |
| 2 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 22.7 | 11.8 |
| 3 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.7 | 10.8 |
| 4 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 22.7 | 11.8 |
| 5 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.7 | 10.8 |
| 6 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 22.7 | 13.8 |
| 7 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 22.7 | 11.8 |
| 8 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.7 | 10.8 |
| 9 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 22.7 | 11.8 |
| 10 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.7 | 10.8 |
| 11 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 22.7 | 11.8 |
| 12 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 13 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4 | 22.7 | 13.8 |
| 14 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 1 | 17.5 | 5 | 22.7 | 11.8 |
| 15 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 16 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 22.7 | 11.8 |
| 17 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 18 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4 | 22.7 | 13.8 |
| 19 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 22.7 | 13.8 |
| 20 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 22.7 | 13.8 |
| 21 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 22.7 | 13.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-2: UE Power Class test requirements (network signalled value "NS_03")
(for Band 2 Mid range and Band 4 with TC,c=0 dB)

| Config ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2.0 | 25.7 | 20.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 3 | 2 | 21 | 2.0 | 25.7 | 18.3 | 2 | 18 | 4.0 | 22.7 | 13.3 |
| 4 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 5 | 2 | 21 | 2.0 | 25.7 | 18.3 | 2 | 18 | 4.0 | 22.7 | 13.3 |
| 6 | 0 | 23 | 2.0 | 25.7 | 20.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 7 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 8 | 2 | 21 | 2.0 | 25.7 | 18.3 | 2 | 18 | 4.0 | 22.7 | 13.3 |
| 9 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 10 | 2 | 21 | 2.0 | 25.7 | 18.3 | 2 | 18 | 4.0 | 22.7 | 13.3 |
| 11 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 12 | 0 | 23 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 13 | 1 | 22 | 2.0 | 25.7 | 19.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 14 | 2 | 21 | 2.0 | 25.7 | 18.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 15 | 0 | 23 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 16 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 17 | 0 | 23 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 18 | 1 | 22 | 2.0 | 25.7 | 19.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 19 | 0 | 23 | 2.0 | 25.7 | 20.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 20 | 0 | 23 | 2.0 | 25.7 | 20.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 21 | 0 | 23 | 2.0 | 25.7 | 20.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-3: UE Power Class test requirements (network signalled value "NS_04")
(for Band 41)

FFS

Table 6.2.4EA.5-4: UE Power Class test requirements (network signalled value "NS_05")
(for Band 1)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 19 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-5: UE Power Class test requirements (network signalled value "NS_06")
(for Band 12 Low/High range with TC,c=1.5 dB)

| Configuration ID |  | Power class 3 |  |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 2 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 3 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5.0 | 22.7 | 10.8 |
| 4 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 5 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5.0 | 22.7 | 10.8 |
| 6 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 7 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 8 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5.0 | 22.7 | 10.8 |
| 9 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 10 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5.0 | 22.7 | 10.8 |
| 11 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 12 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 13 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 14 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 15 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 16 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 17 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 18 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-6: UE Power Class test requirements (network signalled value "NS_06")
(for Band 12 Mid range and Band 13 with TC,c=0 dB)

| Config ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2.0 | 25.7 | 20.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 3 | 2 | 21 | 2.0 | 25.7 | 18.3 | 2 | 18 | 4.0 | 22.7 | 13.3 |
| 4 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 5 | 2 | 21 | 2.0 | 25.7 | 18.3 | 2 | 18 | 4.0 | 22.7 | 13.3 |
| 6 | 0 | 23 | 2.0 | 25.7 | 20.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 7 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 8 | 2 | 21 | 2.0 | 25.7 | 18.3 | 2 | 18 | 4.0 | 22.7 | 13.3 |
| 9 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 10 | 2 | 21 | 2.0 | 25.7 | 18.3 | 2 | 18 | 4.0 | 22.7 | 13.3 |
| 11 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 12 | 0 | 23 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 13 | 1 | 22 | 2.0 | 25.7 | 19.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 14 | 2 | 21 | 2.0 | 25.7 | 18.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 15 | 0 | 23 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 16 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 17 | 0 | 23 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 18 | 1 | 22 | 2.0 | 25.7 | 19.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-7: UE Power Class test requirements (network signalled value "NS_07")
(for band 13)

| Configuration ID | A-MPR (dB) |  | Power class 3 |  |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 2 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 | 0 | 19 | 3.5 | 22.7 | 14.8 |
| 3 | 1 | 1 | 21 | 2.0 | 25.7 | 18.3 | 1 | 18 | 4.0 | 22.7 | 13.3 |
| 4 | 1 | 1 | 21 | 2.0 | 25.7 | 18.3 | 0 | 19 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-7a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_07")
(for band 13)

| Configuration ID | A-MPR (dB) |  | Power class 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 | 0 | 22.5 | 2.0 | 25.7 | 19.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-8: UE Power Class test requirements (network signalled value "NS_08")
(for band 19)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 19 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-9: UE Power Class test requirements (network signalled value "NS_09")
(for band 21)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 19 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-10: UE Power Class test requirements (network signalled value "NS_10")
(for band 20)

Void, not tested

Table 6.2.4EA.5-11: UE Power Class 3 test requirements (network signalled value "NS_12")
(for band 26)

FFS

Table 6.2.4EA.5-12: UE Power Class 3 test requirements (network signalled value "NS_13")
(for band 26)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2.0 | 25.7 | 20.3 | 20 | 2.0 | 22.7 | 17.3 |
| 2 | 1 | 21 | 2.0 | 25.7 | 18.3 | 18 | 4.0 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c is 0 dB for band 26 UL 819.7 MHz (NUL = 26747) as indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-13: UE Power Class 3 test requirements (network signalled value "NS_14")
(for band 26)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2.0 | 25.7 | 20.3 | 20 | 2.0 | 22.7 | 17.3 |
| 2 | 1 | 21 | 2.0 | 25.7 | 18.3 | 18 | 4.0 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c is 0 dB for band 26 UL 829 MHz (NUL = 26840) as indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |

Table 6.2.4.5-14: UE Power Class test requirements (network signalled value "NS_15")
(for band 26)

| Configuration ID | DTC,c (dB) | A-MPR (dB) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power class 3 |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 23 | 2 | 25.7 | 20.3 |
| 2 | 0 | 0 | 2 | 21 | 2 | 25.7 | 18.3 |
| 3 | 1.5 | 0 | 0 | 21.5 | 2 | 25.7 | 18.8 |
| 4 | 1.5 | 3 | 2 | 16.5 | 5 | 25.7 | 10.8 |
| 5 | 1.5 | 0 | 0 | 21.5 | 2 | 25.7 | 18.8 |
| 6 | 1.5 | 0 | 2 | 21.5 | 2 | 25.7 | 18.8 |
| 7 | 1.5 | 9 | 2 | 10.5 | 6 | 25.7 | 3.8 |
| Power class 5 |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 20 | 2 | 22.7 | 17.3 |
| 2 | 0 | 0 | 2 | 18 | 4 | 22.7 | 13.3 |
| 3 | 1.5 | 0 | 0 | 18.5 | 4 | 22.7 | 13.8 |
| 4 | 1.5 | 3 | 2 | 13.5 | 5 | 22.7 | 7.8 |
| 5 | 1.5 | 0 | 0 | 18.5 | 4 | 22.7 | 13.8 |
| 6 | 1.5 | 0 | 2 | 18.5 | 4 | 22.7 | 13.8 |
| 7 | 1.5 | 9 | 2 | 7.5 | 7 | 22.7 | -0.2 |

Table 6.2.4EA.5-15: UE Power Class test requirements (network signalled value "NS_16") 
(for band 27)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 19 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4.5-16: UE Power Class test requirements (network signalled value "NS_17")
(for band 28)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 19.8 | 20 | 2 | 22.7 | 16.8 |
| 2 | 1 | 22 | 2 | 25.7 | 18.8 | 19 | 3.5 | 22.7 | 14.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-17: UE Power Class test requirements (network signalled value "NS_18")
(for band 28)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 19.8 | 20 | 2 | 22.7 | 16.8 |
| 2 | 1 | 22 | 2 | 25.7 | 18.8 | 19 | 3.5 | 22.7 | 14.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EA.5-18: UE Power Class test requirements (network signalled value "NS_19")

FFS

Table 6.2.4EA.5-19: UE Power Class test requirements (network signalled value "NS_20")

FFS

Table 6.2.4EA.5-20: UE Power Class test requirements (network signalled value "NS_21")

FFS

Table 6.2.4EA.5-21: UE Power Class test requirements (network signalled value "NS_22")
(for band 42 and 43)

| Configuration ID | MPR (dB) | PCMAX_L,c (dBm) |  | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Power class 3 |  |  |  |  |  |  |
| 1 | 0 | 23 | 2 |  | 26 | 20 |
| 2 | 1 | 22 | 2 |  | 26 | 19 |
| Power class 5 |  |  |  |  |  |  |
| 1 | 0 | 20 | 2 |  | 23 | 17 |
| 2 | 1 | 19 | 3.5 |  | 24.5 | 14.5 |
| Power class 6 |  |  |  |  |  |  |
| 1 | 0 | 14 | 2.5 |  | 17.5 | 10.5 |
| 2 | 1 | 13 | 2.5 |  | 17.5 | 9.5 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |

Table 6.2.4EA.5-22: UE Power Class test requirements (network signalled value "NS_23")
(for band 42 and 43)

| Configuration ID | MPR (dB) | PCMAX_L,c (dBm) |  | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Power class 3 |  |  |  |  |  |  |
| 1 | 0 | 23 | 2 |  | 26 | 20 |
| 2 | 1 | 22 | 2 |  | 26 | 19 |
| Power class 5 |  |  |  |  |  |  |
| 1 | 0 | 20 | 2 |  | 23 | 17 |
| 2 | 1 | 19 | 3.5 |  | 24.5 | 14.5 |
| Power class 6 |  |  |  |  |  |  |
| 1 | 0 | 14 | 2.5 |  | 17.5 | 10.5 |
| 2 | 1 | 13 | 2.5 |  | 17.5 | 9.5 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |

<< Clauses skipped here>>

#### 6.6.2.2EA	Additional Spectrum Emission Mask for UE category M1

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- NS_04 (B41): Test configuration table missing due to minimum requirements for A-MPR in 36.101 are incomplete.

##### 6.6.2.2EA.1	Test purpose

Same test purpose as in clause 6.6.2.2.1.

##### 6.6.2.2EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

##### 6.6.2.2EA.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2.3

##### 6.6.2.2EA.4	Test description

6.6.2.2EA.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2EA.4.1-1 through 6.6.2.2EA.4.1-4. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2EA.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 0 |
| 8 | 3MHz |  | QPSK | 6 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 5 | 0 |
| 11 | 5MHz |  | QPSK | 6 | 0 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 13 | 5MHz |  | 16QAM | 3 | 0 |
| 14 | 5MHz |  | 16QAM | 5 | 0 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 16 | 10MHz |  | QPSK | 6 | 0 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 18 | 10MHz |  | 16QAM | 5 | 0 |
| 19 | 15MHz |  | QPSK | 6 | 0 |
| 20 | 15MHz |  | 16QAM | 5 | 0 |
| 21 | 20MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 1 |
| 8 | 3MHz |  | QPSK | 6 | 1 |
| 9 | 3MHz |  | 16QAM | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 5 | 1 |
| 11 | 5MHz |  | QPSK | 6 | 3 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 13 | 5MHz |  | 16QAM | 3 | 3 |
| 14 | 5MHz |  | 16QAM | 5 | 3 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 16 | 10MHz |  | QPSK | 6 | 7 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 18 | 10MHz |  | 16QAM | 5 | 7 |
| 19 | 15MHz |  | QPSK | 6 | 11 |
| 20 | 15MHz |  | 16QAM | 5 | 11 |
| 21 | 20MHz |  | 16QAM | 6 | 15 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. Note 4: Only for power class 3 UE |  |  |  |  |  |

Table 6.6.2.2EA.4.1-2: Test Configuration Table (network signalled value "NS_04")

TBD

Table 6.6.2.2EA.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 0 |
| 8 | 3MHz |  | QPSK | 6 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 5 | 0 |
| 11 | 5MHz |  | QPSK | 6 | 0 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 13 | 5MHz |  | 16QAM | 3 | 0 |
| 14 | 5MHz |  | 16QAM | 5 | 0 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 16 | 10MHz |  | QPSK | 6 | 0 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 18 | 10MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 1 |
| 8 | 3MHz |  | QPSK | 6 | 1 |
| 9 | 3MHz |  | 16QAM | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 5 | 1 |
| 11 | 5MHz |  | QPSK | 6 | 3 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 13 | 5MHz |  | 16QAM | 3 | 3 |
| 14 | 5MHz |  | 16QAM | 5 | 3 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 16 | 10MHz |  | QPSK | 6 | 7 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 18 | 10MHz |  | 16QAM | 5 | 7 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. Note 4: Only for power class 3 UE |  |  |  |  |  |

Table 6.6.2.2EA.4.1-4: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 10MHz |  | QPSK | 1 | 0 |
| 2 | 10MHz |  | QPSK | 4 | 0 |
| 3 | 10MHz |  | QPSK | 6 | 0 |
| 4 | 10MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 10MHz |  | QPSK | 1 | 7 |
| 2 | 10MHz |  | QPSK | 4 | 7 |
| 3 | 10MHz |  | QPSK | 6 | 7 |
| 4 | 10MHz |  | 16QAM | 5 | 7 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.2.2EA.4.1-4a: Test Configuration Table, subPRB allocation (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) | SCstart |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 10MHz |  | π/2-BPSK | ¼ | 0 | 0 |
| High range |  |  |  |  |  |  |
| 1 | 10MHz |  | π/2-BPSK | ¼ | 0 | 8 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |  |

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Tables 6.6.2.2EA.4.1-1 to 6.6.2.2EA.4.1-4.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.2EA.4.3.

6.6.2.2EA.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to applicable table from Table 6.6.2.2EA.4.1-1 to 6.6.2.2.4EA.1-4. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.4EA.5-1 to 6.2.4EA.5-22 as appropriate. The period of the measurement shall be at least one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.2.2EA.5.1-1, 6.6.2.2EA.5.2-1 and, 6.6.2.2.5EA.3-1 as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.	For UE supporting subPRB allocation, repeat step 1-5 with UL RMC according to applicable Table from Table 6.6.2.2EA.4.1-1a to 6.6.2.2.4EA.1-4a.

6.6.2.2EA.4.3	Message contents

Message contents are same as in clause 6.2.4EA.4.3.

##### 6.6.2.2EA.5	Test requirements

6.6.2.2EA.5.1	Test requirements (network signalled value "NS_03"”)

When "NS_03” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EA.5-1 and 6.2.4EA.5-2 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EA.5.1-1, as applicable.

Table 6.6.2.2EA.5.1-1: Additional requirements (network signalled value "NS_03”)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 E Table 6.2.4 E -1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2EA.5.2	Test requirements (network signalled value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EA.5-3 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EA.5.2-1, as applicable.

Table 6.6.2.2EA.5.2-1: Additional requirements (network signalled value "NS_04")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -23.5 | -23.5 | -23.5 | -23.5 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4 E Table 6.2.4 E -1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Test requirements (network signalled value "NS_06" or "NS_07")

When "NS_06" or "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EA.5-5, 6.2.4EA.5-6 and 6.2.4EA.5-7 as appropriate ,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EA.5.3-1, as applicable.

Table 6.6.2.2EA.5.3-1: Additional requirements (network signalled value "NS_06" or "NS_07")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 E Table 6.2.4 E -1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<< Clauses skipped here >>

#### 6.6.3EA.3	Additional spurious emissions for UE category M1

Editor's note:	Following aspects missing or not yet determined:

- Test configuration tables and test requirement tables FFS for NS_12

##### 6.6.3EA.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

##### 6.6.3EA.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

##### 6.6.3EA.3.3	Minimum conformance requirements

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.6.3EA.3.3.1	Minimum requirement (network signalled value "NS_05")

When "NS_05" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.1-1: Additional requirements (PHS)

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| 1884.5 f 1915.7 | -41 | -41 | -41 | -41 | 300 KHz | 1 |

6.6.3EA.3.3.2	Minimum requirement (network signalled value "NS_07")

When "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.2-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz |  |
| 769 ≤ f ≤ 775 | -57 | 6.25 kHz |
| NOTE: The emissions measurement shall be sufficiently power averaged to ensure standard deviation < 0.5 dB. |  |  |

6.6.3EA.3.3.3	Minimum requirement (network signalled value "NS_08")

When "NS_08" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.3-1. This requirement also applies for the frequency ranges that are less than ΔfOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.3-1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5MHz | 10MHz | 15MHz |  |
| 860 ≤ f ≤ 895 | -40 | -40 | -40 | 1 MHz |

6.6.3EA.3.3.4	Minimum requirement (network signalled value "NS_09")

When "NS 09" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.4-1. This requirement also applies for the frequency ranges that are less than ΔfOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.4-1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5MHz | 10MHz | 15MHz |  |
| 1475.9 ≤ f ≤ 1510.9 | -35 | -35 | -35 | 1 MHz |

NOTE 1:	To improve measurement accuracy, A-MPR values for NS_09 specified Table 6.2.4EA -1 in sub-clause 6.2.4EA are derived based on 100 kHz RBW.

6.6.3EA.3.3.5	Minimum requirement (network signalled value "NS_12")

When "NS 12" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.5-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4 MHz, 3 MHz, 5 MHz, 10 MHz, 15 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 814.2 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

6.6.3EA.3.3.6	Minimum requirement (network signalled value "NS_13")

When "NS 13" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.6-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4, 3, 5 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 819 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

6.6.3EA.3.3.7	Minimum requirement (network signalled value "NS_14")

When "NS 14" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.7-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.7-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz, 15 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 824 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

6.6.3EA.3.3.8	Minimum requirement (network signalled value "NS_15")

When "NS 15" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.8-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.8-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4 MHz, 3 MHz, 5 MHz, 10 MHz, 15 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

6.6.3EA.3.3.9	Minimum requirement (network signalled value "NS_16")

When "NS_16" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.9-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.9-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4, 3, 5, 10 MHz |  |  |
| 790 ≤ f ≤ 803 | -32 | 1 MHz |  |

6.6.3EA.3.3.10	Minimum requirement (network signalled value "NS_17")

When "NS_17" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.10-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.10-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10 MHz |  |  |
| 470 ≤ f ≤ 710 | -26.2 | 6 MHz | 1 |
| NOTE 1: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. |  |  |  |

6.6.3EA.3.3.11	Minimum requirement (network signalled value "NS_18")

When "NS_18" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.11-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.11-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |  |
| 692-698 | -26.2 | 6 MHz |  |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.3.3.

6.6.3EA.3.3.12	Minimum requirement (network signalled value "NS_19")

FFS

6.6.3EA.3.3.13	Minimum requirement (network signalled value "NS_20")

FFS

6.6.3EA.3.3.14	Minimum requirement (network signalled value "NS_21")

FFS

6.6.3EA.3.3.15	Minimum requirement (network signalled value "NS_22")

When "NS_22" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.15-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.15-1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | MBW |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 3400 ≤ f ≤ 3800 | -23 (Note 1, Note 3) | 5 MHz |
|  | -40 (Note 2) | 1 MHz |
| NOTE 1: This requirement applies within an offset between 5 MHz and 25 MHz from the lower and from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. NOTE 2: This requirement applies from 3400 MHz to 25 MHz below the lower E-UTRA channel edge and from 25 MHz above the upper E-UTRA channel edge to 3800 MHz. NOTE 3: This emission limit might imply risk of harmful interference to UE(s) operating in the protected operating band |  |  |

The normative reference for this requirement is TS 36.101[2] subclause 6.6.3.3.16.

6.6.3EA.3.3.16	Minimum requirement (network signalled value "NS_23")

When "NS_23" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.3.16-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.3.16-1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | MBW |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 3400 ≤ f ≤ 3800 | -23 (Note 1, Note 4) | 5 MHz |
|  | -40 (Note 2) | 1 MHz |
| NOTE 1: This requirement applies within an offset between 5 MHz + Foffset_NS_23 and 25 MHz + Foffset_NS_23 from the lower and from the upper edges of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. NOTE 2: This requirement applies from 3400 MHz to 25 MHz + Foffset_NS_23 below the lower E-UTRA channel edge and from 25 MHz + Foffset_NS_23 above the upper E-UTRA channel edge to 3800 MHz. NOTE 3: Foffset_NS_23 is: 0 MHz for 5 MHz channel BW, 5 MHz for 10 MHz channel BW, 9 MHz for 15 MHz channel BW and 12 MHz for 20 MHz channel BW. NOTE 4: This emission limit might imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |

##### The normative reference for this requirement is TS 36.101[2] subclause 6.6.3.3.17.6.6.3EA.3.4	Test description

6.6.3EA.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3EA.3.4.1-1 to Table 6.6.3EA.3.4.1-16. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.3EA.3.4.1-1: Test Configuration Table (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-2: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 10MHz |  | QPSK | 1 | 0 |
| 2 | 10MHz |  | QPSK | 4 | 0 |
| 3 | 10MHz |  | QPSK | 6 | 0 |
| 4 | 10MHz |  | 16QAM | 5 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-2a: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) | SCstart |
| Low range |  |  |  |  |  |  |
| 1 | 10MHz |  | π/2-BPSK | ¼ | 0 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |  |

Table 6.6.3EA.3.4.1-3: Test Configuration Table (network signalled value "NS_08")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-4: Test Configuration Table (network signalled value "NS_09")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-5: Test Configuration Table (network signalled value "NS_12")

FFS

Table 6.6.3EA.3.4.1-6: Test Configuration Table (network signalled value "NS_13")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | UL 819.7 MHz (NUL = 26747) |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 1.4MHz |  | QPSK | 1 | 0 |
| 2 | 1.4MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-7: Test Configuration Table (network signalled value "NS_14")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | UL 829 MHz (NUL = 26840) |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 10MHz |  | QPSK | 1 | 0 |
| 2 | 10MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-8: Test Configuration Table (network signalled value "NS_15")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Configuration ID 1-2: UL 844.3 MHz Configuration ID 3-8: High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 1.4MHz, 3MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 1.4MHz |  | QPSK | 1@0, 1@5 | 0 |
| 2 | 1.4MHz |  | QPSK | 6 | 0 |
| 3 | 1.4MHz |  | QPSK | 1@5 | 0 |
| 4 | 1.4MHz |  | QPSK | 6 | 0 |
| 5 | 3MHz |  | QPSK | 1@1 | 1 |
| 6 | 3MHz |  | QPSK | 1@5 | 1 |
| 7 | 3MHz |  | QPSK | 6 | 1 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 |  |  |  |  |  |

Table 6.6.3EA.3.4.1-9: Test Configuration Table (network signalled value "NS_16")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 1.4MHz |  | QPSK | 1 | 0 |
| 2 | 1.4MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-10: Test Configuration Table (network signalled value "NS_17")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 720.5 MHz |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-11: Test Configuration Table (network signalled value "NS_18")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-12: Test Configuration Table (network signalled value "NS_19")

FFS

Table 6.6.3EA.3.4.1-13: Test Configuration Table (network signalled value "NS_20")

FFS

Table 6.6.3EA.3.4.1-14: Test Configuration Table (network signalled value "NS_21")

FFS

Table 6.6.3EA.3.4.1-15: Test Configuration Table (network signalled value "NS_22")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5 MHz |  | QPSK | 1 | 0 |
| 2 | 5 MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS 36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.3EA.3.4.1-16: Test Configuration Table (network signalled value "NS_23")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5 MHz |  | QPSK | 1 | 0 |
| 2 | 5 MHz |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS 36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.3EA.3.4.1-1 to Table 6.6.3EA.3.4.1-16.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.3EA.3.4.3.

6.6.3EA.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.3EA.3.4.1-1 to Table 6.6.3EA.3.4.1-16 depending on NS-value. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.4EA.5-1 to 6.2.4EA.5-22 as appropriate. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.3EA.3.1-1, 6.6.3EA.3.3.3-1 to 6.6.3EA.3.3.16-1, and 6.6.3EA.3.5.1-1 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table. For NS_07 measurements made in a bandwidth of 6.25kHz, measurement parameter settings defined in table 6.6.3EA.3.4.2-1 shall be used. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

5.	For UE supporting subPRB allocation, repeat step 1-3 with UL RMC according to applicable Table from Table 6.2.4EA.5-1a to 6.2.4EA.5-22a.

Table 6.6.3EA.3.4.2-1: Measurement setup for NS_07

|  | Option 1: Measurement with No RMS VBW available | Option2 Measurement with VBW Filtering on Power scale |
| --- | --- | --- |
| VBW | >=62.5 kHz (10 times or more the RBW) | <=43Hz |
| RBW | <=6.25kHz | <=6.25kHz |
| Detector type | Averages signal envelope during each measurement point, such as "RMS detector" | Peak |
| Averaging mode (Trace averaging) | Power (RMS voltage) | Power (RMS voltage), as controlled by "Average Type" |
| Average Type (applies to detector) | Power (RMS voltage) (automatically occurs with "RMS detector") | Not applicable |
| Average Type (applies to VBW filter) | Not applicable | Power (RMS voltage) |
| Number of averages | 30, to reduce variance as required, or use an even longer sweep time | 1 or use an even narrower VBW filter, thus a longer sweep time |
| Sweep time | [Don't specify] | Sweep rate (span divided by sweep time) <= 0.8 * RBW*VBW |

6.6.3EA.3.4.3	Message contents

Message contents are same as in clause 6.2.4EA.4.3.

##### 6.6.3EA.3.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.4EA.5-1 to 6.2.4EA.5-22 as appropriate,

and

With the exception of NS_07, test requirements for Additional Spurious Emissions are the same as the minimum requirements and are not repeated in this section.

6.6.3EA.3.5.1	Test requirement (network signalled value "NS_07")

When "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EA.3.5.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EA.3.5.1-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz |  |
| 769 ≤ f ≤ 775 | -55.5 | 6.25 kHz |
| NOTE: The emissions measurement shall be sufficiently power averaged to ensure standard deviation < 0.5 dB. |  |  |

<< End of changes>>