3GPP TSG-RAN5 Meeting #92-e	R5-214729

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0143 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 37.571-4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The module CommonAspDefs.ttcn is erroneously listed in clause D.3. This module is not imported by the positioning ASPs and therefore shall be removed from the references. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleted module CommonAspDefs.ttcn from clause D.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | List of imported modules will be wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex D (informative)
Positioning System Simulator ASP Definitions in TTCN-3

# D.1	PosSystem_ASP_TypeDefs

PosSystem_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_MaxPosSystems | integer | 12 |  |

PositioningSystemType

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | PositioningSystemType |
| Comment |  |
| gps |  |
| modernizedGPS |  |
| glonass |  |
| galileo |  |
| qzss |  |
| otdoa |  |
| ecid |  |
| bds |  |
| mbs |  |
| wlan |  |
| bluetooth |  |
| sensor |  |

PositioningSystemList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PositioningSystemList_Type |
| Comment |  |
| record length (1..tsc_MaxPosSystems) of PositioningSystemType |  |

Altitude_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Altitude_Type |  |  |
| Comment |  |  |  |
| Height | integer |  | Height above a sea level in meters |

## D.1.1	PosSystem_Load_Scenario

Load scenarios follow 3GPP TS 37.571-5 [5]

PosSystem_Load_Scenario: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| ScenarioNumber_Type | integer |  |

PositioningScenario_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PositioningScenario_Type |  |
| Comment |  |  |
| LTE_Positioning | ScenarioNumber_Type | Reference to 3GPP TS 37.571-5 [5] |
| AGNSS | ScenarioNumber_Type | Reference to 3GPP TS 37.571-5 [5] |
| V2X | ScenarioNumber_Type | V2X Reference to 3GPP TS 36.508 [7] |
| NR_Positioning | ScenarioNumber_Type | Reference to 3GPP TS 37.571-5 [5] |
| Aerial | ScenarioNumber_Type | Reference to 3GPP TS 36.508 [7] clause 4.12 |

## D.1.2	PosSystem_Retrieve_Data

### D.1.2.1	SatSystem_Retrieve_UTRAN_Data

UTRAN_GPS_AssistanceDataRequest_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_GPS_AssistanceDataRequest_Type |  |  |
| Comment |  |  |  |
| AssistanceDataReqGPS | bitstring |  | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GPS-AdditionalAssistanceDataRequest |

UTRAN_GANSS_AssistanceDataRequest_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_GANSS_AssistanceDataRequest_Type |  |  |
| Comment |  |  |  |
| AssistanceDataReqGANSS | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AdditionalAssistanceDataRequest |
| AssistanceDataReqGANSSv860ext | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AdditionalAssistanceDataRequest-v860ext |
| AssistanceDataReqGANSSvc50ext | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AdditionalAssistanceDataRequest-vc50ext |

UTRAN_AssistanceDataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UTRAN_AssistanceDataRequest_Type |  |
| Comment |  |  |
| GPS | UTRAN_GPS_AssistanceDataRequest_Type |  |
| GANSS | UTRAN_GANSS_AssistanceDataRequest_Type |  |

UTRAN_AssistanceDataRequestList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | UTRAN_AssistanceDataRequestList_Type |
| Comment |  |
| record length(1..tsc_MaxPosSystems) of UTRAN_AssistanceDataRequest_Type |  |

UTRAN_GPS_AssistanceData_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_GPS_AssistanceData_Type |  |  |
| Comment | The returned Almanac information is split into two fields: Almanac for satellites 1 to 16 in AssistanceDataGPSr7 together with other information; Almanac for satellites 17 to 31: in 'almanacSat17To31' |  |  |
| AssistanceDataGPSr7 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GPS-AssistanceData-r7 |
| AssistanceDataGPSr12 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GPS-AssistanceData-r12 |
| AlmanacSatInfoList17To31 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type AlmanacSatInfoList |

UTRAN_GANSS_AssistanceData_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_GANSS_AssistanceData_Type |  |  |
| Comment |  |  |  |
| AssistanceDataGANSS_r8 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AssistanceData-r8 |
| AssistanceDataGANSS_r12 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AssistanceData-r12 |

UTRAN_AssistanceData_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UTRAN_AssistanceData_Type |  |
| Comment |  |  |
| GPS | UTRAN_GPS_AssistanceData_Type |  |
| GANSS | UTRAN_GANSS_AssistanceData_Type |  |

UTRAN_AssistanceDataList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | UTRAN_AssistanceDataList_Type |
| Comment |  |
| record length(1..tsc_MaxPosSystems) of UTRAN_AssistanceData_Type |  |

### D.1.2.2	PosSystem_Retrieve_LPP_Data

When data is to be requested for more than one positioning system, they will be included into one single template. This template will include a list with one element for each positioning system.
See template "cs_RequestAssistData_UEB_GpsAndGlonass" as an example.

LPP_AssistanceDataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | LPP_AssistanceDataRequest_Type |  |
| Comment |  |  |
| LPP_AssistanceData | bitstring | bitstring containing 3GPP TS 37.355 [10] type RequestAssistanceData-r9-IEs |

LPP_AssistanceDataProvide_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | LPP_AssistanceDataProvide_Type |  |
| Comment |  |  |
| LPP_AssistanceData | bitstring | bitstring containing 3GPP TS 37.355 [10] type ProvideAssistanceData-r9-IEs |

LPP_AssistanceDataRequestList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | LPP_AssistanceDataRequestList_Type |
| Comment |  |
| record length(1..tsc_MaxPosSystems) of LPP_AssistanceDataRequest_Type |  |

LPP_AssistanceDataList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | LPP_AssistanceDataList_Type |
| Comment |  |
| record length(1..tsc_MaxPosSystems) of LPP_AssistanceDataProvide_Type |  |

### D.1.2.3	PosSystem_Retrieve_Data_Common

AssistanceDataRetrieveRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | AssistanceDataRetrieveRequest_Type |  |
| Comment |  |  |
| LPP | LPP_AssistanceDataRequestList_Type | LTE/NR positioning |
| UTRA_GNSS | UTRAN_AssistanceDataRequestList_Type | Retrieve A-GNSS data list in the order of positioning system provided in PositioningSystemList |

AssistanceDataRetrieveResponse_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | AssistanceDataRetrieveResponse_Type |  |
| Comment |  |  |
| LPP | LPP_AssistanceDataList_Type | LTE/NR positioning |
| UTRA_AGNSS | UTRAN_AssistanceDataList_Type | Retrieve A-GNSS data list in the order of positioning system provided in PositioningSystemList |

## D.1.3	PosSystem_PowerControl

PowerControl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PowerControl_Type |  |
| Comment | V2X. The GNSS simulator is switched on/off without stopping the UTC time sequence |  |
| PowerOn | Null_Type | By default the power is on in the GNSS simulator |
| PowerOff | Null_Type |  |

## D.1.4	System_Interface

Common ASP definitions

PosSystemCrtlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PosSystemCrtlRequest_Type |  |
| Comment |  |  |
| Start | Null_Type |  |
| Stop | Null_Type |  |
| LoadScenario | PositioningScenario_Type |  |
| RetrieveData | AssistanceDataRetrieveRequest_Type |  |
| TriggerMove | Null_Type | V2X. Start moving to the next area/zone in the scenario |
| TriggerPowerOnOff | PowerControl_Type | V2X. Switch on/off the transmitter without stopping UTC time sequence |
| SetAltitude | Altitude_Type | Aerial Set the height above the sea level to the provided value |
| TriggerAerialMove | Altitude_Type | Aerial Start moving towards target height |

PosSystemCrtlConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PosSystemCrtlConfirm_Type |  |
| Comment | In general, to be sent after the configuration has been done |  |
| Start | Null_Type |  |
| Stop | Null_Type |  |
| LoadScenario | Null_Type |  |
| RetrieveData | AssistanceDataRetrieveResponse_Type |  |
| TriggerMove | Null_Type | V2X |
| TriggerPowerOnOff | Null_Type | V2X |
| SetAltitude | Null_Type | Aerial |
| TriggerAerialMove | Null_Type | Aerial |

POS_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | POS_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| PositioningSystemList | PositioningSystemList_Type |  | The list of positioning systems to be configured; there is only one system of one type in the list e.g. gps, galileo etc. |
| CnfFlag | boolean |  | true => SS shall send CNF |
| Request | PosSystemCrtlRequest_Type |  |  |

POS_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | POS_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| PositioningSystemList | PositioningSystemList_Type |  |  |
| Confirm | PosSystemCrtlConfirm_Type |  |  |

POS_MTC_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | POS_MTC_SYSTEM_PORT |  |
| Comment | Positioning: port at the MTC to be mapped to the system |  |
| out | POS_SYSTEM_CTRL_REQ |  |
| in | POS_SYSTEM_CTRL_CNF |  |

POS_MTC_PTC_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | POS_MTC_PTC_PORT |  |
| Comment | Positioning: port at the MTC to hand over information from the positioning system to the PTC(s) |  |
| out | POS_SYSTEM_CTRL_CNF |  |
| in | POS_SYSTEM_CTRL_REQ |  |

POS_PTC_MTC_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | POS_PTC_MTC_PORT |  |
| Comment | Positioning: port at the PTC(s) to send commands to the positioning system via the MTC |  |
| out | POS_SYSTEM_CTRL_REQ |  |
| in | POS_SYSTEM_CTRL_CNF |  |

# D.2	CommonDefs

CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| Null_Type | boolean (true) | dummy type for 'typeless' fields in unions |

# D.3	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| PosSystem_ASP_TypeDefs | POS/PosSystem_ASP_TypeDefs.ttcn | Rev 30522 |
| CommonDefs | Common/CommonDefs.ttcn | Rev 30597 |