3GPP TSG-RAN5 Meeting #92-e	R5-214753

# Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2344 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR-DC RRC test case 8.2.5.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies, Anritsu Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Since RLC transparent mode is configured, NULL ciphering algorithm should be configured - NR-1 system information combination would be inline with real-deployment rather than NR-4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Configured NULL ciphering algorithm d - NEA0(NULL) ciphering algorithm configured |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.2.5.3.2	Radio link failure / rlc-MaxNumRetx failure / NR-DC

8.2.5.3.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with NR-DC }

ensure that {

when { the transmitting side of the UE’s AM RLC entity for the SCG DRB has retransmitted an RLC SDU until RETX_COUNT = maxRetxThreshold }

then { the UE shall transmit a SCGFailureInformation message with failureType set to ‘rlc-MaxNumRetx’ }

}

8.2.5.3.2.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.322, clause 5.3.2, TS 38.331, clauses 5.3.10.3, 5.7.3.2, 5.7.3.5. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.2]

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

-	if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

-	set the RETX_COUNT associated with the RLC SDU to zero.

-	else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

-	increment the RETX_COUNT.

-	if RETX_COUNT = maxRetxThreshold:

-	indicate to upper layers that max retransmission has been reached.

…

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached; or

1>	if connected as an IAB-node, upon BH RLF indication received on BAP entity from the SCG; or

1>	upon consistent uplink LBT failure indication from SCG MAC:

2>	if the indication is from SCG RLC and CA duplication is configured and activated for SCG, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

[TS 38.331, clause 5.7.3.2]

A UE initiates the procedure to report SCG failures when neither MCG nor SCG transmission is suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with subclause 5.3.10.3;

…

Upon initiating the procedure, the UE shall:

1>	suspend SCG transmission for all SRBs, DRBs and, if any, BH RLC channels;

1>	reset SCG MAC;

1>	stop T304 for the SCG, if running;

1>	stop conditional reconfiguration evaluation for CPC, if configured;

1>	if the UE is in (NG)EN-DC:

2>	initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10], clause 5.6.13a.

1>	else:

2> initiate transmission of the SCGFailureInformation message in accordance with 5.7.3.5.

[TS 38.331, clause 5.7.3.5]

The UE shall set the contents of the SCGFailureInformation message as follows:

…

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

…

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

8.2.5.3.2.3	Test description

8.2.5.3.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell and NR Cell 10 is the PSCell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC) with DC bearer(MCG and SCG), and Test Loop Function (On) with UE test loop mode A according to TS 38.508-1 [4], Table 4.5.1-1 with the exception that ciphering algorithm ‘nea0 (NULL)’ is configured.

8.2.5.3.2.3.2	Test procedure sequence

Table 8.2.5.3.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits AMD PDU#1 on the DRB associated with SCG | <-- | AMD PDU#1 | - | - |
| 2 | The UE transmits one AMD PDU#1 | --> | AMD PDU#1 (SN=0) | - | - |
| - | EXCEPTION: Steps 3-4 are repeated maxRetxThreshold times | - | - | - | - |
| 3 | The SS transmits an RLC STATUS PDU. ACK_SN =1 and NACK_SN =0. | <-- | STATUS PDU | - | - |
| 4 | The UE transmits one AMD PDU#1 | --> | AMD PDU#1 (SN=0) | - | - |
| 5 | The SS transmits an RLC STATUS PDU. ACK_SN =1 and NACK_SN =0. | <-- | STATUS PDU | - | - |
| 6 | Check: Does the UE transmit in the next 5 sec (arbitrary value) a SCGFailureInformation message with failureType set to ‘rlc-MaxNumRetx’? | --> | SCGFailureInformation | 1 | P |

8.2.5.3.2.3.3	Specific message contents

Table 8.2.5.3.2.3.3-1: SCGFailureInformation (step 6, Table 8.2.5.3.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-24A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SCGFailureInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| scgFailureInformation SEQUENCE { |  |  |  |  |
| FailureReportSCG SEQUENCE { |  |  |  |  |
| failureType |  | rlc-MaxNumRetx |  |  |
| measResultFreqList |  | Not checked |  |  |
| measResultSCG-Failure |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |