3GPP TSG-RAN5 Meeting #92-e	R5-214807

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1986 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IEs for V2X communication over PC5 in NR-PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, NR V2X. default configuration is not complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Define default configuration of IEs for V2X communication over PC5 in NR-PC5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X. test case will have no default configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR... CR... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.7.5.5	V2X information elements of UE policies for V2X communication over PC5

…

–	V2X communication over PC5 in NR-PC5

Table 4.7.5.5-31: V2X communication over PC5 in NR-PC5

| Derivation Path: TS 24.588 Figure 5.3.1.31 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X communication over PC5 in NR-PC5 contents |  | Set to the actual length of 'V2X communication over PC5 in NR-PC5 contents' in bytes |  |  |
| Spare |  | ’00 0000’B |  |  |
| VSINFMRI |  | '1'B | V2X service identifier to V2X NR frequency mapping rules field is present |  |
| DDL2IBI |  | '1'B | Default destination layer-2 ID for broadcast field is present |  |
| V2X service identifier to V2X NR frequency mapping rules |  | See Table 4.7.5.5-32 |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rules |  | See Table 4.7.5.5-37 |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rules |  | See Table 4.7.5.5-39 |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules |  | See Table 4.7.5.5-41 |  |  |
| V2X service identifier to PC5 QoS parameters mapping rules |  | See Table 4.7.5.5-43 |  |  |
| AS configuration |  | See Table 4.7.5.5-45 |  |  |
| Default destination layer-2 ID for broadcast |  | '00 00 00 00 00 50'H |  |  |
| NR-PC5 unicast security policies |  | See Table 4.7.5.5-49 |  |  |
| V2X service identifier to default mode of communication mapping rules |  | See Table 4.7.5.5-52 |  |  |

–	V2X service identifier to V2X NR frequency mapping rules

Table 4.7.5.5-32: V2X service identifier to V2X NR frequency mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.32 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X NR frequency mapping rules contents |  | Set to the actual length of 'V2X service identifier to V2X NR frequency mapping rules contents' in bytes |  |  |
| V2X service identifier to V2X NR frequency mapping rule 1 |  | See Table 4.7.5.5-33 |  |  |
| V2X service identifier to V2X NR frequency mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to V2X NR frequency mapping rule

Table 4.7.5.5-33: V2X service identifier to V2X NR frequency mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X NR frequency mapping rule contents |  | Set to the actual length of 'V2X service identifier to V2X NR frequency mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| V2X NR frequencies with geographical areas list |  | See Table 4.7.5.5-34 |  |  |

–	V2X NR frequencies with geographical areas list

Table 4.7.5.5-34: V2X NR frequencies with geographical areas list

| Derivation Path: TS 24.588 Figure 5.3.1.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies with geographical areas list contents |  | Set to the actual length of 'V2X NR frequencies with geographical areas list contents' in bytes |  |  |
| V2X NR frequencies with geographical areas info 1 |  | See Table 4.7.5.5-35 |  |  |
| V2X NR frequencies with geographical areas info 2 |  | FFS |  |  |

–	V2X NR frequencies with geographical areas info

Table 4.7.5.5-35: V2X NR frequencies with geographical areas info

| Derivation Path: TS 24.588 Figure 5.3.1.35 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies with geographical areas info contents |  | Set to the actual length of 'V2X NR frequencies with geographical areas info contents' in bytes |  |  |
| V2X NR frequencies |  | See Table 4.7.5.5-36 |  |  |
| Geographical areas |  | See Table 4.7.5.5-18 |  |  |

–	V2X NR frequencies

Table 4.7.5.5-36: V2X NR frequencies

| Derivation Path: TS 24.588 Figure 5.3.1.36 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies contents |  | Set to the actual length of 'V2X NR frequencies contents' in bytes |  |  |
| V2X NR frequency 1 |  | 791000 | 5865MHz of N47 in TS 38.101-1 |  |
| V2X NR frequency 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for broadcast mapping rules

Table 4.7.5.5-37: V2X service identifier to destination layer-2 ID for broadcast mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.37 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for broadcast mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 1 |  | See Table 4.7.5.5-38 |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for broadcast mapping rule

Table 4.7.5.5-38: V2X service identifier to destination layer-2 ID for broadcast mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.38 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for broadcast mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| Destination layer-2 ID for broadcast |  | '00 00 00 00 00 51'H |  |  |

–	V2X service identifier to destination layer-2 ID for groupcast mapping rules

Table 4.7.5.5-39: V2X service identifier to destination layer-2 ID for groupcast mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.39 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for groupcast mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 1 |  | See Table 4.7.5.5-40 |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for groupcast mapping rule

Table 4.7.5.5-40: V2X service identifier to destination layer-2 ID for groupcast mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.40 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for groupcast mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| Destination layer-2 ID for groupcast |  | '00 00 00 00 00 52'H |  |  |

–	V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

Table 4.7.5.5-41: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.41 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 1 |  | See Table 4.7.5.5-42 |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

Table 4.7.5.5-42: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.42 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| Destination layer-2 ID for unicast initial signalling |  | '00 00 00 00 00 53'H |  |  |

–	V2X service identifier to PC5 QoS parameters mapping rules

Table 4.7.5.5-43: V2X service identifier to PC5 QoS parameters mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.43 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 QoS parameters mapping rules contents |  | Set to the actual length of 'V2X service identifier to PC5 QoS parameters mapping rules contents' in bytes |  |  |
| V2X service identifier to PC5 QoS parameters mapping rule 1 |  | See Table 4.7.5.5-44 |  |  |
| V2X service identifier to PC5 QoS parameters mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to PC5 QoS parameters mapping rule

Table 4.7.5.5-44: V2X service identifier to PC5 QoS parameters mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.46 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 QoS parameters mapping rule contents |  | Set to the actual length of 'V2X service identifier to PC5 QoS parameters mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| Spare |  | ‘0000’B |  |  |
| RI |  | '1'B | Range field is present |  |
| PLAMBRI |  | '1'B | Per-link aggregate maximum bit rate field is present |  |
| MFBRI |  | '1'B | Maximum flow bit rate field is present |  |
| GFBRI |  | '1'B | Guaranteed flow bit rate field is present |  |
| PQI |  | 22 | See Table 5.4.4-1 in TS 23.287 [xx] |  |
| Guaranteed flow bit rate |  | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Maximum flow bit rate |  | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Per-link aggregate maximum bit rate |  | '0000 0110 0000 0000 0000 0010'B | 2 * 1Mbps = 2Mbps. |  |
| Range |  | '0000 0001 1111 0100'B | 500 meters |  |

–	AS configuration

Table 4.7.5.5-45: AS configuration

| Derivation Path: TS 24.588 Figure 5.3.1.46a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of AS configuration contents |  | Set to the actual length of 'AS configuration contents' in bytes |  |  |
| SLRB mapping rules |  | See Table 4.7.5.5-46 |  |  |

–	SLRB mapping rules

Table 4.7.5.5-46: SLRB mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.47 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of SLRB mapping rules contents |  | Set to the actual length of 'SLRB mapping rules contents' in bytes |  |  |
| SLRB mapping rule 1 |  | See Table 4.7.5.5-47 |  |  |
| SLRB mapping rule 2 |  | FFS |  |  |

–	SLRB mapping rule

Table 4.7.5.5-47: SLRB mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.48 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of SLRB mapping rule contents |  | Set to the actual length of 'SLRB mapping rule contents' in bytes |  |  |
| PC5 QoS profile |  | See Table 4.7.5.5-48 |  |  |
| Length of SLRB |  | Set to the actual length of 'SLRB' in bytes |  |  |
| SLRB |  | See Table 4.10.1-1 |  |  |

–	PC5 QoS profile

Table 4.7.5.5-48: PC5 QoS profile

| Derivation Path: TS 24.588 Figure 5.3.1.49 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of PC5 QoS profile contents |  | Set to the actual length of 'PC5 QoS profile contents' in bytes |  |  |
| Spare |  | ‘0’B |  |  |
| MDBVI |  | '0'B | Maximum data burst volume field is absent |  |
| AWI |  | '1'B | Averaging window field is present |  |
| PLOI |  | '1'B | The octet of the priority level is present |  |
| RI |  | '1'B | Range field is present |  |
| PLAMBRI |  | '1'B | Per-link aggregate maximum bit rate field is present |  |
| MFBRI |  | '1'B | Maximum flow bit rate field is present |  |
| GFBRI |  | '1'B | Guaranteed flow bit rate field is present |  |
| PQI |  | 22 | See Table 5.4.4-1 in TS 23.287[xx] |  |
| Guaranteed flow bit rate |  | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Maximum flow bit rate |  | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Per-link aggregate maximum bit rate |  | '0000 0110 0000 0000 0000 0010'B | 2 * 1Mbps = 2Mbps. |  |
| Range |  | '0000 0001 1111 0100'B | 500 meters |  |
| Priority level |  | 4 |  |  |
| Spare |  | ‘0000 0’B |  |  |
| Averaging window |  | '0000 0111 1101 0000'B | 2000ms |  |
| Maximum data burst volume |  | Not Present |  |  |

–	NR-PC5 unicast security policies

Table 4.7.5.5-49: NR-PC5 unicast security policies

| Derivation Path: TS 24.588 Figure 5.3.1.50 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policies contents |  | Set to the actual length of 'NR-PC5 unicast security policies contents' in bytes |  |  |
| NR-PC5 unicast security policy 1 |  | See Table 4.7.5.5-50 |  |  |
| NR-PC5 unicast security policy 2 |  | FFS |  |  |

–	NR-PC5 unicast security policy

Table 4.7.5.5-50: NR-PC5 unicast security policy

| Derivation Path: TS 24.588 Figure 5.3.1.51 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policy contents |  | Set to the actual length of 'NR-PC5 unicast security policy contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| Security policy |  | See Table 4.7.5.5-51 |  |  |
| Geographical areas |  | See Table 4.7.5.5-18 |  |  |

–	Security policy

Table 4.7.5.5-51: Security policy

| Derivation Path: TS 24.588 Figure 5.3.1.52 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Signalling integrity protection policy |  | '000'B | Signalling integrity protection not needed |  |
| Spare |  | ‘0’B |  |  |
| Signalling ciphering policy |  | '000'B | Signalling ciphering not needed |  |
| Spare |  | ‘0’B |  |  |
| User plane integrity protection policy |  | '000'B | User plane integrity protection not needed |  |
| Spare |  | ‘0’B |  |  |
| User plane ciphering policy |  | '000'B | User plane ciphering not needed |  |
| Spare |  | ‘0’B |  |  |

–	V2X service identifier to default mode of communication mapping rules

Table 4.7.5.5-52: V2X service identifier to default mode of communication mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.53 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to default mode of communication mapping rules contents |  | ‘Set to the actual length of 'V2X service identifier to default mode of communication mapping rules contents' in bytes |  |  |
| V2X service identifier to default mode of communication mapping rule 1 |  | See Table 4.7.5.5-53 |  |  |
| V2X service identifier to default mode of communication mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to default mode of communication mapping rule

Table 4.7.5.5-53: V2X service identifier to default mode of communication mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.54 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to default mode of communication mapping rule contents |  | ‘Set to the actual length of 'V2X service identifier to default mode of communication mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| DMC |  | '00'B | Default mode of communication is set to unicast |  |
| Spare |  | ‘0000 00’B |  |  |

<End of modified section 1>