3GPP TSG-RAN5 Meeting #92-e	R5-214831

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0168 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MDT Applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New MDT test cases are introduced to 38.523-1, thus relevent applicapability definition needs to be added in 38.523-2 as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Following applicapabilities were added for NR MDT inter-system relevent test case in cl 4.1: - 8.1.6.3.4.1 Inter-System MDT / Connection Establishment Failure / Logging and reporting / Bluetooth measurement collection - 8.1.6.3.4.2 Inter-System MDT / Connection Establishment Failure / Logging and reporting / WLAN measurement collection - 8.1.6.3.4.3 Inter-System MDT / Connection Establishment Failure / Logging and reporting / sensor information (Note that, the above TCs re-use current condtions. Definition of new conditions are not necessary.) - A small editorial nit was fixed in Table 4.1-3a, regarding to 8.1.6.1.4.8 Connection Establishment Failure / Logging and reporting / RACH failure report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of relevent testing applicapabilites for MDT inter-system features. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 4.1-3a: Applicability of Protocol conformance RRC test cases, ref. TS 38.523-1 [2]

…

| 8.1.6 | SON and MDT support for NR |  |  |  |
| --- | --- | --- | --- | --- |

…

| 8.1.6.3.3 | Inter-System MDT / Radio Link Failure |  |  |  |
| --- | --- | --- | --- | --- |
| 8.1.6.3.3.1 | Inter-System MDT / Radio Link Failure / Logging and reporting / Bluetooth measurement collection | Rel-16 | C137 | UEs supporting 5G coer and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |
| 8.1.6.3.3.2 | Inter-System MDT / Radio Link Failure / Logging and reporting / WLAN measurement collection | Rel-16 | C138 | UEs supporting 5G coer and WLAN measurements in RRC_IDLE and RRC_INACTIVE state |
| 8.1.6.3.3.3 | Inter-System MDT / Radio Link Failure / Logging and reporting / sensor information | Rel-16 | C139 | UEs supporting 5G Core and collection of sensor information such as Barometeric pressure, UE speed, and UE orientation information as defined in TS 37.355. |
| 8.1.6.3.4 | Inter-System MDT / Connection Establishment Failure |  |  |  |
| 8.1.6.3.4.1 | Inter-System MDT / Connection Establishment Failure / Logging and reporting / Bluetooth measurement collection | Rel-16 | C137 | UEs supporting 5G coer and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |
| 8.1.6.3.4.2 | Inter-System MDT / Connection Establishment Failure / Logging and reporting / WLAN measurement collection | Rel-16 | C138 | UEs supporting 5G coer and WLAN measurements in RRC_IDLE and RRC_INACTIVE state |
| 8.1.6.3.4.3 | Inter-System MDT / Connection Establishment Failure / Logging and reporting / sensor information | Rel-16 | C139 | UEs supporting 5G Core and collection of sensor information such as Barometeric pressure, UE speed, and UE orientation information as defined in TS 37.355. |

<End of modified section 1>

<Start of modified section 2>

Table 4.1-3a: Applicability of Protocol conformance RRC test cases, ref. TS 38.523-1 [2]

…

| 8.1.6.1.4 | Connection Establishment Failure |  |  |  |
| --- | --- | --- | --- | --- |
| 8.1.6.1.4.1 | Connection Establishment Failure / Logging and reporting / T300 expiry | Rel-16 | C126 | UEs supporting 5G Core, NR measurements and CEF (Connection Establishment Failure) logging and reporting. |
| 8.1.6.1.4.2 | Connection Establishment Failure / Logging and reporting / RRC Resume | Rel-16 | C126 | UEs supporting 5G Core, NR measurements and CEF (Connection Establishment Failure) logging and reporting. |
| 8.1.6.1.4.3 | Connection Establishment Failure / Logging and reporting / Reporting at intra-NR handover | Rel-16 | C21 | UEs supporting 5G Core. |
| 8.1.6.1.4.4 | Connection Establishment Failure / Logging and reporting / Reporting at RRC connection re-establishment | Rel-16 | C21 | UEs supporting 5G Core. |
| 8.1.6.1.4.5 | Connection Establishment Failure / Logging and reporting / Location Information | Rel-16 | C126 | UEs supporting 5G Core and equipped with a GNSS or A-GNSS receiver to provide detailed location information. |
| 8.1.6.1.4.6 | Connection Establishment Failure / Logging and reporting / Reporting of Intra-frequency measurements | Rel-16 | C21 | UEs supporting 5G Core.. |
| 8.1.6.1.4.7 | Connection Establishment Failure / Logging and reporting / Reporting of Inter-frequency measurements | Rel-16 | C21 | UEs supporting 5G Core. |
| 8.1.6.1.4.8 | Connection Establishment Failure / Logging and reporting / RACH failure report | Rel-16 | C136 | UEs supporting 5G Core and delivery of rachReport upon request from the network |

<End of modified section 2>