3GPP TSG-RAN5 Meeting #92-e	R5-214845

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0579 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MU and TT for spurious emission band UE co-existence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MU for protected frequency 23.6 GHz ≤ f ≤ 24.0 GHz is missing in spurious emission band UE co-existence test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added MU for protected frequency 23.6 GHz ≤ f ≤ 24.0 GHz in Annex F. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MU for protected frequency 23.6 GHz ≤ f ≤ 24.0 GHz remains missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

Annex F (normative):
Measurement uncertainties and Test Tolerances

F.1	Acceptable uncertainty of Test System (normative)

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. Care should be taken to ensure that each conformance test implementation including the OTA chamber aspects meets the specified measurement uncertainty for each test case by requiring the test laboratory to maintain a detailed measurement uncertainty test report showing compliance to all the measurement uncertainty requirements. The detailed measurement uncertainty report would contain the justification for each measurement uncertainty component and its value and distribution. The derivation of these values is based on the minimum conformance requirements plus relaxation, i.e., test tolerance is not to be considered. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

The downlink signal uncertainties apply at the defined quiet zone with the UE properly positioned in the quiet zone. The uplink signal uncertainties apply at the measurement equipment with the UE positioned properly in the quiet zone.

F.1.1	Measurement of test environments

Editor’s note: Various measurement accuracies for UE test environments, e.g., pressure, relative humidity, DC&AC voltage, vibration, and vibration frequency, are FFS:

The measurement accuracy of the UE test environments defined in TS 38.508-1 [5] subclause 4.1, Test environments shall be

-	Temperature			±4 degrees.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2-2 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Quiet Zone size ≤ 30 cm ±4.42 dB (FR2a) ±4.62 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-2 in TR 38.903) |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC3 Quiet Zone size ≤ 30 cm ±4.60 dB (FR2a) ±5.20 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-4 in TR 38.903) |
| 6.2.2 UE maximum output power for modulation / channel bandwidth | PC3 Quiet Zone size ≤ 30 cm ±4.92 dB (FR2a) ±5.10 dB (FR2b) | MTSU = 1.00 x MU (from Table B.4.4.2 in TR 38.903) |
| 6.2.3 UE maximum output power with additional requirements | TBD |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD | MTSU = 1.00 x MU (from Table B.4.2-2 in TR 38.903) |
| 6.3.1 Minimum output power | PC1 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm FFS (FR2a) FFS dB (FR2b) PC3 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm ±6.15 dB (FR2a & FR2b) |  |
| 6.3.2 Transmit OFF power | PC3: Quiet Zone size ≤ 30 cm ±5.49 dB (FR2a) | MTSU = 1.00 x MU (from Table B.8-2-2 in TR 38.903) |
| 6.3.3.2 General ON/OFF time mask | PC3: ON power: TBD OFF power: Quiet Zone size ≤ 30 cm ±6.15 dB (FR2a & FR2b) |  |
| 6.3.3.4 PRACH time mask | PC3: PRACH power: TBD OFF power: Quiet Zone size ≤ 30 cm ±6.15 dB (FR2a & FR2b) |  |
| 6.3.3.6 SRS time mask | TBD |  |
| 6.3.4.2 Absolute power tolerance | Same as 6.3.1 |  |
| 6.3.4.3 Relative power tolerance | PC3 Quiet Zone size ≤ 30 cm [±1.4 dB] (FR2a) [±1.4 dB] (FR2b) | MTSU = 1.00 x MU (from Table B.9a2.2-2 in TR 38.903) |
| 6.3.4.4 Aggregate power tolerance | PC3 Quiet Zone size ≤ 30 cm [±1.4 dB] (FR2a) [±1.4 dB] (FR2b) | MTSU = 1.00 x MU (from Table B.9a2.2-2 in TR 38.903) |
| 6.3A.1.1 Minimum output power for CA (2UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.2.1 Transmit OFF power for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.2 Transmit OFF power for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.3 Transmit OFF power for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO | PC3: OFF Power Quiet Zone size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |
| 6.3D.3.4 SRS time mask for UL MIMO | PC3: OFF Power Quiet Zone size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4.1 Frequency error | ± 0.01 ppm (NTC & ETC testing) | MTSU = 1.00 x MU (from B.10.1 and B.10.2 in TR 38.903) |
| 6.4.2.1 Error vector magnitude | TBD |  |
| 6.4.2.2 Carrier leakage | TBD |  |
| 6.4.2.3 In-band emissions | TBD |  |
| 6.4.2.4 EVM equalizer spectrum flatness | TBD |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation | TBD |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) | TBD |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) | TBD |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) | TBD |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) | TBD |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) | TBD |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) | TBD |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) | TBD |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) | TBD |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) | TBD |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) | TBD |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) | TBD |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) | TBD |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) | TBD |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) | TBD |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) | TBD |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) | TBD |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) | TBD |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) | TBD |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) | TBD |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) | TBD |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) | TBD |  |
| 6.5.1 Occupied bandwidth | Quiet Zone size ≤ 30cm PC3: FR2a: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.2 [%CBW] (BW 200MHz) ±1.2 [%CBW] (BW 400MHz) FR2b: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.3 [%CBW] (BW 200MHz) ±1.3 [%CBW] (BW 400MHz) FR2c: TBD |  |
| 6.5.2.1 Spectrum Emission Mask | Quiet Zone size ≤ 30 cm ±4.94 dB (FR2a) ±5.32 dB (FR2b) | MTSU = 1.00 x MU (from Table B.16.2-2, B.16.2-3 in TR 38.903) |
| 6.5.2.3 Adjacent Channel Leakage Ratio | Quiet Zone size ≤ 30cm FR2a: ±5.63 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) FR2b: ±6.09 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) | MTSU = 1.00 x MU (from Table B.17-1 in TR 38.903) |
| 6.5.3.1 Transmitter Spurious emissions | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz) ±5.11 dB (12.75GHz < f ≤ 23.45GHz) ±5.41 dB (23.45GHz < f < 40.8GHz) ±7.42 dB (40.8GHz ≤ f ≤ 66GHz) ±7.72 dB (66GHz ≤ f ≤ 80GHz) | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |
| 6.5.3.2 Spurious emission band UE co-existence | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz Protected band n260, n261, n257: ±6.00 dB Protected frequency 23.6 GHz ≤ f ≤ 24.0 GHz: ±6.00 dB Protected frequency 57 GHz ≤ f ≤ 66GHz: ±8.01 dB Protected frequency 36 GHz ≤ f ≤ 37GHz: ±6.00 dB | MTSU = 1.00 x MU (from Table B.18-1a in TR 38.903) |
| 6.5.3.3 Additional Spurious emission | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz), NS_202 ±5.70 dB (12.75GHz < f ≤ 23.45GHz), NS_202 ±6.00 dB (23.45GHz < f < 40.8GHz), NS_202, NS_203 ±8.01 dB (40.8GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band), NS_202 | MTSU = 1.00 x MU (from Table B.18-1b in TR 38.903) |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Quiet Zone size ≤ 30cm PC3: FR2a: TBD FR2b: TBD FR2c: TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) | TBD |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) | TBD |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) | TBD |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) | TBD |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD | MTSU = 1.00 x MU (from Table B.17.2-2 and Table B.17.2-4 in TR 38.309) |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) | TBD |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) | TBD |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) | TBD |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) | TBD |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.4 Transmitter Spurious emissions for CA (5UL CA) | TBD |  |
| 6.5A.3.1.5 Transmitter Spurious emissions for CA (6UL CA) | TBD |  |
| 6.5A.3.1.6 Transmitter Spurious emissions for CA (7UL CA) | TBD |  |
| 6.5A.3.1.7 Transmitter Spurious emissions for CA (8UL CA) | TBD |  |
| 6.5A.3.2.1 Spurious emission band UE co-existence for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.2.2 Spurious emission band UE co-existence for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.2.3 Spurious emission band UE co-existence for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.2.4 Spurious emission band UE co-existence for CA (5UL CA) | TBD |  |
| 6.5A.3.2.5 Spurious emission band UE co-existence for CA (6UL CA) | TBD |  |
| 6.5A.3.2.6 Spurious emission band UE co-existence for CA (7UL CA) | TBD |  |
| 6.5A.3.2.7 Spurious emission band UE co-existence for CA (8UL CA) | TBD |  |
| 6.5A.3.3.1 Additional spurious emissions for CA (2UL CA) | TBD |  |
| 6.5A.3.3.2 Additional spurious emissions for CA (3UL CA) | TBD |  |
| 6.5A.3.3.3 Additional spurious emissions for CA (4UL CA) | TBD |  |
| 6.5A.3.3.4 Additional spurious emissions for CA (5UL CA) | TBD |  |
| 6.5A.3.3.5 Additional spurious emissions for CA (6UL CA) | TBD |  |
| 6.5A.3.3.6 Additional spurious emissions for CA (7UL CA) | TBD |  |
| 6.5A.3.3.7 Additional spurious emissions for CA (8UL CA) | TBD |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause |  | Maximum Test System Uncertainty |  | Derivation of MTSU |  |
| --- | --- | --- | --- | --- | --- |
| 7.3.2 Reference sensitivity power level |  | PC3 ±5.19 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b, NTC testing) ±5.45 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.19-2-2 in TR 38.903) |  |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3.4 EIS spherical coverage |  | PC3 ±4.90 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b) |  | MTSU = 1.00 x MU (from Table B.19-2-2 in TR 38.903) |  |
| 7.4 Maximum input level |  | TBD |  |  |  |
| 7.5 Adjacent channel selectivity |  | PC3 ±7.84 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b) |  | MTSU = 1.00 x MU (from Table B.21-1 in TR 38.903) |  |
| 7.6.2 In-band blocking |  | Same as 7.5 |  |  |  |
| 7.9 Spurious emissions |  | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz For Band n257: ±5.50dB (6GHz ≤ f ≤ 12.75GHz) ±5.46dB (12.75GHz < f ≤ 23.45GHz) ±6.11dB (23.45GHz < f < 40.8GHz) ±7.65dB (40.8GHz ≤ f ≤ 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) |  | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |  |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.1.2-1. |  |  |  |  |  |

F.2	Interpretation of measurement results (normative)

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause F.1 of the present document.

If the Test System using one of the permitted test methods defined in TR38.903 [20] for a test is known to have a measurement uncertainty greater than that specified in clause F.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows:

Any additional uncertainty in the Test System over and above that specified in clause F.1 shall be used to tighten the Test Requirement, making the test harder to pass. For some tests, for example receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with clause F.1does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause F.1 had been used.

F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

F.3.1	Measurement of test environments

TBD

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) |  | PC3 Minimum peak EIRP IFF (Quiet Zone size ≤ 30 cm) 2.87 dB (FR2a), NTC 2.87 dB (FR2b), NTC 3.04 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) Max EIRP 0 dB |  | Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) |  |
| 6.2.1.1 UE maximum output power (TRP) |  | PC3 Max TRP IFF (Quiet Zone size ≤ 30 cm) 2.65 dB (FR2a) 2.77 dB (FR2b) |  | Max TRP TT = 0.60 x MTSUIFF |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) |  | PC1 TBD PC2 TBD PC3 IFF (Quiet Zone size ≤ 30 cm) 2.58 dB (FR2a) 2.58 dB (FR2b) PC4 TBD |  | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) |  |
| 6.2.2 UE maximum output power for modulation / channel bandwidth |  | PC3 Minimum peak EIRP IFF (Quiet Zone size ≤ 30 cm) 3.11 dB (FR2a) 3.11 dB (FR2b) |  | Minimum peak EIRP TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) |  |
| 6.2.3 UE maximum output power with additional requirements |  | TBD |  |  |  |
| 6.2.4 Configured transmitted power |  | TBD |  |  |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) |  | Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3.1 Minimum output power |  | PC3 Minimum EIRP IFF (Quiet Zone size ≤ 30 cm) 3.35 dB (FR2a) 3.35 dB (FR2b) |  | Minimum EIRP TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) |  |
| 6.3.2 Transmit OFF power |  | 0 dB |  |  |  |
| 6.3.3.2 General ON/OFF time mask |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD |  | ON Power TBD |  |
| 6.3.3.4 PRACH time mask |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD |  | ON Power TBD |  |
| 6.3.4.2 Absolute power tolerance |  | Same as 6.3.1 |  |  |  |
| 6.3.4.3 Relative power tolerance |  | PC3 IFF (Quiet Zone size ≤ 30 cm) [0.26 dB] (FR2a) [0.26 dB] (FR2b) |  | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |  |
| 6.3.4.4 Aggregate power tolerance |  | PC3 IFF (Quiet Zone size ≤ 30 cm) [0.26 dB] (FR2a) [0.26 dB] (FR2b) |  | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.2.1 Transmit OFF power for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.2.2 Transmit OFF power for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.2.3 Transmit OFF power for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3D.3.4 SRS time mask for UL MIMO |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4.1 Frequency error |  | 0.005 ppm (NTC & ETC testing) |  | TT = 0.5 x MTSU |  |
| 6.4.2.1 Error vector magnitude |  | 0%, up to 64QAM |  | Minimum requirement + TT |  |
| 6.4.2.2 Carrier leakage |  | TBD |  |  |  |
| 6.4.2.3 In-band emissions |  | TBD |  |  |  |
| 6.4.2.4 EVM equalizer spectrum flatness |  | TBD |  |  |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation |  | TBD |  |  |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) |  | TBD |  |  |  |
| 6.5.1 Occupied bandwidth |  | 0 kHz |  | Minimum requirement + TT |  |
| 6.5.2.1 Spectrum Emission Mask |  | IFF (Quiet Zone size ≤ 30 cm) 3.21 dB (FR2a) 3.46 dB (FR2b) |  | TT = 0.65 x MTSUIFF |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio |  | Absolute requirement 0 dB Relative requirement IFF (Quiet Zone size ≤ 30 cm) FR2a: ±4.66 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) FR2b: ±4.96 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5.3.1 Transmitter Spurious emissions |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.2 Spurious emission band UE co-existence |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.3 Additional spurious emission |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |  |  |  |

F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 7.3.2 Reference sensitivity power level |  | PC3 IFF (Quiet Zone size ≤ 30 cm) 2.34 dB (FR2a, FR2b, NTC) 2.45 dB (FR2a, FR2b, ETC) |  | TT = 0.45 x MTSUIFF |  |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3.4 EIS spherical coverage |  | IFF (Quiet Zone size ≤ 30 cm, FR2a, FR2b) 2.21 dB |  | PC3 TT = 0.45 x MTSUIFF |  |
| 7.4 Maximum input level |  | TBD |  |  |  |
| 7.5 Adjacent channel selectivity |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6.2 In-band blocking |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.9 Spurious emissions |  | 0 dB |  | Minimum requirement + TT T-put limit unchanged |  |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.3.2-1. |  |  |  |  |  |

Annex G (normative): Uplink Physical Channels

<<End of change>>