3GPP TSG-RAN WG5 Meeting #92-e	R5-214899

# Electronic Meeting, 16th Aug 2021 – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2407 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC TC 8.2.3.11.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR-DC TC 8.2.3.11.3 Processing delay / Latency check / NR-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR-DC TC 8.2.6.2.2 Measurement configuration control and reporting / Measurement Gaps / NR-DC is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC WP will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.3.11.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.2.3.11	Measurement configuration control and reporting / Measurement Gaps

##### 8.2.3.11.3	Measurement configuration control and reporting / Measurement Gaps / NR-DC

8.2.3.11.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with NR-DC, and SCG }

ensure that {

when { UE receives RRCReconfiguration message containing MeasConfig for the PCell to setup gapFR2 and refServCellIndicator set to pCell to report periodical measurements of NR serving PCell and neighbor cell on FR2 frequency }

then { UE applies gapFR2, calulates the gaps using pCell as the reference cell and sends periodical measurements for NR serving PCell and neighbor cell on FR2 frequency }

}

8.2.3.11.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.5.2.9. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.5.2.9]

The UE shall:

…

1>	if gapFR2 is set to setup:

2>	if an FR2 measurement gap configuration is already setup, release the FR2 measurement gap configuration;

2>	setup the FR2 measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	if mgta is configured, apply the specified timing advance to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR2 is set to release:

2>	release the FR2 measurement gap configuration;

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	if mgta is configured, apply the specified timing advance to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapUE is set to release:

2>	release the per UE measurement gap configuration.

…

NOTE 1: For gapFR2 configuration, the SFN and subframe of a serving cell on FR2 frequency is used in the gap calculation

8.2.3.11.3.3	Test description

8.2.3.11.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 10 is the PSCell and NR Cell 3 is Neighbour Cell on FR2.

UE:

-	None.

Preamble:

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC) and Bearers (MCG and SCG) established according to TS 38.508-1 [4] Table 4.5.1-1.

8.2.3.11.3.3.2	Test procedure sequence

Table 8.2.3.11.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.3.11.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -82 | -91 | Switch on NR neighbour Cell and UE start to perform NR interRAT measurement. |

Table 8.2.3.11.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to setup gapFR2 to report periodical measurements for NR serving Cell 1 and NR neighbor cell 3 on FR2 frequency. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message to confirm the setup of gapFR2 and report periodical measurements for NR serving Cell 1 and NR neighbor cell 3 on FR1 frequency. | --> | RRCReconfigurationComplete | - | - |
| 3 | Wait and ignore MeasurementReport messages for 8 s to allow for UE to measure the neighbouring cells. | - | - | - | - |
| - | EXCEPTION: In parallel to events described in step 14 the steps specified in table 8.2.3.11.1.3.2-3 shall take place. | - | - | - | - |
| 4 | Wait for 30 s to ensure that the UE performs a periodical inter frequency reporting. | - | - | - | - |
| - | EXCEPTION: Steps 15 to 17 shall be repeated for k=1 to 11 (increment=1). Note: skip the gap pattern among #2 - #11 not supported by the UE according to the PICS setting given in TS 38.508-2[5] table A.4.3.6-1. | - | - | - | - |
| 5 | The SS transmits an RRCReconfiguration including measConfig to change gapFR2. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message to confirm the change of gapFR2. | --> | RRCReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to events described in step 17 the steps specified in table 8.2.3.11.1.3.2-3 shall take place. | - | - | - | - |
| 7 | Wait for 30 s to ensure that the UE performs a periodical inter frequency reporting. | - | - | - | - |
| 8 | SS transmits an RRCReconfiguration message including measConfig to release gapFR2 and measid of periodical measurements. | <-- | RRCReconfiguration | - | - |
| 9 | The UE transmits an RRCReconfigrationComplete message. | --> | RRCReconfigurationComplete | - | - |

Table 8.2.3.11.3.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: After the 1st message is received, step 1 below shall be repeated every time the duration indicated in the IE reportInterval has elapsed. | - | - | - | - |
| 1 | Check: Does the UE transmit a MeasurementReport message to perform periodical interRAT reporting for NR serving Cell 1 and NR Cell 3? | --> | MeasurementReport | 1 | P |

8.2.3.11.3.3.3	Specific message contents

Table 8.2.3.11.3.3.3-1: RRCReconfiguration

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 8.2.3.11.3.3.3-2: MeasConfig (Table 8.2.3.11.3.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList | Not present |  | Step 5 |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  | Step 1 |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 | Table 8.2.3.11.3.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 | Table 8.2.3.11.3.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList | Not present |  | Step 5 |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  | Step 1 |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR with condition PERIODICAL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE | Not present |  | Step 5 |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  | Step 1 |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-gapFR2 | Table 8.2.3.11.3.3.3-5 |  |
| } |  |  |  |

Table 8.2.3.11.3.3.3-3: MeasObjectNR-f1 (Table 8.2.3.11.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| smtc1 SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 10 | To make sure SMTC for intra-frequency measurement is non-overlapping with MG |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.2.3.11.3.3.3-4: MeasObjectNR-f2 (Table 8.2.3.11.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 3 |  |  |
| smtc1 SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.2.3.11.3.3.3-5: MeasGapConfig-gapFR2 (Table 8.2.3.11.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-70 with Condition GAP_FR2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 0 |  |  |
| mgl | ms1dot5 |  | Pattern #20, #21, #22 or #23 |
|  | ms3dot5 |  | Pattern #16, #17, #18 or #19 |
|  | ms5dot5 |  | Pattern #12, #13, #14 or #15 |
| mgrp | ms20 |  | Pattern #12, #16 or #20 |
|  | ms40 |  | Pattern #13, #17 or #21 |
|  | ms80 |  | Pattern #14, #18 or #22 |
|  | ms160 |  | Pattern #15, #19 or #23 |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.11.3.3.3-6: MeasurementReport (step 1, Table 8.2.3.11.3.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry | Measurement report for NR Cell 1 |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 10 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | (0..127) |  | pc_ss_SINR_Meas |
|  | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry | Measurement report for NR Cell 3 |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 3 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | (0..127) |  | pc_ss_SINR_Meas |
|  | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |