3GPP TSG-RAN5 Meeting #92-e	R5-215038

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1390 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR HST RRM TC 6.1.1.7-HST intra-freq cell reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1．Intention of step 7a in test procedure of TC 6.1.1.7 is to ensure that UE falls back to idle state. However, It seems that step 7 is forgotten to be updated accordingly: 7. If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 8. Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 12.. As a result, the new-added step 7a won't take effect since UE will directly go to step 8 if UE reselects to Cell 2 within T2. BTW, RRC release procedure is also needed after step 11 of 6.1.1.1 to ensure UE camps on cell 1. So step 11 is also needed to be updated accordingly. |
| Summary of change: |  | - Test procedure is updated. - Editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confirmant UE may fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

6.1.1.7.1	Test purpose

The purpose of this test is to verify that when the current and target cell operates on the same carrier frequency the UE configured with highSpeedMeasFlag-r16 is able to search and measure cells to meet the intra-frequency NR cell re-selection requirements.

6.1.1.7.2	Test applicability

This test applies to all types of NR UE from release 16 onwards that supports measurement enhancements in high speed scenario

6.1.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.1.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.1.1.7.

6.1.1.7.4	Test description

6.1.1.7.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 6.1.1.7.4.1-1.

Table 6.1.1.7.4.1-1: Supported test configurations for NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 6.1.1.7-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.1.1.7-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.1.1.7-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.1.1.7.4.1-2.

Table 6.1.1.7.4.1-2: Initial conditions for NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.1.1.7.4.1-3.

2. Message contents are defined in clause 6.1.1.7.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 6.1.1.7.4.1-3: General test parameters for NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial | Active cell |  | 1, 2, 3 | Cell1 |  |
| condition | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.32 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

6.1.1.7.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. highSpeedMeasFlag-r16 is broadcasted to UE.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1. Set Cell 2 physical cell identity = initial Cell 2 physical cell identity.

2.	Set the parameters according to T1 in Table 6.1.1.7.5-1. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.1.1.7.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 34 seconds from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7a.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 12.

7a	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 6.1.1.7.5-1. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 8 seconds from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, continues with step 11a.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 12.

11a.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5 on Cell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

6.1.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.1.1.7.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks on condition of HighSpeedMeas with exceptions | Table H.2.1-1 with Condition SMTC.2 and Asynchronous cells for configuration 6.1.1.7-1 Table H.2.1-1 with Condition SMTC.1 and synchronous cells for configuration 6.1.1.7-2 Table H.2.1-1 with Condition SMTC.1 and synchronous cells for configuration 6.1.1.7-3 Table H.2.1-2 Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions |  |

6.1.1.7.5	Test requirement

Tables 6.1.1.7.4.1-3 and 6.1.1.7.5-1 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16.

Table 6.1.1.7.5-1: Cell specific test parameters for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 |  | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.2.1-1 |  |  | OP.1 defined in A.2.1-1 |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |  |
|  | dB | 1 | 16+TT | -3.11+ TT | 2.79+TT | -infinity |  | 2.79+TT | -3.11+ TT |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98+TT |  |  |  |  |  |  |
|  |  | 2 | -98+TT |  |  |  |  |  |  |
|  |  | 3 | -95+TT |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98+TT |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
|  | dB | 1 | 16+TT | 13+TT | 16+TT | -infinity |  | 16+TT | 13+TT |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82+TT | -85+TT | -82+TT | -infinity |  | -82+TT | -85+TT |
|  |  | 2 | -82+TT | -85+TT | -82+TT | -infinity |  | -82+TT | -85+TT |
|  |  | 3 | -79+TT | -82+TT | -79+TT | -infinity |  | -79+TT | -82+TT |
| Io | dBm/9.36 MHz | 1 | -53.94+ TT | -52.21+ TT | -52.21+ TT | specified in Cell 1 columns- |  |  |  |
|  | dBm/9.36 MHz | 2 | -53.94+ TT | -52.21+ TT | -52.21+ TT |  |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85+ TT | -46.12+ TT | -46.12+ TT |  |  |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | N50 |  |  | N50 |  |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  | AWGN 1944Hz Note4 |  |  |
| Propagation Condition |  | 3 | AWGN |  |  |  | AWGN 3334Hz Note5 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. Note 5: The AWGN 3334 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell test requirement in this case is expressed as:

Cell re-selection delay to a newly detectable cell = Tdetect,NR_Intra + TSI-NR

Tdetect,NR_Intra = 2.56 s; as specified in Table 4.2.2.3-2 in clause 4.2.2.3 TS 38.133 [6].

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a newly detectable cell shall be less than a total of 3.84 seconds in this test case (note: this gives a total of 3.84 seconds but the test allows 4 seconds).

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell test requirement in this case is expressed as:

Cell re-selection to an already detected cell delay = Tevaluate,NR_Intra + TSI-NR

Tevaluate,NR_Intra = 0.96 s; as specified in Table 4.2.2.3-2 in clause 4.2.2.3 TS 38.133 [6].

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to an already detected cell shall be less than a total of 2.24 seconds in this test case (note: this gives a total of 2.24 seconds but the test allows 3 seconds).

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

<End of modified section 1>