3GPP TSG-RAN5 Meeting #92-e	R5-215040

# Electronic Meeting, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5333 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of band 42 and 43 to A-MPR tests for a UE category M2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_bands_R16_M2_NB2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A-MPR tests for cat M2 of Rel-16 band 42 and band 43 for NS_22 and NS_23 is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NS_22 and NS_23 requirements added to Table 6.2.4EC-1. - Test configuration tables for NS_22 and NS_23 added. - Test requirements for NS_22 and NS_23 added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4EC.2, 6.2.4EC.3, 6.2.4EC.4, 6.2.4EC.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

### 6.2.4EC	Additional Maximum Power Reduction (A-MPR) for UE category M2

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- NS_03 (B2, B4), NS_04 (B41), NS_07 (B13), NS_12 (B26), NS_13 (B26), NS_14 (B26), NS_15 (B26), NS_16 (B27), NS_17 (B28) and NS_18 (B28): Minimum requirements in 36.101 are incomplete. Test configuration table and test requirements are missing

#### 6.2.4EC.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2EC-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

#### 6.2.4EC.2	Test applicability

The requirements of this test apply in test case 6.6.2.2EC Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07 and NS_35 to all types of E-UTRA UE release 14 and forward of UE category M2.

The requirements of this test apply in test case 6.6.3EC.3 Additional Spurious Emissions for network signalled values NS_05, NS_07, NS_08, NS_09, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_22, NS_23 to all types of E-UTRA UE release 14 and forward of UE category M2.

NOTE:	As a result TC 6.2.4EC has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4EC.3	Minimum conformance requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2.2EA-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2.4EC.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4EC.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2.3EC.3.

Table 6.2.4EC.3-1: Additional Maximum Power Reduction (A-MPR) for category M2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Narrowband bandwidth | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.3 | Table 5.2-1 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_03 | 6.6.2.2.3.1 | 2, 4 | [TBD] | [TBD] | [TBD] |
| NS_04 | 6.6.2.2.3.2 | 41 | [TBD] | [TBD] | [TBD] |
| NS_05 | 6.6.3.3.3.2 | 1 | 1.4, 3, 5 | Table 6.2.4EC.3-2 |  |
| NS_06 | 6.6.2.2.3.3 | 12, 13, 14, 85 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_07 | 6.6.2.2.3.3 6.6.3.3.3.3 | 13 |  | [TBD] |  |
| NS_08 | 6.6.3.3.3.4 | 19 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_09 | 6.6.3.3.3.5 | 21 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_10 |  | 20 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_12 | 6.6.3.3.3.7 | 26 | [TBD] | [TBD] |  |
| NS_13 | 6.6.3.3.3.8 | 26 | [TBD] | [TBD] | [TBD] |
| NS_14 | 6.6.3.3.3.9 | 26 | N/A | Table 5.4.2-1 | N/A |
| NS_15 | 6.6.3.3.3.10 | 26 | [TBD] | [TBD] |  |
| NS_16 | 6.6.3.3.3.11 | 27 | [TBD] | [TBD] | [TBD] |
| NS_17 | 6.6.3.3.3.12 | 28 | [TBD] | [TBD] | [TBD] |
| NS_18 | 6.6.3.3.3.13 | 28 | [TBD] | [TBD] | [TBD] |
| NS_22 | 6.6.3.3.3.17 | 42, 43 | Table 5.4.2-1 | N/A | NS_22 |
| NS_23 | 6.6.3.3.3.18 | 42, 43 | Table 5.4.2-1 | N/A | NS_23 |
| NS_32 | - | - |  | - | - |
| NS_35 | 6.6.2.2.3.5 | 71 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |

No other A-MPR requirement than those specified in table 6.2.4EC.3-1 applies to category M2 UE.

…

Table 6.2.4EC.3-2: A-MPR requirements for "NS_05" for Cat-M2 power class 3 UE

| BW [MHz] | 20 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Fc [MHz] | 1930 |  |  |  |  |
| (NBindex,RBstart) | (0,<6) | (0,<6) | (1,<6) | (2,<6) | (2,<6) |
| LCRB | 9, 12 | ≥15 | ≥15 | 18 | 24 |
| A-MPR [dB] | ≤1 | ≤4 | ≤3 | ≤1 | ≤2 |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [8]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [9]. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 clause 6.2.4 and clause 6.2.4E.

#### 6.2.4EC.4	Test description

##### 6.2.4EC.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.2.4EC.4.1-1 to 6.2.4EC.4.1-16. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2.4EC.4.1-1: Test Configuration Table (network signalled value "NS_03")

TBD

Table 6.2.4EC.4.1-2: Test Configuration Table (network signalled value "NS_04")

TBD

Table 6.2.4EC.4.1-3: Test Configuration Table (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz, 20MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 24 | 0 |
| 3 (Note 4) | 20MHz |  | QPSK | 6 | 0 |
| 4 (Note 4) | 20MHz |  | QPSK | 9 | 0 |
| 5 (Note 4) | 20MHz |  | QPSK | 12 | 0 |
| 6 (Note 4) | 20MHz |  | QPSK | 18 | 0 |
| 7 (Note 4) | 20MHz |  | QPSK | 12 | 1 |
| 8 (Note 4) | 20MHz |  | QPSK | 18 | 1 |
| 9 (Note 4) | 20MHz |  | QPSK | 12 | 2 |
| 10 (Note 4) | 20MHz |  | QPSK | 18 | 2 |
| 11 (Note 4) | 20MHz |  | QPSK | 24 | 2 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0. Note 4: Only for power class 3 UE |  |  |  |  |  |

Table 6.2.4EC.4.1-4: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 (Note 4) | 5MHz |  | QPSK | 1 | 0 |
| 7 (Note 5) | 5MHz |  | QPSK | 3 | 0 |
| 8 | 5MHz |  | QPSK | 24 | 0 |
| 9 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 10 | 5MHz |  | 16QAM | 3 | 0 |
| 11 (Note 5) | 5MHz |  | 16QAM | 5 | 0 |
| 12 | 5MHz |  | 16QAM | 24 | 0 |
| 13 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 14 (Note 5) | 10MHz |  | QPSK | 5 | 0 |
| 15 | 10MHz |  | QPSK | 24 | 0 |
| 16 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 17 | 10MHz |  | 16QAM | 5 | 0 |
| 18 | 10MHz |  | 16QAM | 24 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 (Note 4) | 5MHz |  | QPSK | 1 | 3 |
| 7 (Note 5) | 5MHz |  | QPSK | 3 | 3 |
| 8 | 5MHz |  | QPSK | 24 | 0 |
| 9 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 10 | 5MHz |  | 16QAM | 3 | 3 |
| 11 (Note 5) | 5MHz |  | 16QAM | 5 | 3 |
| 12 | 5MHz |  | 16QAM | 24 | 0 |
| 13 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 14 (Note 5) | 10MHz |  | QPSK | 5 | 7 |
| 15 | 10MHz |  | QPSK | 24 | 4 |
| 16 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 17 | 10MHz |  | 16QAM | 5 | 7 |
| 18 | 10MHz |  | 16QAM | 24 | 4 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 UE |  |  |  |  |  |

Table 6.2.4EC.4.1-5: Test Configuration Table (network signalled value "NS_07")

TBD

Table 6.2.4EC.4.1-6: Test Configuration Table (network signalled value "NS_08")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 3 |
| 2 | 5MHz |  | QPSK | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB# (6 - (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

Table 6.2.4EC.4.1-7: Test Configuration Table (network signalled value "NS_09")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 3 |
| 2 | 5MHz |  | QPSK | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB# (6 - (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

Table 6.2.4EC.4.1-8: Test Configuration Table (network signalled value "NS_10")

Void, not tested

Table 6.2.4EC.4.1-9: Test Configuration Table (network signalled value "NS_12")

TBD

Table 6.2.4EC.4.1-10: Test Configuration Table (network signalled value "NS_13")

TBD

Table 6.2.4EC.4.1-11: Test Configuration Table (network signalled value "NS_14")

TBD

Table 6.2.4EC.4.1-12: Test Configuration Table (network signalled value "NS_15")

TBD

Table 6.2.4EC.4.1-13: Test Configuration Table (network signalled value "NS_16")

TBD

Table 6.2.4EC.4.1-14: Test Configuration Table (network signalled value "NS_22")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5 MHz |  | QPSK | 1 | 3 |
| 2 | 5 MHz |  | QPSK | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EC.4.1-15: Test Configuration Table (network signalled value "NS_23")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5 MHz |  | QPSK | 1 | 3 |
| 2 | 5 MHz |  | QPSK | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.2.4EC.4.1-16: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 (Note 4) | 5MHz |  | QPSK | 1 | 0 |
| 2 (Note 5) | 5MHz |  | QPSK | 3 | 0 |
| 3 | 5MHz |  | QPSK | 24 | 0 |
| 4 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 5 | 5MHz |  | 16QAM | 3 | 0 |
| 6 (Note 5) | 5MHz |  | 16QAM | 5 | 0 |
| 7 | 5MHz |  | 16QAM | 24 | 0 |
| 8 | 10MHz |  | QPSK | 4 | 0 |
| 9 (Note 5) | 10MHz |  | QPSK | 5 | 0 |
| 10 | 10MHz |  | QPSK | 24 | 0 |
| 11 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 12 | 10MHz |  | 16QAM | 5 | 0 |
| 13 | 10MHz |  | 16QAM | 24 | 0 |
| 14 | 20MHz |  | QPSK | 18 | 0 |
| 15 | 20MHz |  | QPSK | 24 | 0 |
| 16 | 20MHz |  | 16QAM | 18 | 0 |
| 17 | 20MHz |  | 16QAM | 24 | 0 |
| High range |  |  |  |  |  |
| 1 (Note 4) | 5MHz |  | QPSK | 1 | 3 |
| 2 (Note 5) | 5MHz |  | QPSK | 3 | 3 |
| 3 | 5MHz |  | QPSK | 24 | 0 |
| 4 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 5 | 5MHz |  | 16QAM | 3 | 3 |
| 6 (Note 5) | 5MHz |  | 16QAM | 5 | 3 |
| 7 | 5MHz |  | 16QAM | 24 | 0 |
| 8 | 10MHz |  | QPSK | 4 | 7 |
| 9 (Note 5) | 10MHz |  | QPSK | 5 | 7 |
| 10 | 10MHz |  | QPSK | 24 | 4 |
| 11 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 12 | 10MHz |  | 16QAM | 5 | 7 |
| 13 | 10MHz |  | 16QAM | 24 | 4 |
| 14 | 20MHz |  | QPSK | 18 | 13 |
| 15 | 20MHz |  | QPSK | 24 | 12 |
| 16 | 20MHz |  | 16QAM | 18 | 13 |
| 17 | 20MHz |  | 16QAM | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 UE |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.4EC.4.1-1 to Table 6.2.4EC.4.1-16.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.2.4EC.4.3.

##### 6.2.4EC.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via M-PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.4EC.4.1-1 to Table 6.2.4EC.4.1-16. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

##### 6.2.4EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and with the same exceptions as in clauses 6.2.4.4.3.1 to 6.2.4.4.3.30 for each network signalled value.

#### 6.2.4EC.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4EC.5-1 to Table 6.2.4EC.5-16. The allowed A-MPR values specified in Table 6.2.4EC.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3EC. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5EA.3-1 and 6.2.5EA.3-2 apply.

Table 6.2.4EC.5-1: UE Power Class test requirements (network signalled value "NS_03")
(for band 2 and band 4)

FFS

Table 6.2.4EC.5-2: UE Power Class test requirements (network signalled value "NS_04")
(for band 41)

FFS

Table 6.2.4EC.5-3: UE Power Class test requirements (network signalled value "NS_05")
(for band 1)

| Configuration ID | A-MPR (dB) | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 2 | 0 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 3 | 0 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 4 | 1 | 0 | 22.0 | 2.0 | 25.7 | 19.3 | N/A |  |  |  |  |
| 5 | 1 | 0 | 22.0 | 2.0 | 25.7 | 19.3 | N/A |  |  |  |  |
| 6 | 4 | 0 | 19.0 | 3.5 | 25.7 | 14.8 | N/A |  |  |  |  |
| 7 | 0 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 8 | 3 | 0 | 20.0 | 2.5 | 25.7 | 16.8 | N/A |  |  |  |  |
| 9 | 0 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 10 | 1 | 0 | 22.0 | 2.0 | 25.7 | 19.3 | N/A |  |  |  |  |
| 11 | 2 | 1 | 20.0 | 2.5 | 25.7 | 16.8 | N/A |  |  |  |  |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EC.5-4A: UE Power Class test requirements without TC,c (network signalled value "NS_06")
(for band 13, band 14 and for Mid range in band 12, band 85)

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 2 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 3 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 4 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 5 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 6 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 7 | N/A |  |  |  |  | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 8 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 9 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 10 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 11 | N/A |  |  |  |  | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 12 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 13 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 14 | N/A |  |  |  |  | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 15 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 16 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 17 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 18 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EC.5-4B: UE Power Class test requirements with TC,c (network signalled value "NS_06")
(for Low/High range in band 12 and band 85)

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 2 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 3 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5.0 | 22.7 | 10.8 |
| 4 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 5 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5.0 | 22.7 | 10.8 |
| 6 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 7 | N/A |  |  |  |  | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 8 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 9 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 10 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 11 | N/A |  |  |  |  | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 12 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 13 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 14 | N/A |  |  |  |  | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 15 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 16 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 17 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 18 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EC.5-5: UE Power Class test requirements (network signalled value "NS_07")
(for band 13)

FFS

Table 6.2.4EC.5-6: UE Power Class test requirements (network signalled value "NS_08")
(for band 19)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 19 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EC.5-7: UE Power Class test requirements (network signalled value "NS_09")
(for band 21)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 19 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EC.5-8: UE Power Class test requirements (network signalled value "NS_10")
(for band 20)

Void, not tested

Table 6.2.4EC.5-9: UE Power Class test requirements (network signalled value "NS_12")
(for band 26)

FFS

Table 6.2.4EC.5-10: UE Power Class test requirements (network signalled value "NS_13")
(for band 26)

FFS

Table 6.2.4EC.5-11: UE Power Class test requirements (network signalled value "NS_14")
(for band 26)

FFS

Table 6.2.4EC.5-12: UE Power Class test requirements (network signalled value "NS_15")
(for band 26)

FFS

Table 6.2.4EC.5-13: UE Power Class test requirements (network signalled value "NS_16")
(for band 27)

FFS

Table 6.2.4EC.5-14: UE Power Class test requirements (network signalled value "NS_22")
(for band 42 and 43)

| Configuration ID | MPR (dB) | PCMAX_L,c (dBm) |  | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Power class 3 |  |  |  |  |  |  |
| 1 | 0 | 23 | 2 |  | 26 | 20 |
| 2 | 1 | 22 | 2 |  | 26 | 19 |
| Power class 5 |  |  |  |  |  |  |
| 1 | 0 | 20 | 2 |  | 23 | 17 |
| 2 | 1 | 19 | 3.5 |  | 24.5 | 14.5 |
| Power class 6 |  |  |  |  |  |  |
| 1 | 0 | 14 | 2.5 |  | 17.5 | 10.5 |
| 2 | 1 | 13 | 2.5 |  | 17.5 | 9.5 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |

Table 6.2.4EC.5-15: UE Power Class test requirements (network signalled value "NS_23")
(for band 42 and 43)

| Configuration ID | MPR (dB) | PCMAX_L,c (dBm) |  | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Power class 3 |  |  |  |  |  |  |
| 1 | 0 | 23 | 2 |  | 26 | 20 |
| 2 | 1 | 22 | 2 |  | 26 | 19 |
| Power class 5 |  |  |  |  |  |  |
| 1 | 0 | 20 | 2 |  | 23 | 17 |
| 2 | 1 | 19 | 3.5 |  | 24.5 | 14.5 |
| Power class 6 |  |  |  |  |  |  |
| 1 | 0 | 14 | 2.5 |  | 17.5 | 10.5 |
| 2 | 1 | 13 | 2.5 |  | 17.5 | 9.5 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |

Table 6.2.4EC.5-16: UE Power Class test requirements (network signalled value "NS_35")
(for band 71)

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 2 | 1 | N/A |  |  |  | 0 | 19.0 | 3.5 | 0 | 17.3 |
| 3 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 4 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 5 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 0 | 19.0 | 3.5 | 22.7 | 17.3 |
| 6 | 2 | N/A |  |  |  | 1 | 18.0 | 4.0 | 1 | 13.3 |
| 7 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 8 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 9 | 1 | N/A |  |  |  | 0 | 19.0 | 3.5 | 0 | 14.8 |
| 10 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 11 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 12 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 0 | 19.0 | 3.5 | 22.7 | 17.3 |
| 13 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 1 | 18.0 | 4.0 | 22.7 | 14.8 |
| 14 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 15 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 16 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 17 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |

<< End of changes>>