3GPP TSG-RAN5 Meeting #92-e	R5-215128

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0235 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of PC2 EN-DC configuration into 38.508-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ENDC_UE_PC2_R17_NR_TDD-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | EN-DC PC2 UE configuration DC_1A-n78C are are already defined in 38.101-3. The RF baseline implementation capabilities for HPUE DC_1A-n78C is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added DC_1A_n78C into EN-DC PC2 baseline implementation capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RF baseline implementation capabilities for PC2 DC_1A_n78C will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2B.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
|  |  |  |  |  |  |  |  |  |  |  | x |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

Table A.4.3.2B.2.3.1-3: Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities

| Item | Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | LTE Frequency band: 1880-1920 MHz NR Frequency band: 2496-2690 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band39_nrBand41_ PC2_Supp |  | DC_39A_n41A |  |
| 2 | LTE Frequency band: 1880-1920 MHz NR Frequency band: 4400-5000 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band39_nrBand79_ PC2_Supp |  | DC_39A_n79A |  |
| 3 | LTE Frequency band: 2496-2690 MHz NR Frequency band: 4400-5000 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band41_nrBand79_ PC2_Supp |  | DC_41A_n79A |  |
| 4 | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 3300-3800 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band3_nrBand78_ PC2_Supp |  | DC_3A_n78A DC_3C_n78A |  |
| 5 | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 2496-2690 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band3_nrBand41_ PC2_Supp |  | DC_3A_n41A DC_3A_n41C DC_3C_n41A |  |
| 6 |  | LTE Frequency band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) NR Frequency band: 3300-3800 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band1_nrBand78_ PC2_Supp |  | DC_1A_n78A DC_1A_n78C |

Table A.4.3.2B.2.3.1-3a: Inter-band EN-DC within FR1 (two bands) NR part power class UE RF Baseline Implementation Capabilities

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported NR part power class |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_39A_n41A | DC_39A_n41A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band39_nrBand41_powerClassNRPart_r16 |  |
| 2 | DC_39A_n79A | DC_39A_n79A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band39_nrBand79_powerClassNRPart_r16 |  |
| 3 | DC_41A_n79A | DC_41A_n79A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band41_nrBand79_powerClassNRPart_r16 |  |
| 4 | DC_3A_n78A DC_3C_n78A | DC_3A_n78A NR part power class DC_3A_n78A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band3_nrBand78_powerClassNRPart_r16 |  |
| 5 | DC_3A_n41A DC_3A_n41C DC_3C_n41A | DC_3A_n41A NR part power class DC_3A_n41C NR part power class DC_3C_n41A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band3_nrBand41_powerClassNRPart_r16 |  |
| 6 | DC_1A_n78A DC_1A_n78C | DC_1A_n78A NR part power class DC_1A_n78A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band1_nrBand78_powerClassNRPart_r17 |  |

<<< END OF CHANGES >>>