3GPP TSG-RAN5 Meeting #92-e			R5-215197

Electronic Meeting, 16th – 27th August 2021

Source:	Apple Italia S.R.L.

Title:	Discussion on AMPR edge RB allocation for NS

Agenda Item:	5.3.2.17

Document for:	Discussion and Endorsement

- Introduction:

In 38.521-1 specification, it is observed that for edge RB, A-MPR value is not explicitly mentioned for several NS scenarios. However, A-MPR values for “Outer” and “Inner” values are indicated under a row below values for different regions but edge RB allocation is omitted.

- Discussion:

We can take an example of NS_21. A-MPR values for “Outer” and “Inner” are indicated under a row below values for different regions (Regions A1a, A1b, A2, A3b and A3a). Interpreting regions A1a and A3a based on formula stated covers outermost allocations at the channel edges.  If we consider 1RB@0 scenario which falls in Region A1a (RBstart ≤ 8, LCRB ≤ 3), we should choose 6 dB AMPR for edge allocation but because of “Outer/Inner” row it creates confusion about selecting AMPR value for such scenario.

It appears that row with phrases like “Outer/Inner” and “Outer” is redundant information and creating confusion for the A-MPR table for NS_21.

Similar issues are found for other NS (example: NS_24, NS_27, NS_46 etc.). This issue requires clarification and further analysis.

One option to clarify this table is omitting the redundant row in the table.

However, any changes in these tables will require a detail study and clarification from RAN4. RAN5 to request RAN4 group to clarify band edge requirement for NS_21 and other NS values where AMPR requirement is defined as “Outer/Inner” but edge requirement is omitted.

- Proposal:

RAN5 to send LS to RAN4 group to clarify band edge requirement for NS_21 and other NS values where AMPR requirement is defined as “Outer/Inner” but edge requirement is omitted.

Note: Appendix A added for draft LS

Appendix A

Proposed Draft LS

3GPP TSG-RAN5 Meeting #92-e			R5-21xxxx

Electronic Meeting, 16th – 27th August 2021

Title:	LS on band NS21 AMPR for edge RB scenario

Response to:

Release:	Release 15

Work Item:	5GS_NR_LTE-UEConTest

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:	N/A

Contact Person:

#### Name:	Mohammad Omar Farooq

Tel. Number:

E-mail Address:	mohammad_farooq@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

In 38.521-1 specification, it is observed that for edge RB, A-MPR value is not explicitly mentioned for several NS scenarios. However, A-MPR values for “Outer” and “Inner” values are indicated under a row below values for different regions but edge RB allocation is omitted.

We can take an example of NS_21. A-MPR values for “Outer” and “Inner” are indicated under a row below values for different regions (Regions A1a, A1b, A2, A3b and A3a). Interpreting regions A1a and A3a based on formula stated covers outermost allocations at the channel edges.  If we consider 1RB@0 scenario which falls in Region A1a (RBstart ≤ 8, LCRB ≤ 3), we should choose 6 dB AMPR for edge allocation but because of “Outer/Inner” row it creates confusion about selecting AMPR value for such scenario.

It appears that row with phrases like “Outer/Inner” and “Outer” is redundant information and creating confusion for the A-MPR table for NS_21.

Similar issues are found for other NS (example: NS_24, NS_27, NS_46 etc.). This issue requires clarification and further analysis.

2. Actions:

To RAN4 group.

ACTION: 	RAN5 respectfully requesting RAN4 group to clarify band edge requirement for NS_21 and other NS values where AMPR requirement is defined as “Outer/Inner” but edge requirement is omitted.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#92e 	 23rd – 27th August 2021	Electronic Meeting

TSG-RAN5 Meeting#93e 	 8th – 19th November 2021	Electronic Meeting

--- END ---