3GPP TSG-RAN5 Meeting #92-e	R5-215249

# Online, , 16th Aug 2021 - 27th Aug 2021

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.918 | CR | n/a | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Text Proposal on Test Procedure A.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2021-08-02 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test procedure of 5G NR / URSP Configuration / Signalling needs to be defined according to the work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test procedure A.2.1.1 5G NR / URSP Configuration / Signalling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 38.198 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

A.2.1.1	5G NR / URSP Configuration / Signalling

A.2.1.1.1	Definition

The provisioning of URSP rules from network to UE is one of the fundamental functional requirements for network slicing.

A.2.1.1.2	Test Purpose

To verify that UE could support the provisioning of URSP rules from network to UE over NAS messages

A.2.1.1.3	Test Parameters

Void

A.2.1.1.4	Test Description

A.2.1.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.1.1.4.2	Test Procedure

1.	UE is switched on.

2.	Steps 2 to 13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed.

3.	The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Configured NSSAI.

4.	The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message.

5.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to configure URSP rules

6.	The UE transmits an UL NAS TRANSPORT message including UE Policy Container and a MANAGE UE POLICY COMPLETE message is included

7.	The SS transmits an RRCRelease message

Table A.2.1.1.4.2-1: REGISTRATION ACCEPT (step 3)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000001 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000002 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000002 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

Table A.2.1.1.4.2-2: DL NAS TRANSPORT (step 5)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0101’B | UE policy container |  |
| Payload container |  | See Table A.2.1.1.4.2-2 |  |  |

Table A.2.1.1.4.2-2: MANAGE UE POLICY COMMAND (step 5)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘10001000’B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000001 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000100’B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘10100000’B |  | OS App Id type |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of 'OS App Id' in bytes |  |  |
| OS App Id |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000002 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000100’B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

<<< END OF CHANGES >>>