3GPP TSG-RAN5 Meeting # 92-e	R5-215267

# Electronic Meeting, 16th - 27h Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.544 | CR | 0031 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of D.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intertek, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UTRA FDD Band XIX was missed in Table D.2-1.The frequency range of Band XIX is inside the range of Band V |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Band XIX in the Table D.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test spec. will be incompleted |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of the change>>

# D.2	Test tolerances

- Test tolerances in Table D.2-1 are used to relax the Minimum Requirements in the present document and to derive the Test Requirements.

- Table D.2-1: Test Tolerances for OTA tests

| - Clause | - Test Tolerance |
| --- | --- |
| - 6.1.1 TRP for UTRA FDD in Speech Mode with beside the Head Phantom | Bands I,II,III,IV,V,VI,VII,VIII,IX,XIX 1.0 dB for minimum requirement 0.7 dB for average requirement |
| - 6.1.2 TRP for UTRA TDD in Speech Mode with beside the Head Phantom | - Bands a,e,f - 1.0 dB for minimum requirement - 0.7 dB for average requirement |
| - 6.1.3 TRP for UTRA FDD in Speech Mode with beside the Head and Hand Phantom | 1.0 dB for minimum requirement - 0.7 dB for average requirement |
| - 6.1.4 TRP for UTRA TDD in Speech Mode with beside the Head and Hand Phantom | 1.0 dB for minimum requirement - 0.7 dB for average requirement |
| - 6.1.5 TRP for E-UTRA FDD UE in Speech Mode with beside the Head and Hand Phantom | - [TBD] |
| - 6.1.6 TRP for E-UTRA TDD UE in Speech Mode with beside the Head and Hand Phantom | - [TBD] |
| - 6.1.7 TRP for UTRA FDD UE in Browsing Mode with Hand Phantom | - [TBD] |
| - 6.1.8 TRP for UTRA TDD UE in Browsing Mode with Hand Phantom | - [TBD] |
| - 6.1.9 TRP for E-UTRA FDD UE in Browsing Mode with Hand Phantom | - [TBD] |
| - 6.1.10 TRP for E-UTRA TDD UE in Browsing Mode with Hand Phantom | - [TBD] |
| - 6.2.1 TRP for UTRA FDD LME | Bands I,II,III,IV,V,VI,VII,VIII,IX,XIX 1.0 dB for minimum requirement 0.7 dB for average requirement |
| - 6.2.2 TRP for UTRA TDD LME | - Bands a,e,f - 1.0 dB for minimum requirement - 0.7 dB for average requirement |
| - 6.2.3 TRP for E-UTRA FDD LME | - [TBD] |
| - 6.2.4 TRP for E-UTRA TDD LME | - [TBD] |
| - 6.3.1 TRP for UTRA FDD LEE | Bands I,II,III,IV,V,VI,VII,VIII,IX,XIX 1.0 dB for minimum requirement 0.7 dB for average requirement |
| - 6.3.2 TRP for UTRA TDD LEE | - Bands a,e,f - 1.0 dB for minimum requirement - 0.7 dB for average requirement |
| - 6.3.3 TRP for E-UTRA FDD LEE | 1.0 dB for minimum requirement - 0.7 dB for average requirement |
| - 6.3.4 TRP for E-UTRA TDD LEE | 1.0 dB for minimum requirement - 0.7 dB for average requirement |
| - 7.1.1 TRS for UTRA FDD in Speech Mode with beside the Head Phantom | Bands I,II,III,IV,V,VI,VII,VIII,IX,XIX 1.2 dB for maximum requirement 0.9 dB for average requirement |
| - 7.1.2 TRS for UTRA TDD in Speech Mode with beside the Head Phantom | Bands a,e,f 1.2 dB for minimum requirement 0.9 dB for average requirement |
| - 7.1.3 TRS for UTRA FDD in Speech Mode with beside the Head and Hand Phantom | 1.2 dB for maximum requirement - 0.9 dB for average requirement |
| - 7.1.4 TRS for UTRA TDD in Speech Mode with beside the Head and Hand Phantom | 1.2 dB for maximum requirement - 0.9 dB for average requirement |
| - 7.1.5 TRS for E-UTRA FDD in Speech Mode with beside the Head and Hand Phantom | - [TBD] |
| - 7.1.6 TRS for E-UTRA TDD in Speech Mode with beside the Head and Hand Phantom | - [TBD] |
| - 7.1.7 TRS for UTRA FDD in Browsing Mode with Hand Phantom | - [TBD] |
| - 7.1.8 TRS for UTRA TDD in Browsing Mode with Hand Phantom | - [TBD] |
| - 7.1.9 TRS for E-UTRA FDD in Browsing Mode with Hand Phantom | - [TBD] |
| - 7.1.10 TRS for E-UTRA TDD in Browsing Mode with Hand Phantom | - [TBD] |
| - 7.2.1 TRS for UTRA FDD LME | Bands I,II,III,IV,V,VI,VII,VIII,IX,XIX 1.2 dB for maximum requirement 0.9 dB for average requirement |
| - 7.2.2 TRS for UTRA TDD LME | Bands a,e,f 1.2 dB for minimum requirement 0.9 dB for average requirement |
| - 7.2.3 TRS for E-UTRA FDD LME | - [TBD] |
| - 7.2.4 TRS for E-UTRA TDD LME | - [TBD] |
| - 7.3.1 TRS for UTRA FDD LEE | Bands I,II,III,IV,V,VI,VII,VIII,IX,XIX 1.2 dB for maximum requirement - 0.9 dB for average requirement |
| - 7.3.2 TRS for UTRA TDD LEE | Bands a,e,f 1.2 dB for minimum requirement - 0.9 dB for average requirement |
| - 7.3.3 TRS for E-UTRA FDD LEE | 1.2 dB for maximum requirement - 0.9 dB for average requirement |
| - 7.3.4 TRS for E-UTRA TDD LEE | 1.2 dB for maximum requirement - 0.9 dB for average requirement |
| - 7.4.1 Radiated Performance of Multiple-antenna Receivers FDD | 1 dB |
| - 7.4.2 Radiated Performance of Multiple-antenna Receivers TDD | 1 dB |

<< end of the change>>