Source: 	LG Electronics, Huawei

Title: 	Test point analysis on 6.5A.1.1 Occupied Bandwidth for CA (2UL CA)

Document for:	Endorsement

Introduction

The specific analysis for generic initial test conditions, environmental conditions, test frequencies, channel bandwidth, SCS, RB allocation is required in accordance with the agreement in RAN5#78 [1], [2]. The general agreement is that FR1 testing mostly follows similar requirement as for LTE.

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Occupied Bandwidth measurement for CA (2UL CA) in FR1.

.

Discussion

Occupied Bandwidth for CA FR1 is defined in Output RF spectrum emissions for CA section in [6] where its scope is described as follows:

For inter-band carrier aggregation with uplink assigned to two NR bands, the occupied bandwidth is defined per component carrier.

For intra-band contiguous carrier aggregation the occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated power of the transmitted spectrum. The OBW shall be less than the aggregated channel bandwidth defined in subclause 5.3A.3.

For intra-band non-contiguous carrier aggregation, the OBW requirement is met when the ratio of the transmitted power in all sub-blocks of the uplink CA configuration to the total integrated power of the transmitted spectrum is greater than 99%.

In LTE specification [5], Occupied Bandwidth for inter-band CA (2UL CA) Test case is defined as follows:

Table 6.6.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 15 | 25 |  | QPSK | 40 | P_15@0 | S_25@0 | - | - |
| 25 | 6 |  | QPSK |  | S_25@0 | P_6@0 |  |  |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 25 | 75 |  | QPSK |  | S_25@0 | P_75@0 |  |  |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 100 |  | QPSK | 175 | P_75@0 | S_100@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. |  |  |  |  |  |  |  |  |

For intra-band contiguous CA (2UL CA) Test case in LTE is defined as follows:

Table 6.6.1A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | All |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 |  | QPSK | 50 | P_25@0 | P_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | P_50@0 |  |  |
| 25 | 75 |  | QPSK | 100 | P_25@0 | P_75@0 | - | - |
| 25 | 100 | N/A for this test | QPSK | 125 | P_25@0 | S_100@0 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | P_50@0 |  |  |
| 50 | 75 |  | QPSK | 125 | P_50@0 | S_75@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 100 |  | QPSK | 175 | P_75@0 | S_100@0 | - | - |
| 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |

For intra-band non-contiguous CA (2UL CA) Test case in LTE is defined as follows:

Table 6.6.1A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Low Wgap, Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test points All PCC NRB = SCC NRB test points |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | PCC & SCC RB allocation |  | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 5 | NA |  | QPSK | P_25@0 | QPSK | S_25@0 |
| 2 | 50 | 50 | 25 |  |  | QPSK | P_50@0 | QPSK | S_50@0 |
| 3 | 75 | 75 | 15 |  |  | QPSK | P_75@0 | QPSK | S_100@0 |
| 4 | 100 | 100 | 5 |  |  | QPSK | P_100@0 | QPSK | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. |  |  |  |  |  |  |  |  |  |

In NR FR1 specification [6], Occupied Bandwidth for SC Test case is defined in [3] as follows:

Table 6.5.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range by default, exceptions listed in Table 6.5.1.4.1-2 |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | All |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for occupied bandwidth test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

Table 6.5.1.4.1-2: Test frequency exceptions for Occupied Bandwidth

| 5G NR Band | Test Frequency |
| --- | --- |
| n77 | Low Range, Mid Range, High Range |
| n78 | Low Range, Mid Range, High Range |
| n79 | Low Range, Mid Range, High Range |

Test Environment

Based on the baseline agreed in [1], test environment for Occupied Bandwidth measurement for inter-band CA (2UL CA) and intra-band contiguous and non-contiguous 2UL CA in FR1 can be defined as in LTE.

Proposal 1: Define Test Environment as Normal Conditions (NC) for Occupied Bandwidth measurement for inter-band CA (2UL CA) and intra-band contiguous and non-contiguous 2UL CA in FR1.

Test Subcarrier Spacing

In [3] lowest SCS for Occupied Bandwidth is selected for single carrier Occupied Bandwidth testing. This could be applicable for 2UL CA to ensure test coverage.

Proposal 2: Select the lowest Subcarrier Spacing to test for Occupied Bandwidth for inter-band CA (2UL CA) and intra-band contiguous and non-contiguous 2UL CA in FR1.

  - Test Frequencies

In LTE, Occupied Bandwidth for CA is tested for Mid range frequency for inter-band CA and intra-band contiguous CA. It is proposed to apply the same rule in FR1. Low Wgap is defined for Tx intra-band non-contiguous CA test cases in LTE, Low Wgap and Maximum Wgap are both tested in LTE. However in NR, testing with max Wgap can represent the worst case and avoid costing double test time. So it’s reasonable to test with Maximum Wgap for intra-band non-contiguous CA in NR.

Proposal 3: Select Mid range as test frequency for Occupied Bandwidth for inter-band CA (2UL CA) and intra-band contiguous 2UL CA in FR1. Test frequencies for each CC depend on specific CA configurations with max Wgap for Occupied Bandwidth for intra-band non-contiguous CA.

Test Channel Bandwidth

In LTE, Occupied Bandwidth for inter-band CA is tested for Lowest and Highest channel bandwidths. Occupied Bandwidth for CA (2UL CA) in FR1 can follow this approach as well.

For intra-band contiguous CA, all applicable aggregated channel bandwidth for each CA configuration in table 5.5A.1-1 in TS 38.521-1 should be tested according to the minimum requirement. In LTE, all applicable CC combinations are tested. For NR, testing all applicable CC combinations will cost too much testing time. In order to reduce testing time without losing test coverage, only testing the CC combination with the highest NRB_PCC when UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg will be the best choice.

For intra-band non-contiguous CA, the OBW requirement is specified and measured for each subblock in LTE. So only CBW combinations with PCC NRB = SCC NRB will be tested. However in NR, the OBW in specified and measured as the sum of powers over all subblocks. So it’ll be reasonable to test all aggregated channel bandwidths for each band combination. Also only the CC combination with the highest NRB_PCC will be tested when UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg.

Proposal 4: Select Lowest and Highest channel bandwidth for Occupied Bandwidth for inter-band CA (2UL CA) in FR1. Testing all aggregated channel bandwidth for intra-band contiguous and non-contiguous 2 UL CA. If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested for intra-band CA.

DL and UL configuration

In LTE, Occupied Bandwidth for inter-band CA is tested with maximum uplink allocation. For NR the same configuration can be adopted.

For NR SC test case, only CP-OFDM waveform is tested. CP-OFDM waveform obtains maximum spectrum utilization per channel bandwidth and SCS compared to DFT-s-OFDM waveform.

Proposal 5: Select maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS.

Proposal 6: Select CP-OFDM waveform to get the maximum spectrum utilization.

In LTE, the lowest modulation scheme is used to measure the occupied bandwidth, as it is the most robust modulation. Similarly to LTE, for NR, the lowest modulation scheme supported for the CP-OFDM is selected.

Proposal 7: Select QPSK modulation to test Occupied Bandwidth for inter-band CA (2UL CA) and intra-band contiguous and non-contiguous 2UL CA.

Conclusion

In conclusion, the following selection of test points are proposed for Occupied Bandwidth for CA (2UL CA):

Proposal 1: Define Test Environment as Normal Conditions (NC) for Occupied Bandwidth measurement for inter-band CA (2UL CA) and intra-band contiguous and non-contiguous 2UL CA in FR1.

Proposal 2: Select the lowest Subcarrier Spacing to test for Occupied Bandwidth for inter-band CA (2UL CA) and intra-band contiguous and non-contiguous 2UL CA in FR1.

Proposal 3: Select Mid range as test frequency for Occupied Bandwidth for inter-band CA (2UL CA) and intra-band contiguous 2UL CA in FR1. Test frequencies for each CC depend on specific CA configurations with max Wgap for Occupied Bandwidth for intra-band non-contiguous CA.

Proposal 4: Select Lowest and Highest channel bandwidth for Occupied Bandwidth for inter-band CA (2UL CA) in FR1. Testing all aggregated channel bandwidth for intra-band contiguous and non-contiguous 2 UL CA. If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested for intra-band CA.

Proposal 5: Select maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS.

Proposal 6: Select CP-OFDM waveform to get the maximum spectrum utilization.

Proposal 7: Select QPSK modulation to test Occupied Bandwidth for inter-band CA (2UL CA) and intra-band contiguous and non-contiguous 2UL CA.

Number of test points

Along this document several proposals have been stated for different configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing quality in UE testing for NR. Table 4-1 and table 4-2 show the total number of test points for Occupied Bandwidth for inter-band CA (2UL CA) and intra-band contiguous 2UL CA test case.

Table 4-1 Maximum Number of Test Steps for Occupied Bandwidth for inter-band CA (2UL CA) test case

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 1 | 1 | 2 |

Table 4-2 Maximum Number of Test Steps for Occupied Bandwidth for intra-band contiguous CA (2UL CA) test case

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

Table 4-3 Maximum Number of Test Steps for Occupied Bandwidth for intra-band non-contiguous CA (2UL CA) test case

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

References

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-180885, Discussion on test point selection for NR Occupied Bandwidth in FR1

[4] R5-192024, Discussion on the waveform selection for UL MIMO tests

[5] 3GPP TS 36.521-1 v16.9.0 User Equipment (UE) radio transmission and reception; Part 1: Conformance Testing (Release 16)

[6] 3GPP TS 38.521-1 v17.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 17)