3GPP TSG-RAN5 Meeting #92-e	R5-215328

# Online, 16th Aug – 27th Aug 2021

Title:							On MU and TT for FR2 EVM

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

This paper provides FR2 EVM MU for FR2a, PUSCH, PC3.

# 2.	Discussion

FR2 EVM MU for FR2a, PUSCH, PC3 was calculated in RAN5#91e as follows, based on the available SNR at TE receiver, when a marginal UE using the whole MPR relaxation is considered.

Table 1 EVM MU based on available SNR

|  | Parameter |  | CBW=50MHz | CBW=100MHz | CBW=200MHz | CBW=400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| Pi/2 BPSK | DFT-s-OFDM | Inner_Full | 1.3% | 1.8% | 2.5% | 3.5% |
|  |  | Outer_Full | 1.8% | 2.6% | 3.7% | 6.2% |
| QPSK | DFT-s-OFDM | Inner_Full | 1.3% | 1.8% | 2.5% | 3.5% |
|  |  | Outer_Full | 1.8% | 2.6% | 3.7% | 6.2% |
|  | CP-OFDM | Inner_Full | 2.6% | 3.7% | 5.2% | 9.8% |
|  |  | Outer_Full | 2.8% | 3.9% | 5.5% | 9.8% |
| 16 QAM | DFT-s-OFDM | Inner_Full | 2.2% | 3.1% | 4.4% | 9.2% |
|  |  | Outer_Full | 2.6% | 3.7% | 5.2% | 9.2% |
|  | CP-OFDM | Inner_Full | 3.5% | 4.9% | 6.9% | 13.0% |
|  |  | Outer_Full | 3.5% | 4.9% | 6.9% | 13.0% |
| 64 QAM | DFT-s-OFDM | Inner_Full | 3.5% | 4.9% | 6.9% | 13.0% |
|  |  | Outer_Full | 4.1% | 5.8% | 8.2% | 13.0% |
|  | CP-OFDM | Inner_Full | 5.2% | 7.3% | 10.3% | 17.3% |
|  |  | Outer_Full | 5.2% | 7.3% | 10.3% | 17.3% |

We propose the following residual EVM by considering TE’s uncertainty in addition to the above EVM based on available SNR.

Table 2 Residual EVM for DFT-s-OFDM

| Parameter | Average EVM Level | Reference Signal EVM Level | CBW=50MHz | CBW=100MHz | CBW=200MHz | CBW=400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| Pi/2 BPSK | 30%+TT | 30%+TT | 2.5% | 3.5% | 5.0% | 7.5% |
| QPSK | 17.5%+TT | 17.5%+TT |  |  |  |  |
| 16 QAM | 12.5%+TT | 12.5%+TT | 3.5% | 4.5% | 6.5% | 11.5% |
| 64 QAM | 8%+TT | 8%+TT | 5.0% | 7.0% | N/A | N/A |

Table 3 Residual EVM for CP-OFDM

| Parameter | Average EVM Level | Reference Signal EVM Level | CBW=50MHz | CBW=100MHz | CBW=200MHz | CBW=400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 17.5%+TT | 17.5%+TT | 3.5% | 5.0% | 7.0% | 12.0% |
| 16 QAM | 12.5%+TT | 12.5%+TT | 4.5% | 6.0% | 8.5% | N/A |
| 64 QAM | 8%+TT | 8%+TT | 6.5% | N/A | N/A |  |

Proposal 1 : Adopt the values in Table 2 and Table 3 to MU of EVM for FR2a, PUSCH, PC3.

Proposal 2 : Remove test points with ‘N/A’ in Table 2 and Table 3 from EVM for FR2a, PUSCH, PC3.

However, some of marginal UEs could fail the test when the residual EVM is over half of the minimum requirement. To ensure that a compliant UE does not fail in such a case, we propose to apply TT as shown below.

Table 4 EVM TT for DFT-s-OFDM

| Parameter | Average EVM Level | Reference Signal EVM Level | CBW=50MHz | CBW=100MHz | CBW=200MHz | CBW=400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| Pi/2 BPSK | 30%+TT | 30%+TT | 0% | 0% | 0% | 0% |
| QPSK | 17.5%+TT | 17.5%+TT |  |  |  |  |
| 16 QAM | 12.5%+TT | 12.5%+TT |  |  | 1.6% | 4.5% |
| 64 QAM | 8%+TT | 8%+TT | 1.4% | 2.6% | N/A | N/A |

Table 5 EVM TT for CP-OFDM

| Parameter | Average EVM Level | Reference Signal EVM Level | CBW=50MHz | CBW=100MHz | CBW=200MHz | CBW=400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 17.5%+TT | 17.5%+TT | 0% | 0% | 0% | 3.7% |
| 16 QAM | 12.5%+TT | 12.5%+TT |  |  | 2.6% | N/A |
| 64 QAM | 8%+TT | 8%+TT | 2.3% | N/A | N/A |  |

Proposal 3 : Adopt the values in Table 4 and Table 5 to TT of EVM for FR2a, PUSCH, PC3.

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

Proposal 1 : Adopt the values in Table 2 and Table 3 to MU of EVM for FR2a, PUSCH, PC3.

Proposal 2 : Remove test points with ‘N/A’ in Table 2 and Table 3 from EVM for FR2a, PUSCH, PC3.

Proposal 3 : Adopt the values in Table 4 and Table 5 to TT of EVM for FR2a, PUSCH, PC3.

# 4.	References

[1] R5-213817, “On FR2 EVM measurement uncertainties and test tolerances”, Keysight Technologies, RAN5#91e