3GPP TSG-RAN5 Meeting #92-e	R5-215359

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2017 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to introduce Handling of PDU Session Release during switch off/Power off procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the current Prose implementation of 38.508 Table 4.9.6.1-1 (Switch off/Power off in RRC_IDLE) a UE that has previously performed IMS registration will be performing the Service REQ procedure in order to perform IMS De-registration. After this, it is expected that the UE will transmit a De-registration REQ. The Prose does not consider the case that the UE may transmit a PDU Session Release REQ after IMS de-registration but prior to transmitting De-registration REQ. This is valid UE behaviour, as the UE may consider that after the IMS De-registration there is no need to maintain the PDU Session. This needs to be accommodated. Similarly, for 38.508 Table 4.9.6.2.1-1 (Switch off/Power off in RRC_INACTIVE) the UE may transmit a PDU Session Release REQ after IMS de-registration but before transmitting De-registration REQ. This is valid UE behaviour, as the UE may consider that after the IMS De-registration there is no need to maintain the PDU Session. This needs to be accommodated. According to 38.508 Table 4.9.6.3-1 (Switch off/Power off in RRC_CONNECTED it is expected that for a UE which is IMS registered, the UE will perform IMS de-registration. After this the UE may transmit a PDU Session Release REQ, as the UE may consider that there is no need to maintain the PDU Session after IMS De-registration. This needs to be handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 38.508 Table 4.9.6.1-1, new Steps 1a4Aa5a1 - 1a4Aa5a3 are introduced that handle the PDU Session Release REQ. For 38.508 Table 4.9.6.2.1-1, new Steps 1a4Aa5a1 - 1a4Aa5a3 are introduced that handle the PDU Session Release REQ. For 38.508 Table 4.9.6.3-1, new Steps 5a2Aa1a1 - 5a2Aa1a3 are proposed that handle the PDU Session Release REQ. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN based on current Prose may fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | IMS testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.9.6	Test procedures for Switch off / Power off UE

#### 4.9.6.1	Switch off / Power off procedure in RRC_IDLE

Table 4.9.6.1-1: Switch off procedure in RRC_IDLE

| Step | Procedure | Message Sequence |  |  |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |  |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) | - | - |  |
| 1a2 | UE transmits an RRCSetupRequest message. | --> | RRC: RRCSetupRequest |  |
| 1a3 | SS transmit an RRCSetup message. | <-- | RRC: RRCSetup |  |
| - | EXCEPTION: Steps 1a4Aa1 to 1a4Aa6 specify optional behaviour if the UE has previously performed IMS registration | - | - |  |
| 1a4Aa1 | The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. | --> | RRC: RRCSetupComplete 5GMM: SERVICE REQUEST |  |
| 1a4Aa2 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |  |
| 1a4Aa3 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |  |
| 1a4Aa4 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT |  |
| - | EXCEPTION: In parallel to the event described in step 1a4Aa5 below, the generic test procedure in TS 34.229-5 [47] Annex A.11 may be performed. | - | - |  |
| 1a4Aa5 | The UE transmits an RRCReconfigurationComplete message. | --> | RRC: RRCReconfigurationComplete |  |
| 1a4Aa5a1 | The UE transmits a PDU SESSION RELEASE REQUEST message. | --> | PDU SESSION RELEASE REQUEST |  |
| 1a4Aa5a2 | PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4] is performed | - | - |  |
| 1a4Aa5a3 | The UE transmits a DEREGISTRATION REQUEST message. | --> | 5GMM: DEREGISTRATION REQUEST |  |
| 1a4Aa5b1 | The UE transmits a DEREGISTRATION REQUEST message | - | 5GMM: DEREGISTRATION REQUEST |  |
| - | EXCEPTION: Step 1a4Ab1 below specifies the behaviour if the UE has not previously performed IMS registration | - | - |  |
| 1a4Ab1 | The UE transmits an RRCSetupComplete message including the DEREGISTRATION REQUEST message. | --> | RRC: RRCSetupComplete 5GMM: DEREGISTRATION REQUEST |  |
| 1a4 | Void | - | - |  |
| 1a5 | The SS transmits an RRCRelease message | <-- | RRC: RRCRelease |  |
| 1b1 | ELSE power off UE (Note 2) | - | - |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |

#### 4.9.6.2	Switch off / Power off procedure in RRC_INACTIVE

4.9.6.2.1	Procedure

Table 4.9.6.2.1-1: Switch off procedure in RRC_INACTIVE

| Step | Procedure | Message Sequence |  |  |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |  |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) | - | - |  |
| 1a2 | UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest |  |
| 1a3 | SS transmit an RRCResume message. | <-- | NR RRC: RRCResume |  |
| - | EXCEPTION: Steps 1a4Aa1 to 1a4Aa6 specify optional behaviour if the UE has previously performed IMS registration | - | - |  |
| 1a4Aa1 | The UE transmits an RRCResumeComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. | --> | RRC: RRCResumeComplete 5GMM: SERVICE REQUEST |  |
| 1a4Aa2 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |  |
| 1a4Aa3 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |  |
| 1a4Aa4 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT |  |
| - | EXCEPTION: In parallel to the event described in step 1a4Aa5 below, the generic test procedure in TS 34.229-5 [47] Annex A.11 may be performed. | - | - |  |
| 1a4Aa5 | The UE transmits an RRCReconfigurationComplete message. | --> | RRC: RRCReconfigurationComplete |  |
| 1a4Aa5a1 | The UE transmits a PDU SESSION RELEASE REQUEST message. | --> | PDU SESSION RELEASE REQUEST |  |
| 1a4Aa5a2 | PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4] is performed | - | - |  |
| 1a4Aa5a3 | The UE transmits a DEREGISTRATION REQUEST message. | --> | 5GMM: DEREGISTRATION REQUEST |  |
| 1a4Aa5b1 | The UE transmits a DEREGISTRATION REQUEST message | - | 5GMM: DEREGISTRATION REQUEST |  |
| - | EXCEPTION: Step 1a4Ab1 below specifies the behaviour if the UE has not previously performed IMS registration | - | - |  |
| 1a4 | Void | - | - |  |
| 1a5 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease |  |
| 1b1 | ELSE power off UE (Note 2) | - | - |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |

4.9.6.2.2	Specific Message contents

Table 4.9.6.2.2-1: RRCResumeRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |  |
| resumeIdentity |  | Not checked |  |  |
| resumeMAC-I |  | Not checked |  |  |
| resumeCause |  | Not checked |  |  |
| spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 4.9.6.3	Switch off / Power off procedure in RRC_CONNECTED

Table 4.9.6.3-1: Switch off procedure in RRC_CONNECTED

| Step | Procedure | Message Sequence |  |  |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |
| 1a1-1a3 | Void | - | - |  |
| 2-4 | Void | - | - |  |
| - | EXCEPTION: Steps 5a1 to 5b1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if a particular implementation is under test. | - | - |  |
| 5a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) |  |  |  |
| - | EXCEPTION : Step 5a1Aa1 to 5a1Aa2 below specifies optional behaviour if the UE has previously performed IMS registration | - | - |  |
| 5a1Aa1 | The UE may perform the generic test procedure described in TS 34.229-5 [47] Annex A.11 | - | - |  |
| 5a2Aa1a1 | The UE transmits a PDU SESSION RELEASE REQUEST message. | --> | PDU SESSION RELEASE REQUEST |  |
| 5a2Aa1a2 | PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4] is performed | - | - |  |
| 5a2Aa1a3 | The UE transmits a DEREGISTRATION REQUEST message. | --> | 5GMM: DEREGISTRATION REQUEST |  |
| 5a2Ab1 | The UE transmits a DEREGISTRATION REQUEST message. | --> | 5GMM: DEREGISTRATION REQUEST |  |
| 5a3 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease |  |
| 5b1 | ELSE power off UE (Note 2) | - | - |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |