3GPP TSG-RAN5 Meeting #92-e	R5-215374

# Electronic Meeting, 16th August – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5347 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to V2X test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_V2X-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The V2X test cases of MPR/AMPR with reference clause and requirement used in the A-SEM and ACLR are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 3GU style correction; - Correction the reference clauses in different test cases; - Updated test applicability to align with TS36.521-2; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | V2X test cases reference clause will keep incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.G.1.1, 6.2.3.G.1.2, 6.2.3.G.1.3, 6.2.3.G.1.4, 6.2.3.G.2.1, 6.2.3.G.2.2, 6.2.4G.1, 6.2.4G.2, 6.6.2.1G.1, 6.6.2.1G.2. 6.6.2.1G.3, 6.6.2.2G.1, 6.6.2.2G.2, 6.6.2.3G.1, 6.6.2.3G.2, 6.6.2.3G.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes also covered WIC “LTE_SL_V2V-UEConTest”. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

<Unchanged Sections Skipped>

### 6.2.3G	Maximum Power Reduction (MPR) for V2X communication

#### 6.2.3G.1	Maximum Power Reduction (MPR) for V2X Communication / Power class 3

##### 6.2.3G.1.1	Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions

6.2.3G.1.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.3G.1.1.2	Test applicability

The requirements of this test apply in the test case 6.6.2.3G.1 Adjacent Channel Leakage power Ratio for V2X Communication to all types of UE that support V2X Sidelink communication and Band 47.

NOTE:	As a result TC 6.2.3G.1.1 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.3G.1.1.3	Minimum conformance requirements

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.2G-1, this subclause specifies the allowed Maximum Power Reduction (MPR) power for V2X physical channels and signals due to PSCCH and PSSCH simultaneous transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2.3G.1.1.3-1 for power class 3.

Table 6.2.3G.1.1.3-1: Maximum Power Reduction (MPR) for power class 3 V2X Communication (Contiguous PSCCH and PSSCH transmission)

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK |  |  |  |  |  |  | ≤ 1.5 |
| 16 QAM |  |  |  |  |  |  | ≤ 2 |

The normative reference for this requirement is in TS 36.101 [2] clause 6.2.3G.

6.2.3G.1.1.4	Test description

6.2.3G.1.1.4.1	Initial conditionInitial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.4.2G.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3G.1.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.2.3G.1.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
|  | V2X Configuration to Transmit |  |  |
| Ch BW | Modulation |  | PSSCH RB allocation |
| 10 MHz | QPSK |  | 48@2 |
| 10 MHz | QPSK |  | 3@2 |
| 10 MHz | QPSK |  | 3@47 |
| 10 MHz | 16QAM |  | 48@2 |
| 10 MHz | 16QAM |  | 3@2 |
| 10 MHz | 16QAM |  | 3@47 |
| 20 MHz | QPSK |  | 96@2 |
| 20 MHz | QPSK |  | 3@2 |
| 20 MHz | QPSK |  | 3@92 |
| 20 MHz | 16QAM |  | 96@2 |
| 20 MHz | 16QAM |  | 3@2 |
| 20 MHz | 16QAM |  | 3@92 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.2.3G.1.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.2.3G.1.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9..

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.2.3G.1.1.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	Measure the mean power of the UE in the channel bandwidth for each test point in Table 6.2.3G.1.1.5-1 according to the test configuration from Table 6.2.3G.1.1.4.1-1. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.2.3G.1.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6, 4.7.I and 4.10.

6.2.3G.1.1.5	Test Requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3G.1.1.5-1.

Table 6.2.3G.1.1.5-1: UE Power Class 3 test requirements

| E-UTRA Band | Class 3 (dBm) | QPSK Tol. (dB)) | 16QAM Tol. (dB) |
| --- | --- | --- | --- |
| 47 | 23 | +3.3 / - 4.8 | +3.3 / - 5.3 |
| Note 1: Void Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply. |  |  |  |

##### 6.2.3G.1.2	Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions

6.2.3G.1.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.3G.1.2.2	Test applicability

The requirements of this test apply in the test case 6.6.2.3G.1 Adjacent Channel Leakage power Ratio for V2X Communication to all types of UE that support V2X Sidelink communication and Band 47.

NOTE:	As a result TC 6.2.3G.1.2 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.3G.1.2.3	Minimum conformance requirements

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.2G-1, this subclause specifies the allowed Maximum Power Reduction (MPR) power for V2X physical channels and signals due to PSCCH and PSSCH simultaneous transmission

For non-contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified as follows

MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =	4.5		; 0.00< A ≤ 0.2

5.5 –5.833A		; 0.2< A ≤0.6

2.0					; 0.6< A ≤1.00

Where

A = NRB_alloc / NRB.

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The normative reference for this requirement is TS 36.101 [2] clause 6.2.3G.

6.2.3G.1.2.4	Test description

6.2.3G.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.4.2G.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3G.1.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.2.3G.1.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Configuration ID | V2X Configuration to Transmit |  |  |  |
|  | Ch BW |  | Modulation | PSSCH RB allocation (PRB @ RBstart) |
| 1 | 10 MHz |  | 16QAM | 5 @ 45 |
| 2 | 10 MHz |  | 16QAM | 25 @ 25 |
| 3 | 10 MHz |  | 16QAM | 40 @ 10 |
| 4 | 20 MHz |  | 16QAM | 10 @ 90 |
| 5 | 20 MHz |  | 16QAM | 50 @ 50 |
| 6 | 20 MHz |  | 16QAM | 80 @ 20 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.2.3G.1.2.4.3.

3.	The V2X reference measurement channel is set according to Table 6.2.3G.1.2.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.2.3G.1.2.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	Measure the mean power of the UE in the channel bandwidth for each test point in Table 6.2.3G.1.2.5-1 according to the test configuration from Table 6.2.3G.1.2.4.1-1. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.2.3G.1.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 6.2.3G.1.2.4.3-1: SL-CommResourcePoolV2X-r14

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-20C SL-CommResourcePoolV2X-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResourcePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | FALSE | BOOLEAN |  |
| numSubchannel-r14 | n8 |  |  |
| startRB-Subchannel-r14 | 10 |  | 10 MHz |
|  | 20 |  | 20 MHz |
| startRB-PSCCH-Pool-r14 | 0 |  |  |

6.2.3G.1.2.5	Test requirement

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3G.1.2.5-1.

Table 6.2.3G.1.2.5-1: UE Power Class 3 test requirements

| Configuration ID | MPR (dB) | PCMAX,c (dBm) | T(PCMAX,c) (dB) | Tol. (dB) |
| --- | --- | --- | --- | --- |
| 1 | 4.5 | 18.5 | 4.0 | +3.3 / -9.8 |
| 2 | 3.0 | 20.0 | 2.5 | +3.3 / -6.8 |
| 3 | 2.0 | 21.0 | 2.0 | +3.3 / -5.3 |
| 4 | 4.5 | 18.5 | 4.0 | +3.3 / -9.8 |
| 5 | 3.0 | 20.0 | 2.5 | +3.3 / -6.8 |
| 6 | 2.0 | 21.0 | 2.0 | +3.3 / -5.3 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply. |  |  |  |  |

##### 6.2.3G.1.3	Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission

6.2.3G.1.3.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.3G.1.3.2	Test applicability

This test applies in test cases 6.6.2.3 and 6.6.2.3G.2 Adjacent Channel Leakage power Ratio to all types of E-UTRA UE release 14 and forward that support V2X communication and is capable of V2X Sidelink communication simultaneous with E-UTRA uplink transmission.

NOTE:	As a result TC 6.2.3G.1.3 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.3G.1.3.3	Minimum conformance requirements

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.2G-2, the allowed MPR requirements in subclause 6.2.3G apply for V2X PSSCH and PSCCH transmission. The allowed MPR requirements in subclause 6.2. 3D apply for other V2X sidelink transmission (PSBCH/PSSS/SSSS). The MPR requirements in subclause 6.2.3 apply for uplink transmission.

The normative reference for this requirement is TS 36.101 [2] clause 6.2.3G.

6.2.3G.1.3.4	Test description

6.2.3G.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3G.1.3.4.1-1. The details of the uplink and V2X reference measurement channels (RMCs) are specified in A.2 and A.8.3 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.3G.1.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Low range for uplink and Low range for V2X, High range for uplink and High range for V2X |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  |  | V2X Configuration |  |  |
| Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  | Ch BW | Mod'n | PSSCH RB allocation |
| 1.4MHz |  | QPSK | 5 |  | 10 MHz | QPSK | 48@2 |
| 1.4MHz |  | QPSK | 6 |  | 10 MHz | 16QAM | 48@2 |
| 1.4MHz |  | 16QAM | 5 |  | 20 MHz | QPSK | 96@2 |
| 1.4MHz |  | 16QAM | 6 |  | 20 MHz | 16QAM | 96@2 |
| 3MHz |  | QPSK | 4 |  |  |  |  |
| 3MHz |  | QPSK | 15 |  |  |  |  |
| 3MHz |  | 16QAM | 4 |  |  |  |  |
| 3MHz |  | 16QAM | 15 |  |  |  |  |
| 5MHz |  | QPSK | 8 |  |  |  |  |
| 5MHz |  | QPSK | 25 |  |  |  |  |
| 5MHz |  | 16QAM | 8 |  |  |  |  |
| 5MHz |  | 16QAM | 25 |  |  |  |  |
| 10MHz |  | QPSK | 12 |  |  |  |  |
| 10MHz |  | QPSK | 50 |  |  |  |  |
| 10MHz |  | 16QAM | 12 |  |  |  |  |
| 10MHz |  | 16QAM | 50 (Note 3) |  |  |  |  |
| 15MHz |  | QPSK | 16 |  |  |  |  |
| 15MHz |  | QPSK | 75 |  |  |  |  |
| 15MHz |  | 16QAM | 16 |  |  |  |  |
| 15MHz |  | 16QAM | 75 (Note 3) |  |  |  |  |
| 20MHz |  | QPSK | 18 |  |  |  |  |
| 20MHz |  | QPSK | 100 |  |  |  |  |
| 20MHz |  | 16QAM | 18 |  |  |  |  |
| 20MHz |  | 16QAM | 100 (Note 3) |  |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: For Low range the E-UTRA carrier uses only test points with RBstart=0. For High range the E-UTRA carrier uses only test points with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.2.3G.1.3.4.3

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to Table 6.2.3G.1.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message content exceptions are defined in clause 6.2.3G.1.3.4.3.

6.2.3G.1.3.4.2	Test procedure

1.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.2.3G.1.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level. Configure the UE to transmit PSCCH and PSSCH with the PUMAX level of each test points.

3.	Measure the mean power over all component carriers in the V2X con-current band configuration. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.2.3G.1.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

6.2.3G.1.3.5	Test requirement

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3G.13.5-1.

Table 6.2.3G.1.3.5-1: Inter-band con-current V2X UE Power Class test requirements

| V2X con-current band Configuration | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | QPSK full RB allocation Tol. (dB)) | 16QAM full RB allocation Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_3A-47A |  |  |  |  | 23 | +3.3 / - 7.8 | +3.3 / - 8.3 |
| V2X_5A-47A |  |  |  |  | 23 | +3.3 / - 7.8 | +3.3 / - 8.3 |
| V2X_7A-47A |  |  |  |  | 23 | +3.3 / - 7.8 | +3.3 / - 8.3 |
| V2X_8A-47A |  |  |  |  | 23 | +3.3 / - 7.8 | +3.3 / - 8.3 |
| V2X_20A-47A |  |  |  |  | 23 | +3.3 / - 7.8 | +3.3 / - 8.3 |
| V2X_34A-47A |  |  |  |  | 23 | +3.3 / - 7.8 | +3.3 / - 8.3 |
| V2X_39A-47A |  |  |  |  | 23 | +3.3 / - 7.8 | +3.3 / - 8.3 |
| V2X_41A-47A |  |  |  |  | 23 | +3.3 / - 7.8 | +3.3 / - 8.3 |
| NOTE 1: The con-current band combinations are used for V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement applies to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |

##### 6.2.3G.1.4	Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Intra-band contiguous multi-carrier operation

6.2.3G.1.4.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.3G.1.4.2	Test applicability

The requirements of this test apply in the test case 6.6.2.3G.3 Adjacent Channel Leakage power Ratio for V2X Communication to all types of UE that support Intra-band contiguous multi-carrier operation V2X Sidelink communication and Band 47.

NOTE:	As a result TC 6.2.3G.1.4 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.3G.1.4.3	Minimum conformance requirements

For intra-band contiguous multi-carrier operation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2G.3.3-1 due to higher order modulation is specified in Table 6.2.3G.1.4.3-1. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2.3G.1.4.3-1: Maximum Power Reduction (MPR) for Power Class 3

| Modulation | 50 RB + 50 RB | 50 RB + 100 RB | 100 RB + 100 RB | MPR (dB) |
| --- | --- | --- | --- | --- |
| QPSK | - | N/A | N/A | ≤ [3] |
| 16 QAM | - | N/A | N/A | ≤ [3] |

The normative reference for this requirement is TS 36.101 [2] clause 6.2.3G.

6.2.3G.1.4.4	Test description

6.2.3G.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.4.2G.1-4. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3G.1.4.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.2.3G.1.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Mid range |  |  |
| Test Channel Bandwidths as specified in subclause 5.4.2G |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  |  | V2X PSSCH Allocation |  |
| CC1 NRB | CC2 NRB |  | CC1 & CC2 Mod | CC1 and CC2 RB Allocation |
| 50 | 50 |  | QPSK | 48@2+48@2 |
| 50 | 50 |  | 16QAM | 48@2+48@2 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.89a.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.2.3G.1.4.4.3.

3.	The V2X reference measurement channel is set according to Table 6.2.3G.1.4.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.2.3G.1. .4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	Measure the mean power of the UE in the channel bandwidth for each test point in Table 6.2.3G.1.4.5-1 according to the test configuration from Table 6.2.3G.1.4.4.1-1. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.2.3G.1.4.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6, 4.7.I and 4.10.

6.2.3G.1.4.5	Test requirement

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3G.1.4.5-1.

Table 6.2.3G.1.4.5-1: UE Power Class 3 test requirements

| E-UTRA Band | Bandwidth combination set | Class 3 (dBm) | QPSK Tol. (dB) | 16QAM Tol. (dB) |
| --- | --- | --- | --- | --- |
| V2X_47B | 0 | 23 | +3.3 / -6.8 | +3.3 / -6.8 |

#### 6.2.3G.2	Maximum Power Reduction (MPR) for V2X Communication / Power class 2

##### 6.2.3G.2.1	Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions

6.2.3G.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.3G.2.1.2	Test applicability

The requirements of this test apply in the test case 6.6.2.3G Adjacent Channel Leakage power Ratio for V2X Communication to all types of UE that support V2X Sidelink communication and Band 47.

NOTE:	As a result TC 6.2.3G.2.1 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.3G.2.1.3	Minimum conformance requirements

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2.3G.2.1.3-1 for power class 2.

Table 6.2.3G.2.1.3-1: Maximum Power Reduction (MPR) for power class 2 V2X Communication (Contiguous PSCCH and PSSCH transmission)

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK |  |  |  |  |  |  | ≤ 2 |
| 16 QAM |  |  |  |  |  |  | ≤ 2.5 |

The normative reference for this requirement is in TS 36.101 [2] clause 6.2.3G.

6.2.3G.2.1.4	Test description

6.2.3G.2.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.4.2G.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3G.2.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.2.3G.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
|  | V2X Configuration to Transmit |  |  |
| Ch BW | Modulation |  | PSSCH RB allocation |
| 10 MHz | QPSK |  | 48@2 |
| 10 MHz | QPSK |  | 3@2 |
| 10 MHz | QPSK |  | 3@47 |
| 10 MHz | 16QAM |  | 48@2 |
| 10 MHz | 16QAM |  | 3@2 |
| 10 MHz | 16QAM |  | 3@47 |
| 20 MHz | QPSK |  | 96@2 |
| 20 MHz | QPSK |  | 3@2 |
| 20 MHz | QPSK |  | 3@92 |
| 20 MHz | 16QAM |  | 96@2 |
| 20 MHz | 16QAM |  | 3@2 |
| 20 MHz | 16QAM |  | 3@92 |

1.	Connect the SS and the GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.2.3G.2.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.2.3G.2.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.2.3G.2.1.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	Measure the mean power of the UE in the channel bandwidth for each test point in Table 6.2.3G.2.1.5-1 according to the test configuration from Table 6.2.3G.2.1.4.1-1. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.2.3G.2.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 6.2.3G.2.1.4.3-1: SL-V2X-PreconfigFreqInfo-r14

| Derivation Path: TS 36.508 [7] clause 4.6, Table 4.6.3-20E SL-V2X-PreconfigFreqInfo-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  |  |  |
| maxTxPower-r12 | 26 |  |  |
| } |  |  |  |

6.2.3G.2.1.5	Test Requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3G.2.1.5-1.

Table 6.2.3G.2.1.5-1: UE Power Class 2 test requirements

| E-UTRA Band | Class 2 (dBm) | QPSK Tol. (dB)) | 16QAM Tol. (dB) |
| --- | --- | --- | --- |
| 47 | 26 | +3.3/-5.3 | +3.3/-5.8 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply. |  |  |  |

##### 6.2.3G.2.2	Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions

6.2.3G.2.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.3G.2.2.2	Test applicability

The requirements of this test apply in the test case 6.6.2.3G Adjacent Channel Leakage power Ratio for V2X Communication to all types of UE Power Class 2 release 14 and forward that support V2X Sidelink communication and Band 47.

NOTE:	As a result TC 6.2.3G.2.2 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.3G.2.2.3	Minimum conformance requirements

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.2G-1, this subclause specifies the allowed Maximum Power Reduction (MPR) power for V2X physical channels and signals due to PSCCH and PSSCH simultaneous transmission

For non-contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified as follows:

MPR = CEIL {MA, 0.5}

Where MA is defined as follows

For 10MHz channel bandwidth         MA =	4.5		         ; 0.0< A ≤ 0.2

8.5 – 20.0A		; 0.2< A ≤0.3

2.5				   ; 0.3< A ≤1.00

For 20MHz channel bandwidth         MA =	9.0		         ; 0.0< A ≤ 0.1

12.0 – 30.0A		; 0.1< A ≤0.3

3.0				   ; 0.3< A ≤1.00

Where

A = NRB_alloc / NRB.

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The normative reference for this requirement is TS 36.101 [2] clause 6.2.3G.

6.2.3G.2.2.4	Test description

6.2.3G.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.2G-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3G.2.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.8.3 3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.2.3G.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Configuration ID | V2X Configuration to Transmit |  |  |  |
|  | Ch BW |  | Modulation | PSSCH RB allocation (PRB @ RBstart) |
| 1 | 10 MHz |  | 16QAM | 8 @ 40 |
| 2 | 10 MHz |  | 16QAM | 10 @ 35 |
| 3 | 10 MHz |  | 16QAM | 40 @ 10 |
| 4 | 20 MHz |  | 16QAM | 8 @ 90 |
| 5 | 20 MHz |  | 16QAM | 20 @ 70 |
| 6 | 20 MHz |  | 16QAM | 80 @ 20 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.2.3G.2.2.4.3.

3.	The V2X reference measurement channel is set according to Table 6.2.3G.2.2.4.1-1.

4. 	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.2.3G.2.2.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	Measure the mean power of the UE in the channel bandwidth for each test point in Table 6.2.3G.2.2.5-1 according to the test configuration from Table 6.2.3G.2.2.4.1-1. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.2.3G.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 6.2.3G.2.2.4.3-1: SL-V2X-PreconfigFreqInfo-r14

| Derivation Path: TS 36.508 [7] clause 4.6, Table 4.6.3-20E SL-V2X-PreconfigFreqInfo-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  |  |  |
| maxTxPower-r12 | 26 |  |  |
| } |  |  |  |

Table 6.2.3G.2.2.4.3-2: SL-CommResourcePoolV2X-r14

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-20C SL-CommResourcePoolV2X-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResourcePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | FALSE | BOOLEAN |  |
| numSubchannel-r14 | n8 |  |  |
| startRB-Subchannel-r14 | 10 |  | 10 MHz |
|  | 20 |  | 20 MHz |
| startRB-PSCCH-Pool-r14 | 0 |  |  |

6.2.3G.2.2.5	Test requirement

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3G.2.2.5-1.

Table 6.2.3G.2.2.5-1: UE Power Class 2 test requirements

| Configuration ID | E-UTRA Band | Class 2 (dBm) | Tol. (dB) |
| --- | --- | --- | --- |
| 1 | 47 | 26 | +3.3 / -7.8 |
| 2 | 47 | 26 | +3.3 / -7.3 |
| 3 | 47 | 26 | +3.3 / -5.8 |
| 4 | 47 | 26 | +3.3 / -15.3 |
| 5 | 47 | 26 | +3.3 / -9.3 |
| 6 | 47 | 26 | +3.3 / -6.3 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply. |  |  |  |

<Unchanged Sections Skipped>

### 6.2.4G	Additional Maximum Power Reduction (A-MPR) for V2X Communication

#### 6.2.4G.1	Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

6.2.4G.1.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2.4G.1.2	Test applicability

The requirements of this test apply in test case 6.6.2.2.4 Additional Spectrum Emission Mask for network signalled values NS_33 and NS_34 to all types of UE that support V2X Sidelink communication and Band 47.

The requirements of this test apply in test case 6.6.3.2 Additional Spurious Emissions for network signalled values NS_33 and NS_34 to all types of UE that support V2X Sidelink communication and Band 47.

NOTE:	As a result TC 6.2.4G.1 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.4G.1.3	Minimum conformance requirements

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.2G-1, the maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =	A-MPRBase + Gpost connector * A-MPRStep

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase  and A-MPRStep  are specified in Tables 6.2.4G.1.3-1, 6.2.4G.1.3-2 is allowed when network signalling value is provided. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in TS 36.101 [2] annex I.

NOTE: the A-MPRstep is the increase in A-MPR allowance to allow UE to meet tighter conducted A-SE and A-SEM requirements with higher value of declared Gpost connector. A-MPRBase is the default A-MPR value when no Gpost connector is declared.  A-MPRBase  and A-MPRstep vary depending on channel frequency and RB allocation. For channel frequencies and RB allocations that are close to the frequency range 5815-5855MHz, those value are much higher due to stringent emission requirement in this range.

Table 6.2.4G.1.3-1: Additional Maximum Power Reduction (A-MPR) for power class 3 V2X UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_33 | 6.6.2.2.3.6 (A-SEM) 6.6.3.2.3 (A-SE) 6.6.2G | 47 | 10 | Table 6.2.4G.1.3-2 |

Table 6.2.4G.1.3-2: A-MPR for NS_33

| Resource pool | Carrier frequency(MHz) | Resources Blocks (NRB) | Start Resource Block | A-MPRBase | A-MPRStep |
| --- | --- | --- | --- | --- | --- |
| Adjacent | 5860 | ≤ 10 | 0 | 15.5 | 0.86 |
|  |  |  | 5, 6 | 8 | 0.64 |
|  |  |  | 10, 12 | 6 | 0.50 |
|  |  |  | ≥ 15 | 5 | 0.93 |
|  |  | >10 & ≤22 | 0 | 15 | 0.71 |
|  |  |  | 5, 6 | 11.5 | 0.64 |
|  |  |  | 10, 12 | 10 | 0.57 |
|  |  |  | 15, 18 | 6 | 0.57 |
|  |  |  | 20, 24, 25 | 5 | 0.57 |
|  |  |  | ≥ 30 | 4.5 | 0.64 |
|  |  | >22 | 0, 5, 6 | 12.5 | 0.71 |
|  |  |  | 10, 12 | 10.5 | 0.57 |
|  |  |  | 15, 18 | 9.5 | 0.64 |
|  |  |  | 20, 24, 25 | 6.5 | 0.71 |
|  | 5870, 5910, 5920 | <20 | ≥ 0 | 3 | 0.64 |
|  |  | ≥ 20 and ≤45 |  | 1.5 | 0.43 |
|  |  | >45 |  | 2.5 | 0.36 |
|  | 5880, 5890, 5900 | <10 | ≥ 0 | 3 | 0.43 |
|  |  | ≥ 10 and ≤38 |  | 1.5 | 0.50 |
|  |  | >38 |  | 2 | 0.43 |
| Non-Adjacent | 5860 | ≤ 5 | ≥ 0 | 13.5 | TBD |
|  |  | >5 |  | 11.5 | TBD |
|  | 5870, 5910, 5920 | ≤ 5 | ≥ 0 | 5 | TBD |
|  |  | >5 & ≤42 |  | 3 | TBD |
|  |  | >42 |  | 4.5 | TBD |
|  | 5880, 5890, 5900 | ≤18 | ≥ 0 | 3.5 | TBD |
|  |  | >18 & ≤42 |  | 2.5 | TBD |
|  |  | >42 |  | 3 | TBD |

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the allowed A-MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2.4G.1.3-3 and 6.2.4G.1.3-4 for V2X UE  power class 2.

Table 6.2.4G.1.3-3: Additional Maximum Power Reduction (A-MPR) for power class 2 V2X UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_34 | 6.6.2.2.3.6 (A-SEM) 6.6.3.2.3 (A-SE) 6.6.2G | 47 | 10 | Table 6.2.4G-4 |

Table 6.2.4G.1.3-4: A-MPR for NS_34

| Resource pool | Carrier frequency(MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- |
| Adjacent | 5860 | <20 | 15 |
|  |  | >20 & <30 | 13 |
|  |  | >30 | 12 |
|  | 5870, 5900, 5880, 5890, 5900 | <15 or >40 | 1 |
| Non-Adjacent | 5860 | - | 12.5 |
|  | 5910, 5920 | <10 | 5.5 |
|  |  | <20 & >10 | 3 |
|  | 5870, 5880, 5890, 5900 | <10 | 5.5 |

The normative reference for this requirement is in TS 36.101 [2] clause 6.2.4G.

6.2.4G.1.4	Test description

6.2.4G.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.2G-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4G.1.4.1-1 and 6.2.4G.1.4.1-2. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.8.3 3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.2.4G.1.4.1-1: Test Configuration Table for Power Class 3

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  | Lowest |  |  |
| Test Parameters for NS_33 A-MPR |  |  |  |  |  |
|  |  | V2X Configuration to Transmit |  |  |  |
| Configura-tion ID | Ch BW | Resource pool | Mod'n | PSSCH RB allocation | PSSCH RBstart |
| 13 | 10MHz | adjacent | QPSK | 3 | 2 |
| 23 | 10MHz |  | QPSK | 3 | 7 |
| 33 | 10MHz |  | QPSK | 3 | 12 |
| 43 | 10MHz |  | QPSK | 3 | 47 |
| 53 | 10MHz |  | QPSK | 8 | 2 |
| 63 | 10MHz |  | QPSK | 8 | 7 |
| 73 | 10MHz |  | QPSK | 8 | 12 |
| 83 | 10MHz |  | QPSK | 8 | 42 |
| 93 | 10MHz |  | QPSK | 18 | 2 |
| 103 | 10MHz |  | QPSK | 18 | 7 |
| 113 | 10MHz |  | QPSK | 18 | 12 |
| 123 | 10MHz |  | QPSK | 18 | 17 |
| 133 | 10MHz |  | QPSK | 18 | 22 |
| 143 | 10MHz |  | QPSK | 18 | 32 |
| 153 | 10MHz |  | QPSK | 27 | 2 |
| 163 | 10MHz |  | QPSK | 27 | 12 |
| 173 | 10MHz |  | QPSK | 27 | 17 |
| 183 | 10MHz |  | QPSK | 27 | 22 |
| 193 | 10MHz |  | 16QAM | 3 | 2 |
| 203 | 10MHz |  | 16QAM | 3 | 7 |
| 213 | 10MHz |  | 16QAM | 3 | 12 |
| 223 | 10MHz |  | 16QAM | 3 | 47 |
| 233 | 10MHz |  | 16QAM | 8 | 2 |
| 243 | 10MHz |  | 16QAM | 8 | 7 |
| 253 | 10MHz |  | 16QAM | 8 | 12 |
| 263 | 10MHz |  | 16QAM | 8 | 42 |
| 273 | 10MHz |  | 16QAM | 18 | 2 |
| 283 | 10MHz |  | 16QAM | 18 | 7 |
| 293 | 10MHz |  | 16QAM | 18 | 12 |
| 303 | 10MHz |  | 16QAM | 18 | 17 |
| 313 | 10MHz |  | 16QAM | 18 | 22 |
| 323 | 10MHz |  | 16QAM | 18 | 32 |
| 333 | 10MHz |  | 16QAM | 27 | 2 |
| 343 | 10MHz |  | 16QAM | 27 | 12 |
| 353 | 10MHz |  | 16QAM | 27 | 17 |
| 363 | 10MHz |  | 16QAM | 27 | 22 |
| 374 | 10MHz |  | QPSK | 8 | 2 |
| 384 | 10MHz |  | QPSK | 18 | 2 |
| 394 | 10MHz |  | QPSK | 48 | 2 |
| 404 | 10MHz |  | 16QAM | 8 | 2 |
| 414 | 10MHz |  | 16QAM | 18 | 2 |
| 424 | 10MHz |  | 16QAM | 48 | 2 |
| 435 | 10MHz |  | QPSK | 3 | 2 |
| 445 | 10MHz |  | QPSK | 8 | 2 |
| 455 | 10MHz |  | QPSK | 48 | 2 |
| 465 | 10MHz |  | 16QAM | 3 | 2 |
| 475 | 10MHz |  | 16QAM | 8 | 2 |
| 485 | 10MHz |  | 16QAM | 48 | 2 |
| 493 | 10MHz | non- adjacent | QPSK | 3 | 5 |
| 503 | 10MHz |  | QPSK | 45 | 5 |
| 514 | 10MHz |  | QPSK | 3 | 5 |
| 524 | 10MHz |  | QPSK | 40 | 5 |
| 534 | 10MHz |  | QPSK | 45 | 5 |
| 545 | 10MHz |  | QPSK | 16 | 5 |
| 555 | 10MHz |  | QPSK | 40 | 5 |
| 565 | 10MHz |  | QPSK | 45 | 5 |
| 573 | 10MHz | adjacent | QPSK | 48 | 2 |
| 584 | 10MHz |  | QPSK | 3 | 2 |
| 594 | 10MHz |  | QPSK | 3 | 47 |
| 605 | 10MHz |  | QPSK | 3 | 47 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4G as not all combinations are necessarily required based on the applicability of the UE. Note 3: Only applicable for Low range test frequency. Note 4: Only applicable for High range test frequency. Note 5: Only applicable for Mid range test frequency. |  |  |  |  |  |

Table 6.2.4G.1.4.1-2: Test Configuration Table for Power Class 2

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  | Lowest |  |  |
| Test Parameters for NS_34 A-MPR |  |  |  |  |  |
|  |  | V2X Configuration to Transmit |  |  |  |
| Configura-tion ID | Ch BW | Resource pool | Mod'n | PSSCH RB allocation | PSSCH RBstart |
| 13 | 10MHz | adjacent | QPSK | 16 | 2 |
| 23 | 10MHz |  | QPSK | 27 | 2 |
| 33 | 10MHz |  | QPSK | 48 | 2 |
| 43 | 10MHz |  | 16QAM | 16 | 2 |
| 53 | 10MHz |  | 16QAM | 27 | 2 |
| 63 | 10MHz |  | 16QAM | 48 | 2 |
| 75 | 10MHz |  | QPSK | 12 | 2 |
| 85 | 10MHz |  | QPSK | 36 | 2 |
| 95 | 10MHz |  | QPSK | 48 | 2 |
| 105 | 10MHz |  | 16QAM | 12 | 2 |
| 115 | 10MHz |  | 16QAM | 36 | 2 |
| 125 | 10MHz |  | 16QAM | 48 | 2 |
| 133 | 10MHz | non- adjacent | QPSK | 45 | 5 |
| 144 | 10MHz |  | QPSK | 6 | 5 |
| 154 | 10MHz |  | QPSK | 16 | 5 |
| 164 | 10MHz |  | QPSK | 45 | 5 |
| 175 | 10MHz |  | QPSK | 6 | 5 |
| 185 | 10MHz |  | QPSK | 45 | 5 |
| 193 | 10MHz | adjacent | QPSK | 3 | 2 |
| 203 | 10MHz |  | QPSK | 3 | 47 |
| 214 | 10MHz |  | QPSK | 3 | 2 |
| 224 | 10MHz |  | QPSK | 3 | 47 |
| 234 | 10MHz |  | QPSK | 48 | 2 |
| 245 | 10MHz |  | QPSK | 3 | 2 |
| 255 | 10MHz |  | QPSK | 3 | 47 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4G as not all combinations are necessarily required based on the applicability of the UE. Note 3: Only applicable for Low range test frequency. Note 4: Only applicable for High range test frequency. Note 5: Only applicable for Mid range test frequency. |  |  |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.2.4G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.2.4G.1.4.1-1 and Table 6.2.4G.1.4.1-2.

4. 	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.2.4G.1.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC. (UE should be already transmitting PUMAX after Initial Conditions setting)

2.	Measure the mean power of the UE in the channel bandwidth for each test point in table 6.2.4G.1.5-1 according to the test configuration from Table 6.2.4G.1.4.1-1 for Power Class 3 and Table 6.2.4G.1.4.1-2 for Power Class 2. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.2.4G.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 6.2.4G.1.4.3-1: SL-CommResourcePoolV2X-r14 for non- adjacent resource pool

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-20C SL-CommResourcePoolV2X-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResourcePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | FALSE | BOOLEAN |  |
| numSubchannel-r14 | n9 |  |  |
| startRB-Subchannel-r14 | 5 |  | 10 MHz |
| startRB-PSCCH-Pool-r14 | 0 |  |  |

6.2.4G.1.4.3.1	Message contents exceptions (network signalled value "NS_33")

1.	Information element additionalSpectrumEmission is set to NS_33. This can be set in the SL-PreconfigGeneral-r12 as part of the SL-V2X-Preconfiguration. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4G.1.4.3.1-1: SL-V2X-PreconfigFreqInfo-r14-DEFAULT: Additional spurious emissions test requirement for "NS_33"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-20K SL-V2X-PreconfigFreqInfo-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission-v1440 | 33 (NS_33) |  |  |

6.2.4G.1.4.3.2	Message contents exceptions (network signalled value "NS_34")

1.	Information element additionalSpectrumEmission is set to NS_34. This can be set in the SL-PreconfigGeneral-r12 as part of the SL-V2X-Preconfiguration. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4G.1.4.3.2-1: SL-V2X-PreconfigFreqInfo-r14-DEFAULT: Additional spurious emissions test requirement for "NS_34"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-20K SL-V2X-PreconfigFreqInfo-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission-v1440 | 34 (NS_34) |  |  |

6.2.4G.1.5	Test Requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table 6.2.4G.1.4.1-1 and Table 6.2.4G.1.4.1-2. For the UE maximum output power modified by A-MPR specified in sub-clause 6.2.4G.1.3, the power limits specified in Table 6.2.5.3-1 apply.

Table 6.2.4G.1.5-1: Test requirement (network signalled value NS_33)

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.5 | 20 | 0 | 1.5 | 7.0 | 26.3 | -6.8 |
| 2 | 1.5 | 8 | 0 | 13.5 | 5.0 | 26.3 | 7.2 |
| 3 | 1.5 | 6 | 0 | 15.5 | 5.0 | 26.3 | 9.2 |
| 4 | 1.5 | 5 | 0 | 16.5 | 5.0 | 26.3 | 10.2 |
| 5 | 1.5 | 15.5 | 0 | 6 | 7.0 | 26.3 | -2.3 |
| 6 | 1.5 | 8 | 0 | 13.5 | 5.0 | 26.3 | 7.2 |
| 7 | 1.5 | 6 | 0 | 15.5 | 5.0 | 26.3 | 9.2 |
| 8 | 1.5 | 5 | 0 | 16.5 | 5.0 | 26.3 | 10.2 |
| 9 | 1.5 | 15 | 0 | 6.5 | 7.0 | 26.3 | -3.3 |
| 10 | 1.5 | 11.5 | 0 | 10 | 6.0 | 26.3 | 2.7 |
| 11 | 1.5 | 10 | 0 | 11.5 | 6.0 | 26.3 | 4.2 |
| 12 | 1.5 | 6 | 0 | 15.5 | 5.0 | 26.3 | 9.2 |
| 13 | 1.5 | 5 | 0 | 16.5 | 5.0 | 26.3 | 10.2 |
| 14 | 1.5 | 4.5 | 0 | 17 | 5.0 | 26.3 | 10.7 |
| 15 | 1.5 | 12.5 | 0 | 9 | 6.0 | 26.3 | 1.7 |
| 16 | 1.5 | 10.5 | 0 | 11 | 6.0 | 26.3 | 3.7 |
| 17 | 1.5 | 9.5 | 0 | 12 | 6.0 | 26.3 | 4.7 |
| 18 | 1.5 | 6.5 | 0 | 15 | 5.0 | 26.3 | 8.7 |
| 19 | 2 | 20 | 0 | 1 | 7.0 | 26.3 | -7.3 |
| 20 | 2 | 8 | 0 | 13 | 5.0 | 26.3 | 6.7 |
| 21 | 2 | 6 | 0 | 15 | 5.0 | 26.3 | 8.7 |
| 22 | 2 | 5 | 0 | 16 | 5.0 | 26.3 | 9.7 |
| 23 | 2 | 15.5 | 0 | 5.5 | 7.0 | 26.3 | -2.8 |
| 24 | 2 | 8 | 0 | 13 | 5.0 | 26.3 | 6.7 |
| 25 | 2 | 6 | 0 | 15 | 5.0 | 26.3 | 8.7 |
| 26 | 2 | 5 | 0 | 16 | 5.0 | 26.3 | 9.7 |
| 27 | 2 | 15 | 0 | 6 | 7.0 | 26.3 | -2.3 |
| 28 | 2 | 11.5 | 0 | 9.5 | 6.0 | 26.3 | 2.2 |
| 29 | 2 | 10 | 0 | 11 | 6.0 | 26.3 | 3.7 |
| 30 | 2 | 6 | 0 | 15 | 5.0 | 26.3 | 8.7 |
| 31 | 2 | 5 | 0 | 16 | 5.0 | 26.3 | 9.7 |
| 32 | 2 | 4.5 | 0 | 16.5 | 5.0 | 26.3 | 10.2 |
| 33 | 2 | 12.5 | 0 | 8.5 | 6.0 | 26.3 | 1.2 |
| 34 | 2 | 10.5 | 0 | 10.5 | 6.0 | 26.3 | 3.2 |
| 35 | 2 | 9.5 | 0 | 11.5 | 6.0 | 26.3 | 4.2 |
| 36 | 2 | 6.5 | 0 | 14.5 | 5.0 | 26.3 | 8.2 |
| 37 | 1.5 | 3 | 0 | 18.5 | 4.0 | 26.3 | 13.2 |
| 38 | 1.5 | 1.5 | 0 | 20 | 2.5 | 26.3 | 16.2 |
| 39 | 1.5 | 2.5 | 0 | 19 | 3.5 | 26.3 | 14.2 |
| 40 | 2 | 3 | 0 | 18 | 4.0 | 26.3 | 12.7 |
| 41 | 2 | 1.5 | 0 | 19.5 | 3.5 | 26.3 | 14.7 |
| 42 | 2 | 2.5 | 0 | 18.5 | 4.0 | 26.3 | 13.2 |
| 43 | 1.5 | 3 | 0 | 18.5 | 4.0 | 26.3 | 13.2 |
| 44 | 1.5 | 1.5 | 0 | 20 | 2.5 | 26.3 | 16.2 |
| 45 | 1.5 | 2 | 0 | 19.5 | 3.5 | 26.3 | 14.7 |
| 46 | 2 | 3 | 0 | 18 | 4.0 | 26.3 | 12.7 |
| 47 | 2 | 1.5 | 0 | 19.5 | 3.5 | 26.3 | 14.7 |
| 48 | 2 | 2 | 0 | 19 | 3.5 | 26.3 | 14.2 |
| 49 | 4.5 | 13.5 | 0 | 5 | 7.0 | 26.3 | -3.3 |
| 50 | 2 | 11.5 | 0 | 9.5 | 6.0 | 26.3 | 2.2 |
| 51 | 4.5 | 5 | 0 | 13.5 | 5.0 | 26.3 | 7.2 |
| 52 | 2 | 3 | 0 | 18 | 4.0 | 26.3 | 12.7 |
| 53 | 2 | 4.5 | 0 | 16.5 | 5.0 | 26.3 | 10.2 |
| 54 | 3.5 | 3.5 | 0 | 16 | 5.0 | 26.3 | 9.7 |
| 55 | 2 | 2.5 | 0 | 18.5 | 4.0 | 26.3 | 13.2 |
| 56 | 2 | 3 | 0 | 18 | 4.0 | 26.3 | 12.7 |
| 57 | 1.5 | 12.5 | 0 | 9 | 6.0 | 26.3 | 1.7 |
| 58 | 1.5 | 3 | 0 | 18.5 | 4.0 | 26.3 | 13.2 |
| 59 | 1.5 | 3 | 0 | 18.5 | 4.0 | 26.3 | 13.2 |
| 60 | 1.5 | 3 | 0 | 18.5 | 4.0 | 26.3 | 13.2 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: The default value of Gpost connector is 0dBi, if external antenna is not used. |  |  |  |  |  |  |  |

Table 6.2.4G.1.5-2: Test requirement (network signalled value NS_34)

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 15 |  | 9 | 6.0 | 29.3 | 1.7 |
| 2 | 2 | 13 | 0 | 11 | 6.0 | 29.3 | 3.7 |
| 3 | 2 | 12 | 0 | 12 | 6.0 | 29.3 | 4.7 |
| 4 | 2.5 | 15 |  | 8.5 | 6.0 | 29.3 | 1.2 |
| 5 | 2.5 | 13 | 0 | 10.5 | 6.0 | 29.3 | 3.2 |
| 6 | 2.5 | 12 | 0 | 11.5 | 6.0 | 29.3 | 4.2 |
| 7 | 2 | 1 | 0 | 23 | 2.0 | 29.3 | 19.7 |
| 8 | 2 | 0 | 0 | 24 | 2.0 | 29.3 | 20.7 |
| 9 | 2 | 1 | 0 | 23 | 2.0 | 29.3 | 19.7 |
| 10 | 2.5 | 1 | 0 | 22.5 | 2.0 | 29.3 | 19.2 |
| 11 | 2.5 | 0 | 0 | 23.5 | 2.0 | 29.3 | 20.2 |
| 12 | 2.5 | 1 | 0 | 22.5 | 2.0 | 29.3 | 19.2 |
| 13 | 2.5 | 12.5 | 0 | 11.0 | 6.0 | 29.3 | 3.7 |
| 14 | 4.5 | 5.5 | 0 | 16.0 | 5.0 | 29.3 | 9.7 |
| 15 | 2.5 | 3 | 0 | 20.5 | 2.5 | 29.3 | 16.7 |
| 16 | 2.5 | 0 | 0 | 23.5 | 2.0 | 29.3 | 20.2 |
| 17 | 4.5 | 5.5 | 0 | 16.0 | 5.0 | 29.3 | 9.7 |
| 18 | 2.5 | 0 | 0 | 23.5 | 2.0 | 29.3 | 20.2 |
| 19 | 2 | 15 | 0 | 9 | 6.0 | 29.3 | 1.7 |
| 20 | 2 | 15 | 0 | 9 | 6.0 | 29.3 | 1.7 |
| 21 | 2 | 0 | 0 | 24 | 2.0 | 29.3 | 20.7 |
| 22 | 2 | 0 | 0 | 24 | 2.0 | 29.3 | 20.7 |
| 23 | 2 | 0 | 0 | 24 | 2.0 | 29.3 | 20.7 |
| 24 | 2 | 1 | 0 | 23 | 2.0 | 29.3 | 19.7 |
| 25 | 2 | 1 | 0 | 23 | 2.0 | 29.3 | 19.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: The default value of Gpost connector is 0dBi, if external antenna is not used. |  |  |  |  |  |  |  |

#### 6.2.4G.2	Additional Maximum Power Reduction (A-MPR) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions

6.2.4G.2.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2.4G.2.2	Test applicability

The requirements of this test apply in test case 6.6.2.2.4 Additional Spectrum Emission Mask for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for network signalled values NS_04/NS_33 to all types of E-UTRA Power Class 3 UE release 14 and forward that support V2X communication and is capable of V2X Sidelink communication simultaneous with E-UTRA uplink transmissions.

The requirements of this test apply in test case 6.6.3.3 Additional Spurious Emissions for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for network signalled values NS_04/NS_33 to all types of E-UTRA Power Class 3 UE release 14 and forward that support V2X communication and is capable of V2X Sidelink communication simultaneous with E-UTRA uplink transmissions.

NOTE:	As a result TC 6.2.4G.2 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.4G.2.3	Minimum conformance requirements

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.2G-2, the requirements in subclause 6.2.4G.1 apply for V2X PSSCH and PSCCH transmission. The allowed A-MPR requirements in subclause 6.2.4D [2] apply for other V2X sidelink transmission (PSBCH/PSSS/SSSS). The A-MPR requirements in subclause 6.2.4 apply for uplink transmission.

The configured maximum output power PCMAX c,E-UTRA(p) in subframe p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L,c,E-UTRA (p) ≤  PCMAX,c,E-UTRA (p) ≤  PCMAX_H,c,E-UTRA (p)

where PCMAX_L,c,E-UTRA and PCMAX_H,c,E-UTRA are the limits for a serving cell c as specified in subclause 6.2.5.

The configured maximum output power PCMAX c,V2X (q) in subframe q for the configured E-UTRA V2X carrier shall be set within the bounds:

PCMAX,c,V2X (q) ≤  PCMAX_H,c,V2X (q)

where PCMAX_H,c,V2X is the limit as specified in subclause 6.2.5G.

The total UE configured maximum output power PCMAX (p,q) in a subframe p of an E-UTRA uplink carrier and a subframe q of an E-UTRA V2X sidelink that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) =  PCMAX_L,c,E-UTRA (p)

PCMAX_H (p,q) = 10 log10 [pCMAX_H,c,E-UTRA (p) + pCMAX_H,c,V2X (q)]

where pCMAX_H,c,V2X and pCMAX_H,c,E-UTRA are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,E-UTRA (p) expressed in linear scale.

The measured total maximum output power PUMAX over both the E-UTRA uplink and E-UTRA V2X carriers is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,V2X],

where pUMAX,c,E-UTRA  denotes the measured output power of serving cell c for the configured E-UTRA uplink carrier, and pUMAX,c,V2X  denotes the measured output power for the configured E-UTRA V2X carrier expressed in linear scale.

When a UE is configured for synchronous V2X sidelink and uplink transmissions,

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2.5C-1. PCMAX_L may be modified for any overlapping portion of subframes (p, q) and (p +1, q+1).

The normative reference for this requirement is in TS 36.101 [2] clause 6.2.4G.

6.2.4G.2.4	Test description

6.2.4G.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4G.2.4.1-1. The details of the uplink and V2X reference measurement channels (RMCs) are specified in A.2 and A.8.3 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4G.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  |  | Low range for uplink and Low range for V2X, High range for uplink and High range for V2X |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  |  | 5MHz for uplink and 10MHz for V2X 20MHz for uplink and 10MHz for V2X |  |  |  |  |
| Test Parameters for NS_04 and NS_33 A-MPR |  |  |  |  |  |  |  |  |  |
|  | E-UTRA uplink Configuration to Transmit |  |  |  | V2X Configuration to Transmit |  |  |  |  |
| Configura-tion ID | Ch BW | Mod’n | PUSCH RB allocation | RBstart TDD | Ch BW | Resource pool | Mod'n | PSSCH RB allocation | PSSCH RBstart |
| 11 | 5MHz | QPSK | 1 | 0 | 10MHz | adjacent | QPSK | 8 | 2 |
| 21 | 5MHz | QPSK | 16 | 9 | 10MHz |  | QPSK | 27 | 2 |
| 31 | 5MHz | 16QAM | 16 | 9 | 10MHz |  | QPSK | 8 | 2 |
| 41 | 5MHz | 16QAM | 16 | 9 | 10MHz |  | QPSK | 27 | 2 |
| 52 | 5MHz | QPSK | 25 | 0 | 10MHz |  | QPSK | 8 | 2 |
| 62 | 5MHz | QPSK | 25 | 0 | 10MHz |  | QPSK | 48 | 2 |
| 72 | 5MHz | 16QAM | 25 | 0 | 10MHz |  | QPSK | 8 | 2 |
| 82 | 5MHz | 16QAM | 25 | 0 | 10MHz |  | QPSK | 48 | 2 |
| 91 | 20MHz | QPSK | 1 | 0 | 10MHz |  | QPSK | 8 | 2 |
| 101 | 20MHz | QPSK | 100 | 0 | 10MHz |  | QPSK | 27 | 2 |
| 111 | 20MHz | 16QAM | 100 | 0 | 10MHz |  | QPSK | 8 | 2 |
| 121 | 20MHz | 16QAM | 100 | 0 | 10MHz |  | QPSK | 27 | 2 |
| 132 | 20MHz | QPSK | 100 | 0 | 10MHz |  | QPSK | 8 | 2 |
| 142 | 20MHz | QPSK | 100 | 0 | 10MHz |  | QPSK | 48 | 2 |
| 152 | 20MHz | 16QAM | 100 | 0 | 10MHz |  | 16QAM | 8 | 2 |
| 162 | 20MHz | 16QAM | 100 | 0 | 10MHz |  | 16QAM | 48 | 2 |
| 171 | 5MHz | QPSK | 1 | 0 | 10MHz | non- adjacent | QPSK | 3 | 5 |
| 181 | 5MHz | QPSK | 16 | 9 | 10MHz |  | QPSK | 45 | 5 |
| 191 | 5MHz | 16QAM | 16 | 9 | 10MHz |  | QPSK | 3 | 5 |
| 201 | 5MHz | 16QAM | 16 | 9 | 10MHz |  | QPSK | 45 | 5 |
| 212 | 5MHz | QPSK | 25 | 0 | 10MHz |  | QPSK | 3 | 5 |
| 222 | 5MHz | QPSK | 25 | 0 | 10MHz |  | QPSK | 45 | 5 |
| 232 | 5MHz | 16QAM | 25 | 0 | 10MHz |  | QPSK | 3 | 5 |
| 242 | 5MHz | 16QAM | 25 | 0 | 10MHz |  | QPSK | 45 | 5 |
| 251 | 20MHz | QPSK | 1 | 0 | 10MHz |  | QPSK | 3 | 5 |
| 261 | 20MHz | QPSK | 100 | 0 | 10MHz |  | QPSK | 45 | 5 |
| 271 | 20MHz | 16QAM | 100 | 0 | 10MHz |  | QPSK | 3 | 5 |
| 281 | 20MHz | 16QAM | 100 | 0 | 10MHz |  | QPSK | 45 | 5 |
| 292 | 20MHz | QPSK | 100 | 0 | 10MHz |  | QPSK | 3 | 5 |
| 302 | 20MHz | QPSK | 100 | 0 | 10MHz |  | QPSK | 45 | 5 |
| 312 | 20MHz | 16QAM | 100 | 0 | 10MHz |  | QPSK | 3 | 5 |
| 322 | 20MHz | 16QAM | 100 | 0 | 10MHz |  | QPSK | 45 | 5 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4G as not all combinations are necessarily required based on the applicability of the UE. Note 3: Only applicable for Low range for uplink and Low range for V2X. Note 4: Only applicable for High range for uplink and High range for V2X. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.2.4G.2.4.3

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to Table 6.2.4G.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message content exceptions are defined in clause 6.2.4G.2.4.3.

6.2.4G.2.4.2	Test procedure

1.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.2.4G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level. Configure the UE to transmit PSCCH and PSSCH with the PUMAX level of each test points; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power over all component carriers in the V2X con-current band configuration. The period of measurement shall be at least continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

6.2.4G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 6.2.4G.2.4.3-1: SL-CommResourcePoolV2X-r14 for non- adjacent resource pool

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-20C SL-CommResourcePoolV2X-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResourcePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | FALSE | BOOLEAN |  |
| numSubchannel-r14 | n9 |  |  |
| startRB-Subchannel-r14 | 5 |  | 10 MHz |
| startRB-PSCCH-Pool-r14 | 0 |  |  |

6.2.4G.2.4.3.1	Message contents exceptions (network signalled value "NS_04" for E-UTRA uplink transmission)

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4G.2.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_04) |  |  |

6.2.4G.2.4.3.2	Message contents exceptions (network signalled value "NS_33" for E-UTRA V2X sidelink transmission)

1.	Information element additionalSpectrumEmission is set to NS_33. This can be set in the SL-PreconfigGeneral-r12 as part of the SL-V2X-Preconfiguration. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4G.2.4.3.1-2: SL-PreconfigGeneral-r12: Additional spurious emissions test requirement for "NS_33"

| Derivation Path: TS 36.331 [5] clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 33 (NS_33) |  |  |

6.2.4G.2.5	Test Requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table 6.2.4G.2.4.1-1. For the UE maximum output power modified by A-MPR specified in sub-clause 6.2.4G.2.3, the power limits specified in Table 6.2.5.3-1 apply.

Table 6.2.4G.2.5-1: Test requirement (network signalled value NS_04/NS_33)

| Configuration ID | MPRE-UTRA (dB) | A-MPRE-UTRA (dB) | ΔTC,c.E-UTRA (dB) | PCMAX,c,E-UTRA (dBm) | MPRV2X (dB) | A-MPRV2X (dB) | ΔTC,c,V2X (dB) | PCMAX,c,V2X (dBm) | ΔTIB,c (dB) | PCMAX_L(p, q) (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 2 | 1.5 | 19.5 | 1.5 | 15.5 | 1.5 | 4.5 | 0 | 19.6 | 5 | 26.3 | 13.3 |
| 2 | 1 | 0 | 0 | 22 | 1.5 | 12.5 | 0 | 9 | 0 | 22.2 | 5 | 26.3 | 15.9 |
| 3 | 2 | 0 | 0 | 21 | 1.5 | 15.5 | 1.5 | 4.5 | 0 | 21.1 | 5 | 26.3 | 14.8 |
| 4 | 2 | 0 | 0 | 21 | 1.5 | 12.5 | 0 | 9 | 0 | 21.3 | 5 | 26.3 | 15.0 |
| 5 | 1 | 0 | 0 | 22 | 1.5 | 3 | 0 | 18.5 | 0 | 23.6 | 3 | 26.3 | 19.3 |
| 6 | 1 | 0 | 0 | 22 | 1.5 | 2.5 | 0 | 19 | 0 | 23.8 | 3 | 26.3 | 19.5 |
| 7 | 2 | 0 | 0 | 21 | 1.5 | 3] | 0 | 18.5 | 0 | 22.9 | 5 | 26.3 | 16.6 |
| 8 | 2 | 0 | 0 | 21 | 1.5 | 2 | 0 | 19 | 0 | 23.1 | 3 | 26.3 | 18.8 |
| 9 | 0 | 3 | 1.5 | 18.5 | 1.5 | 15.5 | 1.5 | 4.5 | 0 | 18.7 | 5 | 26.3 | 12.4 |
| 10 | 1 | 3 | 0 | 19 | 1.5 | 12.5 | 0 | 9 | 0 | 19.4 | 5 | 26.3 | 13.1 |
| 11 | 2 | 3 | 0 | 18 | 1.5 | 15.5 | 0 | 6 | 0 | 18.3 | 5 | 26.3 | 12.0 |
| 12 | 2 | 3 | 0 | 18 | 1.5 | 12.5 | 0 | 9 | 0 | 18.5 | 5 | 26.3 | 12.2 |
| 13 | 1 | 0 | 0 | 22 | 1.5 | 3 | 0 | 18.5 | 0 | 23.6 | 3 | 26.3 | 19.3 |
| 14 | 1 | 0 | 0 | 22 | 1.5 | 2.5 | 0 | 19 | 0 | 23.8 | 3 | 26.3 | 19.5 |
| 15 | 2 | 0 | 0 | 21 | 2 | 3 | 0 | 18 | 0 | 22.8 | 5 | 26.3 | 16.5 |
| 16 | 2 | 0 | 0 | 21 | 2 | 2.5 | 0 | 18.5 | 0 | 22.9 | 5 | 26.3 | 16.6 |
| 17 | 0 | 2 | 1.5 | 19.5 | 4.5 | 13.5 | 1.5 | 3.5 | 0 | 19.6 | 5 | 26.3 | 13.3 |
| 18 | 1 | 0 | 0 | 22 | 2 | 11.5 | 0 | 9.5 | 0 | 22.2 | 5 | 26.3 | 15.9 |
| 19 | 2 | 0 | 0 | 21 | 4.5 | 13.5 | 1.5 | 3.5 | 0 | 21.1 | 5 | 26.3 | 14.8 |
| 20 | 2 | 0 | 0 | 21 | 2 | 11.5 | 0 | 9.5 | 0 | 21.3 | 5 | 26.3 | 15.0 |
| 21 | 1 | 0 | 0 | 22 | 4.5 | 53.5 | 0 | 13.5 | 0 | 22.6 | 5 | 26.3 | 16.3 |
| 22 | 1 | 0 | 0 | 22 | 2 | 4.5 | 0 | 16.5 | 0 | 23.1 | 3 | 26.3 | 18.8 |
| 23 | 2 | 0 | 0 | 21 | 4.5 | 5 | 0 | 13.5 | 0 | 21.7 | 5 | 26.3 | 15.4 |
| 24 | 2 | 0 | 0 | 21 | 2 | 4.5 | 0 | 16.5 | 0 | 22.3 | 5 | 26.3 | 16.0 |
| 25 | 0 | 3 | 1.5 | 18.5 | 4.5 | 13.5 | 1.5 | 3.5 | 0 | 18.6 | 5 | 26.3 | 12.3 |
| 26 | 1 | 3 | 0 | 19 | 2 | 11.5 | 0 | 9.5 | 0 | 19.5 | 5 | 26.3 | 13.2 |
| 27 | 2 | 3 | 0 | 18 | 4.5 | 13.5 | 1.5 | 3.5 | 0 | 18.2 | 5 | 26.3 | 11.9 |
| 28 | 2 | 3 | 0 | 18 | 2 | 11.5 | 0 | 9.5 | 0 | 18.6 | 5 | 26.3 | 12.3 |
| 29 | 1 | 0 | 0 | 22 | 4.5 | 5 | 0 | 13.5 | 0 | 22.6 | 5 | 26.3 | 16.3 |
| 30 | 1 | 0 | 0 | 22 | 2 | 4.5 | 0 | 16.5 | 0 | 23.1 | 3 | 26.3 | 18.8 |
| 31 | 2 | 0 | 0 | 21 | 4.5 | 5 | 0 | 13.5 | 0 | 21.7] | 5 | 26.3 | 15.4 |
| 32 | 2 | 0 | 0 | 21 | 2 | 4.5 | 0 | 16.5 | 0 | 22.3 | 5 | 26.3 | 16.0 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5G.2.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged Sections Skipped>

#### 6.6.2.1G	Spectrum Emission Mask for V2X Communication

##### 6.6.2.1G.1	Spectrum Emission Mask for V2X Communication Non-concurrent with E-UTRA uplink transmissions

###### 6.6.2.1G.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth when UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands.

###### 6.6.2.1G.1.2	Test applicability

This test case applies to UE which support V2X Sidelink communication and Band 47.

###### 6.6.2.1G.1.3	Minimum conformance requirements

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in TS 36.101 [2] Table 5.5G-1, the requirements in TS 36.101[2] subclause 6.6.2 apply except for the ACLR requirements for power class 2 V2X UE.

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1G.1.3-1.

Table 6.6.2.1G.1.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G.

###### 6.6.2.1G.1.4	Test description

6.6.2.1G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1G.1.4.1-1 the details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.2.1G.1.4.1-1: Test Configuration for V2X sidelink transmission

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |
| Test Channel Bandwidths (as specified in subclause 5.4.2G) |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |
|  | V2X Configuration to Trasmit |  |
| Ch BW | Mod'n | PSSCH RB Allocation |
| 10MHz | QPSK | 12 (Note 3) |
| 10MHz | QPSK | 48@2 |
| 10MHz | 16QAM | 12 (Note 3) |
| 10MHz | 16QAM | 48@2 |
| 20MHz | QPSK | 18 (Note 3) |
| 20MHz | QPSK | 96@2 |
| 20MHz | 16QAM | 18 (Note 3) |
| 20MHz | 16QAM | 96@2 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in specified in subclause 5.4.2G. Note 2: The allowed MPR for maximum output power UE might apply is described in clause 36.101[2] subclause 6.2.3G. Note 3: The test shall be performed with allocated sub-channel(s) at low and high end of the channel bandwidth respectively. |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.6.2.1G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.6.2.1G.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in State 5A-V2X  in Transmit Mode according to TS 36.508 [7] clause 4.5.9 , Message contents are defined in clause 6.6.2.1G.1.4.3

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.6.2.1G.1.4.2	Test procedure

1.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.6.2.1G.1.4.1-1;

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3G.1.1.5-1 for Power Class 3 UEs or in 6.2.3G.2.1.5-1 for Power Class 2 UEs. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1G.1.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1G.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with following exceptions listed in the Table 6.6.2.1G.1.4.3-1 and Table 6.6.2.1G.1.4.3-2.

Table 6.6.2.1G.1.4.3-1: SL-V2X-Preconfiguration

| Derivation Path: 36.331 clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-Preconfiguration-r14 ::= SEQUENCE { |  |  |  |
| v2x-PreconfigFreqList-r14 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
| maxTxPower-r12 | 23 |  | Power class3 |
|  | 26 |  | Power class2 |
| sl-bandwidth-r12 | n50 | 10Mhz |  |
|  | n100 | 20Mhz |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommPreconfigSync-r14 SEQUENCE { |  | SL-PreconfigSync-r12 |  |
| syncOffsetIndicators-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicator1-r14 | Not Present | Double synchronization signal transmission |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommRxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { | 1 entry |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-CommResourcePoolV2X-r14-DEFAULT with exception listed in Table 6.6.2.1G.1.4.3-2 |  |  |
| } |  |  |  |
| v2x-CommTxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { |  |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT with exception listed in Table 6.6.2.1G.1.4.3-2 |  |  |
| } |  |  |  |
| v2x-ResourceSelectionConfig | SL-CommTxPoolSensingConfig-r14-DEFAULT with exception listed in Table 6.6.2.1G.1.4.3-3 |  |  |
| } |  |  |  |

Table 6.6.2.1G.1.4.3-2: SL-V2X-PreconfigCommPool-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResroucePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11111111111111111111 |  | BITMAP_6 |
| } |  |  |  |
| sizeSubchannel-r14 | n15 |  | BW10 |
|  | n50 |  | BW10 |
|  | n20 |  | BW20 |
|  | n100 |  | BW20 |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| p0-r12 | 31 | It’s set to the maximum value to disable its effect on transmission power. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.1G.1.4.3-2: SL-PSSCH-TxParameters-r14

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PSSCH-TxParameters-r14 ::= SEQUENCE { |  |  |  |
| minMCS-PSSCH-r14 | 0 |  | QPSK |
|  | 11 |  | 16QAM |
| maxMCS-PSSCH-r14 | 10 |  | QPSK |
|  | 19 |  | 16QAM |
| } |  |  |  |

###### 6.6.2.1G.1.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3G.1.1.5-1as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1G.1.5-1.

Table 6.6.2.1G.1.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≥ 5GHz

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -16 | -19 | 30 kHz |
| 1-2.5 | -8 | -8 | 1 MHz |
| 2.5-2.8 | -8 | -8 | 1 MHz |
| 2.8-5 | -8 | -8 | 1 MHz |
| 5-6 | -12 | -11 | 1 MHz |
| 6-10 | -12 | -11 | 1 MHz |
| 10-15 | -23 | -11 | 1 MHz |
| 15-20 |  | -11 | 1 MHz |
| 20-25 |  | -23 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.1G.2	Spectrum Emission Mask for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions

###### 6.6.2.1G.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth when UE is configured for configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA band

###### 6.6.2.1G.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that supports V2X communication and is capable of V2X Sidelink communication simultaneous with E-UTRA uplink transmissions.

###### 6.6.2.1G.2.3	Minimum conformance requirements

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in TS 36.101 [2] Table 5.5G-2, the requirements in TS 36.101 [2] subclause 6.6.2 apply per V2X sidelink transmission and E-UTRA uplink transmission as specified for the corresponding inter-band con-current operation with uplink assigned to two bands.

Consequently, the emission power of the V2X UE per sidelink and E-UTRA link shall not exceed the values specified in Tables 6.6.2.1G.2.2-1

Table 6.6.2.1G.2.2-1 General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2G and subclause 6.6.2.

###### 6.6.2.1G.2.4	Test description

6.6.2.1G.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on Inter-band con-current V2X configurations specified in table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1G.2.1.4.1-1. The details of the uplink and V2X reference measurement channels (RMCs) are specified in sub-clause A.2 and A.8.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1G.2.4.1-1: Test Configuration

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Low range for uplink and Low range for V2X, High range for uplink and High range for V2X |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  |  | V2X Configuration |  |  |
| Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  | Ch BW | Mod'n | PSSCH RB allocation |
| 1.4MHz |  | QPSK | 5 |  | 10 MHz | QPSK | 48@2 |
| 1.4MHz |  | QPSK | 6 |  | 10 MHz | 16QAM | 48@2 |
| 1.4MHz |  | 16QAM | 5 |  | 20 MHz | QPSK | 96@2 |
| 1.4MHz |  | 16QAM | 6 |  | 20 MHz | 16QAM | 96@2 |
| 3MHz |  | QPSK | 4 |  |  |  |  |
| 3MHz |  | QPSK | 15 |  |  |  |  |
| 3MHz |  | 16QAM | 4 |  |  |  |  |
| 3MHz |  | 16QAM | 15 |  |  |  |  |
| 5MHz |  | QPSK | 8 |  |  |  |  |
| 5MHz |  | QPSK | 25 |  |  |  |  |
| 5MHz |  | 16QAM | 8 |  |  |  |  |
| 5MHz |  | 16QAM | 25 |  |  |  |  |
| 10MHz |  | QPSK | 12 |  |  |  |  |
| 10MHz |  | QPSK | 50 |  |  |  |  |
| 10MHz |  | 16QAM | 12 |  |  |  |  |
| 10MHz |  | 16QAM | 50 (Note 3) |  |  |  |  |
| 15MHz |  | QPSK | 16 |  |  |  |  |
| 15MHz |  | QPSK | 75 |  |  |  |  |
| 15MHz |  | 16QAM | 16 |  |  |  |  |
| 15MHz |  | 16QAM | 75 (Note 3) |  |  |  |  |
| 20MHz |  | QPSK | 18 |  |  |  |  |
| 20MHz |  | QPSK | 100 |  |  |  |  |
| 20MHz |  | 16QAM | 18 |  |  |  |  |
| 20MHz |  | 16QAM | 100 (Note 3) |  |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: For Low range the E-UTRA carrier uses only test points with RBstart=0. For High range the E-UTRA carrier uses only test points with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.6.2.1G.2.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to table 6.6.2.1G.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message content exceptions are defined in clause 6.2.3G.1.3.4.3.

6.6.2.1G.2.4.2	Test procedure

SS sends RRC reconfiguration message specified in clause 6.6.2.1G.2.4.3 to enable PSCCH and PSSCH transmission at the PUMAX level.;

1.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.6.2.1G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

2.	SS sends continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3G.1.3.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal for E-UTRA uplink with a measurement filter of bandwidths according to table 6.6.2.1G.2.5-1 or 6.6.2.1G.2.5-2, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

5.	Measure the power of the transmitted signal for V2X sidelink with a measurement filter of bandwidths according to table 6.6.2.1G.2.5-3, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1G.2.4.3	Message content

Message contents are according to TS 36.508 [7] subclause 4.6.

###### 6.6.2.1G.2.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3 and step 4 shall fulfil requirements in Table 6.2.3G.1.3.5-1, and the power of any UE emission shall fulfil requirements in Table 6.6.2.1G.1.5-1~ Table 6.6.2.1G.2.5-3.

Table 6.6.2.1G.2.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1G.2.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1G.2.5-3: General E-UTRA V2X spectrum emission mask, E-UTRA bands ≥ 5GHz

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -16 | -19 | 30 kHz |
| 1-2.5 | -8 | -8 | 1 MHz |
| 2.5-2.8 | -8 | -8 | 1 MHz |
| 2.8-5 | -8 | -8 | 1 MHz |
| 5-6 | -12 | -11 | 1 MHz |
| 6-10 | -12 | -11 | 1 MHz |
| 10-15 | -23 | -11 | 1 MHz |
| 15-20 |  | -11 | 1 MHz |
| 20-25 |  | -23 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.1G.3	Spectrum Emission Mask for V2X Communication / Intra-band contiguous MCC operation

###### 6.6.2.1G.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth when UE is configured for Intra-band contiguous MCC operation V2X sidelink transmissions.

###### 6.6.2.1G.3.2	Test Applicability

This test case applies to UE which support Intra-band contiguous multi-carrier operation V2X Sidelink communication and Band 47.

###### 6.6.2.1G.3.3	Minimum conformance requirements

For intra-band contiguous multi-carrier operation, the general CA spectrum emission mask for Bandwidth Class B shall apply, which refers to the minimum conformance requirements defined in clause 6.6.2.1A.0.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G.

###### 6.6.2.1G.3.4	Test Description

6.6.2.1G.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1G.3.4.1-1 the details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.2.1G.3.4.1-1: Test Configuration Table for PSSCH and PSCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range , High range |  |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  | V2X PSSCH Allocation |  |  |
| CC1 NRB | CC2 NRB | CC1 & CC2 Mod |  | CC1 and CC2 RB Allocation |
| 50 | 50 | QPSK |  | 12@2+0@0 |
| 50 | 50 | QPSK |  | 0@0+12@37 |
| 50 | 50 | QPSK |  | 48@2+0@0 |
| 50 | 50 | QPSK |  | 0@0+48@2 |
| 50 | 50 | QPSK |  | 48@2+48@2 |
| 50 | 50 | 16QAM |  | 12@2+0@0 |
| 50 | 50 | 16QAM |  | 0@0+12@37 |
| 50 | 50 | 16QAM |  | 48@2+0@0 |
| 50 | 50 | 16QAM |  | 0@0+48@2 |
| 50 | 50 | 16QAM |  | 48@2+48@2 |
| 50 | 50 | QPSK |  | 1@2+1@47 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Figure A.89a.

2.	Propagation conditions are set according to Annex B.0.

2a	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

3.	Ensure the UE is in test state 5A-V2X, according to TS 36.508 [7] clause 4.5.9. Message contents are defined in clause 6.6.2.1G.3.4.3.

4.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.6.2.1G.3.4.2	Test procedure

1.	Set the V2X sidelink communication parameters for both the V2X sidelink capable UE (hereinafter referred to as V2X UE) and SS according to SL-V2X-Preconfiguration in TS36.508[7] Table subclause 4.10  with the exception specified in table 6.6.2.1G.3.4.3-1;

2.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.6.2.1G.3.4.1-1;

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3G.1.4.5-1 for Power Class 3 UEs. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1G.3.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1G.3.4.3	Message Contents

Message content are according to 36.508[7] subclause 4.10 Table 4.10.1.1-2 with the exception listed in table 6.6.2.1G.3.4.3-1, 6.6.2.1G.3.4.3-2 and 6.6.2.1G.3.4.3-3

Table 6.6.2.1G.3.4.3-1: SL-V2X-PreconfigFreqInfo-r14-DEFAULT for CC1 & CC2

| Derivation Path: 36.331 clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
| maxTxPower-r12 | 23 |  | Power class 3 |
| sl-bandwidth-r12 | n50 | 10Mhz | V2X_47B |
| } |  |  |  |
| } |  |  |  |
| v2x-CommRxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { | 1 entry |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table 6.6.2.1G.3.4.3-2 | SL-V2X-PreconfigCommPool-r14 |  |
| } |  |  |  |
| v2x-CommTxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { |  |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table 6.6.2.1G.3.4.3-2 |  |  |
| } |  |  |  |
| v2x-ResourceSelectionConfig | SL-CommTxPoolSensingConfig-r14-DEFAULT with exception listed in Table 6.6.2.1G.3.4.3-3 |  |  |
| syncPriority-r14 | GNSS |  |  |
| } |  |  |  |

Table 6.6.2.1G.3.4.3-2: SL-V2X-PreconfigCommPool-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResroucePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11111111111111111111 |  | BITMAP_6 |
| } |  |  |  |
| sizeSubchannel-r14 | n1 |  | NRB_alloc = 1 |
|  | n12 |  | NRB_alloc = 12 |
|  | n48 |  | NRB_alloc = 48, NRB_alloc = 96 |
| numSubchannel-r14 | n1 |  |  |
| startRB-Subchannel-r14 | According to the test configuration in table 6.6.2.1G.3.4.1-1 |  |  |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| p0-r12 | 31 | It’s set to the maximum value to disable its effect on transmission power. |  |
| } |  |  |  |
| syncAllowed-r14 ::= SEQUENCE { |  |  |  |
| gnss-Sync-r14 | True | The resource of CC1 can be used if the UE is directly or indirectly synchronized to GNSS | CC with LCRB >0 |
| enb-Sync-r14 | True | The resource of CC1 can be used if the UE is directly or indirectly synchronized to enb | CC with LCRB =0 |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.1G.3.4.3-3: SL-PSSCH-TxParameters-r14

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PSSCH-TxParameters-r14 ::= SEQUENCE { |  |  |  |
| minMCS-PSSCH-r14 | 0 |  | QPSK |
|  | 11 |  | 16QAM |
| maxMCS-PSSCH-r14 | 10 |  | QPSK |
|  | 19 |  | 16QAM |
| } |  |  |  |

###### 6.6.2.1G.3.5	Test Requirement

The power of any UE emission shall fulfil requirements in Table 6.6.2.1G.3.5-1 Bandwidth Class B.

Table 6.6.2.1G.3.5-1: General E-UTRA spectrum emission mask for CA Bandwidth Class B, E-UTRA bands > 5GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | 19 | 30 kHz |
| 1-5 | 8 | 1 MHz |
| 5-14.95 | 11 | 1 MHz |
| 14.95-19.90 | 11 | 1 MHz |
| 19.90-19.95 | 23 | 1 MHz |
| 19.95-24.90 | 23 | 1 MHz |

<Unchanged Sections Skipped>

#### 6.6.2.2G	Additional Spectrum Emission Mask for V2X Communication

##### 6.6.2.2G.0	Minimum Requirements

TS 36.101 [2] clause 6.6.2G

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the requirements in subclause 6.6.2 apply except for the ACLR requirements for power class 2 V2X UE.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.6.2 apply per V2X sidelink transmission and E-UTRA uplink transmission as specified for the corresponding inter-band con-current operation with uplink assigned to two bands.

For intra-band contiguous multi-carrier operation, the general CA spectrum emission mask for CA Bandwidth Class B specified in subclause 6.6.2.1A shall apply for V2X Bandwidth Class B, the general CA spectrum emission mask for CA Bandwidth Class C specified in subclause 6.6.2.1A shall apply for V2X Bandwidth Class C and C1.

TS 36.101 [2] clause 6.2.2.4

The additional spectrum mask in Table 6.6.2.2.4-1 applies for E-UTRA V2X UE within 5 855 MHz to 5 950 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" or “NS_34” is indicated in the cell, the power of any V2X UE emission shall not exceed the levels specified in Table 6.6.2.2.4-1.

Table 6.6.2.2.4-1: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [] | 100 kHz |
| 0.5-5 | [] | 100 kHz |
| 5-10 | [] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I.

The normative reference for this requirement is TS 36.101 [2] clauses 6.6.2G, and 6.6.2.2.4 and 6.2.4G where the A-MPR requirement is specified.

##### 6.6.2.2G.1	Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

###### 6.6.2.2G.1.1	Test purpose

To verify that the power of V2X UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

###### 6.6.2.2G.1.2	Test applicability

This test case applies to UE which support V2X Sidelink communication and Band 47.

###### 6.6.2.2G.1.3	Minimum conformance requirements

The additional spectrum mask in Table 6.6.2.2G.1.3-1 applies for E-UTRA V2X UE within 5855 MHz to 5950 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" or "NS_34" is indicated in the cell, the power of any V2X UE emission shall not exceed the levels specified in Table 6.6.2.2G.1.3-1.

Table 6.6.2.2G.1.3-1: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [] | 100 kHz |
| 0.5-5 | [] | 100 kHz |
| 5-10 | [] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.4.

###### 6.6.2.2G.1.4	Test description

6.6.2.2G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4G.1.4.1-1 the details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508[7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1 with the exception specified in clause 6.6.2.2G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.2.4G.1.4.1-1

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.6.2.2G.1.4.2	Test procedure

1.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.2.4G.1.4.1-1;

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclass 6.2.4G.1.5 The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.2G.1.3-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2G.1.4.3	Message contents

Refer to the subclause 6.2.4G.1.4.3

###### 6.6.2.2G.1.5	Test requirements

When "NS_33" or "NS_34" is indicated in the cell,

-	the measured UE mean power in the channel bandwidth, derived in step 2, shall fulfil the requirements described in subclass 6.2.4G.1.5;

And

-	The power of any UE emission shall fulfil requirements in Table 6.6.2.2G.1.5-1 as applicable.

The power of any V2X UE emission shall not exceed the levels specified in Table 6.6.2.2G.1.3-1.

Table 6.6.2.2G.1.5-1: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [] +2 | 100 kHz |
| 0.5-5 | []+2 | 100 kHz |
| 5-10 | []+2 | 100 kHz |

##### 6.6.2.2G.2	Additional Spectrum Emission Mask for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions

###### 6.6.2.2G.2.1	Test purpose

To verify that the power of V2X UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

###### 6.6.2.2G.2.2	Test applicability

This test case applies to E-UTRA UE release 14 and forward which support V2X communication and is capable of V2X Sidelink communication simultaneous with E-UTRA uplink transmissions.

###### 6.6.2.2G.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.2G.0.

###### 6.6.2.2G.2.4 	Test description

6.6.2.2G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC bandwidths based on Inter-band con-current V2X specified in sub-clause 5.4.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4G.2.4.1-1 the details of the uplink and V2X reference measurement channels (RMCs) are specified in sub-clause A.2 and A.8.3 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.6.2.2G.2.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to subclause 6.2.4G.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in the state 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message contents are defined in clause 6.6.2.2G.2.4.3.

6.6.2.2G.2.4.2	Test procedure

1.	SS sends RRC reconfiguration message specified in clause 6.6.2.2G.2.4.3 to enable PSCCH and PSSCH transmission at the PUMAX level.

2.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.6.2.2G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

3.	SS sends continuously “uplink power control” up commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclass 6.2.4G.2.5. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

5.	Measure the power of the transmitted signal for E-UTRA uplink with a measurement filter of bandwidths according to table 6.6.2.2G.2.5-1or 6.6.2.2G.2.5-2, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.	Measure the power of the transmitted signal for V2X sidelink with a measurement filter of bandwidths according to table 6.6.2.2G.2.5-3, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions as defined in sub-clause 6.2.4G.2.4.3

###### 6.6.2.2G.2.5	Test requirements

When "NS_33" or "NS_34" is indicated in the cell,

-	the measured UE mean power in the channel bandwidth, derived in step 4, shall fulfil the requirements described in subclause 6.2.4G.1.5 for Power Class 3 or Power Class 2 UEs;

And

-	The power of any UE emission shall fulfil requirements in Table 6.6.2.2G.2.5-1, 6.6.2.2G.2.5-2 or 6.6.2.2G.2.5-3 as applicable.

The E-UTRA uplink emission derived in step 5 shall fulfil requirements in Table 6.6.2.2G.2.5-1 or Table 6.6.2.2G.2.5-1 as applicable.

Table 6.6.2.2G.2.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2G.2.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

The power of any V2X sidelink emission derived in step 6 shall not exceed the levels specified in Table 6.6.2.2G.2.5-3.

Table 6.6.2.2G.2.5-3: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [] + 2 | 100 kHz |
| 0.5-5 | []+ 2 | 100 kHz |
| 5-10 | []+ 2 | 100 kHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<Unchanged Sections Skipped>

#### 6.6.2.3G	Adjacent Channel Leakage power Ratio for V2X Communication

##### 6.6.2.3G.1	Adjacent Channel Leakage power Ratio for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

###### 6.6.2.3G.1.1	Test purpose

To verify that V2X UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) when UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions.

###### 6.6.2.3G.1.2	Test applicability

This test case applies to E-UTRA UE release 14 and forward which support V2X Sidelink communication.

###### 6.6.2.3G.1.3	Minimum conformance requirements

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.2G-1, the requirements in subclause 6.6.2.3 apply except for the ACLR requirements for power class 2 V2X UE.

For power class 2 V2X UE, the assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3G.1.3-1. If the measured adjacent channel power is greater than –50dBm then ACLR shall be higher than the value specified in Table 6.6.2.3G.1.3-1.

Table 6.6.2.3G.1.3-1: ACLR requirements for power class 2 V2X Communication

|  | Channel bandwidth / ACLR / Measurement bandwidth |  |
| --- | --- | --- |
|  | 10 MHz | 20 MHz |
| ACLR | 31 dB | 31 dB |
| E-UTRA channel Measurement bandwidth | 9.0 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +10 / -10 | +20 / -20 |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G, and clause 6.6.2.3.

###### 6.6.2.3G.1.4	Test description

6.6.2.3G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3G.1.4.1-1 the details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.2.3G.1.4.1-1: Test Configuration for V2X sidelink transmission

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |
| Test Channel Bandwidths (as specified in subclause 5.4.2G) |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |
|  | V2X Configuratin to Transmit |  |
| Ch BW | Mod'n | PSSCH RB Allocation |
| 10MHz | QPSK | 12 (Note 2) |
| 10MHz | QPSK | 48@2 |
| 10MHz | 16QAM | 12 (Note 2) |
| 10MHz | 16QAM | 48@2 |
| 20MHz | QPSK | 18 (Note 2) |
| 20MHz | QPSK | 96@2 |
| 20MHz | 16QAM | 18 (Note 2) |
| 20MHz | 16QAM | 96@2 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in specified in subclause 5.4.2G. Note 2: The test shall be performed with allocated sub-channel(s) at low and high end of the channel bandwidth respectively. |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508[7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1 with the exception specified in clause 6.6.2.3G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.6.2.3G.1.4.1-1

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.6.2.3G.1.4.2	Test procedure

1.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.6.2.3G.1.4.1-1;

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclause 6.2.3G.1.1.5 for power class 3 UE, and shall meet the requirements described in subclause 6.2.3G.2.1.5 for power class 2 UE. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

3.	Measure the rectangular filtered mean power for E-UTRA sidelink.

4.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA sidelink channel, respectively.

6.	Calculate the ratios of the power between the values measured in step 3 over step 4 for lower and upper E-UTRAACLR, respectively.

6.6.2.3G.1.4.3	Message contents

Refer to the subclause 6.6.2.1G.1.4.3

###### 6.6.2.3G.1.5	Test requirements

-	The measured UE mean power in the channel bandwidth, derived in step 2, shall fulfil requirements in subclause 6.2.3G.1 for power class 3 UE, and shall meet the requirements described in subclass 6.2.3G.2 for power class 2 UE as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 6, shall be higher than the limits in table 6.6.2.3.5.1-1.

Table6.6.2.3G.1.5-1: E-UTRA UE ACLR for power class 3 UE

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |
| --- | --- | --- |
|  | 10MHz | 20MHz |
| E-UTRAACLR1 | 29.2dB | 29.2dB |
| E-UTRA channel Measurement bandwidth | 9.0 MHz | 18 MHz |
| UE channel | +10MHz or -10MHz | +20MHz or -20MHz |

Table6.6.2.3G.1.5-2: E-UTRA UE ACLR for power class 2 UE

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |
| --- | --- | --- |
|  | 10MHz | 20MHz |
| E-UTRAACLR1 | 30.2dB | 30.2dB |
| E-UTRA channel Measurement bandwidth | 9.0 MHz | 18 MHz |
| UE channel | +10MHz or -10MHz | +20MHz or -20MHz |

##### 6.6.2.3G.2	Adjacent Channel Leakage power Ratio for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions

###### 6.6.2.3G.2.1	Test purpose

To verify that V2X UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) when UE is configured for E-UTRA V2X sidelink and E-UTRA uplink transmissions.

###### 6.6.2.3G.2.2	Test applicability

This test case applies to E-UTRA UE release 14 and forward which support V2X communication and is capable of V2X Sidelink communication simultaneous with E-UTRA uplink transmissions.

###### 6.6.2.3G.2.3	Minimum conformance requirements

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.6.2 apply per V2X sidelink transmission and E-UTRA uplink transmission as specified for the corresponding inter-band con-current operation with uplink assigned to two bands.

The assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3G.2.3-1 and 6.6.2.3G.2.3-2. If the measured adjacent channel power is greater than –50dBm then ACLR shall be higher than the value specified in Table 6.6.2.3G.2.3-1 and 6.6.2.3G.2.3-2.

Table 6.6.2.3G.2.3-1: E-UTRA UE ACLR for power class 3 UE

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

Table 6.6.2.3G.2.3-2: E-UTRA UE ACLR for power class 2 UE

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 31 dB | 31 dB | 31 dB | 31 dB | 31 dB | 31 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G, and clause 6.6.2.3.

###### 6.6.2.3G.2.4	Test description

6.6.2.3G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.6.2.3G.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3.

Table 6.6.2.3G.2.4.1-1: Test Configuration

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Low range for uplink and Low range for V2X, High range for uplink and High range for V2X |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  |  | V2X Configuration |  |  |
| Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  | Ch BW | Mod'n | PSSCH RB allocation |
| 1.4MHz |  | QPSK | 5 |  | 10 MHz | QPSK | 48@2 |
| 1.4MHz |  | QPSK | 6 |  | 10 MHz | 16QAM | 48@2 |
| 1.4MHz |  | 16QAM | 5 |  | 20 MHz | QPSK | 96@2 |
| 1.4MHz |  | 16QAM | 6 |  | 20 MHz | 16QAM | 96@2 |
| 3MHz |  | QPSK | 4 |  |  |  |  |
| 3MHz |  | QPSK | 15 |  |  |  |  |
| 3MHz |  | 16QAM | 4 |  |  |  |  |
| 3MHz |  | 16QAM | 15 |  |  |  |  |
| 5MHz |  | QPSK | 8 |  |  |  |  |
| 5MHz |  | QPSK | 25 |  |  |  |  |
| 5MHz |  | 16QAM | 8 |  |  |  |  |
| 5MHz |  | 16QAM | 25 |  |  |  |  |
| 10MHz |  | QPSK | 12 |  |  |  |  |
| 10MHz |  | QPSK | 50 |  |  |  |  |
| 10MHz |  | 16QAM | 12 |  |  |  |  |
| 10MHz |  | 16QAM | 50 (Note 3) |  |  |  |  |
| 15MHz |  | QPSK | 16 |  |  |  |  |
| 15MHz |  | QPSK | 75 |  |  |  |  |
| 15MHz |  | 16QAM | 16 |  |  |  |  |
| 15MHz |  | 16QAM | 75 (Note 3) |  |  |  |  |
| 20MHz |  | QPSK | 18 |  |  |  |  |
| 20MHz |  | QPSK | 100 |  |  |  |  |
| 20MHz |  | 16QAM | 18 |  |  |  |  |
| 20MHz |  | 16QAM | 100 (Note 3) |  |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: For Low range the E-UTRA carrier uses only test points with RBstart=0. For High range the E-UTRA carrier uses only test points with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.6.2.3G.2.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to table 6.6.2.3G.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in the state 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C.

6.6.2.3G.2.4.2	Test procedure

1.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.6.2.3G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends MAC padding bits on the UL and V2X RMCs.

2.	SS sends the appropriate TPC commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclass 6.2.3G.1.3.5 for power class 3 UE. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA V2X sidelink and E-UTRA uplink, respectively.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA V2X sidelink and E-UTRA uplink channel, respectively.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

6.6.2.3G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, with the following exception:

Table 6.6.2.3G.2.4.3-1: SL-CommResourcePoolV2X-r14-DEFAULT

| Derivation Path: 36.508 Table 4.6.3-20C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResourcePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sizeSubchannel-r14 | n50 |  | BW10 |
|  | n100 |  | BW20 |
| numSubchannel-r14 | n1 |  |  |
| } |  |  |  |

###### 6.6.2.3G.2.5	Test requirements

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in subclause 6.2.3G.1.3.5 for power class 3 UE as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 6, shall be higher than the limits in table 6.6.2.3G.2.5-1 and 6.6.2.3G.2.5-2.

Table 6.6.2.3G.2.5-1: E-UTRA UE ACLR for power class 3 UE

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

Table 6.6.2.3G.2.5-2: E-UTRA UE ACLR for power class 2 UE

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30.2 dB | 30.2 dB | 30.2 dB | 30.2 dB | 30.2 dB | 30.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

##### 6.6.2.3G.3	Adjacent Channel Leakage power Ratio for V2X Communication / Intra-band contiguous multi-carrier operation

###### 6.6.2.3G.3.1	Test purpose

To verify that V2X UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) when UE is configured for V2X intra-band contiguous multi-carrier operation.

###### 6.6.2.3G.3.2	Test applicability

This test case applies to all types of UE that support Intra-band contiguous multi-carrier operation V2X Sidelink Communication and Band 47.

###### 6.6.2.3G.3.3	Minimum conformance requirements

For intra-band contiguous multi-carrier operation, the E-UTRA ACLR requirment for Bandwidth Class B specified in subclause 6.6.2.3.3A shall apply.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G, and clause 6.6.2.3.3A.

###### 6.6.2.3G.3.4	Test description

6.6.2.3G.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2G.1-4. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3G.3.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.2.3G.3.4.1-1: Test Configuration

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, High range |  |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  | V2X PSSCH Allocation |  |  |
| CC1 NRB | CC2 NRB | CC1 & CC2 Mod |  | CC1 and CC2 RB Allocation |
| 50 | 50 | QPSK |  | 48@2+0@0 |
| 50 | 50 | QPSK |  | 48@2+48@2 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, in Figure A.89a.

2.	The parameter settings for V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 4. 10.1. Message content exceptions are defined in clause 6.6.2.3G.3.4.3.

3.	The UL V2X Reference Measurement channels are set according to Table 6.6.2.3G.3.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.6.2.3G.3.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclass 6.2.3G.1.4.5 for power class 3 UE. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

3.	Measure the rectangular filtered mean power for E-UTRA V2X sidelink channel.

4.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA V2X sidelink channel.

5.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

6.6.2.3G.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, 4.7.I and 4.10.

###### 6.6.2.3G.3.5	Test requirements

-	The measured UE mean power in the channel bandwidth, derived in step 2, shall fulfil requirements in subclause 6.2.3G.1.4.5 for power class 3 UE as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 5, shall be higher than the limits in table 6.6.2.3G.3.5-1.

Table 6.6.2.3G.3.5-1: V2X UE ACLR for intra-band multi carrier operation

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| CA E-UTRAACLR | 29.2 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

## {End of changes}