3GPP TSG-RAN5 Meeting #92-e	R5-215478

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0254 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MU for Tx modulation quality test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MU assessment for FR2 EVM is not documented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MU for EVM has been specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Assessment of EVM MU will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 CR 0607 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related discussion paper in R5-215476. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

# B.10	Frequency error

Following tables summarize the MU threshold for EIRP measurements for Frequency error. The origin MU values for different test setups can be found in following subclauses.

Table B.10-1: MU threshold for beam peak measurement for Frequency error

| Power Class | Frequency | MBW | Power | Threshold MU value for NTC and ETC (NOTE1) |
| --- | --- | --- | --- | --- |
| PC3 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Max Output Power | +/- 0.01 ppm |
|  | 32.125GHz < f <= 40.8GHz |  |  | +/- 0.01 ppm |
| PC1 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Max Output Power | FFS |
|  | 32.125GHz < f <= 40.8GHz |  |  | FFS |
| NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm in section B.10.2 |  |  |  |  |

## B.10.1	Uncertainty budget format and assessment for DFF

+/- 0.01 ppm

-	The uncertainty assessment has been derived for the case of D = [5 cm], f = {23.45 GHz, 32.125 GHz, 40.8 GHz}, P = [Maximum output power]. This uncertainty has no dependency with extreme temperature conditions.

## B.10.2	Uncertainty budget format and assessment for IFF

+/- 0.01 ppm

-	The uncertainty assessment has been derived for the case of Quiet zone size ≤ [30 cm], f = {23.45GHz, 32.125GHz, 40.8GHz}, P = [Maximum output power].

# B.11	EVM

Following tables summarize the MU threshold for measurements EVM. The origin MU values for different test setups can be found in following subclauses.

Table B.11-1: MU threshold for EVM

| Power Class | Frequency | CHBW | Power | Threshold MU value (NOTE1) |
| --- | --- | --- | --- | --- |
| PC3 | FR2a (n257, n258, n261), | BW <= 400MHz | P = Max Output Power | see Table 11-2 |
|  | FR2b (n260) |  |  | see Table 11-3 |
| PC1 | FR2a (n257, n258, n261), | BW <= 400MHz | P = Max Output Power | FFS |
|  | FR2b (n260) |  |  | FFS |
| NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm in section B.10.2 |  |  |  |  |

Table 11-2: MU threshold for EVM (FR2a (n257, n258, n261), IFF, Quiet Zone size ≤ 30cm) for PC3 UEs

| Modulation | EVM MU (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | CHBW |  |  |  |
|  | 400 MHz | 200 MHz | 100 MHz | 50 MHz |
| Pi/2 BPSK DFT-s-OFDM | 2.2 | 1.8 | 1.7 | 1.7 |
| QPSK DFT-s-OFDM | 2.2 | 2.1 | 2.0 | 2.0 |
| 16 QAM DFT-s-OFDM | 3.2 | 2.3 | 2.0 | 2.0 |
| 64 QAM DFT-s-OFDM | 3.2 | 2.3 | 2.0 | 2.0 |
| QPSK CP-OFDM | 3.0 | 2.6 | 2.5 | 2.4 |
| 16 QAM CP-OFDM | 4.2 | 2.9 | 2.5 | 2.4 |
| 64QAM CP-OFDM | 4.2 | 2.9 | 2.5 | 2.4 |

Table 11-3: MU threshold for EVM (FR2b: n260 , IFF, Quiet Zone size ≤ 30cm) for PC3 UEs

| Modulation | EVM MU (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | CHBW |  |  |  |
|  | 400 MHz | 200 MHz | 100 MHz | 50 MHz |
| Pi/2 BPSK DFT-s-OFDM | 2.8 | 2.2 | 2.0 | 1.9 |
| QPSK DFT-s-OFDM | 3.0 | 2.3 | 2.0 | 1.9 |
| 16 QAM DFT-s-OFDM | 4.8 | 3.5 | 2.7 | 2.4 |
| 64 QAM DFT-s-OFDM | 4.8 | 3.5 | 2.7 | 2.4 |
| QPSK CP-OFDM | 4.3 | 2.9 | 2.4 | 2.2 |
| 16 QAM CP-OFDM | 6.4 | 4.3 | 3.2 | 2.7 |
| 64 QAM CP-OFDM | 6.4 | 4.3 | 3.2 | 2.7 |

## B.11.1	Uncertainty budget format and assessment for DFF

TBD

## B.11.2	Uncertainty budget format and assessment for IFF

Table 11.2-1: Uncertainty assessment for EVM (FR2a (n257, n258, n261), IFF, Quiet Zone size ≤ 30cm) for PC3 UEs

| Modulation | EVM MU (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | CHBW |  |  |  |
|  | 400 MHz | 200 MHz | 100 MHz | 50 MHz |
| Pi/2 BPSK DFT-s-OFDM |  | 2.2 |  | 1.8 |  | 1.7 |  | 1.7 |  |
| QPSK DFT-s-OFDM |  | 2.2 |  | 2.1 |  | 2.0 |  | 2.0 |  |
| 16 QAM DFT-s-OFDM |  | 3.2 |  | 2.3 |  | 2.0 |  | 2.0 |  |
| 64 QAM DFT-s-OFDM |  | 3.2 |  | 2.3 |  | 2.0 |  | 2.0 |  |
| QPSK CP-OFDM |  | 3.0 |  | 2.6 |  | 2.5 |  | 2.4 |  |
| 16 QAM CP-OFDM |  | 4.2 |  | 2.9 |  | 2.5 |  | 2.4 |  |
| 64QAM CP-OFDM |  | 4.2 |  | 2.9 |  | 2.5 |  | 2.4 |  |

Table 11.2-2: Uncertainty assessment for EVM (FR2b: n260 , IFF, Quiet Zone size ≤ 30cm) for PC3 UEs

| Modulation | EVM MU (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | CHBW |  |  |  |
|  | 400 MHz | 200 MHz | 100 MHz | 50 MHz |
| Pi/2 BPSK DFT-s-OFDM |  | 2.8 |  | 2.2 |  | 2.0 |  | 1.9 |  |
| QPSK DFT-s-OFDM |  | 3.0 |  | 2.3 |  | 2.0 |  | 1.9 |  |
| 16 QAM DFT-s-OFDM |  | 4.8 |  | 3.5 |  | 2.7 |  | 2.4 |  |
| 64 QAM DFT-s-OFDM |  | 4.8 |  | 3.5 |  | 2.7 |  | 2.4 |  |
| QPSK CP-OFDM |  | 4.3 |  | 2.9 |  | 2.4 |  | 2.2 |  |
| 16 QAM CP-OFDM |  | 6.4 |  | 4.3 |  | 3.2 |  | 2.7 |  |
| 64 QAM CP-OFDM |  | 6.4 |  | 4.3 |  | 3.2 |  | 2.7 |  |

-	The uncertainty assessment has been derived for the case of Quiet zone size ≤ 30 cm, bands n257, n258, n261, n260, P = [Maximum output power].

# B.12 to B.14

< End of changes >