3GPP TSG-RAN5 Meeting #92-e                                                      R5-215531

Electronic Meeting, 17th May – 28th May

Title: 		        Out-of-band emissions for FR2 UL MIMO initial conditions

Source:	        Keysight Technologies UK Ltd, Sporton

Agenda Item:	5.3.2.17

Document for:	Discussion and Endorsement

# 1.	Introduction

Current test specifications TS 38.521-2 ([1]) indicate the following missing aspects for 6.5D.2.1 Spectrum Emission Mask for UL MIMO test case and 6.5D.2.2 Adjacent Channel Leakage Ratio for UL MIMO test case:

This document proposes a way forward to progress on these test cases.

# 2.	Test Point analysis discussion

On one hand, for FR2 Out-of-band emissions test cases for UL MIMO (SEM and ACLR), there are test points using 256QAM modulation in the UL, while in FR2 such modulation has only been defined for DL. Probably an incorrect leveraging form FR1 was considered.

Analyzing core requirements for Out-of-band emissions for UL-MIMO (section 6.5D.2 in [2]), it can be seen that they refer to SISO core requirements while having the UL MIMO configuration in place:

As MPR core requirements are also applicable to some Out-of-band emission test cases, MPR core requirements are also analyzed. It can be seen that they also refer to corresponding SISO core requirements while having the UL MIMO configuration in place:

On the other hand, 38.905 ([3]) already includes a combined test points analysis for MPR, SEM and ACLR test cases.

Taking into account current core requirements definition for out-of-band emissions for FR2 UL-MIMO, it makes sense that test points for SA UL MIMO test cases are exactly the same as for the test cases they refer to (FR2 SISO test cases).

Proposal 1: Leverage FR2 UL SISO test points analysis for MPR, SEM and ACLR for FR2 UL MIMO MPR, SEM and ACLR test cases.

However, as described in 38.905 [3] and according to proposal 1 in [4] the waveform under test must be CP-OFDM.

Proposal 2:  Replace DFT-s- to CP-OFDM waveform and remove duplicated test points once leveraging in proposal 1 is applied.

CRs detailing changes required can be found in [5, 6].

# 3.	Conclusion

This document proposes a way forward to progress on the completion of FR2 Out-of-band emissions test cases for UL-MIMO. The following proposal is made:

Proposal 1: Leverage FR2 UL SISO test points analysis for MPR, SEM and ACLR for FR2 UL MIMO MPR, SEM and ACLR test cases.

Proposal 2:  Replace DFT-s- to CP-OFDM waveform and remove duplicated test points once leveraging in proposal 1 is applied.

# 4.	References

[1] 38.521-2 v16.8.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

[2] 38.101-2 v17.2.0 “User Equipment (UE) Radio transmission and reception; Part 2: Range 2 Standalone

[3] 38.905 v 17.1.0 “Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases”

[4] R5-192024, “Discussion on the waveform selection for UL MIMO tests “, Huawei, HiSilicon, RAN5#4-5G-NR-Adhoc

[5] R5-215532, “FR2 SA UL MIMO Out-of-band emissions initial conditions updates”, Keysight Technologies, RAN5#92-e

[6] R5-215535, “FR2 SA UL MIMO Maximum Power Reduction update”, Keysight Technologies, Sporton RAN5#92-e