3GPP TSG-RAN5 Meeting #92-e	R5-215544

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1373 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to the coherent UL-MIMO test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case of coherent UL-MIMO is not completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement of relative power is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4D.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.4D.4	Requirements for coherent UL MIMO

Editor’s note: This clause is incomplete for relative phase measurement. The following aspects are either missing or not yet determined:

- The test procedure is FFS.

- MU and TT value are still FFS

6.4D.4.1	Test purpose

To verify that the difference of relative phase error and the difference of relative power error between antenna ports in coherent UL MIMO do not exceed the range prescribed by the specified  requirements for coherent UL MIMO and tolerance.

An excess relative phase error or excess relative power error has the possibility to interfere to other channels and decrease UL MIMO performance because of the timing unsynchronization.

6.4D.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support coherent UL MIMO.

6.4D.4.3	Minimum conformance requirements

For coherent UL MIMO, Table 6.4D.4.3-1 lists the maximum allowable difference between the measured relative power and phase errors between different antenna ports in any slot within the specified time window from the last transmitted SRS on the same antenna ports, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS. The requirements in Table 6.4D.4.3-1 apply when the UL transmission power at each antenna port is larger than 0 dBm for SRS transmission and for the duration of time window.

Table 6.4D.4.3-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40 degrees | 4 dB | 20 msec |

The above requirements when all the following conditions are met within the specified time window:

- UE is not signaled with a change in number of SRS ports in SRS-config, or a change in PUSCH-config

- UE remains in DRX active time (UE does not enter DRX OFF time)

- No measurement gap occurs

- No instance of SRS transmission with the usage antenna switching occurs

- Active BWP remains the same

- EN-DC and CA configuration is not changed for the UE (UE is not configured or de-configured with PScell or SCell(s))

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4D.4

6.4D.4.4	Test description

6.4D.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4D.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4D.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range (NOTE 2) |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
| Test ID | SCS | BW | Downlink Configuration | Uplink configuration |  | SRS bandwidth configuration (c-SRS) |  |
|  |  |  |  | Modulation | RB allocation (NOTE 1) | Paired Spectrum | Unpaired Spectrum |
| 1 | 15kHz | 5MHz | N/A | CP-QPSK | 24@0 | 7 (24 RB) | 7 |
|  |  | 10MHz |  | CP-QPSK | 52@0 | 14 (52 RB) | 13 |
|  |  | 15MHz |  | CP-QPSK | 76@0 | 20 (76 RB) | 19 |
|  |  | 20MHz |  | CP-QPSK | 104@0 | 25 (104 RB) | 25 |
|  |  | 25MHz |  | CP-QPSK | 132@0 | 33 (132 RB) | 30 |
|  |  | 30MHz |  | CP-QPSK | 160@0 | 42 (160 RB) | 40 |
|  |  | 40MHz |  | CP-QPSK | 216@0 | 51 (216 RB) | 51 |
|  |  | 50MHz |  | CP-QPSK | 264@0 | 60 (264 RB) | 60 |
|  | 30kHz | 5MHz |  | CP-QPSK | 8@0 | 1 (8 RB) | 1 |
|  |  | 10MHz |  | CP-QPSK | 24@0 | 7 (24 RB) | 6 |
|  |  | 15MHz |  | CP-QPSK | 36@0 | 10 (36 RB) | 10 |
|  |  | 20MHz |  | CP-QPSK | 48@0 | 13 (48 RB) | 12 |
|  |  | 25MHz |  | CP-QPSK | 64@0 | 17 (64 RB) | 17 |
|  |  | 30MHz |  | CP-QPSK | 76@0 | 20 (76 RB) | 20 |
|  |  | 40MHz |  | CP-QPSK | 104@0 | 25 (104 RB) | 25 |
|  |  | 50MHz |  | CP-QPSK | 132@0 | 33 (132 RB) | 30 |
|  |  | 60MHz |  | CP-QPSK | 160@0 | 42 (160 RB) | 40 |
|  |  | 80MHz |  | CP-QPSK | 216@0 | 51 (216 RB) | 51 |
|  |  | 90MHz |  | CP-QPSK | 240@0 | 56 (240 RB) | 53 |
|  |  | 100MHz |  | CP-QPSK | 272@0 | 63 (272 RB) | 61 |
|  | 60kHz | 10MHz |  | CP-QPSK | 8@0 | 1 (8 RB) | 1 |
|  |  | 15MHz |  | CP-QPSK | 16@0 | 4 (16 RB) | 4 |
|  |  | 20MHz |  | CP-QPSK | 24@0 | 7 (24 RB) | 7 |
|  |  | 25MHz |  | CP-QPSK | 28@0 | 8 (28 RB) | 7 |
|  |  | 30MHz |  | CP-QPSK | 36@0 | 10 (36 RB) | 9 |
|  |  | 40MHz |  | CP-QPSK | 48@0 | 13 (48 RB) | 13 |
|  |  | 50MHz |  | CP-QPSK | 64@0 | 17 (64 RB) | 17 |
|  |  | 60MHz |  | CP-QPSK | 76@0 | 20 (76 RB) | 19 |
|  |  | 80MHz |  | CP-QPSK | 104@0 | 25 (104 RB) | 25 |
|  |  | 90MHz |  | CP-QPSK | 120@0 | 29 (120 RB) | 27 |
|  |  | 100MHz |  | CP-QPSK | 132@0 | 33 (132 RB) | 30 |
| NOTE 1: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |  |  |

Table 6.4D.4.4.1-2: Other SRS parameters

| Parameters | Value |
| --- | --- |
| b-SRS | 0 |
| b-hop | 3 |
| freqDomainPosition | 0 |
| SRS-PeriodicityAndOffset | sl5 for SCS 15 KHz |
|  | sl10 for SCS 30 KHz |
|  | sl20 for SCS 60 KHz |
| transmissionComb | n2 |
| CombOffset | 0 |
| cyclicShift | 0 |
| startPosition | 0 for SCS 15 KHz |
|  | 1 for SCS 30 KHz |
|  | 3 for SCS 60 KHz |
| nrofSymbols | n1 for SCS 15 KHz |
|  | n2 for SCS 30 KHz |
|  | n4 for SCS 60 KHz |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.4D.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.4D.4.4.3

6.4D.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4D.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE reaches the Pumax level of the test point.

3.	Measure the mean power of the UE on each antenna connector on SRS symbol in the SRS channel bandwidth according to the test configuration from table 6.4D.4.4.1-1. Calculate the power difference between antennas and save this value as ‘Power_ref’.

4.	On the slots within 20ms following the SRS symbol, measure the mean power of the UE on each antenna connector in the channel bandwidth according to the test configuration from table 6.2.4.4.1-1. The period of measurement shall be at least the continuous duration of one active slot and in the uplink symbols. For TDD slots with transient periods are not under test. Calculate the power difference between antennas and save this value as ‘Power_meas’.

6.4D.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception:

Table 6.4D.4.4.3-1: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1[5], Table 4.6.3-7C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| srs-Config | SRS-Config in Table 6.3.3.6.4.3-1 |  |  |
| } |  |  |  |

Table 6.4D.4.4.3-2: SRS-Config: SRS time mask measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SEQUENCE { | 1 entry |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| alpha | alpha08 |  |  |
| p0 | -100 |  |  |
| } |  |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SEQUENCE { | 1 entry |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  | SCS 15k |
|  | 1 |  | SCS 30k |
|  | 3 |  | SCS 60k |
| nrofSymbols | n1 |  | SCS 15k |
|  | n2 |  | SCS 30k |
|  | n4 |  | SCS 60k |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS |  |  |  |
|  | 7 (for BW 5 MHz) 14 (for BW 10 MHz) 20 (for BW 15 MHz) 25 (for BW 20 MHz) 33 (for BW 25 MHz) 42 (for BW 30 MHz) 51 (for BW 40MHz) 60 (for BW 50MHz) |  | Paired Spectrum for SCS 15 KHz |
|  | 1 (for BW 5 MHz) 7 (for BW 10 MHz) 10 (for BW 15 MHz) 13 (for BW 20 MHz) 17 (for BW 25 MHz) 20 (for BW 30 MHz) 25 (for BW 40MHz) 33 (for BW 50MHz) 42 (for BW 60MHz) 51 (for BW 80MHz) 56 (for BW 90MHz) |  | Paired Spectrum for SCS 30 KHz |
|  | 1 (for BW 10 MHz) 4 (for BW 15 MHz) 7 (for BW 20 MHz) 8 (for BW 25 MHz) 10 (for BW 30 MHz) 13 (for BW 40MHz) 17 (for BW 50MHz) 20 (for BW 60MHz) 25 (for BW 80MHz) 29 (for BW 90MHz) 33 (for BW 100MHz) |  | Paired Spectrum for SCS 60 KHz |
|  | 7 (for BW 5 MHz) 13 (for BW 10 MHz) 19 (for BW 15 MHz) 25 (for BW 20 MHz) 30 (for BW 25 MHz) 40 (for BW 30 MHz) 51 (for BW 40MHz) 60 (for BW 50MHz) |  | Unpaired Spectrum for SCS 15 KHz |
|  | 1 (for BW 5 MHz) 6 (for BW 10 MHz) 10 (for BW 15 MHz) 12 (for BW 20 MHz) 17 (for BW 25 MHz) 20 (for BW 30 MHz) 25 (for BW 40MHz) 30 (for BW 50MHz) 40 (for BW 60MHz) 51 (for BW 80MHz) 53 (for BW 90MHz) 61 (for BW 100MHz) |  | Unpaired Spectrum for SCS 30 KHz |
|  | 1 (for BW 10 MHz) 4 (for BW 15 MHz) 7 (for BW 20, 25 MHz) 9 (for BW 30 MHz) 13 (for BW 40MHz) 17 (for BW 50MHz) 19 (for BW 60MHz) 25 (for BW 80MHz) 27 (for BW 90MHz) 30 (for BW 100MHz) |  | Unpaired Spectrum for SCS 60 KHz |
| b-SRS | 0 |  |  |
| b-hop | 3 |  | Paired Spectrum |
|  | 0 |  | Unpaired Spectrum |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| periodicityAndOffset-p CHOICE{ |  |  |  |
| sl5 | 4 |  | SCS 15 KHz |
| sl10 | 8 |  | SCS 30KHz |
| sl20 | 16 |  | SCS 60 KHz |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4D.4.4.3-3: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  |  |

6.4D.4.5	Test requirement

Maximum allowable difference of ‘power_ref’ measured in step 3 and ‘power_meas’ measured in step 4 shall not exceed the described relative power error in Table 6.4D.4.5-1.

Table 6.4D.4.5-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40+TT degrees | 4+TT dB | 20 msec |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.4D.4.5-2. |  |  |

Table 6.4D.4.5-2: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.3 dB |
| 40MHz < BW ≤ 100MHz | 1.3 dB | 1.3 dB |

<End of Changes>