3GPP TSG-RAN WG5 #92-e		R5-215565
Electronic Meeting, 16th – 27th August 2021

Agenda item:	5.3.2.17

Source:	Qualcomm Austria RFFE GmbH

Title:	Discussion of test applicability for different NS values

Document for:	Endorsement

# 1.	Introduction

UE additional maximum output power reduction (A_MPR) and additional spurious emissions for different NS values were introduced in different releases of TS 38.101-1 [1, 2]

The test applicability for different NS values for UE additional maximum output power reduction (A_MPR) and additional spurious emissions in TS 38.521-1 are not clearly noted and misaligned with core requirement.  In both cases, it is not clear which NS value applies to UE release 15 and forward, or UE release 16 and forward. This leads to different interpretations by different UE vendors.

In this discussion paper, the current discrepancies between core requirement and latest TS 38.521 are discussed and a way forward is proposed.

# 2. 	Discussion

## 2.1 UE additional maximum output power reduction (A_MPR) core requirement

From TS 38.101-1 V15.4.0 [1] and TS 38.101-1 V16.8.0 [2] clause 6.2.3, minimum requirement for different NS values is defined.  Some NS values are only applicable to Release 15 and forward.  NS values required by both Release 15 and Release 16 are listed in Table 2.1-1.  NS values only required by Release 16 are listed in Table 2.1-2.

In Summary, it is clear indicating that all the NA values listed in Table 2.1-2 are not required for a Release 15 complied UE.

Table 2.1-1 NS-values required by both Release 15 and 16

| NS_Value | 4 | 10 | 05, 05U | 40 | 43, 43U | 03, 03U | 37 | 38 | 39 | 41 | 42 | 18 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Rel 16 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

Table 2.1-2 NS-values required by Release 16 only

| NS_Value | 21 | 24 | 27 | 46 | 47 | 44 | 12 | 13 | 14 | 15 | 45 | 48 | 49 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel 15 | No | No | No | No | No | No | No | No | No | No | No | No | No | No |
| Rel 16 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

## 2.2 UE additional maximum output power reduction in TS 38.521-1

In current TS 38.521-1 [3] clause 6.2.3.2 Test applicability, the following statement applies

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, and NS_35 to all types of NR UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply in test case 6.5.2.4 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49 and NS_50 to all types of NR Power Class 2 and 3 UE release 15 and forward.

NS_06 applies to n12 and n14, but n14 is a Release 16 band, NS_35 applies to n71 which is also a Release 16 band.

NS_100 applies to n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89. But n18, n25, n65 and n89 are Release 16 bands.

Further, NS_12, NS_13, NS_14, NS_15, NS_21, NS_44, NS_45, NS_48, and NS_49 are only required by Release 16

Clearly, the test applicability is not fully aligned with core requirement, and this leads different implementations by different UE vendors.

In order to avoid this ambiguity, the following changes are proposed to change the test applicability of A-MPR and related subclauses

Proposal 1: Update the test applicability for A-MPR test

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, and NS_06 (n12) , and NS_35 to all types of NR UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, and NS_35 to all types of NR UE release 16 and forward and NR Power Class 1 UE release 16 and forward in NR Band n14.

The requirements of this test apply in test case 6.5.2.4 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 (applies to n1, n2, n3, n5, n8, n25, n66, n80, n81, n84, n86)   to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 2 and 3 UE release 16 and forward.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49 and NS_50 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49 and NS_50 to all types of NR Power Class 2 and 3 UE release 15 and forward

## 2.3 UE Additional spurious emissions in TS 38.521-1

Similarly, the Test applicability for Additional spurious emissions in current TS 38.521-1 [3] clause 6.5.3 needs to be revised in the same manner.

Proposal 2: Update the test applicability for Additional spurious emissions test

This test case for NS_04, NS_18, NS_05, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, and NS_50 applies to all types of NR UE release 15 and forward.

This test case for all NS values applies to all types of NR UE release 16 and forward.

# 3. 	Conclusion

Based on the discussion, the following modification of test applicability is proposed.

Proposal 1:  same as described in section 2.2

Proposal 2:  same as described in section 2.3

# 4. 	References

- 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1, Standalone", Revision  V15.14.0 (2021-06)

- 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1, Standalone", Revision  V16.8.0 (2021-06)

- 3GPP TS 38.521-1:" User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone.