3GPP TSG-RAN5 Meeting #92-e	R5-215579

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2453 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resubmission of New MAC test case on 2-Step RACH Explicitly signalled |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETSI MCC (Lenovo, Motorola Mobility) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New MAC 2-Step RACH test case ‘Correct selection of RACH parameters / 2-step RACH/MSGA and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure’ needs to be added as per work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new 2-Step RACH Test case 7.1.1.1.8 Correct selection of RACH parameters / 2-step RACH/MSGA and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature will not be adequately tested and hence IoP cannot be guaranteed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.8(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a resubmission of R5-193186 CR 2263 rev - approved at RAN5#91 but not implemented by mistake. |  |  |  |  |  |  |  |  |

Table 7.1.1.1.6.3.3-3: RACH-ConfigCommon (Table 7.1.1.1.6.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 42 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| One | n32 |  |  |
| } |  |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| ra-Msg3SizeGroupA | b208 |  |  |
| messagePowerOffsetGroupB | minusinfinity |  |  |
| numberOfRA-PreamblesGroupA | 28 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| } |  |  |  |

<<< Start of Change>>>

##### 7.1.1.1.8	Correct selection of RACH parameters / 2-step RACH/MSGA and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure

7.1.1.1.8.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected }

ensure that {

when { SS sends an RRCReconfiguration message including RACH-ConfigDedicated information element and the contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated }

then { UE SET RA_TYPE to 2-step AND sends a MSGA on the NR PSCell }

}

(2)

with { UE in RRC_Connected state after transmission of a MSGA on NR SpCell received in RACH-ConfigDedicated on the target cell }

ensure that {

when { UE does not receive a matching MSGB in msgB-ResponseWindow and PREAMBLE_TRANSMISSION_COUNTER is less than msgA-TransMax + 1 }

then { UE retransmits a MSGA in RACH-ConfigDedicated on the target cell }

}

7.1.1.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1, 5.1.2a, 5.1.3a and 5.1.4a . Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, clause 5.1.1]

1>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17) and if the contention-free Random Access Resources for beam failure recovery request for 4-step RA type have been explicitly provided by RRC for the BWP selected for Random Access procedure; or

1>	if the Random Access procedure was initiated for reconfiguration with sync and if the contention-free Random Access Resources for 4-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure:

2>	set the RA_TYPE to 4-stepRA.

1>	else if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources and the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold; or

1>	if the BWP selected for Random Access procedure is only configured with 2-step RA type Random Access resources (i.e. no 4-step RACH RA type resources configured); or

1>	if the Random Access procedure was initiated for reconfiguration with sync and if the contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure:

2>	set the RA_TYPE to 2-stepRA.

1>	else:

2>	set the RA_TYPE to 4-stepRA.

1>	perform initialization of variables specific to Random Access type as specified in clause 5.1.1a;

1>	if RA_TYPE is set to 2-stepRA:

2>	perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a).

1>	else:

2>	perform the Random Access Resource selection procedure (see clause 5.1.2).

[TS 38.321, clause 5.1.2a]

If the selected RA_TYPE is set to 2-stepRA, the MAC entity shall:

1>	if the contention-free 2-step RA type Resources associated with SSBs have been explicitly provided in rach-ConfigDedicated and at least one SSB with SS-RSRP above msgA-RSRP-ThresholdSSB amongst the associated SSBs is available:

2>	select an SSB with SS-RSRP above msgA-RSRP-ThresholdSSB amongst the associated SSBs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB.

1>	else (i.e. for the contention-based Random Access Preamble selection):

2>	if at least one of the SSBs with SS-RSRP above msgA-RSRP-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above msgA-RSRP-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if contention-free Random Access Resources for 2-step RA type have not been configured and if Random Access Preambles group has not yet been selected during the current Random Access procedure:

3>	if Random Access Preambles group B for 2-step RA type is configured:

4>	if the potential MSGA payload size (UL data available for transmission plus MAC subheader and, where required, MAC CEs) is greater than the ra-MsgA-SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – msgA-PreambleReceivedTargetPower – msgA-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-MsgA-SizeGroupA:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

3>	else:

4>	select the Random Access Preambles group A.

2>	else if contention-free Random Access Resources for 2-step RA type have been configured and if Random Access Preambles group has not yet been selected during the current Random Access procedure:

3>	if Random Access Preambles group B for 2-step RA type is configured; and

3>	if the transport block size of the MSGA payload configured in the rach-ConfigDedicated corresponds to the transport block size of the MSGA payload associated with Random Access Preambles group B:

4>	select the Random Access Preambles group B.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Random Access preambles group has been selected during the current Random Access procedure):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the earlier transmission of MSGA.

2>	select a Random Access Preamble randomly with equal probability from the 2-step RA type Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group;

2>	set the PREAMBLE_INDEX to the selected Random Access Preamble.

1>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the msgA-SSB-SharedRO-MaskIndex if configured and ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability among the consecutive PRACH occasions allocated for 2-step RA type according to clause 8.1 of TS 38.213 [6], corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB);

1>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

2>	select a PUSCH occasion from the PUSCH occasions configured in msgA-CFRA-PUSCH corresponding to the PRACH slot of the selected PRACH occasion, according to msgA-PUSCH-resource-Index corresponding to the selected SSB;

2>	determine the UL grant and the associated HARQ information for the MSGA payload in the selected PUSCH occasion;

2>	deliver the UL grant and the associated HARQ information to the HARQ entity.

1>	else:

2>	select a PUSCH occasion corresponding to the selected preamble and PRACH occasion according to clause 8.1A of TS 38.213 [6];

2>	determine the UL grant for the MSGA payload according to the PUSCH configuration associated with the selected Random Access Preambles group and determine the associated HARQ information;

2>	if the selected preamble and PRACH occasion is mapped to a valid PUSCH occasion as specified in clause 8.1A of TS 38.213 [6]:

3>	deliver the UL grant and the associated HARQ information to the HARQ entity.

1>	perform the MSGA transmission procedure (see clause 5.1.3a).

NOTE:	To determine if there is an SSB with SS-RSRP above msgA-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

[TS 38.321, clause 5.1.3a]

The MAC entity shall, for each MSGA:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last MSGA Random Access Preamble transmission; and

1>	if SSB selected is not changed from the selection in the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to msgA-PreambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP;

1>	if this is the first MSGA transmission within this Random Access procedure:

2>	if the transmission is not being made for the CCCH logical channel:

3>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

2>	if the Random Access procedure was initiated for SpCell beam failure recovery and spCell-BFR-CBRA with value true is configured:

3>	indicate to the Multiplexing and assembly entity to include a BFR MAC CE or a Truncated BFR MAC CE in the subsequent uplink transmission.

2>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity according to the HARQ information determined for the MSGA payload (see clause 5.1.2a) and store it in the MSGA buffer.

1>	compute the MSGB-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted;

1>	instruct the physical layer to transmit the MSGA using the selected PRACH occasion and the associated PUSCH resource of MSGA (if the selected preamble and PRACH occasion is mapped to a valid PUSCH occasion), using the corresponding RA-RNTI, MSGB-RNTI, PREAMBLE_INDEX, PREAMBLE_RECEIVED_TARGET_POWER, msgA-PreambleReceivedTargetPower, and the amount of power ramping applied to the latest MSGA preamble transmission (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP);

[TS 38.321, clause 5.1.4a]

Once the MSGA preamble is transmitted, regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	start the msgB-ResponseWindow at the PDCCH occasion as specified in TS 38.213 [6], clause 8.2A;

1>	monitor the PDCCH of the SpCell for a Random Access Response identified by MSGB-RNTI while the msgB-ResponseWindow is running;

1>	if C-RNTI MAC CE was included in the MSGA:

2>	monitor the PDCCH of the SpCell for Random Access Response identified by the C-RNTI while the msgB-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers:

2>	if the C-RNTI MAC CE was included in MSGA:

3>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17) and the PDCCH transmission is addressed to the C-RNTI:

4>	consider this Random Access Response reception successful;

4>	stop the msgB-ResponseWindow;

4>	consider this Random Access procedure successfully completed.

3>	else if the timeAlignmentTimer associated with the PTAG is running:

4>	if the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission:

5>	consider this Random Access Response reception successful;

5>	stop the msgB-ResponseWindow;

5>	consider this Random Access procedure successfully completed.

3>	else:

4>	if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded:

5>	if the MAC PDU contains the Absolute Timing Advance Command MAC CE:

6>	process the received Timing Advance Command (see clause 5.2);

6>	consider this Random Access Response reception successful;

6>	stop the msgB-ResponseWindow;

6>	consider this Random Access procedure successfully completed and finish the disassembly and demultiplexing of the MAC PDU.

2>	if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the MSGB-RNTI and the received TB is successfully decoded:

3>	if the MSGB contains a MAC subPDU with Backoff Indicator:

4>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

3>	else:

4>	set the PREAMBLE_BACKOFF to 0 ms.

3>	if the MSGB contains a fallbackRAR MAC subPDU; and

3>	if the Random Access Preamble identifier in the MAC subPDU matches the transmitted PREAMBLE_INDEX (see clause 5.1.3a):

4>	consider this Random Access Response reception successful;

4>	apply the following actions for the SpCell:

5>	process the received Timing Advance Command (see clause 5.2);

5>	indicate the msgA-PreambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP);

5>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

6>	consider the Random Access procedure successfully completed;

6>	process the received UL grant value and indicate it to the lower layers.

5>	else:

6>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

6>	if the Msg3 buffer is empty:

7>	obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer;

6>	process the received UL grant value and indicate it to the lower layers and proceed with Msg3 transmission.

NOTE:	If within a 2-step RA type procedure, an uplink grant provided in the fallback RAR has a different size than the MSGA payload, the UE behavior is not defined.

3>	else if the MSGB contains a successRAR MAC subPDU; and

3>	if the CCCH SDU was included in the MSGA and the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU:

4>	stop msgB-ResponseWindow;

4>	if this Random Access procedure was initiated for SI request:

5>	indicate the reception of an acknowledgement for SI request to upper layers.

4>	else:

5>	set the C-RNTI to the value received in the successRAR;

5>	apply the following actions for the SpCell:

6>	process the received Timing Advance Command (see clause 5.2);

6>	indicate the msgA-PreambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP).

4>	deliver the TPC, PUCCH resource Indicator, ChannelAccess-CPext (if indicated), and HARQ feedback Timing Indicator received in successRAR to lower layers.

4>	consider this Random Access Response reception successful;

4>	consider this Random Access procedure successfully completed;

4>	finish the disassembly and demultiplexing of the MAC PDU.

1>	if msgB-ResponseWindow expires, and the Random Access Response Reception has not been considered as successful based on descriptions above:

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	indicate a Random Access problem to upper layers;

3>	if this Random Access procedure was triggered for SI request:

4>	consider this Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1:

4>	set the RA_TYPE to 4-stepRA;

4>	perform initialization of variables specific to Random Access type as specified in clause 5.1.1a;

4>	if the Msg3 buffer is empty:

5>	obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer;

4>	flush HARQ buffer used for the transmission of MAC PDU in the MSGA buffer;

4>	discard explicitly signalled contention-free 2-step RA type Random Access Resources, if any;

4>	perform the Random Access Resource selection procedure as specified in clause 5.1.2.

3>	else:

4>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

4>	if the criteria (as defined in clause 5.1.2a) to select contention-free Random Access Resources is met during the backoff time:

5>	perform the Random Access Resource selection procedure for 2-step RA type Random Access (see clause 5.1.2a).

4>	else:

5>	perform the Random Access Resource selection procedure for 2-step RA type Random Access (see clause 5.1.2a) after the backoff time.

Upon receiving a fallbackRAR, the MAC entity may stop msgB-ResponseWindow once the Random Access Response reception is considered as successful.

7.1.1.1.8.3	Test description

7.1.1.1.8.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that Test loop function(Off).

7.1.1.1.8.3.2	Test procedure sequence

Table 7.1.1.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message with reconfiguration with sync and contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated. Note 1 | <-- | RRCReconfiguration | - | - |
| - | Exception: Steps 2 to 3 are run msgA-TransMax times | - | - | - | - |
| 2 | The UE transmits MSGA including RRCReconfigurationComplete message using the Random Access Resources for 2-step RA type explicitly provided. Note 2 | --> | MAC PDU (including C-RNTI MAC CE, RRCReconfigurationComplete) | 1,2 | P |
| 3 | SS does not schedulePDCCH transmission for UE C_RNTI until msgB-ResponseWindow expires | - | - | - | - |
| 4 | The UE transmits MSGA including RRCReconfigurationComplete message using the Random Access Resources for 2-step RA type explicitly provided. Note 2 | --> | MAC PDU (including C-RNTI MAC CE, RRCReconfigurationComplete) | 2 | P |
| 6 | SS schedules PDCCH transmission for UE C_RNTI and DL MAC PDU containing Absolute Timing Advance Command MAC CE. | <-- | MAC PDU(Absolute Timing Advance Command MAC Control Element) | - | - |
| Note 1: for EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: for EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete and sent in E-UTRA cell. In NR Cell a MAC PDU containing C-RNTI MAC CE is sent. |  |  |  |  |  |

7.1.1.1.8.3.3	Specific message contents

FFS