3GPP TSG-RAN WG5 Meeting #92-e						         R5-215613

Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

Source: MediaTek

Title:	Discussion paper on RRC DL segmentation test method

Agenda item: 6.2.3.20

Document for: Discussion and Decision

- Introduction

At RAN5#90-e it was agreed that that test cases are required to test RRC DL segmentation [1].

RRC DL segmentation in NR or LTE happens when the RRC message exceeds the respective maximum PDU size.

- Discussion

In order to trigger a situation when the encoded DL RRC message has to be segmented the encoded DL RRC message needs to be large.

At the moment for DL message segmentation is only applicable to RRCReconfiguration and RRCResume messages for NR and to RRCConnectionReconfiguration and RRCConnectionResume messages for LTE.

There are the below options available to achieve a large message size:

Option 1: A very complex configuration needs to be added to DL RRC messages.

- Pro:

  - this method follows how the feature will be used in the field

- Con:

  - the complex configuration might not be supported by all the UEs

  - the complex configuration might be challenging for the SS

Option 2: An ordinary size (shorter than the max size the fits in a PDU) RRC message is segmented:

- Pro:

  - according to TS 36.331/TS 38.331 the UE behaviour is not dependent on the size of the received message segment

  - easy to implement and maintain the test cases

  - offers the flexibility to choose the number of segments without having to make up a new longer RRC message

- Con:

  - this is not be how the feature will be used in the field

  - SS behaviour is not aligned with the TS 36.331/TS 38.331 as that requires that messages get segmented when the size exceeds the maximum PDU size.

Option 2 does not exclude Option1 as it may happen that during testing other features a very large RRC message needs to be sent.

- Proposal

- Use option 2 for testing DL RRC segmentation:

An ordinary size (shorter than the max size the fits in a PDU) RRC message is segmented.

4.	References

[1]	R5-210581	“Discussion to add test cases for RRC DL segmentation”