3GPP TSG-RAN5 Meeting #92-e	R5-215696

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2462 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of RRC messages for MAC TC 7.1.1.5.1 and 7.1.1.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NR RRC messages in test procedure shall align with the ones in message sequence, therefore the specific message contents shall also be updated according to the definition in TS 38.508-1 and TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRC messages in the test procedure and specific message content were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC messages will be inconsistent with other L2 cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.1.3, 7.1.1.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

7.1.1.5.1	DRX operation / Short cycle not configured / Parameters configured by RRC

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { Long DRX cycle is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset }

then { UE starts the OnDurationTimer and monitors the PDCCH for OnDurationTimer PDCCH-Occasions}

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { Long DRX cycle is configured and a new DL transmission is indicated on the PDCCH during Active Time }

then { UE starts or restarts the Drx-InactivityTimer and monitors the PDCCH for Drx-InactivityTimer PDCCH occasions starting from the next PDCCH occasion of the PDCCH occasion where the DL new transmission was indicated }

}

(3)

with { UE in  RRC_CONNECTED state }

ensure that {

when { Long DRX cycle is configured and if a HARQ RTT Timer expires in this PDCCH Occasion and the data in the soft buffer of the corresponding HARQ process was not successfully decoded }

then { UE starts the drx-RetransmissionTimer-DL for the corresponding HARQ process and monitors the PDCCH for drx-RetransmissionTimer consecutive PDCCH Occasion }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { Long DRX cycle is configured and an uplink grant for a pending HARQ retransmission can occur in this PDCCH occasion}

then { UE monitors the PDCCH in this PDCCH occasion }

}

7.1.1.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE’s PDCCH monitoring. . Activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other subclauses of this specification. When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH continuously.

RRC controls DRX operation by configuring the following timers:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates an new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycle StartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the random access preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in subclause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if an drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN x 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN x 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if drx-SlotOffset is configured:

3>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH;

2>	if the PDCCH indicates a DL transmission or if a DL assignment has been configured:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process immediately after the corresponding PUCCH transmission;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission or if a UL grant has been configured:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process immediately after the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer.

1>	else (i.e. not part of the Active Time):

2>	not report CQI/PMI/RI on PUCCH.

7.1.1.5.1.3	Test description

7.1.1.5.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.1.3.2	Test procedure sequence

Table 7.1.1.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DRX parameters. (Note 6) | <-- | - | - | - |
| 2 | The UE transmits RRCReconfigurationComplete. (Note 7) | --> | - | - | - |
| 3 | In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 5 | At least drx-InactivityTimer PDCCH occasions after the transmission of the MAC PDU in Step 3 has been indicated (This means the next DRX cycle or later after Step 2) in the last PDCCH occasion while the drx-onDurationTimer is still running, the SS indicates the transmission a DL MAC PDU on the PDDCH. (Note 4). | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | drx-InactivityTimer PDCCH-occasions after the transmission of the MAC PDU transmitted in step 5 was indicated on the PDCCH, the SS indicates the transmission of a DL MAC PDU on the PDCCH. (Note 4) | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 7? | --> | HARQ ACK | 2 | P |
| 9 | At least drx-InactivityTimer PDCCH occasions after the transmission of the MAC PDU in Step 7 has been indicated (This means the next DRX cycle or later after Step 5) and in the last PDCCH occasion before the Drx-onDurationTimer expires, the SS indicates the transmission of a DL MAC PDU on the PDDCH. The DL MAC PDU transmitted is invalid. (Note 1, Note 4) | <-- | Invalid MAC PDU | - | - |
| 10 | Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 9? | --> | HARQ NACK | 1 | P |
| 11 | In the first PDCCH occasion when the Drx-RetransmissionTimerDL for the MAC PDU in Step 9 is started (i.e. after expiry of drx-HARQ-RTT-TimerDL after step 9), the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 11? | --> | HARQ ACK | 3 | P |
| 13 | At least drx-InactivityTimer PDCCH occasions after the transmission of the DL MAC PDU in Step 11 has been indicated (This means the next DRX cycle or later after Step 11) and last PDCCH occasion before the Drx-onDurationTimer expires, the SS indicates the transmission of DL MAC PDU on the PDCCH. The DL MAC PDU transmitted is invalid. (Note 1, Note 4) | <-- | Invalid MAC PDU | - | - |
| 14 | Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 13? | --> | HARQ NACK | 1 | P |
| 15 | In the last PDCCH occasion when the drx-RetransmissionTimerDL for MAC PDU in Step 13 is still running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 16 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 15? | --> | HARQ ACK | 3 | P |
| 17 | The SS is configured for Uplink Grant Allocation Type [0]. At least drx-InactivityTimer PDCCH subframes after the transmission of the DL MAC PDU in Step 15 has been indicated in the last PDCCH occasion when the onDuratiopnTimer is still running (This means the next DRX cycle or later after Step 9), the SS indicates an UL grant to the UE on the PDCCH. (Note 4) | <-- | UL grant on PDCCH | - | - |
| 18 | Check: Does the UE transmit a Buffer Status Report on the UL indicating an empty buffer? | --> | Buffer Status Report MAC control element | 1 | P |
| 19 | In the last PDCCH occasion when the drx-RetransmissionTimer-UL for MAC PDU from Step 17 is still running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 20 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 19? | --> | HARQ ACK | 4 | P |
| Note 1: Invalid MAC PDU is a MAC PDU that fails the CRC check. Note 2: All the DL MAC PDU are transmitted with the NDI set on the PDCCH. Note 3: Timer tolerances for the MAC DRX related timers measured in PDCCH occasions is 0. These timers are: drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL. Note 4: The drx-InactivityTimer is started in the next PDCCH occasion of the PDCCH occasion where DL new transmission is indicated. Note 5: The timer values expressed in number of slots. Note 6: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 7: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.5.1.3.3	Specific message contents

Table 7.1.1.5.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.1.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  | NR |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| } |  |  |  |
| nonCriticalExtension::= SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.1.3.3-2: CellGroupConfig (Table 7.1.1.5.1.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms20 |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-HARQ-RTT-TimerUL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-RetransmissionTimerDL | sl8 |  |  |
| drx-RetransmissionTimerUL | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.1.1.5.2	DRX operation / Short cycle not configured / Long DRX command MAC control element reception

7.1.1.5.2.1	Test Purpose (TP)

(1)

with { UE in CONNECTED mode }

ensure that {

when { long DRX cycle is configured and a DRX Command MAC control element is received }

then { UE successfully decodes the MAC control PDU }

}

(2)

with { UE in CONNECTED mode }

ensure that {

when { long DRX cycle is configured and the HARQ RTT Timer is running and a DRX Command MAC control element is received }

then { UE continues running the HARQ RTT timer }

}

(3)

with { UE in CONNECTED mode }

ensure that {

when { long DRX cycle is configured and the drx-RetransmissionTimer is running and a DRX Command MAC control element is received }

then { UE continues running the drx-RetransmissionTimer and monitors the PDCCH }

}

7.1.1.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE’s PDCCH monitoring.  Activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other subclauses of this specification. When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH continuously.

RRC controls DRX operation by configuring the following timers:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay  before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycle StartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the random access preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in subclause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if an drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN x 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN x 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if drx-SlotOffset is configured:

3>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe..

2>	else:

3>	start drx-onDurationTimer.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH;

2>	if the PDCCH indicates a DL transmission or if a DL assignment has been configured:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process immediately after the corresponding PUCCH transmission;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission or if a UL grant has been configured:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process immediately after the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer.

1>	else (i.e. not part of the Active Time):

2>	not report CQI/PMI/RI on PUCCH.

7.1.1.5.2.3	Test description

7.1.1.5.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.2.3.2	Test procedure sequence

For FDD, NormalSLT(current SFN,current sub-frame, current slot,y)=y; For TDD, NormalSLT(current SFN, current slot,y) counts the minimum number of normal slots needed to cover y number of PDCCH-occasions(slots) until next PDCCH-occasion(slot) available, starting from current slot on current SFN.

Table 7.1.1.5.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DRX parameters. (Note 5) | <-- | - | - | - |
| 2 | The UE transmits RRCReconfigurationComplete. (Note 6) | --> | - | - | - |
| 3 | In a PDCCH occasion which is X PDCCH sub frames before the PDCCH occasion in which the onDurationTimer expires, with X < drx-onDurationTimer, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits an MAC PDU. | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 5 | In a PDCCH occasion before the drx-onDurationTimer expires, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU with DRX MAC Control element. UE successfully decodes the MAC PDU and starts the long DRX cycle. | <-- | MAC PDU(DRX MAC Control element) | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 6A | In a PDCCH occasion before the Long DRX cycle ends, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU | <-- | MAC PDU |  |  |
| 6B | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | F |
| 7 | On the next or later DRX cycle than the one used for Step 3 and on a PDCCH occasion which is X PDCCH sub frames before the PDCCH occasion in which the onDurationTimer expires, with X < drx-onDurationTimer,the SS indicates the transmission of a DL MAC PDU. The SS transmits an invalid MAC PDU. (Note 1) | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ NACK? | --> | HARQ NACK | - | P |
| 8A | In a PDCCH occasion before the Drx-HARQ-RTT-TimerDL for the MAC PDU indicated in Step 7 expires, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU with DRX MAC Control element. | <-- | MAC PDU(DRX MAC Control element) | - | - |
| 8B | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2,3 | P |
| 9 | In a PDCCH occasion when the drx-RetransmissionTimer for the MAC PDU indicated in Step 7 is still running,, the SS indicates the transmission of a DL MAC PDU. The SS transmits a DL MAC PDU with DRX MAC Control element. | <-- | MAC PDU(DRX MAC Control element) | - | - |
| 10 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2,3 | P |
| 11 | In the last sub frame when the Drx-RetransmissionTimer for the DL MAC PDU indicated on the PDCCH in Step 7 is still running, the SS indicates the transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2,3 | P |
| Note 1: Invalid MAC PDU is a MAC PDU that fails the CRC check. Note 2: All DL MAC PDUs are transmitted with the NDI set on the PDCCH. Note 3: Timer tolerances for the MAC DRX related timers measured in PDCCH occasions(slots). These timers are: drx-InactivityTimer, drx-RetransmissionTimer, Drx-HARQ-RTT-TimerDL. Note 5: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 6: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete |  |  |  |  |  |

7.1.1.5.2.3.3	Specific message contents

Table 7.1.1.5.2.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.2.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  | NR |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| } |  |  |  |
| nonCriticalExtension::= SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.2.3.3-2: CellGroupConfig (Table 7.1.1.5.2.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms40 |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-HARQ-RTT-TimerUL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-RetransmissionTimerDL | sl80 |  |  |
| drx-RetransmissionTimerUL | sl80 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]