3GPP TSG-RAN5 Meeting #92-e	R5-215711

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0209 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of updated GNSS scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications, Rohde & Schwarz, CATT, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. RAN 5 has agreed that, for a number of reasons, the GNSS scenarios need to be updated. These reasons include the need to update the BDS scenarios to align with the latest version of LPP and the RAN 4 specifications, the need to include the possibility of using the BDS B1C signal and the need to update the time(s) of applicability of the scenarios. 2. In addition, the IE IODA is only required for Galileo. For other GNSSs it is not relevant and should be set to “Not Present” |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The GNSS scenarios are replaced with new ones for UTRA, E-UTRA and NR with the following improvements. Note that currently the GPS-only scenarios for UTRA are not changed, but this could be done in a future meeting if found necessary by companies. Note that currently the QZSS scenarios are not provided (and are set to “FFS”), but this could be done in a future meeting if found necessary by companies. The new scenarios have the following changes: a) The new scenarios provided use Rinex navigation data format which gives the ephemeris (navigation model) and other useful data. Note however that this does not provide the almanac data which is set to “FFS” at this time. This almanc data will be provided at some future time if found necessary by companies. b) the BDS scenarios comply with recent changes in LPP and the RAN 4 specifications and allow for the use of the B1C signal including new tables to define the BDS satellite types. c) the time of applicability of the scenarios is set to 17th September 2020. 2. To allow for the gradual adoption of this new data and the gradual re-verification and re-validation of test cases, a 24 month ”transition period” is proposed where either this new data or the existing data can be used. 3. To avoid unnecessary duplication of data and likely associated errors, most of the Assistance Data that can be easily derived from the Rinex files has been deleted. 4. The following errors are corrected and editorial changes made: a) Some sub-clauses are now numbered for ease of reference. b) The IE IODA is only required for Galileo. For other GNSSs it is not relevant and is set to “Not Present” c) Other minor clarifications and formatting changes made. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. The GNSS scenarios will remain out-of-date and they will become more and more unusable. The BDS scenarios will not align with the latest version of LPP and the RAN 4 specifications and will not be able to use the BDS B1C signal. 2. A number of errors will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 6.1.1, 6.1.2, 6.1.3.2, 6.1.3.4, Annex B, data files |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Equivalent CR for RF Instructions to editor: 1. Remove zip file GNSS_Data_Sig_V16.zip from the zip file 2. Add the attached zip file GNSS_Orbital_Data_Sig_V1.zip into the zip file |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Result of one revision of R5-214185 |  |  |  |  |  |  |  |  |

# 4	General

## 4.1	GPS and GNSS orbital model information, assistance data and assistance data files

## << The changes in this section are repeated in the CR for RF! >>

<< start of changes>>

The following subclauses 5 and 6 define the GPS and GNSS orbital model information, the assistance data and the assistance data files (subclause 5 only) for the test cases as follows:

Subclause 5.1: data for UTRA A-GPS Signalling test cases defined in TS 37.571-2 [7] subclauses 6.1.1 to 6.1.3.

Subclause 5.2: data for UTRA A-GPS Minimum Performance test cases defined in TS 37.571-1 [6] subclause 5.

Subclause 6.1: data for UTRA, E-UTRA and NR A-GNSS Signalling test cases defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9.

Subclause 6.2: data for UTRA, E-UTRA and NR A-GNSS Minimum Performance test cases defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

For subclause 5 the orbital model information is defined and where appropriate is given in Yuma format in .txt files for each scenario in the appropriate data file specified in Annex A.

For subclause 6 the orbital model information is defined and where appropriate is given in Rinex navigation data file format for each scenario in the appropriate data file specified in Annex B.

For subclause 5, where the assistance data is fixed or is not required on a per-satellite basis, then it is defined in the following subclauses. Where assistance data is required on a per-satellite basis, or where the values of the data also vary with time then it is specified in comma-separated-variable files in the appropriate data file specified in Annex A. These files specify the values to be used for each satellite, indexed by satellite PRN or SV ID, and, where applicable, the values to be used indexed by both time and satellite PRN or SV ID.

For subclause 6, the assistance data is defined in the following subclauses.

For the aerial GNSS scenarios defined in TS 36.508 [20], the orbital model information is defined and where appropriate is given in Yuma format in .txt files for each scenario in the appropriate data file specified in Annex B.

## << Next changed section >>

# 6	GNSS information

## 6.1	GNSS Scenarios and Assistance Data for Assisted GNSS signalling tests

### 6.1.1	General

This subclause defines the GNSS scenario and the associated assistance data that shall be used where required for UTRA, E-UTRA and NR Assisted GNSS signalling tests defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9.

In all cases the Assistance Data is given in the two necessary formats, RRC format for TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and LPP format for TS 37.571-2 [7] subclauses 7 and 9. Other information is also given separately for TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9 where it differs between the subclauses.

The satellite simulator shall generate all the UE supported GNSS satellite signals defined in subclause 6.1.2 and/or shall provide assistance data dependent on the UE capabilities defined in subclause 6.1.3. Note that some tests require assistance data to be provided even though satellite signals are not required.

The A-GNSS signalling test cases may include several sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined below. In some cases the detailed assistance data content defined in subclause 6.1.3 depends on the particular sub-test case.

Table 6.1.1-1: Sub-Test Case Number Definition for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS only |
| 2 | UE supporting A-Galileo only |
| 3 | UE supporting A-GPS and Modernized GPS only |
| 4 | UE supporting A-GPS and A-GLONASS only |
| 8 | UE supporting A-GPS and A-Galileo only |
| 9 | UE supporting A-BDS only |
| 10 | UE supporting A-GPS and A-BDS only |

Table 6.1.1-2: Sub-Test Case Number Definition for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | Void |
| 2 | Void |
| 3 | Void |
| 4 | Void |
| 7 | UE supporting GNSS(1) and OTDOA |
| 8 | Void |
| 9 | Void |
| 10 | Void |
| 15 | UE supporting GNSS(1) |
| Note 1: The GNSS combination of GPS, GLONASS, Galileo or BDS supported by the UE |  |

The term SV ID used in this subclause is defined as the satellite PRN for GPS, as Code Number for Galileo, as the satellite Slot Number for GLONASS and as the Ranging Code Number for BDS.

As an alternative, the contents of clause 6.1 in version 16.5.0 of this current specification may be used until September 2023.

### 6.1.2	GNSS Scenario

The following GNSS scenario shall be used. The assistance data specified in the following subclauses is consistent with this GNSS scenario:

-	Rinex navigation data files: the required file(s) in the GNSS orbital data sig zip file specified in Annex B are given below.

Table 6.1.2-1: Rinex navigation data files for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | Rinex navigation datafile(s) |
| --- | --- |
| 1 | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| 2 | Sig GNSS Galileo 2020_9_17 Rinex.txt |
| 3 | Sig GNSS GPS 2020_9_17 Rinex.txt |
| 4 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| 8 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS Galileo 2020_9_17 Rinex.txt |
| 9 | Sig GNSS BDS 2020_9_17 Rinex.txt |
| 10 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS BDS 2020_9_17 Rinex.txt |

Table 6.1.2-2: Rinex navigationdata files for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | Rinex navigation datafile(s) (1) |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15 | GPS | Sig GNSS GPS 2020_9_17 Rinex.txt |
|  | GLONASS | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
|  | Galileo | Sig GNSS Galileo 2020_9_17 Rinex.txt |
|  | BDS | Sig GNSS BDS 2020_9_17 Rinex.txt |
| Note 1: Where the UE supports more than one GNSS then all the relevant Rinex navigationdata files are used |  |  |

-	UE location and Reference location:

Static at latitude: 35 degrees 44 minutes 39.432 seconds north, longitude: 139 degrees 40 minutes 48.633 seconds east, (Tokyo Japan), height: = 300m.

-	Nominal start time:

17th September 2020 23:40:00  (GPS time).

-	The visible satellites available for simulation and for which Assistance Data (other than Almanac) shall be generated are given in Table 6.1.2-2A.

Table 6.1.2-2A: SV IDs of Visible satellites

| GNSS | SV IDs of Visible satellites |
| --- | --- |
| GPS | 3, 4, 6, 17, 19, 28 |
| GLONASS | 3, 4, 5, 10, 18, 19 |
| Galileo | 3, 5, 13, 15, 21, 27 |
| BDS | 38, 40, 42, 43, 59, 60 |

-	For BDS, the satellite types are given in Table 6.1.2-2B

Table 6.1.2-2B: BDS satellite types

| BDS Satellite type | SV IDs of Satellites |
| --- | --- |
| GEO | 59, 60 |
| IGSO | 38, 40 |
| MEO | 42, 43 |

-	The visible satellites to be simulated in each sub-test case are given in Table 6.1.2-3 and Table 6.1.2-4

Table 6.1.2-3: Satellites to be simulated for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 3, 4, 5, 10, 18, 19 (GLONASS) |
| 2 | 3, 5, 13, 15, 21, 27 (Galileo) |
| 3 | 3, 4, 6, 17, 19, 28 (GPS) (Note) |
| 4 | GPS: 3, 4, 6, 28. GLONASS: 5, 16, 18, 19. |
| 8 | GPS: 3, 4, 6, 28. Galileo: 3, 5, 13, 21. |
| 9 | 38, 40, 42, 43, 59, 60 (BDS) |
| 10 | GPS: 3, 4, 6, 28. BDS: 38, 40, 59, 60. |
| Note: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. |  |

Table 6.1.2-4: Satellites to be simulated for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | SV IDs of Satellites to be simulated |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15 (Note) | GPS | 3, 4, 6, 17, 19, 28 |
|  | GLONASS | 3, 4, 5, 10, 18, 19 |
|  | Galileo | 3, 5, 13, 15, 21, 27 |
|  | BDS | 38, 40, 42, 43, 59, 60 |
| Note: For this sub-test the satellite simulator shall generate all the GNSS signals supported by the UE for all the simulated satellites. |  |  |

-	Ionospheric model: see values in subclause 6.1.3

-	The levels of the simulated satellites shall all be at -125dBm +/- 6dB

### 6.1.3	Assistance Data

This subclause defines the GNSS scenarios and assistance data IEs which shall be available for use where required in A-GNSS signalling test cases defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9.

#### 6.1.3.1	Default Assistance Data for TS 37.571-2 subclauses 6.2.1 to 6.2.3

The assistance data listed in subclause 6.1.3.1 are the assistance data elements pushed by the SS in some tests defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3. During the test the UE may request additional assistance data as specified in the tests and the SS shall then provide any other assistance data available as defined in subclause 6.1.3.

Table 6.1.3.1-1: GNSS assistance data to be provided to the UE

| GNSS Assistance Data IE to be provided to the UE | Mode used in test case |  |
| --- | --- | --- |
|  | UE-based | UE-assisted |
| GPS reference time | Yes for sub-tests 3, 4, 8, 10 | Yes for sub-tests 3, 4, 8, 10 |
| GPS reference UE position | Yes for sub-tests 3, 4, 8, 10 | No |
| GPS navigation model | Yes for sub-tests 3, 4, 8, 10 | No |
| GPS ionospheric model | Yes for sub-tests 3, 4, 8, 10 | No |
| GPS UTC model | Yes for sub-tests 4, 8 | Yes for sub-tests 4, 8 |
| GPS acquisition assistance | No | Yes for sub-tests 3, 4, 8, 10 |
| GANSS reference time | Yes for sub-tests 1, 2, 9 | Yes for sub-tests 1, 2, 9 |
| GANSS reference UE position | Yes for sub-tests 1, 2, 9 | No |
| GANSS ionospheric model | Yes for sub-tests 2, 8 | No |
| GANSS additional ionospheric model | Yes for sub-tests 1, 9, 10 | No |
| GANSS Time Models | Yes for sub-tests 4, 8, 10 | No |
| GANSS navigation model | Yes for sub-tests 2, 8 | No |
| GANSS additional navigation models | Yes for sub-tests 1, 4, 9, 10 | No |
| GANSS reference measurement information | No | Yes for sub-tests 1, 2, 4, 8, 9, 10 |
| GANSS auxiliary information | Yes for sub-tests 1, 4 (for GLONASS). Yes for sub-test 3 (for multiple GPS signals). Yes for sub-tests 4, 8, 10 if the UE supports multiple GPS signals | Yes for sub-tests 1, 4 (for GLONASS). Yes for sub-test 3 (for multiple GPS signals). Yes for sub-tests 4, 8, 10 if the UE supports multiple GPS signals |

#### 6.1.3.2	Assistance Data values for TS 37.571-2 subclauses 6.2.1 to 6.2.3

Assistance data that is marked as “time varying” and the GPS TOW msec or GANSS TOD field are created and used in 1 second increments.

The accuracy of the GPS TOW msec or GANSS TOD and assistance data that is marked as “time varying” in the provided assistance data shall be within +/- 2 s relative to the GNSS time in the system simulator. In the case that assistance data is required but satellite signals are not required then this clause does not apply.

Assistance data Information Elements and fields that are not specified shall not be used.

The information elements detailed below are fully defined in TS 25.331 [16]

##### 6.1.3.2.1	Assistance Data GPS Reference Time

# GPS Reference Time (Fields occurring once per message)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GPS Week | weeks | Derived from data in clause 6.1.2 |  |
| GPS Week Cycle Number |  | Derived from data in clause 6.1.2 | Rel-10 onwards |
| GPS TOW msec | msec | Start time is derived from data in clause 6.1.2. Add integer number of 1 seconds as required. (Note) |  |
| UE Positioning GPS Reference Time Uncertainty |  | 125 (2.127 seconds) |  |
| Note: GPS TOW msec This is the value of GPS TOW msec when the GNSS scenario is started in the GNSS simulator. The value of GPS TOW msec to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current GPS TOW msec” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.2. In the case that the (hardware) GPS simulator is switched off or not present then the value of GPS TOW msec given above may be used. |  |  |  |

##### 6.1.3.2.2	Assistance Data GPS Reference UE Position

# GPS Reference UE Position

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Latitude sign |  | Derived from data in clause 6.1.2 |
| Degrees Of Latitude | degrees | Derived from data in clause 6.1.2 |
| Degrees Of Longitude | degrees | Derived from data in clause 6.1.2 |
| Altitude Direction |  | Derived from data in clause 6.1.2 |
| Altitude | m | Derived from data in clause 6.1.2 |
| Uncertainty semi-major | m | 3000 |
| Uncertainty semi-minor | m | 3000 |
| Orientation of major axis | degrees | 0 |
| Uncertainty Altitude | m | 500 |
| Confidence | % | 68 |

##### 6.1.3.2.3	Assistance Data GPS Navigation Model

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GPS Navigation Model (Fields occurring once per satellite)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SatID | - | Derived from data in clause 6.1.2 |
| Satellite Status |  | 0 |

# GPS Ephemeris and Clock correction Information Elements (Fields occurring once per satellite)

Derived from data in clause 6.1.2

##### 6.1.3.2.4	Assistance Data GPS Ionospheric Model

# GPS Ionospheric Model

Derived from data in clause 6.1.2

##### 6.1.3.2.5	Assistance Data GPS UTC model

# GPS UTC Model

Derived from data in clause 6.1.2 and the following information:

A1: 0

A0: 0

##### 6.1.3.2.6	Assistance Data GPS Almanac

# GPS Almanac (Fields occurring once per message)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| WNa | weeks | Derived from data in clause 6.1.2 |  |
| Complete Almanac Provided |  | TRUE | Rel-10 onwards |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 31 |

# GPS Almanac (Fields occurring once per satellite)

FFS

##### 6.1.3.2.7	Assistance Data GPS Acquisition Assistance

# GPS Acquisition Assist - Information Elements appearing once per message

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GPS TOW msec | msec | Start time is derived from data in clause 6.1.2. Add integer number of 1 seconds as required. (Note) |
| UE Positioning GPS Reference Time Uncertainty |  | 125 (2.127 seconds) |
| Note: GPS TOW msec This is the value of GPS TOW msec when the GNSS scenario is started in the GNSS simulator. The value of GPS TOW msec to be used in the Acquisition Assistance IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. In the case that the (hardware) GPS simulator is switched off or not present then the value of GPS TOW msec given above may be used. |  |  |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GPS Acquisition Assist - Information Elements appearing once per satellite

These fields are time varying (see clause 6.1.3.2) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

##### 6.1.3.2.8	Assistance Data GANSS reference time

# GANSS reference time: sub-test 1

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Day |  | Derived from data in clause 6.1.2 |  |
| GANSS Day Cycle Number |  | Derived from data in clause 6.1.2 | Rel-10 onwards |
| GANSS TOD | Seconds | Start time is derived from data in clause 6.1.2. Add integer number of 1 seconds as required. (Note) |  |
| GANSS TOD Uncertainty |  | 125 (2.127 seconds) |  |
| GANSS Time ID |  | 2 (GLONASS) |  |
| Note: GANSS TOD This is the value of GANSS TOD when the GNSS scenario is started in the GNSS simulator. The value of GANSS TOD to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current GANSS TOD” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.3. In the case that the (hardware) GNSS simulator is switched off or not present then the value of GANSS TOD given above may be used. |  |  |  |

# GANSS reference time: sub-test 2

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Day |  | Derived from data in clause 6.1.2 |  |
| GANSS Day Cycle Number |  | Derived from data in clause 6.1.2 | Rel-10 onwards |
| GANSS TOD | Seconds | Start time is derived from data in clause 6.1.2. Add integer number of 1 seconds as required. (Note) |  |
| GANSS TOD Uncertainty |  | 125 (2.127 seconds) |  |
| GANSS Time ID |  | Not present (Galileo) |  |
| Note: GANSS TOD This is the value of GANSS TOD when the GNSS scenario is started in the GNSS simulator. The value of GANSS TOD to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current GANSS TOD” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.3. In the case that the (hardware) GNSS simulator is switched off or not present then the value of GANSS TOD given above may be used. |  |  |  |

# GANSS reference time: sub-test 9

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Day |  | Derived from data in clause 6.1.2 |  |
| GANSS Day Cycle Number |  | Derived from data in clause 6.1.2 | Rel-12 onwards |
| GANSS TOD | Seconds | Start time is derived from data in clause 6.1.2. Add integer number of 1 second as required. (Note) |  |
| GANSS TOD Uncertainty |  | 125 (2.127 seconds) |  |
| GANSS Time ID |  | 3 (BDS system time) |  |
| Note: GANSS TOD This is the value of GANSS TOD when the GNSS scenario is started in the GNSS simulator. The value of GANSS TOD to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current GANSS TOD” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.3. In the case that the (hardware) GNSS simulator is switched off or not present then the value of GANSS TOD given above may be used. |  |  |  |

##### 6.1.3.2.9	Assistance Data GANSS reference UE position

# GANSS reference UE position

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Latitude sign |  | Derived from data in clause 6.1.2 |
| Degrees Of Latitude | degrees | Derived from data in clause 6.1.2 |
| Degrees Of Longitude | degrees | Derived from data in clause 6.1.2 |
| Altitude Direction |  | Derived from data in clause 6.1.2 |
| Altitude | m | Derived from data in clause 6.1.2 |
| Uncertainty semi-major | m | 3000 |
| Uncertainty semi-minor | m | 3000 |
| Orientation of major axis | degrees | 0 |
| Uncertainty Altitude | m | 500 |
| Confidence | % | 68 |

##### 6.1.3.2.10	Assistance Data GANSS ionospheric model

# GANSS ionospheric model

Derived from data in clause 6.1.2

##### 6.1.3.2.11	Assistance Data GANSS additional ionospheric model

# GANSS additional ionospheric model (QZSS)

Derived from data in clause 6.1.2 and the following information:

Data Id: 00

# GANSS additional ionospheric model (BDS)

Derived from data in clause 6.1.2 and the following information:

Data Id: 01

##### 6.1.3.2.12	Assistance Data GANSS time model

# GANSS time model (GLONASS)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Time Model Reference Time | s | Derived from data in clause 6.1.2 |  |
| TA0 | Seconds | 0 |  |
| GNSS_TO_ID |  | 0 (GPS) |  |
| Delta_T | Seconds | Derived from data in clause 6.1.2 | Rel-10 onwards |

# GANSS time model (Galileo)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Time Model Reference Time | s | Derived from data in clause 6.1.2 |  |
| TA0 | Seconds | 0 |  |
| GNSS_TO_ID |  | 0 (GPS) |  |
| Delta_T | Seconds | Derived from data in clause 6.1.2 | Rel-10 onwards |

# GANSS time model (BDS)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Time Model Reference Time | s | Derived from data in clause 6.1.2 |  |
| TA0 | Seconds | 0 |  |
| GNSS_TO_ID |  | 0 (GPS) |  |
| Delta_T | Seconds | Derived from data in clause 6.1.2 | Rel-12 onwards |

##### 6.1.3.2.13	Assistance Data GANSS navigation model

# GANSS navigation model (Galileo)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Non-Broadcast Indication |  | Not present |

# Satellite Information (Galileo)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# Satellite Information (Fields occurring once per satellite) (Galileo)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SatID |  | Derived from data in clause 6.1.2 |
| SV Health |  | 0 |
| IOD |  | Derived from data in clause 6.1.2 |

# GANSS Clock Model (Fields occurring once per satellite) (Galileo)

GANSS Clock Model: Satellite clock model (Model 1)

Derived from data in clause 6.1.2

# GANSS Orbit Model (Fields occurring once per satellite) (Galileo)

GANSS Orbit Model: Keplerian Parameters (Model 1)

Derived from data in clause 6.1.2

##### 6.1.3.2.14	Assistance Data GANSS additional navigation models

# GANSS additional navigation models (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Non-Broadcast Indication |  | Not present |

# Satellite Information (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# Satellite Information (Fields occurring once per satellite) (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SatID |  | Derived from data in clause 6.1.2 |
| SV Health |  | 000000 |
| IOD |  | Derived from data in clause 6.1.2 |

# GANSS additional Clock Models (Fields occurring once per satellite) (GLONASS)

GANSS additional clock models: GLONASS Satellite Clock Model (Model 4)

Derived from data in clause 6.1.2

# GANSS additional orbit models (Fields occurring once per satellite) (GLONASS)

GANSS additional orbit models: GLONASS Earth-Centered, Earth-fixed Parameters (Model 4)

Derived from data in clause 6.1.2

# GANSS additional navigation model (BDS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Non-Broadcast Indication |  | Not present |

# Satellite Information (BDS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# Satellite Information (Fields occurring once per satellite) (BDS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SatID |  | Derived from data in clause 6.1.2 |
| SV Health |  | 0 |
| IOD |  | Derived from data in clause 6.1.2 |

# GANSS additional Clock Model (Fields occurring once per satellite) (BDS)

GANSS additional clock models: BDS Satellite Clock Model (Model 6)

Derived from data in clause 6.1.2

# GANSS additional Orbit Models (Fields occurring once per satellite) (BDS)

GANSS additional orbit models: BDS Keplerian Parameters (Model 6)

Derived from data in clause 6.1.2

##### 6.1.3.2.15	Assistance Data GANSS reference measurement information

# GANSS reference measurement information: sub-tests 1 and 4 (Fields occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS Signal ID |  | Not present |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GANSS reference measurement information: sub-tests 1 and 4 (Fields occurring once per satellite)

These fields are time varying (see clause 6.1.3.2) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GANSS reference measurement information: sub-tests 2 and 8 (Fields occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS Signal ID |  | Not present |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GANSS reference measurement information: sub-tests 2 and 8 (Fields occurring once per satellite)

These fields are time varying (see clause 6.1.3.2) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GANSS reference measurement information: sub-tests 9 and 10 (Fields occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS Signal ID |  | Not present |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GANSS reference measurement information: sub-tests 9 and 10 (Fields occurring once per satellite)

These fields are time varying (see clause 6.1.3.2) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

##### 6.1.3.2.16	Assistance Data GANSS almanac

# GANSS almanac: sub-tests 1 and 4 (Fields occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Week Number | Weeks | N/A |

# Satellite Information GLO-KP: sub-tests 1 and 4

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 24 |

# GANSS almanac: sub-tests 1 and 4 (Fields occurring once per satellite)

GLONASS Keplerian Parameters (Model 5)

FFS

# GANSS almanac: sub-tests 2 and 8 (Fields occurring once per message)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| Week Number | Weeks | Derived from data in clause 6.1.2 |  |
| Complete Almanac Provided |  | TRUE | Rel-10 onwards |

# GANSS almanac: sub-tests 2 and 8 (Field occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Toa |  | Derived from data in clause 6.1.2 |
| IODa |  | Derived from data in clause 6.1.2 |

# Satellite Information KP: sub-tests 2 and 8

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 29 |

# GANSS almanac: sub-tests 2 and 8 (Fields occurring once per satellite)

Keplerian parameters (Model 1)

FFS

# GANSS almanac: sub-tests 9 and 10 (Fields occurring once per message)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| Week Number | Weeks | Derived from data in clause 6.1.2 |  |
| Complete Almanac Provided |  | TRUE | Rel-12 onwards |

# GANSS almanac: sub-tests 9 and 10 (Field occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Toa |  | Derived from data in clause 6.1.2 |
| IODa |  | Not present |

# Satellite Information BDS-KP: sub-tests 9 and 10

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 30 |

# GANSS almanac: sub-tests 9 and 10 (Fields occurring once per satellite)

BDS Keplerian Parameters (Model 7)

FFS

##### 6.1.3.2.17	Assistance Data GANSS auxiliary information

# GANSS auxiliary information (Fields occurring once per message) (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS-ID-3 |  | Present (GLONASS) |

# Aux Info List (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GANSS auxiliary information (Fields occurring once per satellite) (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Sat ID |  | Derived from data in clause 6.1.2 |
| Signals Available |  | G1 |
| Channel Number |  | Derived from data in clause 6.1.2 |

# GANSS auxiliary information (Fields occurring once per message) (multiple GPS signals)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS-ID-1 |  | Present (Modernized GPS) |

Aux Info List (multiple GPS signals)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

GANSS auxiliary information (Fields occurring once per satellite) (multiple GPS signals)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Sat ID |  | Derived from data in clause 6.1.2 |
| Signals Available |  | As supported by the UE |

##### 6.1.3.2.18	Assistance Data GANSS ID

GANSS ID: sub-tests 1 and 4

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS ID |  | 3 (GLONASS) |

GANSS ID: sub-tests 2 and 8

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS ID |  | Not present (Galileo) |

GANSS ID: sub-test 3

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS ID |  | 1 (Modernized GPS) |

GANSS ID: sub-tests 9 and 10

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS ID |  | 4 (BDS) |

#### 6.1.3.3	Default Assistance Data for TS 37.571-2 subclauses 7 and 9

This subclause defines the GNSS assistance data elements which shall be provided to the UE in certain tests in TS 37.571-2 [7] subclauses 7 and 9 in the LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message. The GNSS assistance data provided depends on the mode being used in the test case, the assistance data supported by the UE and the GNSS(s) supported by the UE. GNSS assistance data IEs not supported by the UE shall not be sent. GNSS assistance data IEs supported by the UE but not listed below shall not be sent.

Table 6.1.3.3-1: Default GNSS assistance data to be provided to the UE

| GNSS Assistance Data IE supported by the UE | Mode used in test case |  |  |
| --- | --- | --- | --- |
|  | UE-based | UE-assisted. GNSS-Acquisition Assistance supported by the UE | UE-assisted. GNSS-Acquisition Assistance not supported by the UE |
| GNSS-Reference Time | Yes | Yes | Yes |
| GNSS-ReferenceLocation | Yes | No | Yes |
| GNSS-IonosphericModel | Yes | No | No |
| GNSS-TimeModelList | Yes(1) | No | Yes(1) |
| GNSS-NavigationModel | Yes | No | Yes |
| GNSS-AcquisitionAssistance | No | Yes | No |
| GNSS-Almanac | No | No | Yes |
| GNSS-UTC-Model | Yes(2) | Yes(2) | Yes(2 |
| GNSS-AuxiliaryInformation | Yes(3) | Yes(3) | Yes(3) |
| Note1: Only if more than one GNSS supported by the UE Note2: Only if GLONASS and at least one other GNSS supported by the UE. Note3: Only if GLONASS supported by the UE, and/or if the UE supports multiple GPS signals and/or if BDS B1C supported by the UE. |  |  |  |

#### 6.1.3.4	Assistance Data values for TS 37.571-2 subclauses 7 and 9

Assistance data that is marked as “time varying” and the gnss-TimeOfDay field are created and used in 1 second increments.

The accuracy of the gnss-TimeOfDay and assistance data that is marked as “time varying” in the provided assistance data shall be within +/- 2 s relative to the GNSS time in the system simulator. In the case that assistance data is required but satellite signals are not required then this clause does not apply.

Assistance data Information Elements and fields that are not specified shall not be used.

The information elements detailed below are fully defined in TS 37.355 [8].

##### 6.1.3.4.1	GNSS REFERENCE TIME:

# GNSS-ReferenceTime: If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-SystemTime |  |  |
| gnss-TimeID |  | 0 (gps) |
| gnss-DayNumber |  | Derived from data in clause 6.1.2 |
| gnss-TimeOfDay |  | Start time is derived from data in clause 6.1.2. Add integer number of 1 seconds as required. (Note) |
| gnss-TimeOfDayFrac-msec |  | Not present |
| notificationOfLeapSecond |  | Not present |
| gps-TOW-Assist |  |  |
| satelliteID |  | Derived from data in clause 6.1.2 |
| tlmWord |  | Derived from data in clause 6.1.2 |
| antiSpoof |  | 1 (for all PRNs) |
| alert |  | 0 (for all PRNs) |
| tlmRsvdBits |  | Derived from data in clause 6.1.2 |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Not present |
| Note: gnss-TimeOfDay This is the value of gnss-TimeOfDay when the GNSS scenario is started in the GNSS simulator. The value of gnss-TimeOfDay to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current gnss-TimeOfDay” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.4. In the case that the (hardware) GNSS simulator is switched off or not present then the value of gnss-TimeOfDay given above may be used. |  |  |

# GNSS-ReferenceTime: If GLONASS is the only GNSS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-SystemTime |  |  |
| gnss-TimeID |  | 4 (glonass) |
| gnss-DayNumber |  | Derived from data in clause 6.1.2 |
| gnss-TimeOfDay |  | Start time is derived from data in clause 6.1.2. Add integer number of 1 seconds as required. (Note) |
| gnss-TimeOfDayFrac-msec |  | Not present |
| notificationOfLeapSecond |  | 00 |
| gps-TOW-Assist |  | Not present |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Not present |
| Note: gnss-TimeOfDay This is the value of gnss-TimeOfDay when the GNSS scenario is started in the GNSS simulator. The value of gnss-TimeOfDay to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current gnss-TimeOfDay” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.4. In the case that the (hardware) GNSS simulator is switched off or not present then the value of gnss-TimeOfDay given above may be used. |  |  |

# GNSS-ReferenceTime: If Galileo is the only GNSS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-SystemTime |  |  |
| gnss-TimeID |  | 3 (galileo) |
| gnss-DayNumber |  | Derived from data in clause 6.1.2 |
| gnss-TimeOfDay |  | Start time is derived from data in clause 6.1.2. Add integer number of 1 seconds as required. (Note) |
| gnss-TimeOfDayFrac-msec |  | Not present |
| notificationOfLeapSecond |  | Not present |
| gps-TOW-Assist |  | Not present |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Not present |
| Note: gnss-TimeOfDay This is the value of gnss-TimeOfDay when the GNSS scenario is started in the GNSS simulator. The value of gnss-TimeOfDay to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current gnss-TimeOfDay” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.4. In the case that the (hardware) GNSS simulator is switched off or not present then the value of gnss-TimeOfDay given above may be used. |  |  |

# GNSS-ReferenceTime: If BDS is the only GNSS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-SystemTime |  |  |
| gnss-TimeID |  | 5 (bds) |
| gnss-DayNumber |  | Derived from data in clause 6.1.2 |
| gnss-TimeOfDay |  | Start time is derived from data in clause 6.1.2. Add integer number of 1 seconds as required. (Note) |
| gnss-TimeOfDayFrac-msec |  | Not present |
| notificationOfLeapSecond |  | Not present |
| gps-TOW-Assist |  | Not present |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Not present |
| Note: gnss-TimeOfDay This is the value of gnss-TimeOfDay when the GNSS scenario is started in the GNSS simulator. The value of gnss-TimeOfDay to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current gnss-TimeOfDay” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.4. In the case that the (hardware) GNSS simulator is switched off or not present then the value of gnss-TimeOfDay given above may be used. |  |  |

##### 6.1.3.4.2	GNSS REFERENCE LOCATION:

# GNSS-ReferenceLocation

| Information Element | Units | Value/remark |
| --- | --- | --- |
| threeDlocation |  |  |
| latitudeSign |  | Derived from data in clause 6.1.2 |
| degreesLatitude | degrees | Derived from data in clause 6.1.2 |
| degreesLongitude | degrees | Derived from data in clause 6.1.2 |
| altitudeDirection |  | Derived from data in clause 6.1.2 |
| altitude | m | Derived from data in clause 6.1.2 |
| uncertaintySemiMajor | m | 3000 |
| uncertaintySemiMinor | m | 3000 |
| orientationMajorAxis | degrees | 0 |
| uncertaintyAltitude | m | 500 |
| confidence | % | 68 |

##### 6.1.3.4.3	GNSS IONOSPHERIC MODEL:

# GNSS-IonosphericModel (Klobuchar Model): If GPS or GLONASS or BDS B1I supported by the UE

Derived from data in clause 6.1.2 and the following information:

dataID: 00

neQuickModel: not present unless Galileo is also supported by the UE

klobucharModel2: not present unless BDS B1C is also supported by the UE

# GNSS-IonosphericModel (NeQuick Model): If Galileo supported by the UE

Derived from data in clause 6.1.2 and the following information:

klobucharModel: not present unless GPS or GLONASS or BDS B1I is also supported by the UE

klobucharModel2: not present unless BDS B1C is also supported by the UE

# GNSS-IonosphericModel (Klobuchar2 Model): If BDS B1C supported by the UE

Derived from data in clause 6.1.2 and the following information:

klobucharModel: not present unless GPS or GLONASS or BDS B1I is also supported by the UE

neQuickModel: not present unless Galileo is also supported by the UE

##### 6.1.3.4.4	GNSS TIME MODEL:

# GNSS-TimeModelList: If GLONASS and GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-TimeModelRefTime | seconds | Derived from data in clause 6.1.2 |
| tA0 |  | 0 |
| gnss-TO-ID |  | 1 (GPS) |
| weekNumber |  | Derived from data in clause 6.1.2 |
| deltaT |  | Derived from data in clause 6.1.2 |

# GNSS-TimeModelList: If Galileo and GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-TimeModelRefTime | Seconds | Derived from data in clause 6.1.2 |
| tA0 |  | 0 |
| gnss-TO-ID |  | 1 (GPS) |
| weekNumber |  | Derived from data in clause 6.1.2 |
| deltaT |  | Derived from data in clause 6.1.2 |

# GNSS-TimeModelList: If BDS and GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-TimeModelRefTime | seconds | Derived from data in clause 6.1.2 |
| tA0 |  | 0 |
| gnss-TO-ID |  | 1 (GPS) |
| weekNumber |  | Derived from data in clause 6.1.2 |
| deltaT |  | Derived from data in clause 6.1.2 |

##### 6.1.3.4.5	GNSS NAVIGATION MODEL:

# GNSS-NavigationModel (Model-2): If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-2): If GPS supported by the UE

Derived from data in clause 6.1.2 and the following information:

GNSS-ClockModel: nav-ClockModel, Model-2

GNSS-OrbitModel: nav-KeplerianSet, Model-2

Note: in the case that the UE supports Modernized GPS then the UE may also support the GNSS-NavigationModel (Model-3), however in this case the GNSS-NavigationModel (Model-2) for GPS shall still be used.

# GNSS-NavigationModel (Model-4): If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-4): If GLONASS supported by the UE

Derived from data in clause 6.1.2 and the following information:

svHealth: 00000000

GNSS-ClockModel: glonass-ClockModel, Model-4

GNSS-OrbitModel: glonass-ECEF, Model-4

# GNSS-NavigationModel (Model-1): If Galileo supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-1): If Galileo supported by the UE

Derived from data in clause 6.1.2 and the following information:

svHealth: 0

GNSS-ClockModel: standardClockModelList, Model-1.

standardClockModelList: (SIZE) 1 if the UE supports only Galileo E1, (SIZE) 2 if the UE supports multiple Galileo signals.

StandardClockModelElement (I/NAV):

- stanClockTgd: Not present if the UE supports multiple Galileo signals.

- stanModelID: 0 (I/NAV). Present only if the UE supports multiple Galileo signals

StandardClockModelElement (F/NAV): Present only if the UE supports multiple Galileo signals

- stanClockTgd: Not present

- stanModelID: 1 (F/NAV)

GNSS-OrbitModel: keplerianSet, Model-1

# GNSS-NavigationModel (Model-6): If BDS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-6): If BDS supported by the UE

Derived from data in clause 6.1.2 and the following information:

svHealth: 0

GNSS-ClockModel: BDS-ClockModel-r12, Model-6

GNSS-OrbitModel: BDS-KeplerianSet-r12, Model-6

Note: in the case that the UE supports BDS B1C then the UE may also support the GNSS-NavigationModel (Model-7), however in this case the GNSS-NavigationModel (Model-6) for BDS shall still be used.

##### 6.1.3.4.6	GNSS ACQUISITION ASSISTANCE:

In the case that the UE only supports GPS L1 C/A for GPS then the GNSS-AcquisitionAssistance (GPS L1 C/A) shall be used for GPS.

In the case that the UE supports Modernized GPS then the GNSS-AcquisitionAssistance to be used for GPS depends on the GNSS-AcquisitionAssistance(s) supported by the UE for GPS. The possible GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (GPS L1 C/A)

GNSS-AcquisitionAssistance (Modernized GPS L5)

The GNSS-AcquisitionAssistance to be used shall be determined by the PICs pc_GNSS_AcquAssist_GPS_L1CA and pc_GNSS_AcquAssist_GPS_L5, in the case that both GNSS-AcquisitionAssistances are supported by the UE then the GNSS-AcquisitionAssistance (GPS L1 C/A) shall be used.

# GNSS-AcquisitionAssistance (GPS L1 C/A)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (GPS L1 C/A) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (GPS L1 C/A)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (Modernized GPS L5)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 3 (GPS L5) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (Modernized GPS L5)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance: If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (GLONASS G1) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement: If GLONASS supported by the UE

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

In the case that the UE only supports Galileo E1 for Galileo then the GNSS-AcquisitionAssistance (Galileo E1) shall be used for Galileo.

In the case that the UE supports more than one Galileo signal then the GNSS-AcquisitionAssistance to be used for Galileo depends on the GNSS-AcquisitionAssistance(s) supported by the UE for Galileo. The possible GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (Galileo E1)

GNSS-AcquisitionAssistance (Galileo E5A)

The GNSS-AcquisitionAssistance to be used shall be determined by the PICs pc_GNSS_AcquAssist_Galileo_E1 and pc_GNSS_AcquAssist_Galileo_E5A, in the case that both GNSS-AcquisitionAssistances are supported by the UE then the GNSS-AcquisitionAssistance (Galileo E1) shall be used.

# GNSS-AcquisitionAssistance (Galileo E1)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (Galileo E1) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (Galileo E1)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (Galileo E5A)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 1 (Galileo E5A) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (Galileo E5A)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

In the case that the UE only supports BDS B1I for BDS then the GNSS-AcquisitionAssistance (BDS B1I) shall be used for BDS.

In the case that the UE supports more than one BDS signal then the GNSS-AcquisitionAssistance to be used for BDS depends on the GNSS-AcquisitionAssistance(s) supported by the UE for BDS. The possible GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (BDS B1I)

GNSS-AcquisitionAssistance (BDS B1C)

The GNSS-AcquisitionAssistance to be used shall be determined by the PICs pc_GNSS_AcquAssist_BDS_B1I and pc_GNSS_AcquAssist_BDS_B1C, in the case that both GNSS-AcquisitionAssistances are supported by the UE then the GNSS-AcquisitionAssistance (BDS B1I) shall be used.

# GNSS-AcquisitionAssistance (BDS B1I)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (BDS B1I) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (BDS B1I)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (BDS B1C)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | FFS |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (BDS B1C)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

##### 6.1.3.4.7	GNSS ALMANAC:

# GNSS-Almanac (Model-2): If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| weekNumber |  | Derived from data in clause 6.1.2 |
| toa |  | Derived from data in clause 6.1.2 |
| ioda |  | Not present |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 31 |

# GNSS-AlmanacElement (Model-2): If GPS supported by the UE

FFS

GNSS-AlmanacElement:keplerianNAV-Almanac (Model-2)

Note: in the case that the UE supports Modernized GPS then the UE may also support the GNSS-Almanac (Model-3) and/or GNSS-Almanac (Model-4), however in this case the GNSS-Almanac (Model-2) for GPS shall still be used.

# GNSS-Almanac (Model-5): If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 24 |

# GNSS-AlmanacElement (Model-5): If GLONASS supported by the UE

FFS

GNSS-AlmanacElement: keplerianGLONASS (Model-5)

# GNSS-Almanac (Model-1): If Galileo supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| weekNumber |  | Derived from data in clause 6.1.2 |
| toa |  | Derived from data in clause 6.1.2 |
| ioda |  | 0 |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 29 |

# GNSS-AlmanacElement (Model-1): If Galileo supported by the UE

FFS

GNSS-AlmanacElement: keplerianAlmanacSet (Model-1)

kepSV-StatusFNAV: 0. Present only if the UE supports multiple Galileo signals.

# GNSS-Almanac (Model-7): If BDS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| weekNumber |  | Derived from data in clause 6.1.2 |
| toa |  | Derived from data in clause 6.1.2 |
| ioda |  | Not present |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 30 |

# GNSS-AlmanacElement (Model-7): If BDS supported by the UE

FFS

GNSS-AlmanacElement: BDS-AlmanacSet-r12 (Model-7)

Note: in the case that the UE supports BDS B1C then the UE may also support the GNSS-Almanac (Model-3) and/or GNSS-Almanac (Model-4), however in this case the GNSS-Almanac (Model-7) for BDS shall still be used.

##### 6.1.3.4.8	GNSS UTC MODEL:

# GNSS-UTC-Model: If both GPS and GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-UTC-Model |  |  |
| utcModel1 |  |  |

# UTC-ModelSet1: If both GPS and GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-Utc-A1 |  | 0 |
| gnss-Utc-A0 |  | 0 |
| gnss-Utc-Tot |  | Derived from data in clause 6.1.2 |
| gnss-Utc-WNt |  | Derived from data in clause 6.1.2 |
| gnss-Utc-DeltaTls |  | Derived from data in clause 6.1.2 |
| gnss-Utc-WNlsf |  | Derived from data in clause 6.1.2 |
| gnss-Utc-DN |  | Derived from data in clause 6.1.2 |
| gnss-Utc-DeltaTlsf |  | Derived from data in clause 6.1.2 |

##### 6.1.3.4.9	GNSS AUXILIARY INFORMATION:

# GNSS-AuxiliaryInformation: If multiple GPS signals supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |
| gnss-ID-GPS |  | (SIZE) 6 |
| svID |  | Derived from data in clause 6.1.2 |
| signalsAvailable |  | As supported by the UE |

# GNSS-AuxiliaryInformation: If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |
| gnss-ID-GLONASS |  | (SIZE) 6 |
| svID |  | Derived from data in clause 6.1.2 |
| signalsAvailable |  | G1 |
| channelNumber |  | Derived from data in clause 6.1.2 |

# GNSS-AuxiliaryInformation: If BDS B1C supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |
| gnss-ID-BDS-r16 |  | (SIZE) 6 |
| svID-r16 |  | Derived from data in clause 6.1.2 |
| satType-r16 |  | Derived from data in clause 6.1.2 |

## << Next changed section >>

Annex A (normative): GPS data files

## A.1	GPS data files for signalling tests

The GPS data files for use in GPS signalling tests defined in TS 37.571-2 [7] subclauses 6.1.1 to 6.1.3 are contained in archive GPS_Data_Sig_V7.zip which accompanies this document.

The acquisition assistance data files contained in the archive are recommended but not mandatory.

## A.2	GPS data files for Minimum Performance tests

The GPS data files for use in GPS Minimum Performance tests defined in TS 37.571-1 [6] subclause 5 are contained in archive GPS_Data_Perf_V10.zip which accompanies this document. The different scenarios are designated with suffixes XX in the zip file, where XX is 01, 02, 03 etc. for scenarios #1, #2, #3 etc.

The acquisition assistance data files contained in the archive are recommended but not mandatory.

Annex B (normative): GNSS data files

## B.1	GNSS data files for signalling tests

The GNSS orbital data files for use in GNSS signalling tests defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9 are contained in archive GNSS_Orbital_Data_Sig_V1.zip which accompanies the present document.

## B.2	GNSS data files for Minimum Performance tests

The GNSS data files for use in GNSS Minimum Performance tests defined in TS 37.571-1 [6] subclauses 6,  7 and 13 are contained in archive GNSS_Data_Perf_V16.zip which accompanies the present document. The different scenarios are designated with suffixes XX in the zip file, where XX is 01, 02, 03 etc. for scenarios #1, #2, #3 etc. The acquisition assistance and reference measurement information data files contained in the archive are recommended but not mandatory.

Moreover there are currently two sets of the files Perf GANSS reference measurement information subtest1_4 01.csv and Perf GANSS reference measurement information subtest1_4 02.csv supplied in the archive, designated _set1 and _set2. Either set can be used for testing.

## B.3	GNSS data files for aerial tests

The GNSS data files for use in GNSS signalling tests using the scenario defined in TS 36.508 [20] subclause 4.12 are contained in archive GNSS_Data_Aerial_V1.zip which accompanies the present document.

The data files contained in the archive are recommended but not mandatory.

<<end of changes >>