3GPP TSG-RAN WG5 Meeting #92-e	R5-215746

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0180 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.6.7 - Floor Taken |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to 24.380 and RFC 3550 the SSRC in the common RTCP header is mandatory - According to 24.380 clause 8.2.9 the SSRC of granted floor participant is not present for Multi-Talker - The I bit of the Floor Indicator shall be set for Multi-Talker (NOTE: this is currently done in the specific message content of the test case) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Condition Multi-Talker removed from SSRC in the common RTCP header - Condition Multi-Talker added for SSRC of granted floor participant - Condition Multi-Talker added for Floor Indicator |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid Floor Taken sent to the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-214651 and the outcome of 1 revision to this document |  |  |  |  |  |  |  |  |

#### 5.5.6.7	Floor Taken

Table 5.5.6.7-1: Floor Taken

| Derivation Path: 24.380 [10], Table 8.2.9-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00010 | Floor Taken with acknowledgment not required |  |
| SSRC | The SSRC of the message sender | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPT |  |  |
| User ID | Not present |  | ON-NETWORK |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT user ID of the floor participant sending the Floor Taken message |  |
| Granted Party's Identity |  |  |  |
| Granted Party's Identity | px_MCPTT_ID_User_B | The MCPTT User ID of the floor participant being granted the floor. |  |
| Granted Party's Identity | Not Present |  | Multi-Talker |
| Permission to Request the Floor |  |  |  |
| Permission to Request the Floor | "1" | The receiver is permitted to request floor |  |
| Message Sequence Number |  |  |  |
| Message Sequence Number | The value sent in the previous Floor Taken message, if any, increased with 1 | Any value between '0' and '65535' When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
| Floor Indicator | 1000010010000000 | Normal call, queueing supported, multi-talker | Multi-Talker |
| SSRC of granted floor participant | SS-UE1 (MCPTT Client) SSRC | The SSRC of the granted floor participant. |  |
| SSRC of granted floor participant | Not present |  | Multi-Talker |
| Functional Alias | Not present |  |  |
|  | px_MCPTT_ID_FA_B | Functional Alias = URI | FA AND NOT Multi-Talker |
| List of Granted Users | Not present |  |  |
| List of Granted Users |  |  | Multi-Talker |
| No of users | '10' |  |  |
| User ID | px_MCPTT_ID_User_A |  |  |
| User ID | px_MCPTT_ID_User_B |  |  |
| List of SSRCs of granted floor participants | Not present |  |  |
| List of SSRCs of granted floor participants |  |  | Multi-Talker |
| Number of SSRCs | '10' |  |  |
| SSRC | The SSRC of User A |  |  |
| SSRC | The SSRC of User B |  |  |
| List of Functional Aliases | Not present |  |  |
| List of Functional Aliases |  |  | FA AND Multi-Talker |
| No of FAs | '10' |  |  |
| Functional Alias | px_MCPTT_ID_FA_A |  |  |
| Functional Alias | px_MCPTT_ID_FA_B |  |  |
| Location |  |  | NOT Multi-Talker |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |
| Location | Not present |  | Multi-Talker |
| List of Locations | Not present |  | NOT Multi-Talker |
| List of Locations |  | The location information shall be maintained in the same order as the users in the List of Granted Users to allow location information to be matched to the correct user. | Multi-Talker |
| Number of Locations | '10' |  |  |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |