3GPP TSG-RAN5 Meeting #92-e	R5-215814

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0210 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update GNSS scenarios for multi-GNSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The number of satellites for 3-GNSS scenarios has been increased in TS 36.171 and TS 38.171. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increased the number of satellites in all A-GNSS scenarios with 3-GNSS for minimum performance A-GNSS tests in section 6.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The number of satellites for 3-GNSS scenarios will not be aligned to RAN4 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on R4-21xxxxx |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: correct title on the coverpage |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### 6.2.1.2	GNSS Scenarios for A-GNSS minimum performance testing

This subclause defines the GNSS scenarios that shall be used for all Assisted GNSS minimum performance tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

The GNSS scenarios achieve the required HDOP for the Test Cases and they also satisfy the requirement that for each test instance the reference location shall change sufficiently such that the UE shall have to use the new assistance data.

The viable running time during which the scenario maintains the required HDOP or HDOPs is given. Once this time has been reached the scenario shall be restarted from its nominal start time.

The test cases include sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined below. For each GNSS scenario the parameters that vary with the sub-test are given for each sub-test.

Table 6.2.1.2-1: Sub-Test Case Number Definition for TS 37.571-1 subclause 6

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS only |
| 2 | UE supporting A-Galileo only |
| 3 | UE supporting A-GPS and Modernized GPS only |
| 4 | UE supporting A-GPS and A-GLONASS only |
| 8 | UE supporting A-GPS and A-Galileo only |
| 9 | UE supporting A-BDS only |
| 10 | UE supporting A-GPS and A-BDS only |

Table 6.2.1.2-2: Sub-Test Case Number Definition for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GPS L1 C/A only |
| 2 | UE supporting A-GLONASS only |
| 3 | UE supporting A-Galileo only |
| 4 | UE supporting A-GPS and Modernized GPS only |
| 5 | UE supporting A-GPS and A-GLONASS only (Note) |
| 8 | UE supporting A-GPS and A-Galileo only (Note) |
| 9 | UE supporting A-BDS only |
| 10 | UE supporting A-GPS and A-BDS only (Note) |
| 11 | UE supporting A-GPS and A-GLONASS and A-BDS only (Note) |
| 12 | UE supporting A-GPS and A-Galileo and A-GLONASS only (Note) |
| 13 | UE supporting A-GPS and A-Galileo and A-BDS only (Note) |
| Note: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. |  |

##### 6.2.1.2.1	GNSS Scenario #1

The following GNSS scenario #1 shall be used during the TTFF tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13 with the exception of the Nominal Accuracy test. The assistance data specified in the following subclauses for GNSS scenario #1 is consistent with this GNSS scenario.

Yuma / AGL Almanac data: the required file(s) in the GNSS data perf zip file specified in Annex B are given below.

Table 6.2.1.2.1-1: Yuma / AGL Almanac data files for TS 37.571-1 subclause 6

| Sub-Test Case Number | Yuma / AGL file(s) |
| --- | --- |
| 1 | Perf GNSS 1-1 AGL.txt |
| 2 | Perf GNSS 1-2 Yuma.txt |
| 3 | Perf GNSS 1-3 Yuma.txt |
| 4 | Perf GNSS 1-3 Yuma.txt and Perf GNSS 1-1 AGL.txt |
| 8 | Perf GNSS 1-3 Yuma.txt and Perf GNSS 1-2 Yuma.txt |
| 9 | Perf GNSS 1-9 Yuma.txt |
| 10 | Perf GNSS 1-3 Yuma.txt and Perf GNSS 1-9 Yuma.txt |

Table 6.2.1.2.1-2: Yuma / AGL Almanac data files for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | Yuma / AGL file(s) |
| --- | --- |
| 1 | Perf GNSS 1-3 Yuma.txt |
| 2 | Perf GNSS 1-1 AGL.txt |
| 3 | Perf GNSS 1-2 Yuma.txt |
| 4 | Perf GNSS 1-3 Yuma.txt |
| 5 | Perf GNSS 1-3 Yuma.txt and Perf GNSS 1-1 AGL.txt |
| 8 | Perf GNSS 1-3 Yuma.txt and Perf GNSS 1-2 Yuma.txt |
| 9 | Perf GNSS 1-9 Yuma.txt |
| 10 | Perf GNSS 1-3 Yuma.txt and Perf GNSS 1-9 Yuma.txt |
| 11 | Perf GNSS 1-3 Yuma.txt and Perf GNSS 1-1 AGL.txt and Perf GNSS 1-9 Yuma.txt |
| 12 | Perf GNSS 1-3 Yuma.txt and Perf GNSS 1-2 Yuma.txt and Perf GNSS 1-1 AGL.txt |
| 13 | Perf GNSS 1-3 Yuma.txt and Perf GNSS 1-2 Yuma.txt and Perf GNSS 1-9 Yuma.txt |

UE location: the UE location is calculated as a random offset from the reference location using the method described in subclause 6.2.1.2.6. The reference location is: latitude: 35 degrees 44 minutes 39.432 seconds north, longitude: 139 degrees 40 minutes 48.633 seconds east, (Tokyo Japan 2012), height: = 300m.

Nominal start time: 1st January 2012 00:31:00 (GPS time).

Viable running time to maintain specified HDOP values: 19 minutes.

Visible satellites available for simulation and for which Assistance Data (other than Almanac) shall be generated are given below.

Table 6.2.1.2.1-3: Visible satellites for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 3, 4, 5, 9, 10, 18, 19, 20 |
| 2 | 4, 5, 10, 11, 18, 19, 20, 21 |
| 3 | 1, 4, 11, 17, 19, 20, 23, 28, 32 |
| 4 | GPS: 1, 4, 11, 17, 19, 20, 23, 28, 32. GLONASS: 3, 4, 5, 9, 10, 18, 19, 20 |
| 8 | GPS: 1, 4, 11, 17, 19, 20, 23, 28, 32. Galileo: 4, 5, 10, 11, 18, 19, 20, 21. |
| 9 | 1, 2, 3, 4, 6, 7, 17, 18, 21, 22, 23, 26, 27, 28 |
| 10 | GPS: 1, 4, 11, 17, 19, 20, 23, 28, 32. BDS: 1, 2, 3, 4, 6, 7, 17, 18, 21, 22, 23, 26, 27, 28. |

Table 6.2.1.2.1-4: Visible satellites for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 1, 4, 11, 17, 19, 20, 23, 28, 32 |
| 2 | 3, 4, 5, 9, 10, 18, 19, 20 |
| 3 | 4, 5, 10, 11, 18, 19, 20, 21 |
| 4 | 1, 4, 11, 17, 19, 20, 23, 28, 32 |
| 5 | GPS: 1, 4, 11, 17, 19, 20, 23, 28, 32. GLONASS: 3, 4, 5, 9, 10, 18, 19, 20 |
| 8 | GPS: 1, 4, 11, 17, 19, 20, 23, 28, 32. Galileo: 4, 5, 10, 11, 18, 19, 20, 21. |
| 9 | 1, 2, 3, 4, 6, 7, 17, 18, 21, 22, 23, 26, 27, 28 |
| 10 | GPS: 1, 4, 11, 17, 19, 20, 23, 28, 32. BDS: 1, 2, 3, 4, 6, 7, 17, 18, 21, 22, 23, 26, 27, 28. |
| 11 | GPS: 1, 4, 11, 17, 19, 20, 23, 28, 32. GLONASS: 3, 4, 5, 9, 10, 18, 19, 20. BDS: 1, 2, 3, 4, 6, 7, 17, 18, 21, 22, 23, 26, 27, 28. |
| 12 | GPS: 1, 4, 11, 17, 19, 20, 23, 28, 32. Galileo: 4, 5, 10, 11, 18, 19, 20, 21. GLONASS: 3, 4, 5, 9, 10, 18, 19, 20. |
| 13 | GPS: 1, 4, 11, 17, 19, 20, 23, 28, 32. Galileo: 4, 5, 10, 11, 18, 19, 20, 21. BDS: 1, 2, 3, 4, 6, 7, 17, 18, 21, 22, 23, 26, 27, 28. |

The satellites to be simulated in each test case have been selected in order to achieve the required HDOP. They are defined below.

Table 6.2.1.2.1-5: Satellites to be simulated for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 3, 4, 9, 10, 18, 20 |
| 2 | 5, 10, 11, 18, 19, 20 |
| 3 | 1, 11, 17, 19, 23, 28 (Note) |
| 4 | GPS: 20, 28, 32. GLONASS: 5, 10, 20 |
| 8 | GPS: 20, 28, 32. Galileo: 5, 11, 18. |
| 9 | 1, 2, 7, 18, 21, 27 |
| 10 | GPS: 20, 28, 32. BDS: 1, 7, 21. |
| Note: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. |  |

Table 6.2.1.2.1-6: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | Test case dependant. See Table 6.2.1.2.1-7 |
| 2 | 3, 4, 9, 10, 18, 20 |
| 3 | 5, 10, 11, 18, 19, 20 (Note 2) |
| 4 | 1, 11, 17, 19, 23, 28 (Note 1) |
| 5 | GPS: 20, 28, 32. GLONASS: 5, 10, 20 (Note 1) |
| 8 | GPS: 20, 28, 32. Galileo: 5, 11, 18. (Note 2) (Note 1) |
| 9 | 1, 2, 7, 18, 21, 27 |
| 10 | GPS: 20, 28, 32. BDS: 1, 7, 21. (Note 1) |
| 11 | GPS: 20, 28, 32. GLONASS: 5, 10, 20. BDS: 1, 7, 21. (Note 1) (Note 3) |
| 12 | GPS: 20, 28, 32. Galileo: 5, 11, 18. GLONASS: 5, 10, 20 (Note 1) (Note 2) (Note 3) |
| 13 | GPS: 20, 28, 32. Galileo: 5, 11, 18. BDS: 1, 7, 21. (Note 1) (Note 2) (Note 3) |
| Note 1: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. Note 2: For this sub-test the satellite simulator shall generate all the Galileo signals supported by the UE for all the simulated satellites. Note 3: Only one of the following satellites shall be selected (by the device manufacturer): GPS SVID 28, GLONASS SVID 10, BDS SVID 21 or Galileo SVID 11 (Galileo SVID 18 in Sub-test 13) |  |

Table 6.2.1.2.1-7: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13, sub-test 1

| Test case | SV IDs of Satellites to be simulated |
| --- | --- |
| Sensitivity Coarse Time Assistance | 1, 4, 11, 17, 19, 20, 23, 28 |
| Sensitivity Fine Time Assistance | 1, 4, 11, 17, 19, 20, 23, 28 |
| Nominal Accuracy | 1, 4, 11, 17, 19, 20, 23, 28 |
| Dynamic Range | 1, 11, 17, 19, 23, 28 |
| Multi-Path scenario | 1, 17, 19, 23, 28 |

Ionospheric model: see values in subclause 6.2.7.

Tropospheric model: STANAG with SRI equal to 324.8, as defined in STANAG 4294 [17].

##### 6.2.1.2.2	GNSS Scenario #2

The following GNSS scenario #2 shall be used during the TTFF tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13 with the exception of the Nominal Accuracy test. The assistance data specified in the following subclauses for GNSS scenario #2 is consistent with this GNSS scenario.

Yuma / AGL Almanac data: the required file(s) in the GNSS data perf zip file specified in Annex B are below.

Table 6.2.1.2.2-1: Yuma / AGL Almanac data files for TS 37.571-1 subclause 6

| Sub-Test Case Number | Yuma / AGL file(s) |
| --- | --- |
| 1 | Perf GNSS 2-1 AGL.txt |
| 2 | Perf GNSS 2-2 Yuma.txt |
| 3 | Perf GNSS 2-3 Yuma.txt |
| 4 | Perf GNSS 2-3 Yuma.txt and Perf GNSS 2-1 AGL.txt |
| 8 | Perf GNSS 2-3 Yuma.txt and Perf GNSS 2-2 Yuma.txt |
| 9 | Perf GNSS 2-9 Yuma.txt |
| 10 | Perf GNSS 2-3 Yuma.txt and Perf GNSS 2-9 Yuma.txt |

Table 6.2.1.2.2-2: Yuma / AGL Almanac data files for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | Yuma / AGL file(s) |
| --- | --- |
| 1 | Perf GNSS 2-3 Yuma.txt |
| 2 | Perf GNSS 2-1 AGL.txt |
| 3 | Perf GNSS 2-2 Yuma.txt |
| 4 | Perf GNSS 2-3 Yuma.txt |
| 5 | Perf GNSS 2-3 Yuma.txt and Perf GNSS 2-1 AGL.txt |
| 8 | Perf GNSS 2-3 Yuma.txt and Perf GNSS 2-2 Yuma.txt |
| 9 | Perf GNSS 2-9 Yuma.txt |
| 10 | Perf GNSS 2-3 Yuma.txt and Perf GNSS 2-9 Yuma.txt |
| 11 | Perf GNSS 2-3 Yuma.txt and Perf GNSS 2-1 AGL.txt and Perf GNSS 2-9 Yuma.txt |
| 12 | Perf GNSS 2-3 Yuma.txt and Perf GNSS 2-2 Yuma.txt and Perf GNSS 2-1 AGL.txt |
| 13 | Perf GNSS 2-3 Yuma.txt and Perf GNSS 2-2 Yuma.txt and Perf GNSS 2-9 Yuma.txt |

UE location: the UE location is calculated as a random offset from the reference location using the method described in subclause 6.2.1.2.6. The reference location is: latitude: 37 degrees 24 minutes 53.391 seconds north, longitude: 122 degrees 1 minutes 3.722 seconds west, (Sunnyvale, USA), height: = 50m.

Nominal start time: 1st June 2012, 00:01:00 (GPS time).

Viable running time to maintain specified HDOP values: 19 minutes.

Visible satellites available for simulation and for which Assistance Data (other than Almanac) shall be generated are given below.

Table 6.2.1.2.2-3: Visible satellites for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 1, 8, 9, 10, 16, 19, 20, 21 |
| 2 | 4, 5, 14, 15, 16, 26, 29, 30 |
| 3 | 1, 7, 8, 11, 15, 17, 26, 27, 28 |
| 4 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. GLONASS: 1, 8, 9, 10, 16, 19, 20, 21 |
| 8 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. Galileo: 4, 5, 14, 15, 16, 26, 29, 30. |
| 9 | 4, 15, 16, 17, 18, 25, 26, 27 |
| 10 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. BDS: 4, 15, 16, 17, 18, 25, 26, 27. |

Table 6.2.1.2.2-4: Visible satellites for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 1, 7, 8, 11, 15, 17, 26, 27, 28 |
| 2 | 1, 8, 9, 10, 16, 19, 20, 21 |
| 3 | 4, 5, 14, 15, 16, 26, 29, 30 |
| 4 | 1, 7, 8, 11, 15, 17, 26, 27, 28 |
| 5 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. GLONASS: 1, 8, 9, 10, 16, 19, 20, 21 |
| 8 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. Galileo: 4, 5, 14, 15, 16, 26, 29, 30. |
| 9 | 4, 15, 16, 17, 18, 25, 26, 27 |
| 10 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. BDS: 4, 15, 16, 17, 18, 25, 26, 27. |
| 11 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. GLONASS: 1, 8, 9, 10, 16, 19, 20, 21. BDS: 4, 15, 16, 17, 18, 25, 26, 27. |
| 12 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. Galileo: 4, 5, 14, 15, 16, 26, 29, 30. GLONASS: 1, 8, 9, 10, 16, 19, 20, 21. |
| 13 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. Galileo: 4, 5, 14, 15, 16, 26, 29, 30. BDS: 4, 15, 16, 17, 18, 25, 26, 27. |

The satellites to be simulated in each test case have been selected in order to achieve the required HDOP. They are defined below.

Table 6.2.1.2.2-5: Satellites to be simulated for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 1, 8, 9, 10, 19, 20 |
| 2 | 4, 15, 16, 26, 29, 30 |
| 3 | 1, 7, 8, 17, 27, 28 (Note) |
| 4 | GPS: 1, 8, 26. GLONASS: 8, 10, 20 |
| 8 | GPS: 1, 8, 26. Galileo: 15, 26, 30. |
| 9 | 15, 16, 17, 25, 26, 27 |
| 10 | GPS: 1, 8, 26. BDS: 15, 25, 27. |
| Note: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. |  |

Table 6.2.1.2.2-6: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | Test case dependant. See Table 6.2.1.2.2-7 |
| 2 | 1, 8, 9, 10, 19, 20 |
| 3 | 4, 15, 16, 26, 29, 30 (Note 2) |
| 4 | 1, 7, 8, 17, 27, 28 (Note 1) |
| 5 | GPS: 1, 8, 26. GLONASS: 8, 10, 20 (Note 1) |
| 8 | GPS: 1, 8, 26. Galileo: 15, 26, 30. (Note 2) (Note 1) |
| 9 | 15, 16, 17, 25, 26, 27 |
| 10 | GPS: 1, 8, 26. BDS: 15, 25, 27. (Note 1) |
| 11 | GPS: 1, 8, 26. GLONASS: 8, 10, 20. BDS: 15, 25, 27. (Note 1) (Note 3) |
| 12 | GPS: 1, 8, 26. Galileo: 15, 26, 30. GLONASS: 8, 10, 20. (Note 1) (Note 2) (Note 3) |
| 13 | GPS: 1, 8, 26. Galileo: 15, 16, 26. BDS: 15, 25, 27. (Note 1) (Note 2) (Note 3) |
| Note 1: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. Note 2: For this sub-test the satellite simulator shall generate all the Galileo signals supported by the UE for all the simulated satellites. Note 3: Only one of the following satellites shall be selected (by the device manufacturer): GPS SVID 26, GLONASS SVID 8, BDS SVID 15 or Galileo SVID 15. |  |

Table 6.2.1.2.2-7: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13, sub-test 1

| Test case | SV IDs of Satellites to be simulated |
| --- | --- |
| Sensitivity Coarse Time Assistance | 1, 7, 8, 15, 17, 26, 27, 28 |
| Sensitivity Fine Time Assistance | 1, 7, 8, 15, 17, 26, 27, 28 |
| Nominal Accuracy | 1, 7, 8, 15, 17, 26, 27, 28 |
| Dynamic Range | 1, 7, 8, 17, 27, 28 |
| Multi-Path scenario | 1, 7, 8, 17, 27 |

Ionospheric model: see values in subclause 6.2.7.

Tropospheric model: STANAG with SRI equal to 324.8, as defined in STANAG 4294 [17].

##### 6.2.1.2.3	GNSS Scenario #3

The following GNSS scenario #3 shall be used during the Nominal Accuracy test defined in TS 37.571-1 [6] subclauses 6, 7 and 13. The assistance data specified in the following subclauses for GNSS scenario #3 is consistent with this GNSS scenario.

The scenario used varies dependent on the SBAS supported by the UE and also whether QZSS is supported. The scenario to be used is defined below. Where more than one SBAS is supported use the scenario for MSAS if MSAS and QZSS are supported, otherwise use the scenario for the first supported SBAS in the list.

Table 6.2.1.2.3-1: Scenarios used for Scenario #3

| SBAS supported | Scenarios used |  |
| --- | --- | --- |
|  | UE supports QZSS | UE does not support QZSS |
| None | GNSS Scenario #1 with QZSS Scenario #1 | GNSS Scenario #1 |
| WAAS | [FFS] | GNSS Scenario #2 with WAAS |
| EGNOS | [FFS] | GNSS Scenario #3A with EGNOS |
| MSAS | GNSS Scenario #1 with QZSS Scenario #1 and MSAS | GNSS Scenario #1 with MSAS |
| GAGAN | [FFS] | GNSS Scenario #3B with GAGAN |

###### 6.2.1.2.3.1	GNSS Scenario #3A

[FFS]

###### 6.2.1.2.3.2	GNSS Scenario #3B

[FFS]

###### 6.2.1.2.3.3	QZSS Scenario #1

Almanac data: see file Perf GNSS 1-3 Yuma_QZSS.txt in the GNSS data perf zip file specified in Annex B.

Ephemeris data: see file Perf GNSS Navigation Model subtest1 01_QZSS.csv in the GNSS data perf zip file specified in Annex B.

Acquisition Assistance: see file Perf GNSS Acquisition Assistance subtest1 01_QZSS.csv in the GNSS data perf zip file specified in Annex B.

UE location: as for GNSS scenario #1.

Nominal start time: as for GNSS scenario #1.

Viable running time to maintain specified requirements: as for GNSS scenario #1.

Satellite meeting specified requirements to be used for simulation and for which Assistance Data (other than Almanac) shall be generated: PRN 195.

###### 6.2.1.2.3.4	WAAS Scenario

Satellite positions: (PRN 135)133.0 degrees west, height: 35786037.417m, (PRN 138)107.3 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 135.

###### 6.2.1.2.3.5	EGNOS Scenario

Satellite positions: (PRN 120)15.5 degrees west, height: 35786037.417m, (PRN 124) 21.5 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 120.

###### 6.2.1.2.3.6	MSAS Scenario

Satellite positions: (PRN 129)140.0 degrees east, height: 35786037.417m, (PRN 137)145 degrees east, height: 35786037.417m

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 129.

###### 6.2.1.2.3.7	GAGAN Scenario

[FFS]

##### 6.2.1.2.4	GNSS Scenario #4

The following GNSS scenario #4 shall be used during the Nominal Accuracy test defined in TS 37.571-1 [6] subclauses 6, 7 and 13. The assistance data specified in the following subclauses for GNSS scenario #4 is consistent with this GNSS scenario.

The scenario used varies dependent on the SBAS supported by the UE and also whether QZSS is supported. The scenario to be used is defined below. Where more than one SBAS is supported use the scenario for MSAS if MSAS and QZSS are supported, otherwise use the scenario for the first supported SBAS in the list.

Table 6.2.1.2.4-1: Scenarios used for Scenario #4

| SBAS supported | Scenarios used |  |
| --- | --- | --- |
|  | UE supports QZSS | UE does not support QZSS |
| None | GNSS Scenario #4D with QZSS Scenario #2 | GNSS Scenario #2 |
| WAAS | [FFS] | GNSS Scenario #4C with WAAS |
| EGNOS | [FFS] | GNSS Scenario #4A with EGNOS |
| MSAS | GNSS Scenario #4D with QZSS Scenario #2 and MSAS | GNSS Scenario #4D with MSAS |
| GAGAN | [FFS] | GNSS Scenario #4B with GAGAN |

###### 6.2.1.2.4.1	GNSS Scenario #4A

[FFS]

###### 6.2.1.2.4.2	GNSS Scenario #4B

[FFS]

###### 6.2.1.2.4.3	GNSS Scenario #4C

[FFS]

###### 6.2.1.2.4.4		GNSS Scenario #4D

[FFS]

###### 6.2.1.2.4.5	QZSS Scenario #2

Almanac data: see file Perf QZSS 2 Yuma.txt in the GNSS data perf zip file specified in Annex B.

UE location: as for GNSS scenario #4D.

Nominal start time: as for GNSS scenario #4D.

Viable running time to maintain specified requirements: as for GNSS scenario #4D.

Satellite meeting specified requirements to be used for simulation and for which Assistance Data (other than Almanac) shall be generated: PRN [FFS].

###### 6.2.1.2.4.6	WAAS Scenario

Satellite positions: (PRN 135)133.0 degrees west, height: 35786037.417m, (PRN 138)107.3 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 138.

###### 6.2.1.2.4.7	EGNOS Scenario

Satellite positions: (PRN 120)15.5 degrees west, height: 35786037.417m, (PRN 124) 21.5 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 124.

###### 6.2.1.2.4.8	MSAS Scenario

Satellite positions: (PRN 129)140.0 degrees east, height: 35786037.417m, (PRN 137)145 degrees east, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 137.

###### 6.2.1.2.4.9	GAGAN Scenario

[FFS]

##### 6.2.1.2.5	GNSS Scenario #5

The following GNSS scenario #5 shall be used during the Moving Scenario and Periodic Update test cases defined in TS 37.571-1 [6] subclauses 6, 7 and 13. The assistance data specified in the following subclauses for GNSS scenario #5 is consistent with this GNSS scenario.

Yuma / AGL Almanac data: the required file(s) in the GNSS data perf zip file specified in Annex B are below.

Table 6.2.1.2.5-1: Yuma / AGL Almanac data files for TS 37.571-1 subclause 6

| Sub-Test Case Number | Yuma / AGL file(s) |
| --- | --- |
| 1 | Perf GNSS 5-1 AGL.txt |
| 2 | Perf GNSS 5-2 Yuma.txt |
| 3 | Perf GNSS 5-3 Yuma.txt |
| 4 | Perf GNSS 5-3 Yuma.txt and Perf GNSS 5-1 AGL.txt |
| 8 | Perf GNSS 5-3 Yuma.txt and Perf GNSS 5-2 Yuma.txt |
| 9 | Perf GNSS 5-9 Yuma.txt |
| 10 | Perf GNSS 5-3 Yuma.txt and Perf GNSS 5-9 Yuma.txt |

Table 6.2.1.2.5-2: Yuma / AGL Almanac data files for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | Yuma / AGL file(s) |
| --- | --- |
| 1 | Perf GNSS 5-3 Yuma.txt |
| 2 | Perf GNSS 5-1 AGL.txt |
| 3 | Perf GNSS 5-2 Yuma.txt |
| 4 | Perf GNSS 5-3 Yuma.txt |
| 5 | Perf GNSS 5-3 Yuma.txt and Perf GNSS 5-1 AGL.txt |
| 8 | Perf GNSS 5-3 Yuma.txt and Perf GNSS 5-2 Yuma.txt |
| 9 | Perf GNSS 5-9 Yuma.txt |
| 10 | Perf GNSS 5-3 Yuma.txt and Perf GNSS 5-9 Yuma.txt |
| 11 | Perf GNSS 5-3 Yuma.txt and Perf GNSS 5-9 AGL.txt and Perf GNSS 5-9 Yuma.txt |
| 12 | Perf GNSS 5-3 Yuma.txt and Perf GNSS 5-2 Yuma.txt and Perf GNSS 5-1 AGL.txt |
| 13 | Perf GNSS 5-3 Yuma.txt and Perf GNSS 5-2 Yuma.txt and Perf GNSS 5-9 Yuma.txt |

UE location: the UE location is given as a trajectory as shown in Figure 6.6.1 and Figure 7.1 of TS 37.571-1 [6]. The reference location is at the centre of the trajectory and is at: latitude: 37 degrees 24 minutes 53.391 seconds north, longitude: 122 degrees 1 minutes 3.722 seconds west, (Sunnyvale, USA), height: = 50m.

Start location: at the point between l11 and l12 in Figure 6.6.1 and Figure 7.1 of TS 37.571-1 [6], going in a clock-wise direction.

Nominal start time: 1st June 2012, 00:01:00 (GPS time).

Viable running time to maintain specified HDOP values: 20 minutes.

Visible satellites available for simulation and for which Assistance Data (other than Almanac) shall be generated are given below.

Table 6.2.1.2.5-3: Visible satellites for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 1, 8, 9, 10, 16, 19, 20, 21 |
| 2 | 4, 5, 14, 15, 16, 26, 29, 30 |
| 3 | 1, 7, 8, 11, 15, 17, 26, 27, 28 |
| 4 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. GLONASS: 1, 8, 9, 10, 16, 19, 20, 21 |
| 8 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. Galileo: 4, 5, 14, 15, 16, 26, 29, 30 |
| 9 | 4, 15, 16, 17, 18, 25, 26, 27 |
| 10 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. BDS: 4, 15, 16, 17, 18, 25, 26, 27 |

Table 6.2.1.2.5-4: Visible satellites for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 1, 7, 8, 11, 15, 17, 26, 27, 28 |
| 2 | 1, 8, 9, 10, 16, 19, 20, 21 |
| 3 | 4, 5, 14, 15, 16, 26, 29, 30 |
| 4 | 1, 7, 8, 11, 15, 17, 26, 27, 28 |
| 5 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. GLONASS: 1, 8, 9, 10, 16, 19, 20, 21 |
| 8 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. Galileo: 4, 5, 14, 15, 16, 26, 29, 30 |
| 9 | 4, 15, 16, 17, 18, 25, 26, 27 |
| 10 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. BDS: 4, 15, 16, 17, 18, 25, 26, 27 |
| 11 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. GLONASS: 1, 8, 9, 10, 16, 19, 20, 21. BDS: 4, 15, 16, 17, 18, 25, 26, 27. |
| 12 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. Galileo: 4, 5, 14, 15, 16, 26, 29, 30. GLONASS: 1, 8, 9, 10, 16, 19, 20, 21. |
| 13 | GPS: 1, 7, 8, 11, 15, 17, 26, 27, 28. Galileo: 4, 5, 14, 15, 16, 26, 29, 30. BDS: 4, 15, 16, 17, 18, 25, 26, 27. |

The satellites to be simulated in each test case have been selected in order to achieve the required HDOP. They are defined below.

Table 6.2.1.2.5-5: Satellites to be simulated for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 1, 8, 9, 10, 19, 20 |
| 2 | 4, 15, 16, 26, 29, 30 |
| 3 | 1, 7, 8, 17, 27, 28 (Note) |
| 4 | GPS: 1, 8, 26. GLONASS: 8, 10, 20 |
| 8 | GPS: 1, 8, 26. Galileo: 15, 26, 30 |
| 9 | 15, 16, 17, 25, 26, 27 |
| 10 | GPS: 1, 8, 26. BDS: 15, 25, 27 |
| Note: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. |  |

Table 6.2.1.2.5-6: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 7, 8, 17, 27, 28 |
| 2 | 1, 8, 9, 10, 19, 20 |
| 3 | 4, 15, 16, 26, 29, 30 (Note 2) |
| 4 | 1, 7, 8, 17, 27, 28 (Note 1) |
| 5 | GPS: 1, 8, 26. GLONASS: 8, 10, 20 (Note 1) |
| 8 | GPS: 1, 8, 26. Galileo: 15, 26, 30 (Note 2) (Note 1) |
| 9 | 15, 16, 17, 25, 26, 27 |
| 10 | GPS: 1, 8, 26. BDS: 15, 25, 27 (Note 1) |
| 11 | GPS: 1, 8, 26. GLONASS: 8, 10, 20. BDS: 15, 25, 27 (Note 1) (Note 3) |
| 12 | GPS: 1, 8, 26. Galileo: 15, 26, 30. GLONASS: 8, 10, 20. (Note 1) (Note 2) (Note 3) |
| 13 | GPS: 1, 8, 26. Galileo: 15, 16, 26. BDS: 15, 25, 27. (Note 1) (Note 2) (Note 3) |
| Note 1: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. Note 2: For this sub-test the satellite simulator shall generate all the Galileo signals supported by the UE for all the simulated satellites. Note 3: Only one of the following satellites shall be selected (by the device manufacturer): GPS SVID 26, GLONASS SVID 8, BDS SVID 15 or Galileo SVID 15. |  |

Ionospheric model: see values in subclause 6.2.7.

Tropospheric model: STANAG with SRI equal to 324.8, as defined in STANAG 4294 [17].

##### 6.2.1.2.6	UE Location for TTFF test cases

This subclause defines the method for generating the random UE locations that are required to be used for the TTFF tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

For every Test Instance in each TTFF test case, the UE location shall be randomly selected to be within 3 km of the Reference Location. The Altitude of the UE shall be randomly selected between 0 m to 500 m above WGS-84 reference ellipsoid. These values shall have uniform random distributions.

The UE location is calculated as an offset from the Reference Location.

###### 6.2.1.2.6.1	UE Location Offset

The UE location offset shall be calculated by selecting the next pair of random numbers, representing a pair of latitude and longitude offsets in degrees, from a standard uniform random number generator, with the following properties:

The ranges of the latitude and longitude offsets values shall be such that when translated onto the surface of the earth they shall lie within a 3km radius circle, centred on the Reference location specified for the GNSS scenario under consideration. For the purposes of this calculation make the following assumptions:

a)	Over the 3km radius circle at the Reference location the earth is flat and the meridians and parallels form a rectangular grid

b)	The earth is spherical with a radius of 6371141m (equal to the WGS 84 value at 35 degrees latitude)

The resolution used for the latitude and longitude offsets values shall be 90/2E23 for the latitude offset values and 360/2E24 for the longitude offset values, representing the coding resolution in degrees specified in TS 23.032.

###### 6.2.1.2.6.2	UE Altitude

The UE altitude value shall be calculated by selecting the next random number from a standard uniform random number generator, in the range 0 to 500, representing meters. The resolution used for the random number shall be 1, representing 1 meter.

< End of changes >