3GPP TSG-RAN5 Meeting #92-e	R5-215837

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2014 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CSI report configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For Table 4.6.3-25, nrOfAntennaPorts is defined only more than 2Tx case. For Table 4.6.3-39, the conditions for cqi-Tale are not adequate and PDSCH configuration defined in a test case should be considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Table 4.6.3-25, added the setting for 2Tx condition. For Table 4.6.3-39, removed FR1 and FR2 conditions, and added 64QAM and 256QAM conditions respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configurations and message settings are inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is outcome of 1 revision of R5-215340 r1: - Move ‘2TX’ to the appropriate row in Table 4.6.3-25 - Set 64QAM to the default in Table 4.6.3-39 - Remove 64QAM from the table for condition explanation |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

CodebookConfig

Table 4.6.3-25: CodebookConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| two SEQUENCE { |  |  | 2TX |
| twoTX-CodebookSubsetRestriction | 111111 |  |  |
| } |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| two-one-TypeI-SinglePanel-Restriction | 11111111 |  | FR2 |
| four-one-TypeI-SinglePanel-Restriction | 11111111 11111111 |  | FR1 |
| } |  |  |  |
| typeI-SinglePanel-codebookSubsetRestriction-i2 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 11111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| codebookMode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX | 2 Tx CSI-RS configuration. |

<<Unchaged sections skipped>>

–	CSI-ReportConfig

Table 4.6.3-39: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| nzp-CSI-RS-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER { | 1 entry |  |  |
| INTEGER[1] | 14 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL, |  | FR1 |
| cri-RI-LI-PMI-CQI | NULL |  | FR2 |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand | Not present |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table1 | 64QAM |  |
|  | table2 |  | 256QAM |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 256QAM | Test cases using 256QAM for PDSCH |

<<End of change>>