3GPP TSG-RAN5 Meeting #92-e	R5-216015

# Electronic Meeting, 16th – 27th Aug. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1117 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of general spurious emissions test requirements for Rel-16 inter-band EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the minimum requirements defined in 38.101-3 clause 6.5B.3.3.1, The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band. Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur the general spurious emission for inter-band EN-DC can be verified at the specific frequency where 2nd and 3rd order intermodulation products may occur. Therefore, the test requirements need to be updated for the purpose of reducing test time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating test requirements for the following Rel-16 EN-DC combinations: DC_3A_n41A DC_8A_n41A DC_12A_n78A DC_28A_n3A DC_39A_n41A DC_40A_n41A - Adjust the position of several EN-DC configurations to follow alphabetical order. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above Rel-16 EN-DC combinations may not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.905 CR 0477 - 0482 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-215216r2 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5B.3.3.1	General spurious emissions for Inter-band EN-DC within FR1

6.5B.3.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5B.3.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward supporting inter-band EN-DC.

6.5B.3.3.1.3	Minimum conformance requirements

The general spurious emissions requirements specified in subclause 6.6.3.1 of TS 36.101 [5], subclause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier. For the case of inter-band EN-DC with a single carrier per cell group, the general spurious emissions requirements also apply with both downlink carrier and both uplink carriers active. Limits on configured maximum output power for the uplink according to subclause 6.2B.4 apply.

NOTE:	The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band.  Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.3.1. Exception requirements applicable for both NR or LTE, therefore LTE anchor agnostic approach is not applied

6.5B.3.3.1.4	Test description

6.5B.3.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, DC configuration specified in Table 5.5B.4.1-1 and test channel bandwidths specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1, and sub-carrier spacing based on NR operating bands specified in TS 38.521-1 [8] clause 5.3. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 6.5B.3.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annexe A, clause A.2.3 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.3.3.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [6] clause 4.3.1 and TS 38.508-1 clause 4.3.1 |  | 5 MHz for E-UTRA CC and Lowest for NR CC Highest for E-UTRA CC and Highest for NR CC |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  | Lowest SCS per Channel Bandwidth |  |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Spurious emission. | QPSK | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Left |
| 2 |  | QPSK | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Right |
| 3 |  | QPSK | Outer_Full | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 3: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 4: Only one EN-DC combination per FR1 band is tested for each EN-DC configuration as defined in clause 5.5B.4. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for SS diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

4.	NR downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set according to TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B and TS 38.521-1 [8] Annex B for E-UTRA link and NR link respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.5B.3.3.1.4.3.

8.	For both E-UTRA and NR UL uplink carriers active when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.3.3.1.4.2	Test procedure

1.	E-UTRA SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands to the UE for both NR and E-UTRA carriers until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5B.3.3.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to the same table. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

In addition to test configuration and test procedure above, NSA only capable UEs need to be tested according to LTE anchor agnostic below.

Same test description as in clause 6.5.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.3.3.1.4.3	Message Contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.3.3.1.4.3-1: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | Sa2 |  |  |
| specialSubframePatterns | Ssp7 |  |  |
| } |  |  |  |

Table 6.5B3.3.1.4.3-1a: Void

Table 6.5B.3.3.1.4.3-2: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Table 6.5B.3.3.1.4.3-3: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

6.5B.3.3.1.5	Test Requirement

For EN-DC configurations listed in Table 6.5B.3.3.1.5-1, the according test requirements of the same table apply.

For EN-DC configurations without test requirements specified in Table 6.5B.3.3.1.5-1, the test requirements in clause 6.5B.3.1.1.5 apply.

For EN-DC only capable devices, for EN-DC configurations listed in Table 6.5B.3.3.1.5-1, in addition to Table 6.5B.3.3.1.5-1, test requirements for NR carrier are the same as Table 6.5.3.1.5-1 in TS 38.521-1 [8].

Table 6.5B.3.3.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| Test requirements for DC_3A_n7A Configuration |  |  |  |
| 715 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1070 MHz 3215 MHz ≤ f ≤ 3430 MHz 4210 MHz ≤ f ≤ 4355 MHz 5920 MHz ≤ f ≤ 6140 MHz 6710 MHz ≤ f ≤ 6925 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n41A Configuration |  |  |  |
| 711 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000MHz ≤ f ≤ 1074 MHz 3207MHz ≤ f ≤ 3670 MHz 4206MHz ≤ f ≤ 4475 MHz 5916MHz ≤ f ≤ 6260 MHz 6702MHz ≤ f ≤ 7165 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n78A Configuration |  |  |  |
| 270 MHz ≤ f ≤ 380 MHz | -36 dBm | 100 kHz |  |
| 1515 MHz ≤ f ≤ 2090 MHz 4815 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7370 MHz 8310 MHz ≤ f ≤ 9385 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_8A_n41A Configuration |  |  |  |
| 666 MHz ≤ f < 930 MHz | -36 dBm | 100 kHz |  |
| 1581MHz ≤ f ≤ 1810 MHz 3376MHz ≤ f ≤ 3605 MHz 4077MHz ≤ f ≤ 4520 MHz 5872MHz ≤ f ≤ 6295 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_8A_n77A Configuration |  |  |  |
| 1470 MHz ≤ f ≤ 3320 MHz 4180 MHz ≤ f ≤ 9315 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_12A_n66A Configuration |  |  |  |
| 278 MHz ≤ f ≤ 382 MHz 994 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1081 MHz 2409 MHz ≤ f ≤ 2496 MHz 2704 MHz ≤ f ≤ 2861 MHz 3108 MHz ≤ f ≤ 3212 MHz 4119 MHz ≤ f ≤ 4276 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_12A_n78A Configuration |  |  |  |
| 1868 MHz ≤ f ≤ 2402 MHz 2584 MHz ≤ f ≤ 3101 MHz 3999 MHz ≤ f ≤ 4516 MHz 4698 MHz ≤ f ≤ 5232 MHz 5884 MHz ≤ f ≤ 6901 MHz 7299 MHz ≤ f ≤ 8316 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_26A_n41A Configuration |  |  |  |
| 798 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1062 MHz 1647 MHz ≤ f ≤ 1876 MHz 3310 MHz ≤ f ≤ 3539 MHz 4124 MHz ≤ f ≤ 4566 MHz 5806 MHz ≤ f ≤ 6229 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_26A_n79A Configuration |  |  |  |
| 2702 MHz ≤ f ≤ 3372 MHz 3551 MHz ≤ f ≤ 4186 MHz 5214 MHz ≤ f ≤ 5849 MHz 6028 MHz ≤ f ≤ 6698 MHz 7951 MHz ≤ f ≤ 9186 MHz 9614 MHz ≤ f ≤ 10849 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_28A_n3A Configuration |  |  |  |
| 214 MHz ≤ f ≤ 379 MHz 962 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1082 MHz 2413 MHz ≤ f ≤ 2533 MHz 2672 MHz ≤ f ≤ 2867 MHz 3116 MHz ≤ f ≤ 3281 MHz 4123 MHz ≤ f ≤ 4318 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_30A_n5A Configuration |  |  |  |
| 607 MHz ≤ f ≤ 667 MHz | -36 dBm | 100 kHz |  |
| 1456 MHz ≤ f ≤ 1491 MHz 3129 MHz ≤ f ≤ 3164 MHz 3761 MHz ≤ f ≤ 3806 MHz 3953 MHz ≤ f ≤ 4013 MHz 5434 MHz ≤ f ≤ 5479 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_39A_n41A Configuration |  |  |  |
| 576 MHz ≤ f < 810 MHz | -36 dBm | 100 kHz |  |
| 1070MHz ≤ f ≤ 1344 MHz 3072MHz ≤ f ≤ 3500 MHz 4376MHz ≤ f ≤ 4610 MHz 6256MHz ≤ f ≤ 6530 MHz 6872MHz ≤ f ≤ 7300 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_40A_n41A Configuration |  |  |  |
| 96 MHz ≤ f < 390 MHz | -36 dBm | 100 kHz |  |
| 1910 MHz ≤ f ≤ 2304 MHz 2592 MHz ≤ f ≤ 3080 MHz 4796 MHz ≤ f ≤ 5090 MHz 7096 MHz ≤ f ≤ 7780 MHz | -30 dBm | 1 MHz |  |

<End of Changes>