3GPP TSG-RAN5 Meeting #92-e	R5- 216026

# Electronic Meeting, 16th - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5338 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Test Case 6.2.4A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SGS Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test Case 6.2.4A.4 is in WI, so need to complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of Test Case 6.2.3A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 6.2.4A.4 will be still incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-215146r2 |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2.4A.4	Additional Maximum Power Reduction (A-MPR) for CA (3UL CA)

6.2.4A.4.1	Test purpose

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the CA Power Class as specified in Table 6.2.2A.0.

6.2.4A.4.2	Test applicability

The requirements of this test apply in test case 6.6.3.3A.4 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for network signalled values CA_NS_04 to all types of E-UTRA UE release 13 and forward that support intra-band contiguous 3DL CA and 3UL CA.

The requirements of this test apply in test case 6.6.3.3A.4 Additional spurious emissions for CA (inter-band 3DL CA and 3UL CA) for network signalled values CA_NS_04 to all types of E-UTRA UE release 13 and forward that support inter-band 3DL CA and 3UL CA.

NOTE:	As a result TC 6.2.4A.4 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.4A.4.3	Minimum conformance requirements

For PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For intra-band carrier aggregation, the A-MPR shall be evaluated per Teval period as specified in table 6.2.4A-3 and given by the maximum value taken over the transmission(s) on all component carriers within that period; the maximum A-MPR over TREF is then applied for the entire TREF.

Table 6.2.4A-3: A-MPR evaluation Teval period

| TTI pattern | TREF | Teval |
| --- | --- | --- |
| Subframe | 1 subframe | 1 slot |
| Slot | 7 OS | Min(Tno_hopping, 7OS) |
| Sublot | 2 OS, 3OS | Min(Tno_hopping, 2OS/3OS) |

For combinations of intra-band and inter-band carrier aggregation with the UE configured for transmission on three serving cells (up to two contiguously aggregated carriers per band), the maximum output power reduction is specified as follows. For the band supporting one serving cell the maximum output power reduction specified in Table 6.2.4-1 is allowed according to the Network Signaling value indicated by the field additionalSprectrumEmission for the PCC and the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10 for the SCC. The value of additionalSpectrumEmissionSCell-r10 is equal to that of additionalSprectrumEmission configured on the SCC. MPR as specified in subclause 6.2.3A is allowed in addition. For the band supporting intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2.4A.1.3-1 is allowed for all serving cells of the applicable uplink CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10. Then clause 6.2.3A does not apply, i.e. the carrier aggregation MPR = 0dB, unless the value indicated is CA_NS_31. For uplink 64 QAM and 256 QAM, the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in Table 6.2.3A-1 and A-MPR requirements specified in Table 6.2.4A.1.3-1.

For the UE maximum output power modified by A-MPR specified in table 6.2.4A-1, the power limits specified in subclause 6.2.5A apply.

If the UE is configured to CA_41C or any uplink inter-band CA configuration containing CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmission on two component carriers for contiguously aggregated signals is specified in Table 6.2.4A.3-1A for UE power class 3.

Table 6.2.4A.3-1A: Contiguous Allocation A-MPR for CA_NS_04 (power class 3), Bandwidth Class D

| CA Bandwidth Class D | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK [dB] | A-MPR for 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- | --- | --- |
| 50 RB / 75 RB/ 100 RB | 0 – 64 and 161 – 224 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 65 – 160 | N/A | >161 | ≤3dB | ≤3.5dB |
| 50 RB / 100 RB / 100 RB | 0 – 72 and 178 – 249 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 73 – 177 | N/A | >178 | ≤3dB | ≤3.5dB |
| 75 RB / 75 RB / 100 RB | 0 – 72 and 178 – 249 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 73 – 177 | N/A | >178 | ≤3dB | ≤3.5dB |
| 75 RB / 100 RB / 100 RB | 0 – 78 and 197 – 274 | >0 | N/A | ≤3.5dB | ≤4dB |
|  | 79 - 196 | N/A | >197 | ≤2.5dB | ≤3dB |
| 100 RB / 100 RB / 100 RB | 0 – 96 and 204 – 300 | >0 | N/A | ≤3.5dB | ≤4dB |
|  | 97– 203 | N/A | >204 | ≤2.5dB | ≤3dB |

6.2.4A.4.4	Test description

6.2.4A.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2.4A.4.4.1-1, the power limits specified in subclause 6.2.4A.4.5-1 apply.

Table 6.2.4A.4.4.1-1: Additional Maximum Power Reduction (A-MPR) for intra-band contiguous CA

| CA Network Signalling value |  | Requirements (subclause) |  |  | Uplink CA Configuration |  | A-MPR [dB] (subclause) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_NS_04 |  | 6.6.2.2A.1, 6.6.3.3A.8 |  |  | CA_41C, CA_41D |  | 6.2.4A.4 |  |
| NOTE 1: Applicable for uplink CA configurations listed in Table 5.6A.1-1 for which none of the additional requirements in subclauses 6.6.2.2A or 6.6.3.3A apply. NOTE 2: The index of the sequence CA_NS corresponds to the value of additionalSpectrumEmissionSCell-r10. |  |  |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.2.4A.4.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4A.4.4.1-2: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg DL Allocation |  |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCC1 NRB | SCC2 NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC1 & SCC2 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 75 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 |  |
| 2 | 100 | 75 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 |  |
| 3 | 100 | 75 | 50 |  | QPSK | 225 | P_100@0 | S_75@0 | S_50@0 |  |
| 4 | 100 | 75 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 |  |
| 5 | 100 | 75 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 |  |
| 6 | 100 | 75 | 50 |  | 16QAM | 225 | P_100@0 | S_75@0 | S_50@0 |  |
| 7 | 100 | 75 | 50 |  | QPSK | 3 | P_1@0 | S_1@74 | S_1@49 |  |
| 8 | 100 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 |  |
| 9 | 100 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 |  |
| 10 | 100 | 100 | 50 |  | QPSK | 250 | P_100@0 | S_100@0 | S_50@0 |  |
| 11 | 100 | 100 | 50 |  | 16QAM | 12 | P_18@0 | S_0@0 | S_0@0 |  |
| 12 | 100 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 |  |
| 13 | 100 | 100 | 50 |  | 16QAM | 250 | P_100@0 | S_100@0 | S_50@0 |  |
| 14 | 100 | 100 | 50 |  | QPSK | 3 | P_1@0 | S_1@99 | S_1@49 |  |
| 15 | 100 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 |  |
| 16 | 100 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 |  |
| 17 | 100 | 75 | 75 |  | QPSK | 250 | P_100@0 | S_75@0 | S_75@0 |  |
| 18 | 100 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 |  |
| 19 | 100 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 |  |
| 20 | 100 | 75 | 75 |  | 16QAM | 250 | P_100@0 | S_75@0 | S_75@0 |  |
| 21 | 100 | 75 | 75 |  | QPSK | 3 | P_1@0 | S_1@74 | S_1@74 |  |
| 22 | 100 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 |  |
| 23 | 100 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 |  |
| 24 | 100 | 100 | 75 |  | QPSK | 275 | P_100@0 | S_100@0 | S_75@0 |  |
| 25 | 100 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 |  |
| 26 | 100 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 |  |
| 27 | 100 | 100 | 75 |  | 16QAM | 275 | P_100@0 | S_100@0 | S_75@0 |  |
| 28 | 100 | 100 | 75 |  | QPSK | 3 | P_1@0 | S_1@74 | S_1@74 |  |
| 29 | 100 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 |  |
| 30 | 100 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 |  |
| 31 | 100 | 100 | 100 |  | QPSK | 300 | P_100@0 | S_100@0 | S_100@0 |  |
| 32 | 100 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 |  |
| 33 | 100 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 |  |
| 34 | 100 | 100 | 100 |  | 16QAM | 300 | P_100@0 | S_100@0 | S_100@0 |  |
| 35 | 100 | 100 | 100 |  | QPSK | 3 | P_1@0 | S_1@99 | S_1@99 |  |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS and interfering sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.4A.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4A.4.4.3.

6.2.4A.4.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.2.3A.4.4.3.

3.	SS activates SCC by sending the MAC-CE according to TS 36.321 [13] clauses 5.13 and 6.1.3.8. Wait for at least 2 seconds as per TS 36.133 [4] clause 8.3.3.2.

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.2.4A.4.4.1-1on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test.

6.2.4A.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.2.4A.4.5	Test Requirements

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4A.4.5-1. For the UE maximum output power modified by A-MPR specified in sub-clause 6.2.4A.2.3, the power limits specified in Table 6.2.5A.3.3-1 apply.

Table 6.2.4A.4.5-1: CA UE Power Class test requirements (intra-band continuous),

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7/-7.2 |
| 2 |  |  |  |  | 23 | +2.7/-4.7 |
| 3 |  |  |  |  | 23 | +2.7/-7.7 |
| 4 |  |  |  |  | 23 | +2.7/-7.2 |
| 5 |  |  |  |  | 23 | +2.7/-5.7 |
| 6 |  |  |  |  | 23 | +2.7/-7.7 |
| 7 |  |  |  |  | 23 | +2.7/-7.2 |
| 8 |  |  |  |  | 23 | +2.7/-7.2 |
| 9 |  |  |  |  | 23 | +2.7/-4.7 |
| 10 |  |  |  |  | 23 | +2.7/-7.7 |
| 11 |  |  |  |  | 23 | +2.7/-7.2 |
| 12 |  |  |  |  | 23 | +2.7/-5.7 |
| 13 |  |  |  |  | 23 | +2.7/-7.7 |
| 14 |  |  |  |  | 23 | +2.7/-7.2 |
| 15 |  |  |  |  | 23 | +2.7/-7.2 |
| 16 |  |  |  |  | 23 | +2.7/-4.7 |
| 17 |  |  |  |  | 23 | +2.7/-7.7 |
| 18 |  |  |  |  | 23 | +2.7/-7.2 |
| 19 |  |  |  |  | 23 | +2.7/-5.7 |
| 20 |  |  |  |  | 23 | +2.7/-7.7 |
| 21 |  |  |  |  | 23 | +2.7/-7.2 |
| 22 |  |  |  |  | 23 | +2.7/-6.2 |
| 23 |  |  |  |  | 23 | +2.7/-4.7 |
| 24 |  |  |  |  | 23 | +2.7/-7.7 |
| 25 |  |  |  |  | 23 | +2.7/-7.2 |
| 26 |  |  |  |  | 23 | +2.7/-5.7 |
| 27 |  |  |  |  | 23 | +2.7/-7.7 |
| 28 |  |  |  |  | 23 | +2.7/-8.2 |
| 29 |  |  |  |  | 23 | +2.7/-6.2 |
| 30 |  |  |  |  | 23 | +2.7/-4.7 |
| 31 |  |  |  |  | 23 | +2.7/-6.7 |
| 32 |  |  |  |  | 23 | +2.7/-7.2 |
| 33 |  |  |  |  | 23 | +2.7/-5.7 |
| 34 |  |  |  |  | 23 | +2.7/-7.7 |
| 35 |  |  |  |  | 23 | +2.7/-6.2 |
| NOTE 1: Lower limit is assuming ΔTC,c and ΔTIB,c are zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

<< End of changes >>