Source: 	oppo, Huawei

Title: 	Discussion on test point selection for Absolute power tolerance for CA

Document for:	Endorsement

Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation.

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Absolute power tolerance for CA.

Discussion

Absolute power tolerance for CA is defined in [2] where its scope is described as follows:

For intra-band contiguous carrier aggregation the absolute power control tolerance per component carrier is given in Table 6.3.4.2-1.

For inter-band CA:

No requirements unique to CA operation are defined.

For intra-band non-contiguous carrier aggregation the absolute power control tolerance per component carrier is given in Table 6.3.4.2-1.

Proposal 1: Absolute power tolerance test is no need for Inter-band CA.

The test point selection from the relevant NR single carrier (Table 6.3.4.2.4.1-1 in [1]) and LTE intra-band contiguous CA (Table 6.3.5A.1.1.4.1-1 in [3]) and LTE intra-band non-contiguous CA (Table 6.3.5A.1.3.4.1-1 in [3] ) test case are extracted as below:

Table 6.3.4.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Absolute power tolerance test case |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with High range test frequencies. |  |  |  |  |

Table 6.3.5A.1.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] clause [4.3.1] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 |  |  |
| 25 | 50 | N/A for this test | QPSK | 75 | P_25@0 | S_50@0 |  |  |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 |  |  |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_50@0 |  |  |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 |  |  |
| 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 |  |  |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 |  |  |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 |  |  |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

Table 6.3.5A.1.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Low Wgap, Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | QPSK | P_25@0 | QPSK | S_25@0 |
| 2 | 25 | 25 | 5 | N/A | N/A | QPSK | P_25@0 | QPSK | S_25@0 |
| 3 | 100 | 100 | 5 | N/A | N/A | QPSK | P_100@0 | QPSK | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. |  |  |  |  |  |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

Test Environment

According to general comment in [2] core spec, the power control cases shall be tested “under normal conditions” only:

The requirements on power control accuracy apply under normal conditions.

It is reasonable to use this principle for NR CA power control test cases.

Proposal 2: Define Test Environment as Normal Conditions (NC) for Absolute Power Tolerance for CA.

Test Subcarrier Spacing

Lowest and Highest SCS are tested in MPR for single carrier case in FR1, it’s reasonable to select the same SCS for CA test cases regardless of number of CCs.

Proposal 3: Select Lowest and Highest SCS supported for UE for Absolute Power Tolerance measurement for CA.

  - Test Frequencies

In LTE intra-band contiguous CA and NR single carrier, this test case is tested for mid frequency. We believe this is also reasonable for NR CA.

Low Wgap is defined for Tx intra-band non-contiguous CA test cases in LTE, Low Wgap and Maximum Wgap are both tested in LTE. However in NR, testing with max Wgap can represent the worst case and avoid costing double test time. So it’s reasonable to test with Maximum Wgap for Absolute Power Tolerance measurement for intra-band non-contiguous CA.

Proposal 4: Select Mid range for PCC and SCC as test frequencies for Absolute Power Tolerance measurement for intra-band contiguous CA. Test frequencies for each CC depend on specific CA configurations with max Wgap for Absolute Power Tolerance measurement for intra-band non-contiguous CA.Test Channel Bandwidth

In LTE CA, Lowest NRB_agg and Highest NRB_agg is selected. We believe this is also reasonable for NR CA.

Proposal 5: Test the Lowest NRB_agg and Highest NRB_agg as Channel Bandwidth for Absolute Power Tolerance measurement for CA.

Uplink Modulations and RB allocation

In NR single carrier, CP-OFDM is selected which is the worst case PAPR. QPSK is selected for both LTE CA and NR single carrier. We believe CP-OFDM QPSK is reasonable for NR CA.

In LTE CA, outer full RB is selected for both PCC and SCC. We also achieve consensus that higher BW should be worse. Therefore, we suggest to use outer full for PCC and SCC.

Proposal 6: Use CP-OFDM QPSK, Outer full for PCC and SCC for Absolute Power Tolerance measurement for CA.

Number of Test Points

Based on above proposals, the total test points of Absolute power tolerance for intra-band CA as below.

Table 2.6-1 Total test points for intra-band contiguous CA (2UL CA)

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 2 | 1 | 4 |

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for absolute power control for CA:

Proposal 1: Absolute power tolerance test is no need for Inter-band CA.

Proposal 2: Define Test Environment as Normal Conditions (NC) for Absolute Power Tolerance for CA.

Proposal 3: Select Lowest and Highest SCS supported for UE for Absolute Power Tolerance measurement for CA.

Proposal 4: Select Mid range for PCC and SCC as test frequencies for Absolute Power Tolerance measurement for intra-band contiguous CA. Test frequencies for each CC depend on specific CA configurations with max Wgap for Absolute Power Tolerance measurement for intra-band non-contiguous CA.

Proposal 5: Test the Lowest NRB_agg and Highest NRB_agg as Channel Bandwidth for Absolute Power Tolerance measurement for CA.

Proposal 6: Use CP-OFDM QPSK, Outer full for PCC and SCC for Absolute Power Tolerance measurement for CA.

References

[1] 3GPP TS 38.521-1 V17.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 17)

[2] 3GPP TS 38.101-1 v16.8.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[3] 3GPP TS 36.521-1 V16.9.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing (Release 16)