3GPP TSG-RAN5 Meeting #92-e	R5-216139

# Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.901-5 | CR | 0015 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RAN4 agreed contents for VRC scenarios to 37.901-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_UE_5GNR_App_Data_Perf |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 37.90-5 spec is currently missing outcome of feasibility study for introducing link adaptation scenarios |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added section 5.10 and its subsections inline with RAN4 LS and associated endorsed draftCR listed in other comments section. Removed the R4 Tdoc reference since spec drafting rules does not allow Tdoc references to be included in the TR contents. Editorial corrections to remove change over changes, correcting table format |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 LS Tdoc R4-2114569, RAN4 draftCR R4-2115743, R4-2115744, R4-2115745, R4-2115746 RAN4 request to upgrade TR37.901-5 to Rel17 is not allowed as per 3GPP process as FS_UE_5GNR_App_Data_Perf is a Rel16 SI, hence RAN4 endorsed draftCR contents are included in Rel16 TR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-215586r4 R5-216139 |  |  |  |  |  |  |  |  |

### 5.9.2	Test System Uncertainty and Test Tolerance for FR2 testing

<Start of changes>

## 5.10	Feasibility of Defining Link Adaptation Absolute Physical Layer Requirements

### 5.10.1	General

The purpose of this clause is to analyse whether it is feasible to define absolute physical layer throughput requirements under link adaptation using link-level simulation results based on the agreed set of simulation assumptions. As part of feasibility study, this clause will also conclude on test methodology which includes:

1. 	Alignment criteria for aligning the simulation results across companies and

2. 	Methodology to define the final requirements, if it is found to be feasible to define such requirements.

### 5.10.2	Test Methodology

#### 5.10.2.1	Simulation Alignment Criteria

The absolute throughput simulation results may vary among interested companies which is highly related to different implementation. In this section, the simulation alignment criteria for application layer data throughput performance is provided.

When SNR span (Gspan) can be reached for the T% of maximum throughput, it can be believed the simulation results are aligned among the interested companies, where,

  - -	Maximum throughput is defined with TBS corresponding to CQI index 15 with rank 2 for 2Rx/4Rx UE.

  - -	Gspan = Max (G) – Min (G), where G is the set of SNRs submitted by different companies to achieve T% of maximum throughput

  - -	Gspan is based on simulation results from interested companies. Candidate option is Gspan = [2.5] dB.

#### 5.10.2.2	Methodology for Requirements Definition

After aligning the simulation results based on simulation alignment criteria specified in clause 5.10.2.1, the absolute physical layer throughput requirements can be defined as T% of maximum throughput that needs to be achieved at (average SNR of impairments results to achieve T% of maximum throughput + X dB margin).

### 5.10.3	Simulation Assumptions

The simulation assumptions are captured in Table 5.10.3-1 for studying feasibility of defining link adaptation absolute physical layer requrements.

Table 5.10.3-1: Simulation assumptions for Absolute Physical Layer Throughput alignment with link adaptation

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

### 5.10.4	Simulation Results

In this section link level simulation results from different companies are collected. The simulation results are prepared based on simulation assumptions from Section 5.10.3.

Table 5.10.4-1 provides the information about the average SNR and SNR span of simulations results from different companies. The SNR span is measured on different percentage level on maximum achievable throughput. The maximum achievable throughput is calculated under assumption of Rank 2 transmission and MCS corresponding to the highest CQI (i.e. 15).

Table 5.10.4-1: SNR span in dB of simulation results

|  | FR1 FDD |  |  |  | FR1 TDD |  |  |  | FR2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 Rx UE |  | 4 Rx UE |  | 2 Rx UE |  | 4 Rx UE |  | 2 Rx UE |  |
| % of Max T-put | Avg | Span | Avg | Span | Avg | Span | Avg | Span | Avg | Span |
| 10 | 3.8 | 0.8 | 1.3 | 1.2 | 4.0 | 0.8 | 1.1 | 1.8 | 1.6 | 1.8 |
| 15 | 7.1 | 1.3 | 4.3 | 2.7 | 7.3 | 1.4 | 4.1 | 2.2 | 4.4 | 2.1 |
| 20 | 10.1 | 1.5 | 6.3 | 2.0 | 10.4 | 1.5 | 6.1 | 2.0 | 7.0 | 2.7 |
| 25 | 12.8 | 2.0 | 8.0 | 1.5 | 13.3 | 1.9 | 7.8 | 1.8 | 9.5 | 2.8 |
| 30 | 15.3 | 2.3 | 9.6 | 1.2 | 15.9 | 1.9 | 9.5 | 1.5 | 11.6 | 2.6 |
| 35 | 17.5 | 2.5 | 11.3 | 1.2 | 18.0 | 1.9 | 11.1 | 1.1 | 13.6 | 2.4 |
| 40 | 19.5 | 2.4 | 12.9 | 1.5 | 20.0 | 2.0 | 12.8 | 1.2 | 15.4 | 2.0 |
| 45 |  |  | 14.5 | 1.3 |  |  | 14.4 | 1.4 |  |  |
| 50 |  |  | 16.0 | 0.9 |  |  | 16.0 | 1.5 |  |  |
| 55 |  |  | 17.5 | 1.7 |  |  | 17.7 | 1.7 |  |  |
| 60 |  |  | 18.8 | 1.1 |  |  | 19.0 | 1.3 |  |  |

### 5.10.5	Summary

The feasibility of defining absolute physical layer throughput requirements under link adaptation has been evaluated. Based on the simulation alignment criteria defined in clause 5.10.2.1 and link level simulation results in clause 5.10.4 under the simulation assumptions listed in clause 5.10.3, it can be concluded that it is feasible to define the absolute physical layer throughput requirements under link adaptation, and corresponding requirements shall be defined and specified afterwards using the methodology defined in clause 5.10.2.2.

<End of changes>