3GPP TSG-RAN5 Meeting #92-e	R5-216161

# Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2038 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to reference configurations for IMS video call signaling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For the 2nd QoS rule defined in reference QoS rule#7, the given precendence value is incorrect. - In the definition of “Interworking_with_EPS” condition for reference QoS flows, it is currently mentioned as only applicable in PDU SESSION ESTABLISHMENT ACCEPT message. This could also apply if new QoS flows are added in PDU SESSION MODIFICATION COMMAND (see change 3) - For video call test cases 2 QoS flows are added, one for audio stream and one for video stream, mapped to 2 new DRBs. The masterCellGroup only specifies one DRB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct precedence value in reference QoS rule #7 to align with the comment column. - Expand condition description to include PDU SESSION MODIFICATION COMMAND - Modified the masterCellGroup to refer to CellGroupConfig-DRB(0,2) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail IMS video call test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.1, 4.8.2.1, 4.8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CRs R5s210877 and R5s210879 R5-215688->R5-216161 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.8.1	Radio configurations

#### –	RRCReconfiguration-DRB(n, m)

Table 4.8.1-1: RRCReconfiguration-DRB (n, m)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-DRB(n,m) |  |  |
| secondaryCellGroup | CellGroupConfig-DRB(n.m) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### <<<SKIPPED TEXT>>>

#### –	RRCReconfiguration-Video

Table 4.8.1-1D: RRCReconfiguration-Video

| Derivation Path: Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-Video |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-DRB(0,2) | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### <<<SKIPPED TEXT>>>

#### 4.8.2.1	Reference QoS rules

Table 4.8.2.1-1: Reference QoS rule #1

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filters |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0001’B | QFI 1 (Table 4.8.2.3-1) |  |

Table 4.8.2.1-2: Reference QoS rule #2

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0010’B | 2 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |

Table 4.8.2.1-3: Reference QoS rule #3

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-2 | Packet filter list #2 |  |
| QoS rule precedence | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0001’B | QFI 1 (Table 4.8.2.3-1) |  |

Table 4.8.2.1-4: Reference QoS rule #4

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0100’B | 4 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-3 | Packet filter list #3 |  |
| QoS rule precedence | ‘0000 00100’B | 4 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |

Table 4.8.2.1-4a: Reference QoS rule #4a

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 1111’B | 15 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-3a | Packet filter list #3a |  |
| QoS rule precedence | ‘0000 1111’B | 15 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0100’B | QFI 4 (Table 4.8.2.3-2a) |  |

Table 4.8.2.1-5: Reference QoS rule #5

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0101’B | 5 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-4 | Packet filter list #4 |  |
| QoS rule precedence | ‘0000 0101’B | 5 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0101’B | QFI 5 (Table 4.8.2.3-3) |  |

Table 4.8.2.1-6: Reference QoS rule #6

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0110’B | 6 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-5 | Packet filter list #5 |  |
| QoS rule precedence | ‘0000 0110’B | 6 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0110’B | QFI 6 (Table 4.8.2.3-4) |  |

Table 4.8.2.1-7: Reference QoS rule #7

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  | IMS_VOICE |
| QoS rule identifier | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-6 | Packet filter list #6 |  |
| QoS rule precedence | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0111’B | QFI 7 (Table 4.8.2.3-5) |  |
| QoS rule |  |  | IMS_VIDEO |
| QoS rule identifier | ‘0000 0100’B | 4 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-7 | Packet filter list #7 |  |
| QoS rule precedence | ‘0000 0010’B | 2 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 1000’B | QFI 8 (Table 4.8.2.3-6) |  |

| Condition | Explanation |
| --- | --- |
| IMS_VOICE | If this QoS rule is used to setup an IMS voice session |
| IMS_VIDEO | If this QoS rule is used to setup an IMS video session |

Table 4.8.2.1-8: Reference QoS rule #8

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0111’B | 7 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 1001’B | QFI 9 (Table 4.8.2.3-7) |  |

<UNCHANGED TEXT SKIPPED>

#### 4.8.2.3	Reference QoS flow descriptions

Table 4.8.2.3-1: Reference QoS flow #1

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0001’B | QFI 1 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 1001’B | 5QI 9 |  |
| EPS bearer identity | Any not yet assigned value different to '5' |  | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-2: Reference QoS flow #2

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0010’B | QFI 2 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0101’B | 5QI 5 |  |
| EPS bearer identity | ‘0101 0000’B | EBI 5 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-2a: Reference QoS flow #2a

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0100’B | QFI 4 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-3: Reference QoS flow #3

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0101’B | QFI 5 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-4: Reference QoS flow #4

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0110’B | QFI 6 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-5: Reference QoS flow #5

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0111’B | QFI 7 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0101’B | 5 parameter |  |
| Number of parameters | ’00 0110’B | 6 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0001’B | 5QI 1 |  |
| GFBR uplink | ‘040002’H | 128 Kbps |  |
| GFBR downlink | ‘040002’H | 128 Kbps |  |
| MFBR uplink | ‘040005’H | 320 Kbps |  |
| MFBR downlink | ‘040005’H | 320 Kbps |  |
| EPS bearer identity | Any not yet assigned value different to '5' | EBI 6 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-6: Reference QoS flow #6

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1000’B | QFI 8 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0101’B | 5 parameter |  |
| Number of parameters | ’00 0110’B | 6 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0010’B | 5QI 2 |  |
| GFBR uplink | ‘020012’H | 72 Kbps |  |
| GFBR downlink | ‘020012’H | 72 Kbps |  |
| MFBR uplink | ‘030033’H | 816 Kbps |  |
| MFBR downlink | ‘030033’H | 816 Kbps |  |
| EPS bearer identity | Any not yet assigned value different to '5' | EBI 7 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-7: Reference QoS flow #7

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1001’B | QFI 9 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0101 0010’B | 5QI 82 |  |