3GPP TSG-RAN5 Meeting #92-e	R5-216170

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2424 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC 7.1.1.4.x test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Each test loop takes around 360msec to complete and for PRB 273 the test loop is around 15288 and takes around execution takes around 90 mins to complete. The time taken can be reduced if activation time is not used. In addition to this the UL TBS calculated exceed the DL TBS and the expectation is the PDU provided with the TBS calculated is to sent on a particular slot. The SS cannot fulfil the requirement as the data is more than the DL TBS configured and therefore it can only be sent segmented. It is proposed to rework the test cases based on sending the sending the DL data without activation time and ensure enough time is given for the UL Buffer to be fulfilled and schedule the grant based on SR. With this change the test execution takes around 120msec for one loop to complete and therefore test can be completed around 30mins - The time domain resource allocation of using SLIV 53 is used in as part of the default configuration for DL transmission for the preamble procedure and grant allocation (38.523-3). It is therefore proposed to use this for the preamble and during test procedure to test both sliv values - The current test specification expects the test to be run with the highest common UL/DL bandwidth defined in 38.508-1. 38.508-1 tables are now updated to include Rel-16 bandwidth combination. Taking n1 as an example for Rel-15 maximum is 20MHz and Rel-16 specification, the maximum is 50MHz and test cases will fail UEs not supporting 50MHz. In addition the FR2 bands have 400MHz as optional bandwidth and if UE don’t support it then test cannot be executed. - Lower PRBs and MCS 27 and 28, results in higher coding rate (>0.95) and therefore may cause CRC failure in UL decoding. - When testing 7.1.1.4.2.1 the TBsize goes upto 229576, but any TBsize greater than 144920 Is sent as 13 SDUs, each SDUs at higher TBS will exceed the maxmimum SDU limit of 1500 octets |  |  |  |  |  |  |  |  |
| Summary of change: |  | - UL TBS test cases are modified to remove the timing requirements and to provide grant based on SR. PDSCH-TimeDomainResourceAllocationList is added to configure SLIV 27 (S=0 L=14) to ensure that DL TBS is sufficient to send Data in one slot. - Swapped the order of startand symbol length, so that the SLIV53 is used for preamble transmissions - Clarified that the UL/DL bandwidth used shall comply mandatory bandwidth. - Added MCS 27 and PRB 5 to the combination as this combination results in high code rate and may cause CRC failures. - Extended the condition to 20 SDUs that would cover limit the size of each SDU to 1500 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test execution takes very long time to complete and a Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | TTCN CRs R5s210885 and R5s210884 submitted |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Outcome of R5-215134 with 3 revisions |  |  |  |  |  |  |  |  |

#### 7.1.1.4	Transport Size Selection

##### 7.1.1.4.1	DL-SCH Transport Block Size Selection

###### 7.1.1.4.1.0	Common parameters for DL-SCH Transport Block Size Selection

Table 7.1.1.4.1.0-1: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 86 | S=2, L=7 |  |
| } |  |  |  |
| } |  |  |  |

###### 7.1.1.4.1.1	DL-SCH Transport Block Size selection / DCI format 1_0

7.1.1.4.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE on PDCCH receives DCI format 1_0 indicating a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE decodes the received transport block of size correspondent as per Modulation Coding scheme, time domain resource allocation  and PRB's and forwards it to higher layers }

}

7.1.1.4.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.2.1, TS 38.214 clause 5.1.2.1, 5.1.2.2, 5.1.2.2.2, 5.1.3, 5.1.3.1 and 5.1.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.212, clause 7.3.1.2.1]

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or new-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment – bits

-	 is the size of the active DL bandwidth part in case DCI format 1_0 is monitored in the UE specific search space and satisfying

-	the total number of different DCI sizes monitored per slot is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI monitored per slot is no more than 3 for the cell

otherwise,  is the size of the initial DL bandwidth part.

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows:

-	Random Access Preamble index – 6 bits according to ra-PreambleIndex in Subclause 5.1.2 of [8, TS38.321]

-	UL/SUL indicator – 1 bit. If the value of the “Random Access Preamble index” is not all zeros and if the UE is configured with SUL in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved

-	SS/PBCH index – 6 bits. If the value of the “Random Access Preamble index” is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved.

-	PRACH Mask index – 4 bits. If the value of the “Random Access Preamble index” is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by “SS/PBCH index” for the PRACH transmission, according to Subclause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved

-	Reserved bits – 10 bits

Otherwise, all remaining fields are set as follows:

-	Time domain resource assignment – 4 bits as defined in Subclause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.1.2-33

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3 of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – 2 bits as defined in Subclause 9.1.3 of [5, TS38.213], as counter DAI

-	TPC command for scheduled PUCCH – 2 bits as defined in Subclause 7.2.1 of [5, TS38.213]

-	PUCCH resource indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213]

[TS 38.214, clause 5.1.2.1]

When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in sub-clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception.

Given the parameter values of the indexed row:

-	The slot allocated for the PDSCH is , where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and  and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and

-	The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PDSCH are determined from the start and length indicator SLIV:

if  then

else

where, and

-	The PDSCH mapping type is set to Type A or Type B as defined in sub-clause 7.4.1.1.2 of [4, TS 38.211] .

The UE shall consider the S and L combinations defined in table 5.1.2.1-1 as valid PDSCH allocations:

Table 5.1.2.1-1: Valid S and L combinations

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} (Note 1) | {3,…,14} | {3,…,14} | {0,1,2,3} (Note 1) | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,4,7} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |

[38.214 clause 5.1.2.2]

Two downlink resource allocation schemes, type 0 and type 1, are supported. The UE shall assume that when the scheduling grant is received with DCI format 1_0, then downlink resource allocation type 1 is used.

[38.214 clause 5.1.2.2.2]

In downlink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved or interleaved virtual resource blocks within the active bandwidth part of size  PRBs except for the case when DCI format 1_0 is decoded in any common search space in CORESET 0 in which case the initial bandwidth part of size  shall be used.

A downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

[TS 38.214, clause 5.1.3]

To determine the modulation order, target code rate, and transport block size(s) in the physical downlink shared channel, the UE shall first

-	read the 5-bit modulation and coding scheme field (IMCS) in the DCI to determine the modulation order (Qm) and target code rate (R) based on the procedure defined in Subclause 5.1.3.1, and

-	read redundancy version field (rv) in the DCI to determine the redundancy version..

and second

-	the UE shall use the number of layers (ʋ), the total number of allocated PRBs before rate matching (nPRB) to determine to the transport block size based on the procedure defined in Subclause 5.1.3.2.

The UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.95, where the effective channel code rate is defined as the number of downlink information bits (including CRC bits) divided by the number of physical channel bits on PDSCH. If the UE skips decoding, the physical layer indicates to higher layer that the transport block is not successfully decoded.

[TS 38.214, clause 5.1.3.1]

For the PDSCH scheduled by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI,

if the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with a DCI format 1_1 and the CRC is scrambled by C-RNTI or CS-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with new-RNTI, the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam64LowSE', and the PDSCH is scheduled with C-RNTI, and the PDSCH is assigned by a PDCCH in a UE-specific search space

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with new-RNTI, and the PDSCH is scheduled with new-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256', the PDSCH is scheduled with CS-RNTI, and the PDSCH is assigned by a PDCCH with DCI format 1_1

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with the higher layer parameter mcs-Table given by SPS-config set to 'qam64LowSE', and the PDSCH is scheduled with CS-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

End

The UE is not expected to decode a PDSCH scheduled with P-RNTI, RA-RNTI, SI-RNTI and Qm > 2

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

[TS 38.214, clause 5.1.3.2]

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block, otherwise the transport block is enabled. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

For the PDSCH assigned by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, or SI-RNTI, if Table 5.1.3.1-2 is used and ≤IMCS≤27, or a table other than Table 5.1.3.1-2 is used and ≤IMCS≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1 or as described for format 1_0 in Subclause 5.1.6.2, and  NohPRBis the overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig. If the xOverhead in PDSCH-ServingCellconfig is not configured (a value from 0, 6, 12, or 18), the  is set to 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI,  is assumed to be 0.

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE=NRE'*nPRB, where nPRB is the total number of allocated PRBs for the UE.

2)	Intermediate number of information bits (Ninfo) TBStemp)is obtained by TBStemp=NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the closest TBS that is not less than .

Table 5.1.3.2-2: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.1.1.3	Test description

7.1.1.4.1.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value) and Short_DCI condition is applied in NR Serving cell configuration.

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

7.1.1.4.1.1.3.2	Test procedure sequence

Table 7.1.1.4.1.1.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.1.1.3.2-2: Number of downlink PDCP SDUs and PDCP SDU size used as test data

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 136 ≤ TBS ≤12128 note 2 | 1 | 8*FLOOR((TBS – 128)/8) |
| 12129 ≤ TBS ≤24200 | 2 | 8*FLOOR((TBS – 200)/16) |
| 24201 ≤ TBS ≤ 36272 | 3 | 8*FLOOR((TBS – 272)/24) |
| 36273 ≤ TBS ≤48344 | 4 | 8*FLOOR((TBS – 344)/32) |
| 48345≤ TBS ≤60416 | 5 | 8*FLOOR((TBS – 416)/40) |
| 60417 ≤ TBS ≤ 72488 | 6 | 8*FLOOR((TBS –488)/48) |
| 72489 ≤ TBS ≤84560 | 7 | 8*FLOOR((TBS – 560)/56) |
| 84561 ≤ TBS ≤96632 | 8 | 8*FLOOR((TBS –632)/64) |
| 96633< TBS ≤108704 | 9 | 8*FLOOR((TBS –704)/72) |
| 10705 ≤ TBS ≤120776 | 10 | 8*FLOOR((TBS – 776)/80) |
| 120777≤ TBS ≤132848 | 11 | 8*FLOOR((TBS –848)/88) |
| 132849 ≤ TBS ≤ 144920 | 12 | 8*FLOOR((TBS – 920)/96) |
| 144921 ≤ TBS ≤ 156992 | 13 | 8*FLOOR((TBS – 992)/104) |
| 156993 ≤ TBS ≤ 169064 | 14 | 8*FLOOR((TBS – 1064)/112) |
| 169065 ≤ TBS ≤ 181136 | 15 | 8*FLOOR((TBS – 1136)/120) |
| 181137 ≤ TBS ≤ 193208 | 16 | 8*FLOOR((TBS – 1208)/128) |
| 193209 ≤ TBS ≤ 205280 | 17 | 8*FLOOR((TBS – 1280)/136) |
| 205281 ≤ TBS ≤ 217352 | 18 | 8*FLOOR((TBS – 1352)/144) |
| 217353 ≤ TBS ≤ 229424 | 19 | 8*FLOOR((TBS – 1424)/152) |
| TBS> 229424 | 20 | 8*FLOOR((TBS –1496)/160) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size is 24 bits with 18 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*24- N*24 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 136 bits. |  |  |

Table 7.1.1.4.1.1.3.2-3: Specific Parameters

| Parameter | Value | Comment |
| --- | --- | --- |
| number of layers (ʋ) | 1 |  |
| mcs-Table | qam64 |  |
| xoh-PDSCH | Not Present | Results in value 0(xoh0) |

Table 7.1.1.4.1.1.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of 1 to in BWP, time domain resource as per table 7.1.1.4.1.0-1 and from 0 to 28. NOTE: Skip the execution of steps for which the TBS size results in coding rate exceeding 0.95. | - | - | - | - |
| 1 | The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS is less than or equal to UE capability "Maximum number of DL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.1.1.3.2-1 and larger than or equal to 132 bits as specified in Table 7.1.1.4.1.1.3.2-2 | - | - | - | - |
| 2 | The SS creates one or more PDCP SDUs, depending on TBS, in accordance with Table 7.1.1.4.1.1.3.2-2. | - | - | - | - |
| 3 | The SS transmits the PDCP SDUs concatenated into a MAC PDU and indicates on PDCCH DCI Format 1_0 and values of S, L,and nPRB. | <-- | MAC PDU (NxPDCP SDUs) DCI: (DCI Format 1_0, S, L,and nPRB.) | - | - |
| 4 | At the reception of scheduling request the SS transmits UL Grant for transmitting loop back PDCP SDUs. | <-- | (UL Grant) | - | - |
| 5 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3? | --> | (NxPDCP SDUs) | 1 | P |

7.1.1.4.1.1.3.3	Specific message contents

None.

###### 7.1.1.4.1.2	Void

###### 7.1.1.4.1.3	DL-SCH transport block size selection / DCI format 1_1 / RA type 0/RA Type 1 / 2 Codewords enabled

7.1.1.4.1.3.1	Test Purpose (TP)

(1)

with { UE in  RRC_CONNECTED state and maxNrofCodeWordsScheduledByDCI set to 'n2' }

ensure that {

when { UE on PDCCH receives DCI format 1_1 indicating resource allocation type 0  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE decodes the received transport block of size correspondent as per Modulation Coding scheme, time domain resource allocation  and PRB's and forwards it to higher layers }

}

(2)

with { UE in  RRC_CONNECTED state and maxNrofCodeWordsScheduledByDCI set to 'n2' }

ensure that {

when { UE on PDCCH receives DCI format 1_1 indicating resource allocation type 1  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE decodes the received transport block of size correspondent as per Modulation Coding scheme, time domain resource allocation  and PRB's and forwards it to higher layers }

}

7.1.1.4.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.2.2, TS 38.214 clause 5.1.2.1, 5.1.2.2.1, 5.1.2.2.2, 5.1.3, 5.1.3.1 and 5.1.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.212, clause 7.3.1.2.2]

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or new-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Carrier indicator – 0 or 3 bits as defined in Subclause 10.1 of [5, TS38.213].

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL BWPs  configured by higher layers, excluding the initial DL bandwidth part. The bit width for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active DL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Subclause 5.1.2.2.1 of [6, TS38.214],

-	bits if only resource allocation type 1 is configured, or

-	 bits if both resource allocation type 0 and 1 are configured.

-	If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the LSBs provide the resource allocation as defined in Subclause 5.1.2.2.1 of [6, TS38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as defined in Subclause 5.1.2.2.2 of [6, TS38.214]

If “Bandwidth part indicator” field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bit width of the “Frequency domain resource assignment” field of the active bandwidth part is smaller than the bit width of the “Frequency domain resource assignment” field of the indicated bandwidth part.

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Subclause 5.1.2.1 of [6, TS38.214]. The bit width for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-AllocationList.

-	VRB-to-PRB mapping – 0 or 1 bit

-	0 bit if only resource allocation type 0 is configured;

-	1 bit according to Table 7.3.1.1.2-33 otherwise, only applicable to resource allocation type 1, as defined in Subclause 7.3.1.6  of [4, TS38.211].

-	PRB bundling size indicator – 0 bit if the higher layer parameter prb-BundlingType is not configured or is set to ‘static’, or 1 bit if the higher layer parameter prb-BundlingType is set to ‘dynamic’, according to Subclause 5.1.2.3 of [6, TS38.214].

-	Rate matching indicator – 0, 1, or 2 bits according to higher layer parameter rateMatchPattern.

-	ZP CSI-RS trigger – 0, 1, or 2 bits as defined in Subclause 5.1.4.2 of [6, TS38.214]. The bit width for this field is determined as bits, where  is the number of ZP CSI-RS resource sets in the higher layer parameterzp-CSI-RS-Resource.

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3.1 of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

For transport block 2 (only present if maxNrofCodeWordsScheduledByDCI equals 2

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3.1 of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

If “Bandwidth part indicator” field indicates a bandwidth part other than the active bandwidth part and the value of maxNrofCodeWordsScheduledByDCI for the indicated bandwidth part equals 2 and the value of maxNrofCodeWordsScheduledByDCI for the active bandwidth part equals 1, the UE assumes zeros are padded when interpreting the “Modulation and coding scheme”, “New data indicator”, and “Redundancy version” fields of transport block 2 according to Subclause 12 of [5, TS38.213], and the UE ignores the “Modulation and coding scheme”, “New data indicator”, and “Redundancy version” fields of transport block 2 for the indicated bandwidth part.

-	HARQ process number – 4 bits

-	Downlink assignment index – number of bits as defined in the following

-	4 bits if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	2 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 bits are the counter DAI;

-	0 bits otherwise.

-	TPC command for scheduled PUCCH – 2 bits as defined in Subclause 7.2.1 of [5, TS38.213]

-	PUCCH resource indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 0, 1, 2, or bits as defined in Subclause 9.2.3 of [5, TS38.213]. The bit width for this field is determined as bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK.

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bit width of this field equals , where  is the “Antenna ports” bit width derived according to dmrs-DownlinkForPDSCH-MappingTypeA and  is the “Antenna ports” bit width derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of  and .

-	Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI is not enabled; otherwise 3 bits as defined in Subclause 5.1.5 of [6, TS38.214].

If “Bandwidth part indicator” field indicates a bandwidth part other than the active bandwidth part and the “Transmission configuration indication” field is not present in the DCI format 1_1, the UE assumes tci-PresentInDCI  is not enabled for the indicated bandwidth part.

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with SUL in the cell; 3 bits for UEs configured SUL in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Subclause 6.1.1.2 of [6, TS 38.214].

-	CBG transmission information (CBGTI) – 0, 2, 4, 6, or 8 bits as defined in Subclause 5.1.7 of [6, TS38.214], determined by the higher layer parameters maxCodeBlockGroupsPerTransportBlock and Number-MCS-HARQ-DL-DCI for the PDSCH.

-	CBG flushing out information (CBGFI) – 0 or 1 bit as defined in Subclause 5.1.7 of [6, TS38.214], determined by higher layer parameter codeBlockGroupFlushIndicator.

-	DMRS sequence initialization – 1 bit if both scramblingID0 and scramblingID1 are configured in DMRS-DownlinkConfig for  selection defined in Subclause 7.4.1.1.1 of [4, TS38.211]; 0 bit otherwise.

[TS 38.214, clause 5.1.2.1]

When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in sub-clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception.

Given the parameter values of the indexed row:

-	The slot allocated for the PDSCH is , where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and  and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and

-	The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PDSCH are determined from the start and length indicator SLIV:

if  then

else

where, and

-	The PDSCH mapping type is set to Type A or Type B as defined in sub-clause 7.4.1.1.2 of [4, TS 38.211] .

The UE shall consider the S and L combinations defined in table 5.1.2.1-1 as valid PDSCH allocations:

Table 5.1.2.1-1: Valid S and L combinations

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} (Note 1) | {3,…,14} | {3,…,14} | {0,1,2,3} (Note 1) | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,4,7} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |
| Note 1: S = 3 is applicable only if dmrs-TypeA-Posiition = 3 |  |  |  |  |  |  |

[TS 38.214, clause 5.1.2.2.1]

In downlink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured for PDSCH and the size of the carrier bandwidth part as defined in Table 5.1.2.2.1-1.

Table 5.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

The total number of RBGs () for a downlink carrier bandwidth part i of size PRBs is given by , where

-	the size of the first RBG is ,

-	the size of last RBG is if   and P otherwise,

-	the size of all other RBGs is P.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency and starting at the lowest frequency of the carrier bandwidth part. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

[TS 38.214, clause 5.1.2.2.2]

In downlink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated localized or distributed virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 1_0 is decoded in the common search space in CORESET 0 in which case the initial bandwidth part of size  shall be used.

A downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

[TS 38.214, clause 5.1.3]

To determine the modulation order, target code rate, and transport block size(s) in the physical downlink shared channel, the UE shall first

-	read the 5-bit modulation and coding scheme field (IMCS) in the DCI to determine the modulation order (Qm) and target code rate (R) based on the procedure defined in Subclause 5.1.3.1, and

-	read redundancy version field (rv) in the DCI to determine the redundancy version.

and second

-	the UE shall use the number of layers (ʋ), the total number of allocated PRBs before rate matching (nPRB) to determine to the transport block size based on the procedure defined in Subclause 5.1.3.2.

The UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.95, where the effective channel code rate is defined as the number of downlink information bits (including CRC bits) divided by the number of physical channel bits on PDSCH. If the UE skips decoding, the physical layer indicates to higher layer that the transport block is not successfully decoded.

[TS 38.214, clause 5.1.3.1]

For the PDSCH scheduled by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI,

if the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with a DCI format 1_1 and the CRC is scrambled by C-RNTI or CS-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with new-RNTI, the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam64LowSE', and the PDSCH is scheduled with C-RNTI, and the PDSCH is assigned by a PDCCH in a UE-specific search space

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with new-RNTI, and the PDSCH is scheduled with new-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256', the PDSCH is scheduled with CS-RNTI, and the PDSCH is assigned by a PDCCH with DCI format 1_1

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with the higher layer parameter mcs-Table given by SPS-config set to 'qam64LowSE', and the PDSCH is scheduled with CS-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

End

The UE is not expected to decode a PDSCH scheduled with P-RNTI, RA-RNTI, SI-RNTI and Qm > 2

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

[TS 38.214, clause 5.1.3.2]

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block, otherwise the transport block is enabled. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

For the PDSCH assigned by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, or SI-RNTI, if Table 5.1.3.1-2 is used and ≤IMCS≤27, or a table other than Table 5.1.3.1-2 is used and ≤IMCS≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1 or as described for format 1_0 in Subclause 5.1.6.2, and  NohPRBis the overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig. If the xOverhead in PDSCH-ServingCellconfig is not configured (a value from 0, 6, 12, or 18), the  is set to 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI,  is assumed to be 0.

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE=NRE'*nPRB, where nPRB is the total number of allocated PRBs for the UE.

2)	Intermediate number of information bits (Ninfo) TBStemp)is obtained by TBStemp=NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the closest TBS that is not less than .

Table 5.1.3.2-2: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.1.3.3	Test description

7.1.1.4.1.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set  the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value).

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

7.1.1.4.1.3.3.2	Test procedure sequence

Table 7.1.1.4.1.3.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.1.3.3.2-2: Number of downlink PDCP SDUs and PDCP SDU size used as test data

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 192 ≤ TBS ≤12184 note 2 | 1 | 8*FLOOR((TBS – 184)/8) |
| 12185≤ TBS ≤24256 | 2 | 8*FLOOR((TBS – 256)/16) |
| 24257≤ TBS ≤ 36328 | 3 | 8*FLOOR((TBS – 328)/24) |
| 36329 ≤ TBS ≤48400 | 4 | 8*FLOOR((TBS –400)/32) |
| 48401≤ TBS ≤60472 | 5 | 8*FLOOR((TBS – 472)/40) |
| 60473 ≤ TBS ≤ 72544 | 6 | 8*FLOOR((TBS – 544)/48) |
| 72545≤ TBS ≤84616 | 7 | 8*FLOOR((TBS – 616)/56) |
| 84617 ≤ TBS ≤96688 | 8 | 8*FLOOR((TBS – 688)/64) |
| 96689< TBS ≤108760 | 9 | 8*FLOOR((TBS – 760)/72) |
| 108761 ≤ TBS ≤120832 | 10 | 8*FLOOR((TBS –832)/80) |
| 120833≤ TBS ≤132904 | 11 | 8*FLOOR((TBS – 904)/88) |
| 132905 ≤ TBS ≤ 144976 | 12 | 8*FLOOR((TBS – 976)/96) |
| TBS> 144976 | 13 | 8*FLOOR((TBS – 1048)/104) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU – 32 bit Additional RLC header with SO if one RLC SDU gets split in 2 TBS and 24 bit MAC header for this additional PDU) / N, where PDCP header size is 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size is 24 bits with 18 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). IF RLC SDU does not get split the 32 bits additional padding gets added instead RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*24- N*24– N*24 -112 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 192 bits. |  |  |

Table 7.1.1.4.1.3.3.2-2A: Bandwidth part Dependent Parameters for Resource allocation 0 with start of BWP assumed as 0

| = | Nominal RBG size P (Configuration1) | Size of last RBG | Allowed Values |
| --- | --- | --- | --- |
| 11 | 2 | 1 | All 1…11 |
| 18 | 2 | 2 | 2,4,6,8,10,12,16,18 |
| 24 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24 |
| 25 | 2 | 1 | All 1…25 |
| 31 | 2 | 1 | All 1…31 |
| 32 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32 |
| 38 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38 |
| 51 | 4 | 3 | 3,4,7,8,11,12,15,16,19,20,23,24,27,28,31,32,35,36,39,40,43,44,47,48,51 |
| 52 | 4 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52 |
| 65 | 4 | 1 | 1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,32,33,36,37,40,41,44,45,48,49, 52,53,56,57,60,61,64,65 |
| 66 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38,40,42,44,46,48,50,52, 54,56,58,60,62,64,66 |
| 79 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79 |
| 106 | 8 | 2 | 2,8,10,16,18,24,26,32,34,40,42,48,50,56,58,64,66,72,74,80,82,88,90,96, 92,104,106 |
| 107 | 8 | 3 | 3,8,11,16,19,24,27,32,35,40,43,48,51,56,59,64,67,72,75,80,83,88,91,96, 99,104,107 |
| 132 | 8 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52,56,60,64,68,72,76,80,84,88,92,96, 100,104, 108,112,116,120,124,128,132 |
| 133 | 8 | 5 | 5,8,13,16,21,24,29,32,37,40,45,48,53,56,61,64,69,72,77,80,85,88,93,96, 101,104, 109,112,117,120,125,128,133 |
| 135 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79,80,87,88,95,96, 103,104, 111,112,119,120,127,128,135 |
| 160 | 16 | 16 | 16,32,48,64,80,96,112,128,144,160 |
| 216 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,152,160, 168, 176,184,192,200,208,216 |
| 217 | 16 | 9 | 9,16,25,32,41,48,57,64,73,80,89,96,105,112,121,128,137,144,153,160, 169,176,185,192,201,208,217 |
| 264 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,160,168, 176,184,192,200,208,216,224,232,240,248,256,264 |
| 270 | 16 | 14 | 14,16,30,32,46,44,62,64,78,80,94,96,110,112, 126,128,142,144,158, 160,174, 176,190,192, 206,208,222,224,238,240, 254,256,270 |
| 273 | 16 | 1 | 1,16,17,32,33,48,49,64,65,80,81,96,97,112,113,128,129,144,145,160, 161,176,171, 192,193, 208,209, 224,225,240,241,256,257,272,273 |

Table 7.1.1.4.1.3.3.2-3: Specific Parameter

| Parameter | Value | Comments | Condition |
| --- | --- | --- | --- |
| number of layers (ʋ) | 1 |  |  |
| mcs-Table | qam64 |  |  |
| resourceAllocation | dynamicSwitch |  | pc_dynamicSwitchRA_Type0_1_PDSCH |
|  | resourceAllocationType0 |  | NOT pc_dynamicSwitchRA_Type0_1_PDSCH AND Steps 1-5 |
|  | resourceAllocationType1 |  | NOT pc_dynamicSwitchRA_Type0_1_PDSCH AND Steps 6-10 |
| maxNrofCodeWordsScheduledByDCI | n2 | both codewords enabled |  |
| NstartBWP | 0 |  |  |

Table 7.1.1.4.1.3.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of as per table 7.1.1.4.1.3.3.2-2A in BWP, time domain resource as per table 7.1.1.4.1.0-1 and from 0 to 28. NOTE: Skip the execution of steps for which the TBS size results in coding rate exceeding 0.95. | - | - | - | - |
| 1 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. The SS uses the same and TBS for both transport blocks: = = TBS 1= TBS 2= TBS | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS1 + TBS2 is less than or equal to UE capability "Maximum number of DL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.1.3.3.2-1 and larger than or equal to 192 bits as specified in Table 7.1.1.4.1.3.3.2-2. | - | - | - | - |
| 2 | SS creates one or more PDCP SDUs for transport block 1 and 2 depending on TBS1, and TBS2 in accordance with Table 7.1.1.4.1.3.3.2-2. | - | - | - | - |
| 3 | SS transmits the PDCP SDUs concatenated into a MAC PDU and indicates on PDCCH DCI Format 1_1 resource allocation 0 and values of S, L, , and nPRB. | <-- | Transport block 1: MAC PDU Transport block 2: MAC PDU DCI: (DCI Format 1_1, S, L,, and nPRB.) | - | - |
| 4 | At the reception of scheduling request the SS transmits UL Grant for transmitting loop back PDCP SDUs. | <-- | (UL Grant) | - | - |
| 5 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3? | --> | (NxPDCP SDUs) | 1 | P |
| - | EXCEPTION : Steps 5Aa1 to 5Aa2 are executed if NOT pc_dynamicSwitchRA_Type0_1_PDSCH | - | - | - | - |
| 5Aa1 | The SS transmits a NR RRCReconfiguration message including PDSCH-Config with IE resourceAllocation set to resourceAllocationType1 (Note 1) | <-- | RRCReconfiguration | - | - |
| 5Aa2 | The UE transmit a NR RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 6 to 10 are repeated for allowed values of 1 to in BWP, time domain resource as per table 7.1.1.4.1.0-1 and from 0 to 28. | - | - | - | - |
| 6 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. The SS uses the same and TBS for both transport blocks: = = TBS 1= TBS 2= TBS | - | - | - | - |
| - | EXCEPTION: Steps 7 to 10 are performed if TBS1 + TBS2 is less than or equal to UE capability "Maximum number of DL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.1.3.3.2-1 and larger than or equal to 192 bits as specified in Table 7.1.1.4.1.3.3.2-2. | - | - | - | - |
| 7 | SS creates one or more PDCP SDUs for transport block 1 and 2 depending on TBS1, and TBS2 in accordance with Table 7.1.1.4.1.3.3.2-2. | - | - | - | - |
| 8 | SS transmits the PDCP SDUs concatenated into a MAC PDU and indicates on PDCCH DCI Format 1_1 resource allocation 1 and values of S, L, , and nPRB. | <-- | Transport block 1: MAC PDU Transport block 2: MAC PDU DCI: (DCI Format 1_1, S, L,, and nPRB.) | - | - |
| 9 | At the reception of scheduling request the SS transmits UL Grant for transmitting loop back PDCP SDUs. | <-- | (UL Grant) | - | - |
| 10 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3? | --> | (NxPDCP SDUs) | 2 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.4.1.3.3.3	Specific message contents

None.

###### 7.1.1.4.1.4	DL-SCH transport block size selection / DCI format 1_1 / RA type 0/RA Type 1 / 2 Codewords enabled / 256QAM

7.1.1.4.1.4.1	Test Purpose (TP)

(1)

with { UE in  RRC_CONNECTED state,  maxNrofCodeWordsScheduledByDCI set to 'n2' and mcs-Table is set as ‘qam256‘ }

ensure that {

when { UE on PDCCH receives DCI format 1_1 indicating resource allocation type 0  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE decodes the received transport block of size correspondent as per Modulation Coding scheme, time domain resource allocation  and PRB's and forwards it to higher layers }

}

(2)

with { UE in  RRC_CONNECTED state,  maxNrofCodeWordsScheduledByDCI set to 'n2' and mcs-Table is set as ‘qam256‘ }

ensure that {

when { UE on PDCCH receives DCI format 1_1 indicating resource allocation type 1  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE decodes the received transport block of size correspondent as per Modulation Coding scheme, time domain resource allocation  and PRB's and forwards it to higher layers }

}

7.1.1.4.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.2.2, TS 38.214 clauses 5.1.2.1, 5.1.2.2.1, 5.1.2.2.2, 5.1.3, 5.1.3.1 and 5.1.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.212, clause 7.3.1.2.2]

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or new-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Carrier indicator – 0 or 3 bits as defined in Subclause 10.1 of [5, TS38.213].

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL BWPs  configured by higher layers, excluding the initial DL bandwidth part. The bit width for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active DL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Subclause 5.1.2.2.1 of [6, TS38.214],

-	bits if only resource allocation type 1 is configured, or

-	 bits if both resource allocation type 0 and 1 are configured.

-	If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the LSBs provide the resource allocation as defined in Subclause 5.1.2.2.1 of [6, TS38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as defined in Subclause 5.1.2.2.2 of [6, TS38.214]

If “Bandwidth part indicator” field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bit width of the “Frequency domain resource assignment” field of the active bandwidth part is smaller than the bit width of the “Frequency domain resource assignment” field of the indicated bandwidth part.

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Subclause 5.1.2.1 of [6, TS38.214]. The bit width for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-AllocationList.

-	VRB-to-PRB mapping – 0 or 1 bit

-	0 bit if only resource allocation type 0 is configured;

-	1 bit according to Table 7.3.1.1.2-33 otherwise, only applicable to resource allocation type 1, as defined in Subclause 7.3.1.6  of [4, TS38.211].

-	PRB bundling size indicator – 0 bit if the higher layer parameter prb-BundlingType is not configured or is set to ‘static’, or 1 bit if the higher layer parameter prb-BundlingType is set to ‘dynamic’, according to Subclause 5.1.2.3 of [6, TS38.214].

-	Rate matching indicator – 0, 1, or 2 bits according to higher layer parameter rateMatchPattern.

-	ZP CSI-RS trigger – 0, 1, or 2 bits as defined in Subclause 5.1.4.2 of [6, TS38.214]. The bit width for this field is determined as  bits, where  is the number of ZP CSI-RS resource sets in the higher layer parameterzp-CSI-RS-Resource .

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3.1 of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

For transport block 2 (only present if maxNrofCodeWordsScheduledByDCI equals 2

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3.1 of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

If “Bandwidth part indicator” field indicates a bandwidth part other than the active bandwidth part and the value of maxNrofCodeWordsScheduledByDCI for the indicated bandwidth part equals 2 and the value of maxNrofCodeWordsScheduledByDCI for the active bandwidth part equals 1, the UE assumes zeros are padded when interpreting the “Modulation and coding scheme”, “New data indicator”, and “Redundancy version” fields of transport block 2 according to Subclause 12 of [5, TS38.213], and the UE ignores the “Modulation and coding scheme”, “New data indicator”, and “Redundancy version” fields of transport block 2 for the indicated bandwidth part.

-	HARQ process number – 4 bits

-	Downlink assignment index – number of bits as defined in the following

-	4 bits if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	2 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 bits are the counter DAI;

-	0 bits otherwise.

-	TPC command for scheduled PUCCH – 2 bits as defined in Subclause 7.2.1 of [5, TS38.213]

-	PUCCH resource indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Subclause 9.2.3 of [5, TS38.213]. The bit width for this field is determined as bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK.

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bit width of this field equals , where  is the “Antenna ports” bit width derived according to dmrs-DownlinkForPDSCH-MappingTypeA and  is the “Antenna ports” bit width derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of  and .

-	Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI is not enabled; otherwise 3 bits as defined in Subclause 5.1.5 of [6, TS38.214].

If “Bandwidth part indicator” field indicates a bandwidth part other than the active bandwidth part and the “Transmission configuration indication” field is not present in the DCI format 1_1, the UE assumes tci-PresentInDCI  is not enabled for the indicated bandwidth part.

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with SUL in the cell; 3 bits for UEs configured SUL in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Subclause 6.1.1.2 of [6, TS 38.214].

-	CBG transmission information (CBGTI) – 0, 2, 4, 6, or 8 bits as defined in Subclause 5.1.7 of [6, TS38.214], determined by the higher layer parameters maxCodeBlockGroupsPerTransportBlock and Number-MCS-HARQ-DL-DCI for the PDSCH.

-	CBG flushing out information (CBGFI) – 0 or 1 bit as defined in Subclause 5.1.7 of [6, TS38.214], determined by higher layer parameter codeBlockGroupFlushIndicator.

-	DMRS sequence initialization – 1 bit if both scramblingID0 and scramblingID1 are configured in DMRS-DownlinkConfig for  selection defined in Subclause 7.4.1.1.1 of [4, TS38.211]; 0 bit otherwise.

[TS 38.214, clause 5.1.2.1]

When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in sub-clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception.

Given the parameter values of the indexed row:

-	The slot allocated for the PDSCH is , where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and  and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and

-	The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PDSCH are determined from the start and length indicator SLIV:

if  then

else

where, and

-	The PDSCH mapping type is set to Type A or Type B as defined in sub-clause 7.4.1.1.2 of [4, TS 38.211] .

The UE shall consider the S and L combinations defined in table 5.1.2.1-1 as valid PDSCH allocations:

Table 5.1.2.1-1: Valid S and L combinations

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3 (Note 1)} | {3,…,14} | {3,…,14} | {0,1,2,3 (Note 1)} | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,4,7} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |
| Note 1: S = 3 is applicable only if dmrs-TypeA-Posiition = 3 |  |  |  |  |  |  |

[TS 38.214, clause 5.1.2.2.1]

In downlink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured for PDSCH and the size of the carrier bandwidth part as defined in Table 5.1.2.2.1-1.

Table 5.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

The total number of RBGs () for a downlink carrier bandwidth part i of size PRBs is given by , where

-	the size of the first RBG is ,

-	the size of last RBG is if   and P otherwise,

-	the size of all other RBGs is P.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency and starting at the lowest frequency of the carrier bandwidth part. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

[TS 38.214, clause 5.1.2.2.2]

In downlink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated localized or distributed virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 1_0 is decoded in the common search space in CORESET 0 in which case the initial bandwidth part of size  shall be used.

A downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

[TS 38.214, clause 5.1.3]

To determine the modulation order, target code rate, and transport block size(s) in the physical downlink shared channel, the UE shall first

-	read the 5-bit modulation and coding scheme field (IMCS) in the DCI to determine the modulation order (Qm) and target code rate (R) based on the procedure defined in Subclause 5.1.3.1, and

-	read redundancy version field (rv) in the DCI to determine the redundancy version..

and second

-	the UE shall use the number of layers (ʋ), the total number of allocated PRBs before rate matching (nPRB) to determine to the transport block size based on the procedure defined in Subclause 5.1.3.2.

The UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.95, where the effective channel code rate is defined as the number of downlink information bits (including CRC bits) divided by the number of physical channel bits on PDSCH. If the UE skips decoding, the physical layer indicates to higher layer that the transport block is not successfully decoded.

[TS 38.214, clause 5.1.3.1]

For the PDSCH scheduled by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI,

if the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with a DCI format 1_1 and the CRC is scrambled by C-RNTI or CS-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with new-RNTI, the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam64LowSE', and the PDSCH is scheduled with C-RNTI, and the PDSCH is assigned by a PDCCH in a UE-specific search space

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with new-RNTI, and the PDSCH is scheduled with new-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256', the PDSCH is scheduled with CS-RNTI, and the PDSCH is assigned by a PDCCH with DCI format 1_1

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with the higher layer parameter mcs-Table given by SPS-config set to 'qam64LowSE', and the PDSCH is scheduled with CS-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

End

The UE is not expected to decode a PDSCH scheduled with P-RNTI, RA-RNTI, SI-RNTI and Qm > 2

…

Table 5.1.3.1-2: MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

[TS 38.214, clause 5.1.3.2]

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block, otherwise the transport block is enabled. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

For the PDSCH assigned by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, or SI-RNTI, if Table 5.1.3.1-2 is used and ≤IMCS≤27, or a table other than Table 5.1.3.1-2 is used and ≤IMCS≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1 or as described for format 1_0 in Subclause 5.1.6.2, and  NohPRBis the overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig. If the xOverhead in PDSCH-ServingCellconfig is not configured (a value from 0, 6, 12, or 18), the  is set to 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI,  is assumed to be 0.

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE=NRE'*nPRB, where nPRB is the total number of allocated PRBs for the UE.

2)	Intermediate number of information bits (Ninfo) TBStemp)is obtained by TBStemp=NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the closest TBS that is not less than .

Table 5.1.3.2-2: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.1.4.3	Test description

7.1.1.4.1.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set  the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value).

Test frequency NRf1 is as specified in TS 38.508-1[4] clause 4.3.1 using the common highest UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1[4] clause 6.2.3.1.

7.1.1.4.1.4.3.2	Test procedure sequence

Table 7.1.1.4.1.4.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.1.4.3.2-2: Number of downlink PDCP SDUs and PDCP SDU size used as test data

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 192 ≤ TBS ≤12184 note 2 | 1 | 8*FLOOR((TBS – 184)/8) |
| 12185≤ TBS ≤24256 | 2 | 8*FLOOR((TBS – 256)/16) |
| 24257≤ TBS ≤ 36328 | 3 | 8*FLOOR((TBS – 328)/24) |
| 36329 ≤ TBS ≤48400 | 4 | 8*FLOOR((TBS –400)/32) |
| 48401≤ TBS ≤60472 | 5 | 8*FLOOR((TBS – 472)/40) |
| 60473 ≤ TBS ≤ 72544 | 6 | 8*FLOOR((TBS – 544)/48) |
| 72545≤ TBS ≤84616 | 7 | 8*FLOOR((TBS – 616)/56) |
| 84617 ≤ TBS ≤96688 | 8 | 8*FLOOR((TBS – 688)/64) |
| 96689< TBS ≤108760 | 9 | 8*FLOOR((TBS – 760)/72) |
| 108761 ≤ TBS ≤120832 | 10 | 8*FLOOR((TBS –832)/80) |
| 120833≤ TBS ≤132904 | 11 | 8*FLOOR((TBS – 904)/88) |
| 132905 ≤ TBS ≤ 144976 | 12 | 8*FLOOR((TBS – 976)/96) |
| 144785 ≤ TBS ≤ 157048 | 13 | 8*FLOOR((TBS – 1048)/104) |
| 157049 ≤ TBS ≤ 169120 | 14 | 8*FLOOR((TBS – 1120)/112) |
| 169121< TBS ≤ 181192 | 15 | 8*FLOOR((TBS – 1192)/120) |
| 181193 ≤ TBS ≤193264 | 16 | 8*FLOOR((TBS – 1264)/128) |
| 193337 ≤ TBS ≤ 205336 | 17 | 8*FLOOR((TBS – 1336)/136) |
| 205409 ≤ TBS ≤ 217408 | 18 | 8*FLOOR((TBS – 1408)/144) |
| TBS> 217408 | 19 | 8*FLOOR((TBS – 1480)/152) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU – 32 bit Additional RLC header with SO if one RLC SDU gets split in 2 TBS and 24 bit MAC header for this additional PDU) / N, where PDCP header size is 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size is 24 bits with 18 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). IF RLC SDU does not get split the 32 bits additional padding gets added instead RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*24- N*24– N*24 -112 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 192 bits. |  |  |

Table 7.1.1.4.1.4.3.2-2A: Bandwidth part Dependent Parameters for Resource allocation 0 with start of BWP assumed as 0

| = | Nominal RBG size P (Configuration1) | Size of last RBG | Allowed Values |
| --- | --- | --- | --- |
| 11 | 2 | 1 | All 1…11 |
| 18 | 2 | 2 | 2,4,6,8,10,12,16,18 |
| 24 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24 |
| 25 | 2 | 1 | All 1…25 |
| 31 | 2 | 1 | All 1…31 |
| 32 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32 |
| 38 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38 |
| 51 | 4 | 3 | 3,4,7,8,11,12,15,16,19,20,23,24,27,28,31,32,35,36,39,40,43,44,47,48,51 |
| 52 | 4 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52 |
| 65 | 4 | 1 | 1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,32,33,36,37,40,41,44,45,48,49, 52,53,56,57,60,61,64,65 |
| 66 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38,40,42,44,46,48,50,52, 54,56,58,60,62,64,66 |
| 79 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79 |
| 106 | 8 | 2 | 2,8,10,16,18,24,26,32,34,40,42,48,50,56,58,64,66,72,74,80,82,88,90,96, 92,104,106 |
| 107 | 8 | 3 | 3,8,11,16,19,24,27,32,35,40,43,48,51,56,59,64,67,72,75,80,83,88,91,96, 99,104,107 |
| 132 | 8 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52,56,60,64,68,72,76,80,84,88,92,96, 100,104, 108,112,116,120,124,128,132 |
| 133 | 8 | 5 | 5,8,13,16,21,24,29,32,37,40,45,48,53,56,61,64,69,72,77,80,85,88,93,96, 101,104, 109,112,117,120,125,128,133 |
| 135 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79,80,87,88,95,96, 103,104, 111,112,119,120,127,128,135 |
| 160 | 16 | 16 | 16,32,48,64,80,96,112,128,144,160 |
| 216 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,152,160,168, 176,184,192,200,208,216 |
| 217 | 16 | 9 | 9,16,25,32,41,48,57,64,73,80,89,96,105,112,121,128,137,144,153,160,169, 176,185,192,201,208,217 |
| 264 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,160,168, 176,184,192,200,208,216,224,232,240,248,256,264 |
| 270 | 16 | 14 | 14,16,30,32,46,44,62,64,78,80,94,96,110,112, 126,128,142,144,158,160, 174, 176,190,192, 206,208,222,224,238,240, 254,256,270 |
| 273 | 16 | 1 | 1,16,17,32,33,48,49,64,65,80,81,96,97,112,113,128,129,144,145,160, 161,176,171, 192,193, 208,209, 224,225,240,241,256,257,272,273 |

Table 7.1.1.4.1.4.3.2-3: Specific Parameter

| Parameter | Value | Comments | Condition |
| --- | --- | --- | --- |
| PDSCH mappingType | typeA |  |  |
| starting symbol S | 0 0r 3 to avoid clash with PDCCH symbols |  |  |
| number of consecutive symbols L | 3..14-S |  |  |
| k0 | 0 or 1 (if S=0) |  |  |
| number of layers (ʋ) | 1 |  |  |
| mcs-Table | qam256 |  |  |
| xoh-PDSCH | Not present | Results in value 0(xoh0) |  |
| dmrs-AdditionalPosition | pos0 | Results in 1 DMRS symbol per two carrier ()for Duration in symbols >=3 (TS 38.211 [24], table 7.4.1.1.2-3) |  |
| resourceAllocation | dynamicSwitch |  | pc_dynamicSwitchRA_Type0_1_PDSCH |
|  | resourceAllocationType0 |  | NOT pc_dynamicSwitchRA_Type0_1_PDSCH AND Steps 1-5 |
|  | resourceAllocationType1 |  | NOT pc_dynamicSwitchRA_Type0_1_PDSCH AND Steps 6-10 |
| maxNrofCodeWordsScheduledByDCI | n2 | both codewords enabled |  |
| rbg-Size | Not present | configuration 1 applicable |  |
| NstartBWP | 0 |  |  |

Table 7.1.1.4.1.4.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of as per Table 7.1.1.4.1.4.3.2-2A in BWP, time domain resource as per table 7.1.1.4.1.0-1 and from 0 to 27. NOTE: Skip the execution of steps for which the TBS size results in coding rate exceeding 0.95. | - | - | - | - |
| 1 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. The SS uses the same and TBS for both transport blocks: = = TBS 1= TBS 2= TBS | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS1 + TBS2 is less than or equal to UE capability "Maximum number of DL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.1.4.3.2-1 and larger than or equal to 192 bits as specified in Table 7.1.1.4.1.4.3.2-2. | - | - | - | - |
| 2 | SS creates one or more PDCP SDUs for transport block 1 and 2 depending on TBS1, and TBS2 in accordance with Table 7.1.1.4.1.4.3.2-2. | - | - | - | - |
| 3 | SS transmits the PDCP SDUs concatenated into a MAC PDU and indicates on PDCCH DCI Format 1_1 resource allocation 0 and values of S, L, , and nPRB. | <-- | Transport block 1: MAC PDU Transport block 2: MAC PDU DCI: (DCI Format 1_1, S, L,, and nPRB.) | - | - |
| 4 | At the reception of scheduling request the SS transmits UL Grant for transmitting loop back PDCP SDUs. | <-- | (UL Grant) | - | - |
| 5 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3? | --> | (NxPDCP SDUs) | 1 | P |
| - | EXCEPTION : Steps 5Aa1 to 5Aa2 are executed if NOT pc_dynamicSwitchRA_Type0_1_PDSCH | - | - | - | - |
| 5Aa1 | The SS transmits a NR RRCReconfiguration message including PDSCH-Config with IE resourceAllocation set to resourceAllocationType1 (Note 1) | <-- | RRCReconfiguration | - | - |
| 5Aa2 | The UE transmit a NR RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 6 to 10 are repeated for allowed values of 1 to in BWP, time domain resource length L 3 to 14-S and from 0 to 27. | - | - | - | - |
| 6 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. The SS uses the same and TBS for both transport blocks: = = TBS 1= TBS 2= TBS | - | - | - | - |
| - | EXCEPTION: Steps 7 to 10 are performed if TBS1 + TBS2 is less than or equal to UE capability "Maximum number of DL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.1.4.3.2-1 and larger than or equal to 192 bits as specified in Table 7.1.1.4.1.4.3.2-2 | - | - | - | - |
| 7 | SS creates one or more PDCP SDUs for transport block 1 and 2 depending on TBS1, and TBS2 in accordance with Table 7.1.1.4.1.4.3.2-2. | - | - | - | - |
| 8 | SS transmits the PDCP SDUs concatenated into a MAC PDU and indicates on PDCCH DCI Format 1_1 resource allocation 1 and values of S, L, , and nPRB. | <-- | Transport block 1: MAC PDU Transport block 2: MAC PDU DCI: (DCI Format 1_1, S, L,, and nPRB.) | - | - |
| 9 | At the reception of scheduling request the SS transmits UL Grant for transmitting loop back PDCP SDUs. | <-- | (UL Grant) | - | - |
| 10 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3? | --> | (NxPDCP SDUs) | 2 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.4.1.4.3.3	Specific message contents

None.

##### 7.1.1.4.2	UL-SCH Transport Block Size Selection

###### 7.1.1.4.2.0	Common parameters for UL-SCH Transport Block Size Selection

Table 7.1.1.4.2.0-1: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [4], table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entry |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 | FR1 |
| k2 | 2 |  | FR1 |
|  | 4 |  | FR2 |
| mappingType | typeB |  |  |
| startSymbolAndLength | 52 | Start symbol(S)=10, Length(L)=4 | FR1 |
| startSymbolAndLength | 42 | Start symbol(S)=0, Length(L)=4 | FR2 |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 | FR1 |
| k2 | 2 |  | FR1 |
|  | 4 |  | FR2 |
| mappingType | typeB |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.4.2.0-2: PUSCH-Config

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig | See Table 7.1.1.4.2.0-3 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.4.2.0-3: DMRS-UplinkConfig

Derivation Path: TS 38.508-1 [4], Table 4.6.3-51

Table 7.1.1.4.2.0-4: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508 [4], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceId | SchedulingRequestResourceId |  |  |
| schedulingRequestID | SchedulingRequestId |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl40 | 9 | With SCS = kHz15 results in repetition every 40 ms | SCS15 |
| sl80 | 9 | With SCS = kHz30 results in repetition every 40 ms | SCS30 |
| sl320 | 9 | With SCS = kHz120 results in repetition every 40 ms | SCS120 |
| } |  |  |  |
| resource | 6 | ID of the PUCCH resource as configured by PUCCH-Config (Table 4.6.3-84) |  |
| } |  |  |  |

Table 7.1.1.4.2.0-5: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | 1 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | S=0, L=14 |  |
| } |  |  |  |
| } |  |  |  |

###### 7.1.1.4.2.1	UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled

7.1.1.4.2.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has pending data for transmission and receives on PDCCH DCI format 0_0 indicating a resource block assignment correspondent to physical resource blocks , Time domain resource assignment and modulation and coding }

then { UE transmits MAC PDU on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

7.1.1.4.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.1.1, TS 38.214 clause 6.1.2.1, 6.1.2.2, 6.1.2.2.2, 6.1.4.1, 5.1.3.1, 6.1.4.2 and 5.1.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.212, clause 7.3.1.1.1]

DCI format 0_0 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or new-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Frequency domain resource assignment –  bits where

-	 is the size of the active UL bandwidth part in case DCI format 0_0 is monitored in the UE specific search space and satisfying

-	the total number of different DCI sizes monitored per slot is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI monitored per slot is no more than 3 for the cell

-	otherwise,  is the size of the initial UL bandwidth part.

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 4 bits as defined in Subclause 6.1.2.1 of [6, TS 38.214]

-	Frequency hopping flag – 1 bit.

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.3 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS 38.213]

-	Padding bits, if required.

-	UL/SUL indicator – 1 bit for UEs configured with SUL in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured;

-	If the UL/SUL indicator is not present in DCI format 0_0, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured.

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Frequency domain resource assignment –bits where

-	 is the size of the initial UL bandwidth part.

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where  if  and  otherwise

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 4 bits as defined in Subclause 6.1.2.1 of [6, TS 38.214]

-	Frequency hopping flag – 1 bit.

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.3 of [6, TS 38.214], using Table 5.1.3.1-1

-	New data indicator – 1 bit, reserved

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits, reserved

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS 38.213]

-	Padding bits, if required.

-	UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB

If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, zeros shall be appended to the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to padding is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bit width of the frequency domain resource allocation field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 equals to the size of the DCI format 1_0.

If DCI format 0_0 is monitored in UE specific search space but does not satisfy at least one of the following

-	the total number of different DCI sizes monitored per slot is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI monitored per slot is no more than 3 for the cell

and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, zeros shall be appended to the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

If DCI format 0_0 is monitored in UE specific search space but does not satisfy at least one of the following

-	the total number of different DCI sizes monitored per slot is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI monitored per slot is no more than 3 for the cell

and if the number of information bits in the DCI format 0_0 prior to padding is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bit width of the frequency domain resource allocation field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 equals to the size of the DCI format 1_0.

If DCI format 0_0 is monitored in UE specific search space and satisfies both of the following

-	the total number of different DCI sizes monitored per slot is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI monitored per slot is no more than 3 for the cell

and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in UE specific search space for scheduling the same serving cell, zeros shall be appended to the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

[TS 38.214, clause 6.1.2.1]

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report by a CSI request field on a DCI, the Time-domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the applied resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L,  and the PUSCH mapping type to be applied in the PUSCH transmission and K2 is determined based on the corresponding list entries of the higher layer parameter reportSlotConfig in CSI-ReportConfig for the triggered CSI Reporting Settings. The ith codepoint of K2 s determined as  where  is the ith codepoint of .

-	The slot where the UE shall transmit the PUSCH is determined by K2 as  where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and

-	The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	The PUSCH mapping type is set to Type A or Type B as defined in Subclause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} | {0,…,12} | {1,…,12} | {1,…,12} |

When the UE is configured with aggregationFactorUL > 1, the same symbol allocation is applied across the aggregationFactorUL consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the aggregationFactorUL consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version when aggregationFactorUL > 1

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

If the UE procedure for determining slot configuration, as defined in subclause 11.1 of [6, TS 38.213], determines symbols of a slot allocated for PUSCH as downlink symbols, the transmission on that slot is omitted for multi-slot PUSCH transmission.

[38.214 clause 6.1.2.2]

The UE shall determine the resource block assignment in frequency domain using the resource allocation field in the detected PDCCH DCI. Two uplink resource allocation schemes type 0 and type 1 are supported. Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. Uplink resource allocation scheme type 1 is supported for PUSCH for both cases when transform precoding is enabled or disabled.

If the scheduling DCI is configured to indicate the uplink resource allocation type as part of the Frequency domain resource assignment field by setting a higher layer parameter resourceAllocation in pusch-Config to ‘dynamicswitch’, the UE shall use uplink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the uplink frequency resource allocation type as defined by the higher layer parameter resourceAllocation.

The UE shall assume that when the scheduling PDCCH is received with DCI format 0_0, then uplink resource allocation type 1 is used.

If a bandwidth part indicator field is not configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI, except for the case when DCI format 0_0 is decoded in any PDCCH common search space in CORESET 0 in which case the initial bandwidth part shall be used. The UE shall upon detection of PDCCH intended for the UE determine first the uplink bandwidth part and then the resource allocation within the bandwidth part.

[38.214 clause 6.1.2.2.2]

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in the Type0-PDCCH common search space in CORESET 0 in which case the initial bandwidth part of size  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

[TS 38.214, clause 6.1.4.1]

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, or SP-CSI-RNTI, the transform precoding is enabled if transformPrecoder in PUSCH-Config is set to 'enabled', or if transformPrecoder in PUSCH-Config is not configured and msg3-transformPrecoding in rach-ConfigCommon is set to 'enabled'; otherwise the transform precoding is disabled.

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by CS-RNTI, or the PUSCH with configured grant using CS-RNTI, the transform precoding is enabled if transformPrecoder in ConfiguredGrantConfig is set to 'enabled'; otherwise the transform precoding is disabled.

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI, or SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI,

if transformPrecoder is disabled for this PUSCH transmission

-	if mcs-Table in PUSCH-Config is set to 'qam256', and PUSCH is scheduled with C-RNTI or SP-CSI-RNTI, and PUSCH is assigned by DCI format 0_1,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with new-RNTI, mcs-Table in PUSCH-Config is set to 'qam64LowSE', the PUSCH is scheduled with C-RNTI, or SP-CSI-RNTI, and the PUSCH is assigned by a PDCCH in a UE-specific search space,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with new-RNTI, and the PUSCH is scheduled with new-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in ConfiguredGrantConfig is set to 'qam256', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in ConfiguredGrantConfig is set to 'qam64LowSE', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

[TS 38.214, clause 5.1.3.1]

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

[TS 38.214, clause 6.1.4.2]

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, or SP-CSI-RNTI.

if

-	and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled and , the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  Nsymbslotis the number of symbols of the PUSCH allocation within the slot,  is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 0_1 or as described for DCI format 0_0 in Subclause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 0, 6, 12, or 18), the  is assumed to be 0. For MSG3 transmission the  is always set to 0..

-	A UE determines the total number of REs allocated for PUSCH  by where  is the total number of allocated PRBs for the UE.

-	Next, proceed with steps 2-5 as defined in Subclause 5.1.3.2

else if

-	 and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

[TS 38.214, clause 5.1.3.2]

2	Intermediate number of information bits (Ninfo) TBStemp)is obtained by TBStemp=NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the closest TBS that is not less than .

Table 5.1.3.2-2: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.2.1.3	Test description

7.1.1.4.2.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value) and Short_DCI condition is applied in NR Serving cell configuration.

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

7.1.1.4.2.1.3.2	Test procedure sequence

Table 7.1.1.4.2.1.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.2.1.3.2-2: Number of uplink PDCP SDUs and PDCP SDU size used as test data

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 136 ≤ TBS ≤12128 note 2 | 1 | 8*FLOOR((TBS – 128)/8) |
| 12129 ≤ TBS ≤24200 | 2 | 8*FLOOR((TBS – 200)/16) |
| 24201 ≤ TBS ≤ 36272 | 3 | 8*FLOOR((TBS – 272)/24) |
| 36273 ≤ TBS ≤48344 | 4 | 8*FLOOR((TBS – 344)/32) |
| 48345≤ TBS ≤60416 | 5 | 8*FLOOR((TBS – 416)/40) |
| 60417 ≤ TBS ≤ 72488 | 6 | 8*FLOOR((TBS –488)/48) |
| 72489 ≤ TBS ≤84560 | 7 | 8*FLOOR((TBS – 560)/56) |
| 84561 ≤ TBS ≤96632 | 8 | 8*FLOOR((TBS –632)/64) |
| 96633< TBS ≤108704 | 9 | 8*FLOOR((TBS –704)/72) |
| 10705 ≤ TBS ≤120776 | 10 | 8*FLOOR((TBS – 776)/80) |
| 120777≤ TBS ≤132848 | 11 | 8*FLOOR((TBS –848)/88) |
| 132849 ≤ TBS ≤ 144920 | 12 | 8*FLOOR((TBS – 920)/96) |
| 144921 ≤ TBS ≤ 156992 | 13 | 8*FLOOR((TBS – 992)/104) |
| 156993 ≤ TBS ≤ 169064 | 14 | 8*FLOOR((TBS – 1064)/112) |
| 169065 ≤ TBS ≤ 181136 | 15 | 8*FLOOR((TBS – 1136)/120) |
| 181137 ≤ TBS ≤ 193208 | 16 | 8*FLOOR((TBS – 1208)/128) |
| 193209 ≤ TBS ≤ 205280 | 17 | 8*FLOOR((TBS – 1280)/136) |
| 205281 ≤ TBS ≤ 217352 | 18 | 8*FLOOR((TBS – 1352)/144) |
| 217353 ≤ TBS ≤ 229424 | 19 | 8*FLOOR((TBS – 1424)/152) |
| TBS> 229424 | 20 | 8*FLOOR((TBS – 1496)/160) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size is 24 bits with 18 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*24- N*24 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 136 bits. |  |  |

Table 7.1.1.4.2.1.3.2-3: Specific Parameters

| Parameter | Value | Comment |
| --- | --- | --- |
| number of layers (ʋ) | 1 |  |
| mcs-Table | qam64 |  |

Table 7.1.1.4.2.1.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of 1 to in BWP, time domain resource as per Table 7.1.1.4.2.0-1 and from 0 to 28. | - | - | - | - |
| 1 | The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS is less than or equal to UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.1.3.2-1 and larger than or equal to 136 bits as specified in Table 7.1.1.4.2.1.3.2-2. Skip the execution of steps 2 to 5 for which the TBS size equal to 3824 or 3840. (Note 2) Skip the execution of steps for > 27 and < 5. (Note 1) | - | - | - | - |
| 2 | The SS creates one or more PDCP SDUs, depending on TBS, in accordance with Table 7.1.1.4.2.1.3.2-2. | - | - | - | - |
| 3 | The SS transmits all PDCP SDUs (NSDUs) as created in step 2 in a MAC PDU. | <-- | MAC PDU (NxPDCP SDUs) | - | - |
| 4 | After the reception of 2 Scheduling Request, , SS transmits UL Grant DCI 0_0, and values of S, L,and nPRB.. | <-- | (UL Grant) (DCI Format 0_0, S, L,and nPRB.) | - | - |
| 5 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 4? | --> | MAC PDU (N x PDCP SDU) | 1 | P |
| Note 1: For > 27 and < 5, the combination results in higher coding rate and therefore leading to CRC errors in decoding UL data. Note 2: There is ambiguity of TBS calculation when 3824.0 < Ninfo < 3825.0 in clause 5.1.3.2 of TS 38.214 [15]. |  |  |  |  |  |

7.1.1.4.2.1.3.3	Specific message contents

None.

###### 7.1.1.4.2.2	Void

###### 7.1.1.4.2.3	UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled

7.1.1.4.2.3.1	Test Purpose (TP)

(1)

with { UE in  RRC_CONNECTED state }

ensure that {

when { UE has pending data for transmission and receives DCI format 0_1 indicating resource allocation type 0  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE transmits MAC PDU's on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

(2)

with { UE in  RRC_CONNECTED state }

ensure that {

when { UE has pending data for transmission and receives DCI format 0_1 indicating resource allocation type 1  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE transmits MAC PDU's on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

7.1.1.4.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.1.1, TS 38.214 clause 6.1.2.1, 6.1.2.2, 6.1.2.2.1, 6.1.2.2.2, 6.1.4.1, 5.1.3.1, 6.1.4.2 and 5.1.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.212, clause 7.3.1.1.2]

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or new-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator – 0 or 3 bits, as defined in Subclause 10.1 of [5, TS38.213].

-	UL/SUL indicator – 0 bit for UEs not configured with SUL in the cell or UEs configured with SUL in the cell but only PUCCH carrier in the cell is configured for PUSCH transmission; 1 bit for UEs configured with SUL in the cell as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bit width for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active UL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Subclause 6.1.2.2.1 of [6, TS 38.214],

-	bits if only resource allocation type 1 is configured, or  bits if both resource allocation type 0 and 1 are configured.

-	If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the  LSBs provide the resource allocation as defined in Subclause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bit width of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bit width of the "Frequency domain resource assignment"  field of the indicated bandwidth part.

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Subclause 6.1.2.1 of [6, TS38.214]. The bit width for this field is determined as bits, where I the number of entries in the higher layer parameter pusch-AllocationList.

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if the higher layer parameter frequencyHopping is not configured;

-	1 bit according to Table 7.3.1.1.2-34 otherwise, only applicable to resource allocation type 1, as defined in Subclause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	1st downlink assignment index – 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook.

-	2nd downlink assignment index – 0 or 2 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks;

-	0 bit otherwise.

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS38.213]

-	SRS resource indicator – or  bits, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', and  is the maximum number of supported layers for the PUSCH.

-	 bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'nonCodeBook';

-	 bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook'.

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank, and codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank, and codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters maxRank and codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebookmaxRank and codebookSubset, and according to the values of higher layer parameters .

-	Antenna ports – number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if transformPrecoder=enabled, dmrs-Type=1, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-7, if transformPrecoder=enabled, dmrs-Type=1, and maxLength=2;

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transformPrecoder=disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transformPrecoder=disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transformPrecoder=disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transformPrecoder=disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bit width of this field equals , where  is the “Antenna ports” bit width derived according to dmrs-UplinkForPUSCH-MappingTypeA and  is the “Antenna ports” bit width derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  and .

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with SUL in the cell; 3 bits for UEs configured SUL in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Subclause 6.1.1.2 of [6, TS 38.214].

-	CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize.

-	CBG transmission information (CBGTI) – 0, 2, 4, 6, or 8 bits determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for PUSCH.

-	PTRS-DMRS association – number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured and transformPrecoder=disabled, or if transformPrecoder=enabled, or if maxRank=1;

-	2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field.

If “Bandwidth part indicator” field indicates a bandwidth part other than the active bandwidth part and the “PTRS-DMRS association” field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the “PTRS-DMRS association” field is not present for the indicated bandwidth part.betaOffsets = semiStatic

-	beta_offset indicator – 0 if the higher layer parameter ; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

-	DMRS sequence initialization – 0 if the higher layer parameter transformPrecoder=enabled; 1 bit if the higher layer parameter transformPrecoder=disabled and both scramblingID0 and scramblingID1 are configured in DMRS-UplinkConfig, for  selection defined in Subclause 6.4.1.1.1.1 of [4, TS 38.211].

-	UL-SCH indicator – 1 bit. A value of “1” indicates UL-SCH shall be transmitted on the PUSCH and a value of “0” indicates UL-SCH shall not be transmitted on the PUSCH.

For a UE configured with SUL in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_1 for the SUL is not equal to the number of information bits in format 0_1 for the non-SUL, zeros shall be appended to smaller format 0_1 until the payload size equals that of the larger format 0_1.

Table 7.3.1.1.2-1: Bandwidth part indicator

| Value of BWP indicator field | Bandwidth part |
| --- | --- |
| 2 bits |  |
| 00 | First bandwidth part configured by higher layers |
| 01 | Second bandwidth part configured by higher layers |
| 10 | Third bandwidth part configured by higher layers |
| 11 | Fourth bandwidth part configured by higher layers |

Table 7.3.1.1.2-2: Precoding information and number of layers, for 4 antenna ports, if transformPrecoder=disabled and maxRank = 2 or 3 or 4

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |
| 19 | 1 layer: TPMI=11 | 19 | 1 layer: TPMI=11 |  |  |
| 20 | 2 layers: TPMI=6 | 20 | 2 layers: TPMI=6 |  |  |
| … | … | … | … |  |  |
| 27 | 2 layers: TPMI=13 | 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 | 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 | 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 | 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 | 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layers: TPMI=12 |  |  |  |  |
| … | … |  |  |  |  |
| 47 | 1 layers: TPMI=27 |  |  |  |  |
| 48 | 2 layers: TPMI=14 |  |  |  |  |
| … | … |  |  |  |  |
| 55 | 2 layers: TPMI=21 |  |  |  |  |
| 56 | 3 layers: TPMI=3 |  |  |  |  |
| … | … |  |  |  |  |
| 59 | 3 layers: TPMI=6 |  |  |  |  |
| 60 | 4 layers: TPMI=3 |  |  |  |  |
| 61 | 4 layers: TPMI=4 |  |  |  |  |
| 62-63 | reserved |  |  |  |  |

Table 7.3.1.1.2-3: Precoding information and number of layers for 4 antenna ports, if transformPrecoder= enabled, or if transformPrecoder=disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |  |  |
| … | … | … | … |  |  |
| 11 | 1 layer: TPMI=11 | 11 | 1 layer: TPMI=11 |  |  |
| 12 | 1 layers: TPMI=12 | 12-15 | reserved |  |  |
| … | … |  |  |  |  |
| 27 | 1 layers: TPMI=27 |  |  |  |  |
| 28-31 | reserved |  |  |  |  |

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder=disabled and maxRank = 2

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 | 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 | 3 | reserved |
| 4 | 1 layer: TPMI=3 |  |  |
| 5 | 1 layer: TPMI=4 |  |  |
| 6 | 1 layer: TPMI=5 |  |  |
| 7 | 2 layers: TPMI=1 |  |  |
| 8 | 2 layers: TPMI=2 |  |  |
| 9-15 | reserved |  |  |

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder= enabled, or if transformPrecoder= disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |  |  |
| 3 | 1 layer: TPMI=3 |  |  |
| 4 | 1 layer: TPMI=4 |  |  |
| 5 | 1 layer: TPMI=5 |  |  |
| 6-7 | reserved |  |  |

...

Table 7.3.1.1.2-33: VRB-to-PRB mapping

| Bit field mapped to index | VRB-to-PRB mapping |
| --- | --- |
| 0 | Non-interleaved |
| 1 | Interleaved |

[TS 38.214, clause 6.1.2.1]

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report by a CSI request field on a DCI, the Time-domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the applied resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L,  and the PUSCH mapping type to be applied in the PUSCH transmission and K2 is determined based on the corresponding list entries of the higher layer parameter reportSlotConfig in CSI-ReportConfig for the triggered CSI Reporting Settings. The ith codepoint of K2 s determined as  where  is the ith codepoint of .

-	The slot where the UE shall transmit the PUSCH is determined by K2 as  where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and

-	The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	The PUSCH mapping type is set to Type A or Type B as defined in Subclause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} | {0,…,12} | {1,…,12} | {1,…,12} |

When the UE is configured with aggregationFactorUL > 1, the same symbol allocation is applied across the aggregationFactorUL consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the aggregationFactorUL consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version when aggregationFactorUL > 1

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

If the UE procedure for determining slot configuration, as defined in subclause 11.1 of [6, TS 38.213], determines symbols of a slot allocated for PUSCH as downlink symbols, the transmission on that slot is omitted for multi-slot PUSCH transmission.

[38.214 clause 6.1.2.2]

The UE shall determine the resource block assignment in frequency domain using the resource allocation field in the detected PDCCH DCI. Two uplink resource allocation schemes type 0 and type 1 are supported. Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. Uplink resource allocation scheme type 1 is supported for PUSCH for both cases when transform precoding is enabled or disabled.

If the scheduling DCI is configured to indicate the uplink resource allocation type as part of the Frequency domain resource assignment field by setting a higher layer parameter resourceAllocation in pusch-Config to ‘dynamicswitch’, the UE shall use uplink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the uplink frequency resource allocation type as defined by the higher layer parameter resourceAllocation.

The UE shall assume that when the scheduling PDCCH is received with DCI format 0_0, then uplink resource allocation type 1 is used.

If a bandwidth part indicator field is not configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI, except for the case when DCI format 0_0 is decoded in any PDCCH common search space in CORESET 0 in which case the initial bandwidth part shall be used. The UE shall upon detection of PDCCH intended for the UE determine first the uplink bandwidth part and then the resource allocation within the bandwidth part.

[38.214 clause 6.1.2.2.1]

In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Sizeconfigured for PUSCH and the size of the carrier bandwidth part as defined in Table 6.1.2.2.1-1.

Table 6.1.2.2.1-1: Nominal RBG size P

| Carrier Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

The total number of RBGs () for a uplink carrier bandwidth part i of sizePRBs is given by  where

-	the size of the first RBG is ,

-	the size of the last RBG is if and P otherwise.

-	the size of all other RBG is P.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency of the carrier bandwidth part and starting at the lowest frequency. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB of the bitmap. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

[38.214 clause 6.1.2.2.2]

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in the Type0-PDCCH common search space in CORESET 0 in which case the initial bandwidth part of size  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

[TS 38.214, clause 6.1.4.1]

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, or SP-CSI-RNTI, the transform precoding is enabled if transformPrecoder in PUSCH-Config is set to 'enabled', or if transformPrecoder in PUSCH-Config is not configured and msg3-transformPrecoding in rach-ConfigCommon is set to 'enabled'; otherwise the transform precoding is disabled.

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by CS-RNTI, or the PUSCH with configured grant using CS-RNTI, the transform precoding is enabled if transformPrecoder in ConfiguredGrantConfig is set to 'enabled'; otherwise the transform precoding is disabled.

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI, or SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI,

if transformPrecoder is disabled for this PUSCH transmission

-	if mcs-Table in PUSCH-Config is set to 'qam256', and PUSCH is scheduled with C-RNTI or SP-CSI-RNTI, and PUSCH is assigned by DCI format 0_1,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with new-RNTI, mcs-Table in PUSCH-Config is set to 'qam64LowSE', the PUSCH is scheduled with C-RNTI, or SP-CSI-RNTI, and the PUSCH is assigned by a PDCCH in a UE-specific search space,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with new-RNTI, and the PUSCH is scheduled with new-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in ConfiguredGrantConfig is set to 'qam256', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in ConfiguredGrantConfig is set to 'qam64LowSE', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

[TS 38.214, clause 5.1.3.1]

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

[TS 38.214, clause 6.1.4.2]

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, or SP-CSI-RNTI.

if

-	and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  Nsymbslotis the number of symbols of the PUSCH allocation within the slot,  is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 0_1 or as described for DCI format 0_0 in Subclause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 0, 6, 12, or 18), the  is assumed to be 0. For MSG3 transmission the  is always set to 0..

-	A UE determines the total number of REs allocated for PUSCH  by where  is the total number of allocated PRBs for the UE.

-	Next, proceed with steps 2-4 as defined in Subclause 5.1.3.2

else if

-	 and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

[TS 38.214, clause 5.1.3.2]

2)	Intermediate number of information bits (Ninfo) TBStemp)is obtained by TBStemp=NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the closest TBS that is not less than .

Table 5.1.3.2-2: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.2.3.3	Test description

7.1.1.4.2.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set  the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value).

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

7.1.1.4.2.3.3.2	Test procedure sequence

Table 7.1.1.4.2.3.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.2.3.3.2-2: Number of downlink PDCP SDUs and PDCP SDU size used as test data

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 136 ≤ TBS ≤12128 note 2 | 1 | 8*FLOOR((TBS – 128)/8) |
| 12129 ≤ TBS ≤24200 | 2 | 8*FLOOR((TBS – 200)/16) |
| 24201 ≤ TBS ≤ 36272 | 3 | 8*FLOOR((TBS – 272)/24) |
| 36273 ≤ TBS ≤48344 | 4 | 8*FLOOR((TBS – 344)/32) |
| 48345≤ TBS ≤60416 | 5 | 8*FLOOR((TBS – 416)/40) |
| 60417 ≤ TBS ≤ 72488 | 6 | 8*FLOOR((TBS –488)/48) |
| 72489 ≤ TBS ≤84560 | 7 | 8*FLOOR((TBS – 560)/56) |
| 84561 ≤ TBS ≤96632 | 8 | 8*FLOOR((TBS –632)/64) |
| 96633< TBS ≤108704 | 9 | 8*FLOOR((TBS –704)/72) |
| 10705 ≤ TBS ≤120776 | 10 | 8*FLOOR((TBS – 776)/80) |
| 120777≤ TBS ≤132848 | 11 | 8*FLOOR((TBS –848)/88) |
| 132849 ≤ TBS ≤ 144920 | 12 | 8*FLOOR((TBS – 920)/96) |
| 144921 ≤ TBS ≤ 156992 | 13 | 8*FLOOR((TBS – 992)/104) |
| 156993 ≤ TBS ≤ 169064 | 14 | 8*FLOOR((TBS – 1064)/112) |
| 169065 ≤ TBS ≤ 181136 | 15 | 8*FLOOR((TBS – 1136)/120) |
| 181137 ≤ TBS ≤ 193208 | 16 | 8*FLOOR((TBS – 1208)/128) |
| 193209 ≤ TBS ≤ 205280 | 17 | 8*FLOOR((TBS – 1280)/136) |
| 205281 ≤ TBS ≤ 217352 | 18 | 8*FLOOR((TBS – 1352)/144) |
| 217353 ≤ TBS ≤ 229424 | 19 | 8*FLOOR((TBS – 1424)/152) |
| TBS> 229424 | 20 | 8*FLOOR((TBS – 1496)/160) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size is 24 bits with 18 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*24- N*24 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 136 bits. |  |  |

Table 7.1.1.4.2.3.3.2-2A: Bandwidth part Dependent Parameters for Resource allocation 0 with start of BWP assumed as 0

| = | Nominal RBG size P (Configuration1) | Size of last RBG | Allowed Values |
| --- | --- | --- | --- |
| 11 | 2 | 1 | All 1…11 |
| 18 | 2 | 2 | 2,4,6,8,10,12,16,18 |
| 24 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24 |
| 25 | 2 | 1 | All 1…25 |
| 31 | 2 | 1 | All 1…31 |
| 32 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32 |
| 38 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38 |
| 51 | 4 | 3 | 3,4,7,8,11,12,15,16,19,20,23,24,27,28,31,32,35,36,39,40,43,44,47,48,51 |
| 52 | 4 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52 |
| 65 | 4 | 1 | 1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,32,33,36,37,40,41,44,45,48,49, 52,53,56,57,60,61,64,65 |
| 66 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38,40,42,44,46,48,50,52, 54,56,58,60,62,64,66 |
| 79 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79 |
| 106 | 8 | 2 | 2,8,10,16,18,24,26,32,34,40,42,48,50,56,58,64,66,72,74,80,82,88,90,96, 92,104,106 |
| 107 | 8 | 3 | 3,8,11,16,19,24,27,32,35,40,43,48,51,56,59,64,67,72,75,80,83,88,91,96, 99,104,107 |
| 132 | 8 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52,56,60,64,68,72,76,80,84,88,92,96, 100,104, 108,112,116,120,124,128,132 |
| 133 | 8 | 5 | 5,8,13,16,21,24,29,32,37,40,45,48,53,56,61,64,69,72,77,80,85,88,93,96, 101,104, 109,112,117,120,125,128,133 |
| 135 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79,80,87,88,95,96, 103,104, 111,112,119,120,127,128,135 |
| 160 | 16 | 16 | 16,32,48,64,80,96,112,128,144,160 |
| 216 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,152,160, 168, 176,184,192,200,208,216 |
| 217 | 16 | 9 | 9,16,25,32,41,48,57,64,73,80,89,96,105,112,121,128,137,144,153,160, 169,176,185,192,201,208,217 |
| 264 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,160,168, 176,184,192,200,208,216,224,232,240,248,256,264 |
| 270 | 16 | 14 | 14,16,30,32,46,44,62,64,78,80,94,96,110,112, 126,128,142,144,158, 160,174, 176,190,192, 206,208,222,224,238,240, 254,256,270 |
| 273 | 16 | 1 | 1,16,17,32,33,48,49,64,65,80,81,96,97,112,113,128,129,144,145,160, 161,176,171, 192,193, 208,209, 224,225,240,241,256,257,272,273 |

Table 7.1.1.4.2.3.3.2-3: Specific Parameter

| Parameter | Value | Comment | Condition |
| --- | --- | --- | --- |
| mcs-Table | qam64 |  |  |
| resourceAllocation | dynamicSwitch |  | pc_dynamicSwitchRA_Type0_1_PUSCH |
|  | resourceAllocationType1 |  | NOT pc_dynamicSwitchRA_Type0_1_PUSCH AND Steps 1-5 |
|  | resourceAllocationType0 |  | NOT pc_dynamicSwitchRA_Type0_1_PUSCH AND pc_ra_Type0_PUSCH AND Steps 6-10 |
| rbg-Size | Not present | configuration 1 applicable |  |
| NstartBWP | 0 |  |  |

Table 7.1.1.4.2.3.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of 1 to in BWP, time domain resource as per Table 7.1.1.4.2.0-1 and from 0 to 28. | - | - | - | - |
| 1 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS is less than or equal to UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.3.3.2-1 and larger than or equal to 136 bits as specified in Table 7.1.1.4.2.3.3.2-2 Skip the execution of steps 2 to 5 for which the value of satisfies the condition 3824 < < 3825.(Note 4) Skip the execution of steps 1 to 5 for > 27 and < 5. (Note3) | - | - | - | - |
| 2 | SS creates one or more PDCP SDUs depending on TBS in accordance with Table 7.1.1.4.2.3.3.2-2. | - | - | - | - |
| 3 | The SS transmits all PDCP SDUs (NSDUs) as created in step 2 in a MAC PDU. | <-- | MAC PDU (NxPDCP SDUs) | - | - |
| 4 | After the reception of 2 Scheduling Request, SS transmits UL Grant DCI 0_1, and values of S, L,and nPRB.. | <-- | (UL Grant) (DCI: (DCI Format 0_1, S, L,and nPRB.) | - | - |
| 5 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 4? | --> | (NxPDCP SDUs) | 2 | P |
| - | EXCEPTION : Steps 5Aa1 to 10 are executed if pc_ra_Type0_PUSCH | - | - | - | - |
| - | EXCEPTION : Steps 5Aa1 to 5Aa2 are executed if NOT pc_dynamicSwitchRA_Type0_1_PUSCH | - | - | - | - |
| 5Aa1 | The SS transmits a NR RRCReconfiguration message including PUSCH-Config with IE resourceAllocation set to resourceAllocationType1 (Note 1) | <-- | RRCReconfiguration | - | - |
| 5Aa2 | The UE transmit a NR RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 6 to 10 are repeated for allowed values of as per table 7.1.1.4.2.3.3.2-2A in BWP, time domain resource length L 3 to 14-S and from 0 to 28. | - | - | - | - |
| 6 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 7 to 10 are performed if TBS1 + TBS2 is less than or equal to UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.3.3.2-1 and larger than or equal to 136 bits as specified in Table 7.1.1.4.2.3.3.2-2. Skip the execution of steps 7 to 10 for which the value of satisfies the condition 3824 < < 3825. (Note 4) Skip the execution of steps 6 to 10 for > 27 and < 5. (Note 3) | - | - | - | - |
| 7 | SS creates one or more PDCP SDUs depending on TBS in accordance with Table 7.1.1.4.2.3.3.2-2. | - | - | - | - |
| 8 | The SS transmits all PDCP SDUs (NSDUs) as created in step 7 in a MAC PDU. | <-- | MAC PDU (NxPDCP SDUs) | - | - |
| 9 | After the reception of 2 Scheduling Request SS transmits UL Grant DCI 0_1, and values of S, L,and nPRB.. | <-- | (UL Grant) (DCI: (DCI Format 0_1, S, L,and nPRB.) | - | - |
| 10 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 8 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 9? | --> | (NxPDCP SDUs) | 1 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: For > 27 and < 5, the combination results in higher coding rate and therefore leading to CRC errors in decoding UL data. Note 4: Depending upon UE implementation of TBS determination as per clause 5.1.3.2 of TS 38.214 [15], 3824 < < 3825, different step may be used by UEs for TBS determintation. When Resource Allocation Type is RA Type 1, =5,=123 and Time Domain Allocation Symbols = 4, the resulting is 3824.05. |  |  |  |  |  |

7.1.1.4.2.3.3.3	Specific message contents

None.

###### 7.1.1.4.2.4	UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled

7.1.1.4.2.4.1	Test Purpose (TP)

(1)

with { UE in  RRC_CONNECTED state and mcs-Table is set as ‘qam256‘ }

ensure that {

when { UE has pending data for transmission and receives DCI format 1_1 indicating resource allocation type 0  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE transmits MAC PDU's on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

(2)

with { UE in  RRC_CONNECTED state and mcs-Table is set as ‘qam256‘ }

ensure that {

when { UE has pending data for transmission and receives DCI format 1_1 indicating resource allocation type 1  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE transmits MAC PDU's on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

7.1.1.4.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.1.1, TS 38.214 clause 6.1.2.1, 6.1.2.2, 6.1.2.2.1, 6.1.2.2.2, 6.1.4.1, 5.1.3.1, 6.1.4.2 and 5.1.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.212, clause 7.3.1.1.2]

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or new-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator – 0 or 3 bits, as defined in Subclause 10.1 of [5, TS38.213].

-	UL/SUL indicator – 0 bit for UEs not configured with SUL in the cell or UEs configured with SUL in the cell but only PUCCH carrier in the cell is configured for PUSCH transmission; 1 bit for UEs configured with SUL in the cell as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bit width for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active UL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Subclause 6.1.2.2.1 of [6, TS 38.214],

-	bits if only resource allocation type 1 is configured, or  bits if both resource allocation type 0 and 1 are configured.

-	If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the  LSBs provide the resource allocation as defined in Subclause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bit width of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bit width of the "Frequency domain resource assignment"  field of the indicated bandwidth part.

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Subclause 6.1.2.1 of [6, TS38.214]. The bit width for this field is determined as bits, where I the number of entries in the higher layer parameter pusch-AllocationList.

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if the higher layer parameter frequencyHopping is not configured;

-	1 bit according to Table 7.3.1.1.2-34 otherwise, only applicable to resource allocation type 1, as defined in Subclause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	1st downlink assignment index – 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook.

-	2nd downlink assignment index – 0 or 2 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks;

-	0 bit otherwise.

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS38.213]

-	SRS resource indicator – or  bits, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', and  is the maximum number of supported layers for the PUSCH.

-	 bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'nonCodeBook';

-	 bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook'.

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank, and codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank, and codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters maxRank and codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters maxRank and codebookSubset.

-	Antenna ports – number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if transformPrecoder=enabled, dmrs-Type=1, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-7, if transformPrecoder=enabled, dmrs-Type=1, and maxLength=2;

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transformPrecoder=disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transformPrecoder=disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transformPrecoder=disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transformPrecoder=disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bit width of this field equals , where  is the “Antenna ports” bit width derived according to dmrs-UplinkForPUSCH-MappingTypeA and  is the “Antenna ports” bit width derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  and .

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with SUL in the cell; 3 bits for UEs configured SUL in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Subclause 6.1.1.2 of [6, TS 38.214].

-	CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize.

-	CBG transmission information (CBGTI) – 0, 2, 4, 6, or 8 bits determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for PUSCH.

-	PTRS-DMRS association – number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured and transformPrecoder=disabled, or if transformPrecoder=enabled, or if maxRank=1;

-	2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field.

If “Bandwidth part indicator” field indicates a bandwidth part other than the active bandwidth part and the “PTRS-DMRS association” field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the “PTRS-DMRS association” field is not present for the indicated bandwidth part.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

-	DMRS sequence initialization – 0 if the higher layer parameter transformPrecoder=enabled; 1 bit if the higher layer parameter transformPrecoder=disabled and both scramblingID0 and scramblingID1 are configured in DMRS-UplinkConfig, for  selection defined in Subclause 6.4.1.1.1.1 of [4, TS 38.211].

-	UL-SCH indicator – 1 bit. A value of “1” indicates UL-SCH shall be transmitted on the PUSCH and a value of “0” indicates UL-SCH shall not be transmitted on the PUSCH.

For a UE configured with SUL in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_1 for the SUL is not equal to the number of information bits in format 0_1 for the non-SUL, zeros shall be appended to smaller format 0_1 until the payload size equals that of the larger format 0_1.

Table 7.3.1.1.2-1: Bandwidth part indicator

| Value of BWP indicator field | Bandwidth part |
| --- | --- |
| 2 bits |  |
| 00 | First bandwidth part configured by higher layers |
| 01 | Second bandwidth part configured by higher layers |
| 10 | Third bandwidth part configured by higher layers |
| 11 | Fourth bandwidth part configured by higher layers |

Table 7.3.1.1.2-2: Precoding information and number of layers, for 4 antenna ports, if transformPrecoder=disabled and maxRank = 2 or 3 or 4

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |
| 19 | 1 layer: TPMI=11 | 19 | 1 layer: TPMI=11 |  |  |
| 20 | 2 layers: TPMI=6 | 20 | 2 layers: TPMI=6 |  |  |
| … | … | … | … |  |  |
| 27 | 2 layers: TPMI=13 | 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 | 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 | 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 | 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 | 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layers: TPMI=12 |  |  |  |  |
| … | … |  |  |  |  |
| 47 | 1 layers: TPMI=27 |  |  |  |  |
| 48 | 2 layers: TPMI=14 |  |  |  |  |
| … | … |  |  |  |  |
| 55 | 2 layers: TPMI=21 |  |  |  |  |
| 56 | 3 layers: TPMI=3 |  |  |  |  |
| … | … |  |  |  |  |
| 59 | 3 layers: TPMI=6 |  |  |  |  |
| 60 | 4 layers: TPMI=3 |  |  |  |  |
| 61 | 4 layers: TPMI=4 |  |  |  |  |
| 62-63 | reserved |  |  |  |  |

Table 7.3.1.1.2-3: Precoding information and number of layers for 4 antenna ports, if transformPrecoder= enabled, or if transformPrecoder=disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |  |  |
| … | … | … | … |  |  |
| 11 | 1 layer: TPMI=11 | 11 | 1 layer: TPMI=11 |  |  |
| 12 | 1 layers: TPMI=12 | 12-15 | reserved |  |  |
| … | … |  |  |  |  |
| 27 | 1 layers: TPMI=27 |  |  |  |  |
| 28-31 | reserved |  |  |  |  |

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder=disabled and maxRank = 2

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 | 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 | 3 | reserved |
| 4 | 1 layer: TPMI=3 |  |  |
| 5 | 1 layer: TPMI=4 |  |  |
| 6 | 1 layer: TPMI=5 |  |  |
| 7 | 2 layers: TPMI=1 |  |  |
| 8 | 2 layers: TPMI=2 |  |  |
| 9-15 | reserved |  |  |

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder= enabled, or if transformPrecoder= disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |  |  |
| 3 | 1 layer: TPMI=3 |  |  |
| 4 | 1 layer: TPMI=4 |  |  |
| 5 | 1 layer: TPMI=5 |  |  |
| 6-7 | reserved |  |  |

...

Table 7.3.1.1.2-33: VRB-to-PRB mapping

| Bit field mapped to index | VRB-to-PRB mapping |
| --- | --- |
| 0 | Non-interleaved |
| 1 | Interleaved |

[TS 38.214, clause 6.1.2.1]

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report on PUSCH by a DCI, the Time domain resource assignment field value md of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in sub-clause 6.1.2.1.1.  The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report by a CSI request field on a DCI, the Time-domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the applied resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L,  and the PUSCH mapping type to be applied in the PUSCH transmission and K2 is determined based on the corresponding list entries of the higher layer parameter reportSlotConfig in CSI-ReportConfig for the triggered CSI Reporting Settings. The ith codepoint of K2 s determined as  where  is the ith codepoint of .

-	The slot where the UE shall transmit the PUSCH is determined by K2 as  where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and

-	The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	The PUSCH mapping type is set to Type A or Type B as defined in Subclause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} | {0,…,12} | {1,…,12} | {1,…,12} |

When the UE is configured with aggregationFactorUL > 1, the same symbol allocation is applied across the aggregationFactorUL consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the aggregationFactorUL consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version when aggregationFactorUL > 1

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

If the UE procedure for determining slot configuration, as defined in subclause 11.1 of [6, TS 38.213], determines symbols of a slot allocated for PUSCH as downlink symbols, the transmission on that slot is omitted for multi-slot PUSCH transmission.

[38.214 clause 6.1.2.2]

The UE shall determine the resource block assignment in frequency domain using the resource allocation field in the detected PDCCH DCI. Two uplink resource allocation schemes type 0 and type 1 are supported. Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. Uplink resource allocation scheme type 1 is supported for PUSCH for both cases when transform precoding is enabled or disabled.

If the scheduling DCI is configured to indicate the uplink resource allocation type as part of the Frequency domain resource assignment field by setting a higher layer parameter resourceAllocation in pusch-Config to ‘dynamicswitch’, the UE shall use uplink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the uplink frequency resource allocation type as defined by the higher layer parameter resourceAllocation.

The UE shall assume that when the scheduling PDCCH is received with DCI format 0_0, then uplink resource allocation type 1 is used.

If a bandwidth part indicator field is not configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI, except for the case when DCI format 0_0 is decoded in any PDCCH common search space in CORESET 0 in which case the initial bandwidth part shall be used. The UE shall upon detection of PDCCH intended for the UE determine first the uplink bandwidth part and then the resource allocation within the bandwidth part.

[38.214 clause 6.1.2.2.1]

In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Sizeconfigured for PUSCH and the size of the carrier bandwidth part as defined in Table 6.1.2.2.1-1.

Table 6.1.2.2.1-1: Nominal RBG size P

| Carrier Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

The total number of RBGs () for a uplink carrier bandwidth part i of sizePRBs is given by  where

-	the size of the first RBG is ,

-	the size of the last RBG is if and P otherwise.

-	the size of all other RBG is P.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency of the carrier bandwidth part and starting at the lowest frequency. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB of the bitmap. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

[38.214 clause 6.1.2.2.2]

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in the Type0-PDCCH common search space in CORESET 0 in which case the initial bandwidth part of size  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

[TS 38.214, clause 6.1.4.1]

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, or SP-CSI-RNTI, the transform precoding is enabled if transformPrecoder in PUSCH-Config is set to 'enabled', or if transformPrecoder in PUSCH-Config is not configured and msg3-transformPrecoding in rach-ConfigCommon is set to 'enabled'; otherwise the transform precoding is disabled.

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by CS-RNTI, or the PUSCH with configured grant using CS-RNTI, the transform precoding is enabled if transformPrecoder in ConfiguredGrantConfig is set to 'enabled'; otherwise the transform precoding is disabled.

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI, or SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI,

if transformPrecoder is disabled for this PUSCH transmission

-	if mcs-Table in PUSCH-Config is set to 'qam256', and PUSCH is scheduled with C-RNTI or SP-CSI-RNTI, and PUSCH is assigned by DCI format 0_1,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with new-RNTI, mcs-Table in PUSCH-Config is set to 'qam64LowSE', the PUSCH is scheduled with C-RNTI, or SP-CSI-RNTI, and the PUSCH is assigned by a PDCCH in a UE-specific search space,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with new-RNTI, and the PUSCH is scheduled with new-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in ConfiguredGrantConfig is set to 'qam256', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in ConfiguredGrantConfig is set to 'qam64LowSE', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

[TS 38.214, clause 5.1.3.1]

Table 5.1.3.1-2: MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

[TS 38.214, clause 6.1.4.2]

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, or SP-CSI-RNTI.

if

-	and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  Nsymbslotis the number of symbols of the PUSCH allocation within the slot,  is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 0_1 or as described for DCI format 0_0 in Subclause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 0, 6, 12, or 18), the  is assumed to be 0. For MSG3 transmission the  is always set to 0..

-	A UE determines the total number of REs allocated for PUSCH  by where  is the total number of allocated PRBs for the UE.

-	Next, proceed with steps 2-4 as defined in Subclause 5.1.3.2

else if

-	 and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

[TS 38.214, clause 5.1.3.2]

2)	Intermediate number of information bits (Ninfo) TBStemp)is obtained by TBStemp=NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the closest TBS that is not less than .

Table 5.1.3.2-2: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.2.4.3	Test description

7.1.1.4.2.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set  the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value).

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

7.1.1.4.2.4.3.2	Test procedure sequence

Table 7.1.1.4.2.4.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.2.4.3.2-2: Number of downlink PDCP SDUs and PDCP SDU size used as test data

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 136 ≤ TBS ≤12128 note 2 | 1 | 8*FLOOR((TBS – 128)/8) |
| 12129 ≤ TBS ≤24200 | 2 | 8*FLOOR((TBS – 200)/16) |
| 24201 ≤ TBS ≤ 36272 | 3 | 8*FLOOR((TBS – 272)/24) |
| 36273 ≤ TBS ≤48344 | 4 | 8*FLOOR((TBS – 344)/32) |
| 48345≤ TBS ≤60416 | 5 | 8*FLOOR((TBS – 416)/40) |
| 60417 ≤ TBS ≤ 72488 | 6 | 8*FLOOR((TBS –488)/48) |
| 72489 ≤ TBS ≤84560 | 7 | 8*FLOOR((TBS – 560)/56) |
| 84561 ≤ TBS ≤96632 | 8 | 8*FLOOR((TBS –632)/64) |
| 96633< TBS ≤108704 | 9 | 8*FLOOR((TBS –704)/72) |
| 10705 ≤ TBS ≤120776 | 10 | 8*FLOOR((TBS – 776)/80) |
| 120777≤ TBS ≤132848 | 11 | 8*FLOOR((TBS –848)/88) |
| 132849 ≤ TBS ≤ 144920 | 12 | 8*FLOOR((TBS – 920)/96) |
| TBS> 144920 | 13 | 8*FLOOR((TBS – 992)/104) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size is 24 bits with 18 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*24- N*24 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 136 bits. |  |  |

Table 7.1.1.4.2.4.3.2-2A: Bandwidth part Dependent Parameters for Resource allocation 0 with start of BWP assumed as 0

| = | Nominal RBG size P (Configuration1) | Size of last RBG | Allowed Values |
| --- | --- | --- | --- |
| 11 | 2 | 1 | All 1…11 |
| 18 | 2 | 2 | 2,4,6,8,10,12,16,18 |
| 24 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24 |
| 25 | 2 | 1 | All 1…25 |
| 31 | 2 | 1 | All 1…31 |
| 32 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32 |
| 38 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38 |
| 51 | 4 | 3 | 3,4,7,8,11,12,15,16,19,20,23,24,27,28,31,32,35,36,39,40,43,44,47,48,51 |
| 52 | 4 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52 |
| 65 | 4 | 1 | 1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,32,33,36,37,40,41,44,45,48,49, 52,53,56,57,60,61,64,65 |
| 66 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38,40,42,44,46,48,50,52, 54,56,58,60,62,64,66 |
| 79 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79 |
| 106 | 8 | 2 | 2,8,10,16,18,24,26,32,34,40,42,48,50,56,58,64,66,72,74,80,82,88,90,96, 92,104,106 |
| 107 | 8 | 3 | 3,8,11,16,19,24,27,32,35,40,43,48,51,56,59,64,67,72,75,80,83,88,91,96, 99,104,107 |
| 132 | 8 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52,56,60,64,68,72,76,80,84,88,92,96, 100,104, 108,112,116,120,124,128,132 |
| 133 | 8 | 5 | 5,8,13,16,21,24,29,32,37,40,45,48,53,56,61,64,69,72,77,80,85,88,93,96, 101,104, 109,112,117,120,125,128,133 |
| 135 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79,80,87,88,95,96, 103,104, 111,112,119,120,127,128,135 |
| 160 | 16 | 16 | 16,32,48,64,80,96,112,128,144,160 |
| 216 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,152,160,168, 176,184,192,200,208,216 |
| 217 | 16 | 9 | 9,16,25,32,41,48,57,64,73,80,89,96,105,112,121,128,137,144,153,160,169, 176,185,192,201,208,217 |
| 264 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,160,168, 176,184,192,200,208,216,224,232,240,248,256,264 |
| 270 | 16 | 14 | 14,16,30,32,46,44,62,64,78,80,94,96,110,112, 126,128,142,144,158,160, 174, 176,190,192, 206,208,222,224,238,240, 254,256,270 |
| 273 | 16 | 1 | 1,16,17,32,33,48,49,64,65,80,81,96,97,112,113,128,129,144,145,160, 161,176,171, 192,193, 208,209, 224,225,240,241,256,257,272,273 |

Table 7.1.1.4.2.4.3.2-3: Specific Parameter

| Parameter | Value | Comment | Condition |
| --- | --- | --- | --- |
| number of layers (ʋ) | 1 |  |  |
| mcs-Table | qam256 |  |  |
| resourceAllocation | dynamicSwitch |  | pc_dynamicSwitchRA_Type0_1_PUSCH |
|  | resourceAllocationType1 |  | NOT pc_dynamicSwitchRA_Type0_1_PUSCH AND Steps 1-5 |
|  | resourceAllocationType0 |  | NOT pc_dynamicSwitchRA_Type0_1_PUSCH AND pc_ra_Type0_PUSCH AND Steps 6-10 |
| rbg-Size | Not present | configuration 1 applicable |  |
| NstartBWP | 0 |  |  |

Table 7.1.1.4.2.4.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of 1 to in BWP, time domain resource as per Table 7.1.1.4.2.0-1 and from 0 to 27. | - | - | - | - |
| 1 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS is less than or equal to UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.4.3.2-1 and larger than or equal to 136 bits as specified in Table 7.1.1.4.2.4.3.2-2. Skip the execution of steps 2 to 5 for which the TBS size equal to 3824 or 3840. (Note 3) | - | - | - | - |
| 2 | SS creates one or more PDCP SDUs depending on TBS in accordance with Table 7.1.1.4.2.4.3.2-2. | - | - | - | - |
| 3 | After 300ms, the SS transmits all PDCP SDUs (NSDUs) as created in step 2 in a MAC PDU. | <-- | MAC PDU (NxPDCP SDUs) | - | - |
| 4 | After 60ms of step 3 SS transmits UL Grant DCI 0_1, and values of S, L,and nPRB.. | <-- | (UL Grant) (DCI: (DCI Format 0_1, S, L,and nPRB.) | - | - |
| 5 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 4? | --> | (NxPDCP SDUs) | 2 | P |
| - | EXCEPTION : Steps 5Aa1 to 10 are executed if pc_ra_Type0_PUSCH | - | - | - | - |
| - | EXCEPTION : Steps 5Aa1 to 5Aa2 are executed if NOT pc_dynamicSwitchRA_Type0_1_PUSCH | - | - | - | - |
| 5Aa1 | The SS transmits a NR RRCReconfiguration message including PUSCH-Config with IE resourceAllocation set to resourceAllocationType1 (Note 1) | <-- | RRCReconfiguration | - | - |
| 5Aa2 | The UE transmit a NR RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 6 to 10 are repeated for allowed values of as per Table 7.1.1.4.2.4.3.2-2A in BWP, time domain resource length L 3 to 14-S and from 0 to 27. | - | - | - | - |
| 6 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 7 to 10 are performed if TBS is less than or equal to UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.4.3.2-1 and larger than or equal to 136 bits as specified in Table 7.1.1.4.2.4.3.2-2. Skip the execution of steps 7 to 10 for which the TBS size equal to 3824 or 3840. (Note 3) | - | - | - | - |
| 7 | SS creates one or more PDCP SDUs depending on TBS in accordance with Table 7.1.1.4.2.4.3.2-2. | - | - | - | - |
| 8 | After 300ms, the SS transmits all PDCP SDUs (NSDUs) as created in step 7 in a MAC PDU. | <-- | MAC PDU (NxPDCP SDUs) | - | - |
| 9 | After 60ms of step 8 SS transmits UL Grant DCI 0_1, and values of S, L,and nPRB.. | <-- | (UL Grant) (DCI: (DCI Format 0_1, S, L,and nPRB.) | - | - |
| 10 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 8 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 4? | --> | (NxPDCP SDUs) | 1 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: There is ambiguity of TBS calculation when 3824.0 < Ninfo < 3825.0 in clause 5.1.3.2 of TS 38.214 [15]. |  |  |  |  |  |

7.1.1.4.2.4.3.3	Specific message contents

[None].

###### 7.1.1.4.2.5	UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM

7.1.1.4.2.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and transform precoding enabled}

ensure that {

when { UE has pending data for transmission and receives on PDCCH DCI format 0_0 indicating a resource block assignment correspondent to physical resource blocks , Time domain resource assignment and modulation and coding }

then { UE transmits MAC PDU on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

7.1.1.4.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.1.1, TS 38.214 clause 6.1.2.1, 6.1.2.2, 6.1.2.2.2, 6.1.4.1, 5.1.3.1, 6.1.4.2 and 5.1.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.212, clause 7.3.1.1.1]

DCI format 0_0 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or new-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Frequency domain resource assignment –  bits where

-	 is the size of the active UL bandwidth part in case DCI format 0_0 is monitored in the UE specific search space and satisfying

-	the total number of different DCI sizes monitored per slot is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI monitored per slot is no more than 3 for the cell

-	otherwise,  is the size of the initial UL bandwidth part.

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 4 bits as defined in Subclause 6.1.2.1 of [6, TS 38.214]

-	Frequency hopping flag – 1 bit.

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.3 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS 38.213]

-	Padding bits, if required.

-	UL/SUL indicator – 1 bit for UEs configured with SUL in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured;

-	If the UL/SUL indicator is not present in DCI format 0_0, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured.

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Frequency domain resource assignment –bits where

-	 is the size of the initial UL bandwidth part.

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where  if  and  otherwise

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 4 bits as defined in Subclause 6.1.2.1 of [6, TS 38.214]

-	Frequency hopping flag – 1 bit.

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.3 of [6, TS 38.214], using Table 5.1.3.1-1

-	New data indicator – 1 bit, reserved

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits, reserved

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS 38.213]

-	Padding bits, if required.

-	UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB

If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, zeros shall be appended to the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to padding is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bit width of the frequency domain resource allocation field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 equals to the size of the DCI format 1_0.

If DCI format 0_0 is monitored in UE specific search space but does not satisfy at least one of the following

-	the total number of different DCI sizes monitored per slot is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI monitored per slot is no more than 3 for the cell

and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, zeros shall be appended to the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

If DCI format 0_0 is monitored in UE specific search space but does not satisfy at least one of the following

-	the total number of different DCI sizes monitored per slot is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI monitored per slot is no more than 3 for the cell

and if the number of information bits in the DCI format 0_0 prior to padding is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bit width of the frequency domain resource allocation field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 equals to the size of the DCI format 1_0.

If DCI format 0_0 is monitored in UE specific search space and satisfies both of the following

-	the total number of different DCI sizes monitored per slot is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI monitored per slot is no more than 3 for the cell

and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in UE specific search space for scheduling the same serving cell, zeros shall be appended to the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

[TS 38.214, clause 6.1.2.1]

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report by a CSI request field on a DCI, the Time-domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the applied resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L,  and the PUSCH mapping type to be applied in the PUSCH transmission and K2 is determined based on the corresponding list entries of the higher layer parameter reportSlotConfig in CSI-ReportConfig for the triggered CSI Reporting Settings. The ith codepoint of K2 s determined as  where  is the ith codepoint of .

-	The slot where the UE shall transmit the PUSCH is determined by K2 as  where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and

-	The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	The PUSCH mapping type is set to Type A or Type B as defined in Subclause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} | {0,…,12} | {1,…,12} | {1,…,12} |

When the UE is configured with aggregationFactorUL > 1, the same symbol allocation is applied across the aggregationFactorUL consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the aggregationFactorUL consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version when aggregationFactorUL > 1

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

If the UE procedure for determining slot configuration, as defined in subclause 11.1 of [6, TS 38.213], determines symbols of a slot allocated for PUSCH as downlink symbols, the transmission on that slot is omitted for multi-slot PUSCH transmission.

[38.214 clause 6.1.2.2]

The UE shall determine the resource block assignment in frequency domain using the resource allocation field in the detected PDCCH DCI. Two uplink resource allocation schemes type 0 and type 1 are supported. Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. Uplink resource allocation scheme type 1 is supported for PUSCH for both cases when transform precoding is enabled or disabled.

If the scheduling DCI is configured to indicate the uplink resource allocation type as part of the Frequency domain resource assignment field by setting a higher layer parameter resourceAllocation in pusch-Config to ‘dynamicswitch’, the UE shall use uplink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the uplink frequency resource allocation type as defined by the higher layer parameter resourceAllocation.

The UE shall assume that when the scheduling PDCCH is received with DCI format 0_0, then uplink resource allocation type 1 is used.

If a bandwidth part indicator field is not configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI, except for the case when DCI format 0_0 is decoded in any PDCCH common search space in CORESET 0 in which case the initial bandwidth part shall be used. The UE shall upon detection of PDCCH intended for the UE determine first the uplink bandwidth part and then the resource allocation within the bandwidth part.

[38.214 clause 6.1.2.2.2]

n uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in the Type0-PDCCH common search space in CORESET 0 in which case the initial bandwidth part of size  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

[TS 38.214, clause 6.1.4.1]

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, or SP-CSI-RNTI, the transform precoding is enabled if transformPrecoder in PUSCH-Config is set to 'enabled', or if transformPrecoder in PUSCH-Config is not configured and msg3-transformPrecoding in rach-ConfigCommon is set to 'enabled'; otherwise the transform precoding is disabled.

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by CS-RNTI, or the PUSCH with configured grant using CS-RNTI, the transform precoding is enabled if transformPrecoder in ConfiguredGrantConfig is set to 'enabled'; otherwise the transform precoding is disabled.

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI, or SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI,

if transformPrecoder is disabled for this PUSCH transmission

...

else

-	if mcs-TableTransformPrecoder in PUSCH-Config is set to 'qam256', and the PUSCH is scheduled with C-RNTI or SP-CSI-RNTI, and PUSCH is assigned by DCI format 0_1,

-	the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with new-RNTI, mcs-TableTransformPrecoder in PUSCH-Config is set to 'qam64LowSE', and the PUSCH is scheduled with C-RNTI, or SP-CSI-RNTI, and the PUSCH is assigned by a PDCCH in a UE-specific search space,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with new-RNTI, and the PUSCH is scheduled with new-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in ConfiguredGrantConfig is set to 'qam256', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in ConfiguredGrantConfig is set to 'qam64LowSE', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 6.1.4.1-1to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

end

For Table 6.1.4.1-1 and Table 6.1.4.1-2, if higher layer parameter PUSCH-tp-pi2BPSK is configured, q = 1 otherwise q=2.

Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 240/ q | 0.2344 |
| 1 | q | 314/ q | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 466 | 2.7305 |
| 18 | 6 | 517 | 3.0293 |
| 19 | 6 | 567 | 3.3223 |
| 20 | 6 | 616 | 3.6094 |
| 21 | 6 | 666 | 3.9023 |
| 22 | 6 | 719 | 4.2129 |
| 23 | 6 | 772 | 4.5234 |
| 24 | 6 | 822 | 4.8164 |
| 25 | 6 | 873 | 5.1152 |
| 26 | 6 | 910 | 5.3320 |
| 27 | 6 | 948 | 5.5547 |
| 28 | q | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

[TS 38.214, clause 6.1.4.2]

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, or SP-CSI-RNTI.

if

-	and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  Nsymbslotis the number of symbols of the PUSCH allocation within the slot,  is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 0_1 or as described for DCI format 0_0 in Subclause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 0, 6, 12, or 18), the  is assumed to be 0. For MSG3 transmission the  is always set to 0..

-	A UE determines the total number of REs allocated for PUSCH  by where  is the total number of allocated PRBs for the UE.

-	Next, proceed with steps 2-4 as defined in Subclause 5.1.3.2

else if

-	 and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

[TS 38.214, clause 5.1.3.2]

2)	Intermediate number of information bits (Ninfo) TBStemp)is obtained by TBStemp=NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the closest TBS that is not less than .

Table 5.1.3.2-2: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.2.5.3	Test description

7.1.1.4.2.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value) and Short_DCI condition is applied in NR Serving cell configuration.

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

7.1.1.4.2.5.3.2	Test procedure sequence

Table 7.1.1.4.2.5.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.2.5.3.2-2: Number of uplink PDCP SDUs and PDCP SDU size used as test data

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 136 ≤ TBS ≤12128 note 2 | 1 | 8*FLOOR((TBS – 128)/8) |
| 12129 ≤ TBS ≤24200 | 2 | 8*FLOOR((TBS – 200)/16) |
| 24201 ≤ TBS ≤ 36272 | 3 | 8*FLOOR((TBS – 272)/24) |
| 36273 ≤ TBS ≤48344 | 4 | 8*FLOOR((TBS – 344)/32) |
| 48345≤ TBS ≤60416 | 5 | 8*FLOOR((TBS – 416)/40) |
| 60417 ≤ TBS ≤ 72488 | 6 | 8*FLOOR((TBS –488)/48) |
| 72489 ≤ TBS ≤84560 | 7 | 8*FLOOR((TBS – 560)/56) |
| 84561 ≤ TBS ≤96632 | 8 | 8*FLOOR((TBS –632)/64) |
| 96633< TBS ≤108704 | 9 | 8*FLOOR((TBS –704)/72) |
| 10705 ≤ TBS ≤120776 | 10 | 8*FLOOR((TBS – 776)/80) |
| 120777≤ TBS ≤132848 | 11 | 8*FLOOR((TBS –848)/88) |
| 132849 ≤ TBS ≤ 144920 | 12 | 8*FLOOR((TBS – 920)/96) |
| 144921 ≤ TBS ≤ 156992 | 13 | 8*FLOOR((TBS – 992)/104) |
| 156993 ≤ TBS ≤ 169064 | 14 | 8*FLOOR((TBS – 1064)/112) |
| 169065 ≤ TBS ≤ 181136 | 15 | 8*FLOOR((TBS – 1136)/120) |
| 181137 ≤ TBS ≤ 193208 | 16 | 8*FLOOR((TBS – 1208)/128) |
| 193209 ≤ TBS ≤ 205280 | 17 | 8*FLOOR((TBS – 1280)/136) |
| 205281 ≤ TBS ≤ 217352 | 18 | 8*FLOOR((TBS – 1352)/144) |
| 217353 ≤ TBS ≤ 229424 | 19 | 8*FLOOR((TBS – 1424)/152) |
| TBS> 229424 | 20 | 8*FLOOR((TBS – 1496)/160) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size is 24 bits with 18 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*24- N*24 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 136 bits. |  |  |

Table 7.1.1.4.2.5.3.2-3: Specific Parameters

| Parameter | Value | Comment |
| --- | --- | --- |
| number of layers (ʋ) | 1 |  |
| transformPrecoder | enabled |  |

Table 7.1.1.4.2.5.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of 1 to in BWP, time domain resource as per Table 7.1.1.4.2.0-1 and from 0 to 27. | - | - | - | - |
| 1 | The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS is less than or equal to UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.5.3.2-1 and larger than or equal to 136 bits as specified in Table 7.1.1.4.2.5.3.2-2 Skip the execution of steps 2 to 5 for which the TBS size equal to 3824 or 3840. (Note 1) Skip the execution of steps 1 to 5 for > 27 and < 5. (Note2) | - | - | - | - |
| 2 | The SS creates one or more PDCP SDUs, depending on TBS, in accordance with Table 7.1.1.4.2.5.3.2-2. | - | - | - | - |
| 3 | The SS transmits all PDCP SDUs (NSDUs) as created in step 2 in a MAC PDU. | <-- | MAC PDU (NxPDCP SDUs) | - | - |
| 4 | After the reception of 2 Scheduling Request , SS transmits UL Grant DCI 0_0, and values of S, L,and nPRB. | <-- | (UL Grant) (DCI Format 0_0, S, L,and nPRB.) | - | - |
| 5 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 4? | --> | MAC PDU (N x PDCP SDU) | 1 | P |
| Note 1: There is ambiguity of TBS calculation when 3824.0 < Ninfo < 3825.0 in clause 5.1.3.2 of TS 38.214 [15]. Note 2: For > 27 and < 5, the resulting TBS is very small leading to CRC errors in decoding UL data. |  |  |  |  |  |

7.1.1.4.2.5.3.3	Specific message contents

None.