3GPP TSG-RAN5 Meeting #92-e	R5-216337

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5057 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE Idle mode TC 6.1.2.13 for CAT-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For CAT-M1 UE, any modification in the SIB info must be indicated by increasing the value of corresponding SystemInfoValueTagSI-r13 in SIB1-BR. For TC 6.1.2.13, after modifying the SIB5 on cell3, SIB1 is not modified which is causing the UE to not decode the modified SIB5 on cell3 and hence failing to reselect to cell1 at Step18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | systemInfoValueTagList_r13[2] is set to 1, indicating SIB5 modification on cell3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Associated with TTCN CR(R5s210890) R1: Updated SIB1-BR contents at step16 similar to Step15A This document is final version of R5-215690 and one revision |  |  |  |  |  |  |  |  |

#### 6.1.2.13	Cell reselection, Sintrasearch, Snonintrasearch

6.1.2.13.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Sintrasearch is non-zero in system information }

then { UE perform measurement and reselects the highest ranked cell upon Srxlev < Sintrasearch }

}

(2)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Snonintrasearch is non-zero in system information }

then { UE perform measurement and reselects the cell which belong to the equal priority frequency cell upon Srxlev < Snonintrasearch }

}

(3)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Snonintrasearch is non-zero in system information }

then { UE perform measurement and reselects the cell which belong to the high priority frequency cell upon Srxlev > Snonintrasearch }

}

6.1.2.13.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.304, clauses 5.2.4.2, 5.2.4.5 and 5.2.4.6;  TS 36.331, clause 6.3.1;  TS 36.133, clause 4.2.2.4.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

.....

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

.....

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell.

...

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT,

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.3.1]

The IE SystemInformationBlockType3 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

[TS 36.133, clause 4.2.2.4]

…

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in  TS 36.304 within Kcarrier * Tdetect,EUTRAN_Inter  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5dB for reselections based on ranking or 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

6.1.2.13.3	Test description

6.1.2.13.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 3 have different tracking areas according to table 6.0.1-2.

- 	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1(serving cell) according to [18].

6.1.2.13.3.2	Test procedure sequence

Table 6.1.2.13.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.1.2.13.3.2-2.

Table 6.1.2.13.3.2-1: Time instances of cell power level and parameter change

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -79 | "Off" | Srxlev of Cell 1 is less than Sintrasearch. |
|  | Qrxlevmin | dBm | -106 | -106 | -106 |  |
|  | Sintrasearch | dB | 22 | 22 | 22 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -97 | -79 | Srxlev of Cell 2 is less than Snonintrasearch. |
|  | Snonintrasearch | dB | 16 | 16 | 16 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | "Off" | "Off" | -79 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | -79 | Srxlev of Cell 3 is greater than Snonintrasearch but Cell 1 is high priority cell. |
| Note: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 6.1.2.13.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the cell-specific reference signal levels according to row "T1" in table 6.1.2.13.3.2-1. | - | - | - | - |
| 2 | Check: Does UE transmit an RRCConnectionRequest message on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 3 - 7 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 8 | The SS re-adjusts the cell-specific reference signal levels according to row "T2" in table 6.1.2.13.3.2-1. | - | - | - | - |
| 9 | Check: Does UE transmit an RRCConnectionRequest message on Cell 3? | --> | RRCConnectionRequest | 2 | P |
| 10 - 14 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 3. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 14A | The SS re-adjusts the cell-specific reference signal levels according to row "T3" in table 6.1.2.13.3.2-1. |  |  |  |  |
| 15 | The SS notifies the UE of change of System Information and changes the system information for Cell 3. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 16 | Wait for 15 s for the UE to receive system information. | - | - | - | - |
| 16A | SS modifies SIB3 in Cell 1 and the systemInfoValueTag in the SystemInformationBlockType1 is increased. | - | - | - | - |
| 17 | The SS re-adjusts the cell-specific reference signal levels according to row " T4" in table 6.1.2.13.3.2-1. | - | - | - | - |
| 18 | Check: Does the test result of steps 1 to 5 of the generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? (Note1) NOTE: The UE performs a TAU procedure. Note: UE uses the new SIB3 modified in step 16A. | - | - | 3 | - |
| Note1: UE shall search higher priority layer (i.e. E-UTRA carrier frequency of Cell 1) at least every (60 * Nlayers) seconds, where Nlayers = 1 as specified in [4.2.2, 34]. |  |  |  |  |  |

6.1.2.13.3.3	Specific message contents

Table 6.1.2.13.3.3-0: Conditions for specific message contents
in Table 6.1.2.13.3.3-4

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.13.3.3-1: Void

Table 6.1.2.13.3.3-2: SystemInformationBlockType3 for Cells 1, 2 and 3 (preamble and all steps, table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 8 (16 dB) |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearch | 11 (22 dB) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-3A: SystemInformationBlockType1-BR-r13 for Cell 3 (step 15 when UE under test is CAT M1, Table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | 3 entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType5. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[2] |  |  |  |
| SystemInfoValueTagSI-r13[3] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-3A: SystemInformationBlockType1-BR-r13 for Cell 3 (step 15 when UE under test is CAT M1, Table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.1.2.13.3.3-4: SystemInformationBlockType5 for Cell 3 (step 15, Table 6.1.2.13.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| threshX-High[1] | 5 | 10 dB |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-5: Paging (step 15, Table 6.1.2.13.3.2-2)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| etws-Indication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-6: SystemInformationBlockType3 for Cell 1 (step 16A, table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 8 (16 dB) |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearch | 11 (22 dB) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-7: SystemInformationBlockType1 for Cell 1 (step 16A, Table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.1.2.13.3.3-7A: SystemInformationBlockType1-BR-r13  for Cell 1 (step 16A when UE under test is CAT M1, Table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | 3 entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType3. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[2] |  |  |  |
| SystemInfoValueTagSI-r13[3] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |