3GPP TSG-RAN5 Meeting #92-e	R5-214203r1

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0336 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of FR2 demod test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Achievable SNRBB needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SNRBB and related appilicability of test points have been updated in clauses 7.1.1_1 and 8.1.1_1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testability and assumption of achievable SNR will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1_1, 8.1.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related discussion paper in R5-214202. r1: removed test points 3 and 4 of TC 8.2.2.2.2.1 in Table 8.1.1_1-2 in order to resolve overlap with CR R5-215098. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.1.1_1	Applicability of test requirements due to maximum achievable SNR

Table 7.1.1_1-1 specifies the current assumption of maximum testable SNRBB for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm under fading conditions.

Table 7.1.1_1-1: Current assumption of maximum testable SNRBB under fading conditions

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | [24.0] | [20.8] | [17.8] |
| n258 mid | [24.0] | [20.8] | [17.8] |
| n259 mid | [13.6] | [10.3] | [7.0] |
| n260 mid | [17.7] | [14.5] | [11.4] |
| n261 mid | [24.0] | [20.8] | [17.8] |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 7.1.1_1-2 for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm under fading conditions.

Table 7.1.1_1-2: Current assumption of maximum testable SNRBB

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | - | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 16.0 | x | x | - | - | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | - | x | x |
|  | 2-4 | 100 | Yes | 15.8 | x | x | - | - | x |
|  | 2-5 | 50 | Yes | 16 | x | x | - | x | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | - | - | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | - | - | x |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |

< Unchanged sections omitted >

### 8.1.1_1	Applicability of test requirements due to maximum achievable SNR

The current assumption of maximum testable SNRBB for PC3, Quiet Zone size ≤ 30 cm under fading conditions is specified in Table 7.1.1_1-1.

The current assumption of maximum testable SNRBB for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm without fading conditions is specified in Table 8.1.1_1-1.

Table 8.1.1_1-1: Current assumption of maximum testable SNRBB without fading

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | [28.7] | [25.5] | [22.5] |
| n258 mid | [28.7] | [25.5] | [22.5] |
| n259 mid | [18.4] | [15.2] | [12.1] |
| n260 mid | [22.5] | [19.3] | [16.3] |
| n261 mid | [28.7] | [25.5] | [22.5] |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 8.1.1_1-2 for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm under fading conditions.

Table 8.1.1_1-2: Current assumption of maximum testable SNRBB

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 8.2.2.2.1.1 | 1 | 100 | No | 9 | x | x | x | x | x |
|  | 2 | 100 | No | 15 | x | x | x | x | x |
| 8.2.2.2.2.1 | 1 | 100 | Yes | 7 | x | x | x | x | x |
|  | 2 | 100 | Yes | 13 | x | x | - | x | x |
| 8.4.2.2.1 | 1 | 100 | Yes | 0 | x | x | x | x | x |
|  | 2 | 100 | Yes | 16 | x | x | - | - | x |
|  | 3 | 100 | Yes | 16 | x | x | - | - | x |

< End of changes >