3GPP TSG-RAN5 Meeting #92-e	R5-214204r1

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0242 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of demod SNR testability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Maximum achievable SNR needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on proposed available DL power at CW 1dB compression in the center of quiet zone in the related discussion paper R5-214202 and the current working assumption for the backoff of [17.71] dB for modulations up to 64 QAM, the SNR values have been updated in Table D.3.2.2-1 and Table D.3.2.2-2. This is an interim solution until the backoff depending on the acceptable clipping frequency has been agreed. The attached spreadsheet entitled “Demod SNR range calculator V3” documents the derivation of the values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demod testability analysis will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related discussion paper in R5-214202. r1: clarified that SNRBB is valid for modulations up to 64 QAM. added Anritsu as co-sourcing company |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### D.3.2.2	SNR range for SNRRP - SNRBB ≤ 1dB for IFF

Based on the method of setting the noise from the Test system to give a maximum of 1dB degradation in overall SNR between reference point and baseband, we can then work back through the signal chain to determine how high the SNR can be set. As the noise is set to a fixed level, the maximum SNR is set by the test system power amplifier and the channel bandwidth to be tested.

The SNR upper bound depends on the type of test system. For the Indirect Far field (IFF) setup the diagram below illustrates the principle, and is based on the “IFF 100MHz ( n257, n258, n261)” tab of the accompanying spreadsheet.

The process works back through the signal chain, from left to right in the diagram.

Figure D.3.2.2-1: Estimation of single band UE SNR range for Indirect far field (IFF) when no fading applies

The test equipment must supply at least the wanted noise level at the reference point. If the noise was lower, the degradation in SNR would be greater than 1dB, and may cause a conformant UE to fail.

The accuracy of setting the signal and noise levels has been taken as ±5.19 dB.

Inclusion of this contribution directly reduces the maximum SNR that can be measured by a test system for a given channel bandwidth. To find the maximum SNR that can be measured by a test system with a specific Channel BW, the baseband SNR in the spreadsheet is increased until the value “Wanted signal + headroom, dBm/Ch BW” is just below the “Available DL power at CW 1dB compression at QZ, dBm” value. For fading conditions, the working assumption for the “Backoff from P1dB” with a value of [-17.71] dB valid for modulations up to 64 QAM has been applied. In the case without fading, the “Backoff from P1dB” is [-13] dB valid for modulations up to 64 QAM. The resulting values for SNRBB are given in Table D.3.2.2-1 for tests cases making use of fading and D.3.2.2-2 for test cases without fading. Single band UE values are obtained by setting the UE multi-band relaxation factor to 0 dB.

Table D.3.2.2-1: Predicted SNRBB upper bound values for Indirect far field (IFF) with 30cm QZ, PC3, 100MHz CHBW, modulation up to 64 QAM under fading conditions

|  | Operating Band | Maximum SNRBB (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| Multi-band UE (Note) | n257 | [24.0] | [20.8] | [17.8] |
|  | n258 | [24.0] | [20.8] | [17.8] |
|  | n259 | [13.6] | [10.3] | [7.0] |
|  | n260 | [17.7] | [14.5] | [11.4] |
|  | n261 | [24.0] | [20.8] | [17.8] |
| Note: For ∑MBp from TS 38.101-2 [16] Table 6.2.1.3-4 allow up to 0.75 dB in Rel-15. |  |  |  |  |

Table D.3.2.2-2: Predicted SNRBB upper bound values for Indirect far field (IFF) with 30cm QZ, PC3, 100MHz CHBW, modulation up to 64 QAM when no fading conditions apply

|  | Operating Band | Maximum SNRBB (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| Multi-band UE (Note) | n257 | [28.7] | [25.5] | [22.5] |
|  | n258 | [28.7] | [25.5] | [22.5] |
|  | n259 | [18.4] | [15.2] | [12.1] |
|  | n260 | [22.5] | [19.3] | [16.3] |
|  | n261 | [28.7] | [25.5] | [22.5] |
| Note: For ∑MBp from TS 38.101-2 [16] Table 6.2.1.3-4 allow up to 0.75 dB in Rel-15. |  |  |  |  |

Note that these are UE baseband SNR values (SNRBB), so the Reference point figures used in TS 38.101-4 [19] will be 1 dB higher.

An example of SNR calculation for IFF method is provided in “38.521-4 Spreadsheet - Demod SNR range calculator.zip” file attached to the TR.

< End of changes >

Add the following attached .zip file to TR 38.903:

## See attachment “38.521-4 Spreadsheet - Demod SNR range calculator V3.zip”

Remove the following .zip file from TR 38.903:

## “38.521-4 Spreadsheet - Demod SNR range calculator V2.zip”