3GPP TSG-RAN5 Meeting #92-e	R5-214806r1

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1985 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IEs for V2X communication over PC5 in E-UTRA-PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, NR V2X. default configuration is not complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Define default configuration of IEs for V2X communication over PC5 in E-UTRA-PC5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X. test case will have no default configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR... CR... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: 1, change Layer 2 ID value of Table 4.7.5.5-19/21 to be 3 octets value, change PDB value of Table 4.7.5.5-23 to be 2 octets value |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.7.5.5	V2X information elements of UE policies for V2X communication over PC5

…

–	V2X communication over PC5 in E-UTRA-PC5

Table 4.7.5.5-19: V2X communication over PC5 in E-UTRA-PC5

| Derivation Path: TS 24.588 Figure 5.3.1.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X communication over PC5 in E-UTRA-PC5 contents |  | Set to the actual length of 'V2X communication over PC5 in E-UTRA-PC5 contents' in bytes |  |  |
| Spare |  | ‘0000’B |  |  |
| PPMR |  | '1'B | PPPP to PDB mapping rules field is present |  |
| VSAPI |  | '1'B | V2X services authorized for PPPR field is present |  |
| VSIEFMRI |  | '1'B | V2X service identifier to V2X E-UTRA frequency mapping rules field is present |  |
| DDL2II |  | '1'B | Default destination layer-2 ID field is present |  |
| V2X service identifier to destination layer-2 ID mapping rules |  | See Table 4.7.5.5-20 |  |  |
| PPPP to PDB mapping rules |  | See Table 4.7.5.5-22 |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rules |  | See Table 4.7.5.5-24 |  |  |
| V2X services authorized for PPPR |  | See Table 4.7.5.5-29 |  |  |
| Default destination layer-2 ID |  | '00 00 40'H | 3 bytes, Random value |  |

–	V2X service identifier to destination layer-2 ID mapping rules

Table 4.7.5.5-20: V2X service identifier to destination layer-2 ID mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID mapping rule 1 |  | See Table 4.7.5.5-21 |  |  |
| V2X service identifier to destination layer-2 ID mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID mapping rule

Table 4.7.5.5-21: V2X service identifier to destination layer-2 ID mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| Destination layer-2 ID |  | '00 00 41'H |  |  |

–	PPPP to PDB mapping rules

Table 4.7.5.5-22: PPPP to PDB mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of PPPP to PDB mapping rules contents |  | Set to the actual length of 'PPPP to PDB mapping rules contents' in bytes |  |  |
| PPPP to PDB mapping rule 1 |  | See Table 4.7.5.5-23 |  |  |
| PPPP to PDB mapping rule 2 |  | FFS |  |  |

–	PPPP to PDB mapping rule

Table 4.7.5.5-23: PPPP to PDB mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PPPP |  | '000'B | PPPP value 1 |  |
| Spare |  | ‘0000 0’B |  |  |
| PDB |  | '00 32'H | 50ms |  |

–	V2X service identifier to V2X E-UTRA frequency mapping rules

Table 4.7.5.5-24: V2X service identifier to V2X E-UTRA frequency mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.24 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rules contents |  | Set to the actual length of 'V2X service identifier to V2X E-UTRA frequency mapping rules contents' in bytes |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 1 |  | See Table 4.7.5.5-25 |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to V2X E-UTRA frequency mapping rule

Table 4.7.5.5-25: V2X service identifier to V2X E-UTRA frequency mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rule contents |  | Set to the actual length of 'V2X service identifier to V2X E-UTRA frequency mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| V2X E-UTRA frequencies with geographical areas list |  | See Table 4.7.5.5-26 |  |  |

–	V2X E-UTRA frequencies with geographical areas list

Table 4.7.5.5-26: V2X E-UTRA frequencies with geographical areas list

| Derivation Path: TS 24.588 Figure 5.3.1.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies with geographical areas list contents |  | Set to the actual length of 'V2X E-UTRA frequencies with geographical areas list contents' in bytes |  |  |
| V2X E-UTRA frequencies with geographical areas info 1 |  | See Table 4.7.5.5-27 |  |  |
| V2X E-UTRA frequencies with geographical areas info 2 |  | FFS |  |  |

–	V2X E-UTRA frequencies with geographical areas info

Table 4.7.5.5-27: V2X E-UTRA frequencies with geographical areas info

| Derivation Path: TS 24.588 Figure 5.3.1.27 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies with geographical areas info contents |  | Set to the actual length of 'V2X E-UTRA frequencies with geographical areas info contents' in bytes |  |  |
| V2X E-UTRA frequencies |  | See Table 4.7.5.5-28 |  |  |
| Geographical areas |  | See Table 4.7.5.5-18 |  |  |

–	V2X E-UTRA frequencies

Table 4.7.5.5-28: V2X E-UTRA frequencies

| Derivation Path: TS 24.588 Figure 5.3.1.28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies contents |  | Set to the actual length of 'V2X E-UTRA frequencies contents' in bytes |  |  |
| V2X E-UTRA frequency 1 |  | 54890 | 5890MHz of B47 in TS 36.101 |  |
| V2X E-UTRA frequency 2 |  | FFS |  |  |

–	V2X services authorized for PPPR

Table 4.7.5.5-29: V2X services authorized for PPPR

| Derivation Path: TS 24.588 Figure 5.3.1.29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X services authorized for PPPR contents |  | Set to the actual length of 'V2X services authorized for PPPR contents' in bytes |  |  |
| V2X service authorized for PPPR 1 |  | See Table 4.7.5.5-30 |  |  |
| V2X service authorized for PPPR 2 |  | FFS |  |  |

–	V2X service authorized for PPPR

Table 4.7.5.5-30: V2X service authorized for PPPR

| Derivation Path: TS 24.588 Figure 5.3.1.30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service authorized for PPPR contents |  | Set to the actual length of 'V2X service authorized for PPPR contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| PPPR |  | '000'B | PPPR value 1 |  |
| Spare |  | ‘0000 0’B |  |  |

<End of modified section 1>